62-18.720  Instrumentation.

Equipment used in making vehicle exhaust system sound measurements shall consist of the following:

(1) Sound Level Meter. Sound level meters shall be of the type meeting A-weighting requirements and Type 2 or better specifications of the American National Standards Institute Inc. or its successor bodies. Alternately, a microphone/magnetic tape recorder/indicating meter system whose overall response is equivalent to the above may be used.

(2) Sound Level Calibrator. An acoustic calibrator of the coupler type, accurate to within  +- 0.5 dB, shall be available for the calibration of sound level meters.

(3) Tachometer. A calibrated engine speed tachometer with an accuracy of  +- 100 rpm over a range of 2,200 to 5,000 rpm shall be used. Full scale should have a reading of at least 6,000 rpm. Tachometers used for the testing of motorcycles and motor-driven cycles shall have a steady state accuracy of  +- 5%.

(4) Anemometer. If tests are conducted outdoors during windy periods, an anemometer shall be used to measure the wind speed.

(5) Windscreen. An open-cell foam windscreen recommended by the manufacturer of the sound level meter shall be used.

Rulelmaking Authority 403.061(7), 403.061(11), 403.415 FS. Law Implemented 403.415, 403.415(8) FS. History–New 7-19-77, Formerly 17-18.72, 17-18.720.