62-18.750  Meter Operation.

The sound level meter shall be used according to the instrument manufacturer’s instructions and as follows:

(1) Microphone Orientation. The microphone shall be oriented in relation to the source of the sound as specified by the microphone manufacturer. Where the instrument manual is vague or does not include adequate information, specific orientation recommendations shall be obtained from the manufacturer.

(2) Meter Setting. The meter shall be set for the A-weighted network and slow response. Tests conducted under Section 62-18.770 shall use the fast meter response.

(3) Calibration Check. An external calibration check shall be made before and after each period of use and at intervals not exceeding 2 hours during continued use.

(4) Ambient Sound. Measurements shall be made only when the A-weighted ambient sound level, including wind effects and all sources other than the vehicle being measured, is at least 10 dBA lower than the sound level of the vehicle.

(5) Wind. Measurements shall not be made when the wind velocity at an outdoor test site is more than 12 miles per hour (19.3 kph).

Rulemaking Authority 403.061(7), 403.061(11), 403.415 FS. Law Implemented 403.415, 403.415(8) FS. History–New 7-19-77, Formerly 17-18.75, 17-18.750.