62-18.780  Motorcycles and Motor-driven Cycles.

Exhaust systems for motorcycles and motor-driven cycles shall comply with the specified noise limits of this part when tested in accordance with the following specifications:

(1) Measuring Site. A flat, open space of concrete, asphalt, or hard materials, excluding compressed or other earth surfaces shall be used. No sound reflecting surfaces other than the pavement shall be within 10 feet (3.0 m) of the vehicle being tested.

(2) Microphone Position. The microphone shall be located behind, 20 inches (508 mm) from, and at the same height as the rearmost exhaust outlet on each side of the vehicle, and at a 45  +- 10 degrees angle from the normal line of travel. The longitudinal axis of the microphone shall be in a plane parallel to the ground. If there are exhaust outlets separated by more than 12 inches (305 mm) on a side of the vehicle, separate measurements shall be made for each outlet.

(3) Engine Operation. The engine shall be at normal operating temperature. With the transmission in neutral and the clutch engaged, the engine speed shall be stabilized to one-half of maximum rpm and the sound level measured.

(4) Exhaust Sound Level. The sound level for a given outlet shall be the highest reading obtained during the test, disregarding unrelated peaks due to extraneous ambient noise. The sound level recorded shall be that of the loudest outlet for that vehicle.

(5) Maximum Exhaust Sound Level. The maximum exhaust sound level for motorcycles and motor-driven cycles shall not exceed the following criteria:

(a) Motorcycles and motor-driven cycles manufactured on or after July 1, 1978. The stationary exhaust sound level for each particular model motorcycle and its associated exhaust system shall not exceed the exhaust sound level for that model as determined by the vehicle manufacturer in accordance with the procedures of this part. The exhaust sound level and maximum rpm for each model shall be available from the vehicle manufacturer.

(b) Motorcycles and motor-driven cycles manufactured before July 1, 1978. The stationary exhaust sound level shall not exceed 102 dBA when tested in accordance with the procedures of this part.

(6) In addition to the certification requirements of Section 62-18.800 of this part, the manufacturer, distributor, importer, or designated agent thereof of motorcycles and motor-driven cycles shall file written documentation as to the exhaust sound levels and engine operating speeds for specific models offered for sale in Florida. Replacement exhaust system manufacturers shall file written certification of compliance to applicable sections of this part and to Section 403.415(8)(b), F.S.

Rulemaking Authority 403.061(7), 403.061(11), 403.415 FS. Law Implemented 403.415, 403.415(8) FS. History–New 7-19-77, Formerly 17-18.78, 17-18.780.