62-18.760  Light Trucks, Buses and Passenger Cars.

Exhaust systems for motor vehicles, other than motorcycles, with a manufacturer’s gross vehicle weight rating of 10,000 pounds (4,536 kg) or less and all passenger cars shall comply with the specified noise limits of this part when tested in accordance with the following specifications:

(1) Measuring Site. The vehicle under test shall be positioned either on outdoor pavement or on a shop floor (not over a hoist or pit) in a location where the exhaust outlets face an open shop door. No sound reflecting surface other than the pavement and vehicle being measured shall be within 10 feet (3.0 m) of the exhaust outlets.

(2) Microphone Location. The microphone for the sound level meter shall be at the same height as the center of the exhaust outlet but not closer to the pavement than 8 inches (203 mm). The microphone shall be positioned with its longitudinal axis parallel to the ground, 20 inches (508 mm) from the edge of the exhaust outlet, and 45  +- 10 degrees from the axis of the outlet (Figure 6). For exhaust outlets located 12 inches (305 mm) or more inboard from the vehicle body, the microphone shall be located at the specified angle and 8 inches (203 mm) from the vehicle body.

[Figure 6]

(3) Engine Operation. The engine shall be at normal running temperature. With the transmission in neutral or park position, the engine speed shall be stabilized to three-quarters of maximum rpm. For vehicles manufactured before 1972, an average steady state engine speed of 3,000 rpm shall be used.

(4) Exhaust Sound Level. The sound level for a given outlet shall be the highest reading obtained during the test, disregarding unrelated peaks due to extraneous ambient noise. The sound level recorded shall be that of the loudest outlet for the vehicle. The maximum exhaust sound level for motor vehicles with a manufacturer’s gross vehicle weight rating of 10,000 pounds or less, except motorcycles, shall not exceed 95 dBA for the following schedule of compliance:

| Year Model | Operative Date |
| --- | --- |
| 1970 and later | January 1, 1978 |
| 1968-69 | July 1, 1978 |
| 1967 and earlier | July 1, 1979 |

Rulemaking Authority 403.061(7), 403.061(11), 403.415 FS. Law Implemented 403.415, 403.415(8) FS. History–New 7-19-77, Formerly 17-18.76, 17-18.760.