- 62-304.625 Peace River Basin TMDLs.

(1) Lake Cannon. The Total Maximum Daily Load for Lake Cannon for Total Phosphorus (TP) is 143 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 54 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 129 kg/year of TP, and

(d) The Margin of Safety is 14 kg/year of TP.

(2) Lake Howard. The Total Maximum Daily Load for Lake Howard for Total Phosphorus (TP) is 143 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 63 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 126 kg/year of TP, and

(d) The Margin of Safety is 17 kg/year of TP.

(3) Lake Idylwild. The Total Maximum Daily Load for Lake Idylwild for Total Phosphorus (TP) is 64 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 43 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 59 kg/year of TP, and

(d) The Margin of Safety is 5 kg/year of TP.

(4) Lake Jessie. The Total Maximum Daily Load for Lake Jessie for Total Phosphorus (TP) is 140 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 50 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 127 kg/year of TP, and

(d) The Margin of Safety is 13 kg/year of TP.

(5) Lake Lulu. The Total Maximum Daily Load for Lake Lulu for Total Phosphorus (TP) is 84 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 55 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 75 kg/year of TP, and

(d) The Margin of Safety is 9 kg/year of TP.

(6) Lake May. The Total Maximum Daily Load for Lake May for Total Phosphorus (TP) is 88 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 58 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 79 kg/year of TP, and

(d) The Margin of Safety is 9 kg/year of TP.

(7) Lake Mirror. The Total Maximum Daily Load for Lake Mirror for Total Phosphorus (TP) is 55 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 28 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 51 kg/year of TP, and

(d) The Margin of Safety is 4 kg/year of TP.

(8) Lake Shipp. The Total Maximum Daily Load for Lake Shipp for Total Phosphorus (TP) is 97 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 65 percent reduction of the average TP loading for the 1990 to 1999 period,

(c) The Load Allocation for nonpoint sources is 84 kg/year of TP, and

(d) The Margin of Safety is 13 kg/year of TP.

(9) Wahneta Farms Drainage Canal. The fecal coliform Total Maximum Daily Load for Wahneta Farms Drainage Canal Creek is 400 counts/100 mL, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1999 to 2004 period, will require a 39% reduction at sources contributing to exceedances of the criteria,

(b) The Load Allocation for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1999 to 2004 period, will require a 39% reduction at sources contributing to exceedances of the criteria, and

(c) The Margin of Safety is implicit.

(10) Peace River Above Bowlegs Creek. The fecal coliform Total Maximum Daily Load for the Peace River Above Bowlegs Creek is a median of 2.29 x 1012 colonies/day under “moist conditions,” which are defined as flows ranging from 99 to 665 cubic feet per second (cfs), and a median of 1.66 x 1011 colonies/day under “dry conditions,” which are defined as flows ranging from 6.4 to 35 cfs, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1993 to 2003 period, will require a 23% reduction at sources contributing to exceedances of the criteria during moist conditions and a 52% reduction at sources contributing to exceedances of the criteria during dry conditions,

(b) The Load Allocations for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1993 to 2003 period, will require a 23% reduction at sources contributing to exceedances of the criteria during moist conditions and a 52% reduction at sources contributing to exceedances of the criteria during dry conditions, and

(c) The Margin of Safety is implicit.

(11) Peace Creek Drainage Canal. The fecal coliform Total Maximum Daily Load for Peace Creek Drainage Canal is a median of 3.62 x 1011 colonies/day, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Wastewater Permitting Program is to meet applicable water quality criteria for fecal coliform,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1992 to 2004 period, will require a 62% reduction at sources contributing to exceedances of the criteria,

(c) The Load Allocation for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1992 to 2004 period, will require a 62% reduction at sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(12) While the Load Allocation and Wasteload Allocation for fecal coliform as set forth in subsections (9), (10) and (11) above have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal coliform concentrations. However, it is not the intent of these Total Maximum Daily Loads to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 5-1-07.