62-304.725 Southeast Coast Basin TMDLs.

(1) Wagner Creek Fecal Coliform TMDL. The fecal coliform Total Maximum Daily Load for Wagner Creek is 400 counts/100 mL, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1997 to 2006 period, will require an 86% reduction at sources contributing to exceedances of the criteria,

(b) The Load Allocation for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1997 to 2006 period, will require an 86% reduction at sources contributing to exceedances of the criteria, and

(c) The Margin of Safety is implicit.

(2) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal coliform concentrations. However, it is not the intent of these TMDLs to abate natural background conditions.

Specific Authority 403.061, 403.067 FS. Law implemented 403.031, 403.061, 403.062, 403.067 FS. History–New 5-1-07.