40D-8.624 Guidance and Minimum Levels for Lakes.

(1) Levels for lakes adopted during or after August 7, 2000, are set forth in Table 8-2 in subsection 40D-8.624(13), F.A.C. There are three Guidance Levels for lakes, the Ten Year Flood Guidance Level, the High Guidance Level and the Low Guidance Level. There are two Minimum Levels for lakes, the High Minimum Lake Level and the Minimum Lake Level. After the High Minimum Lake Level and Minimum Lake Level for each lake is a designation indicating which of the categories described below applied and, therefore, which method was used to determine the High Minimum Lake Level and the Minimum Lake Level.

(2) Levels for lakes adopted prior to August 7, 2000, are set forth in the table in subsection 40D-8.624(14), F.A.C.

(3) Renaming of Levels.

(a) Lake levels adopted prior to August 7, 2000, were referred to in the District’s rules as management levels and alternatively as minimum levels. These levels were the Ten Year Flood Warning Level, the Minimum Flood Level, the Low Management Level and, for some lakes, Extreme Low Management Level. As of (effective date of these proposed rule revisions) these levels are now referred to as Guidance Levels. For those lakes with levels adopted prior to August 7, 2000, the Ten Year Flood Warning Level is now named the Ten Year Flood Guidance Level, the Minimum Flood is now named the High Level, the Low Management Level is now named the Low Level and the Extreme Low Management Level is now named the Extreme Low Level.

(b) For lakes with levels adopted before August 7, 2000, a reference within the District’s rules to the applicable minimum level or to established minimum water levels shall refer to the Low Level, or, if adopted for the lake, the Extreme Low Level. Such Low Level or Extreme Low Level shall not be deemed a minimum level pursuant to Section 373.042, F.S.

(c) For lakes with levels adopted during or after August 7, 2000, a reference in the District’s rules to the applicable minimum level or to established minimum water levels shall refer to the adopted High Minimum Lake Level and Minimum Lake Level described in this Rule 40D-8.624, F.A.C.

(4) The High Guidance Level (HGL).

(a) The High Guidance Level (HGL) is provided as an advisory guideline for construction of lake shore development, water dependent structures, and operation of water management structures. Figure 8-2 depicts the method described below for calculating the HGL. The High Guidance Level is the expected Historic P10 of the lake. For Category 2 lakes as described in paragraph 40D-8.624(8)(b), F.A.C., below, the HGL and the High Minimum Lake Level may be calculated to be the same elevation.

(b) The HGL is established using best available information, including:

1. Hydrologic data;

2. Hydrologic Indicators;

3. Where the lake has been Structurally Altered, the apparent effectiveness of the Structural Alterations in controlling water levels; and

4. Other information indicative of previous water levels.

(c) Unless the best available information indicates that a different elevation more accurately approximates the Historic P10 of the lake, the HGL shall be established using the following procedure:

1. The HGL shall be established at the Historic P10, if competent Historic data are available.

2. For a Structurally Altered lake with no Historic data, but with Current data, the HGL shall be the higher of the Current P10 or the Control Point Elevation.

3. For a Structurally Altered lake without Historic or Current data, the HGL shall be the Control Point Elevation.

4. For a lake that is not Structurally Altered without Historic or Current data, the HGL is equal to the higher of the Current P10 or the Hydrologic Indicators.

5. For a lake that is not Structurally Altered with no Historic data, but with Current data the HGLs shall be the higher of the Hydrologic Indicators of normal pool or the Current P10.

(5) Historic P50.

(a) The Historic P50 is derived to support development of minimum lake levels. Figure 8-3 depicts the method described below for calculating the Historic P50.

(b) The Historic P50 shall be established using competent Historic data, if it is available.

(c) In those cases where competent Historic data are unavailable, but Current data are available, the Historic P50 is determined by first calculating the difference between the Current P10 and the Current P50 (“P10-P50”) and comparing this to the region-specific Reference Lake Water Regime 50 (RLWR 50), then:

1. If the Current lake specific P10-P50 difference is greater than the RLWR 50, it is assumed the lake is impacted by water withdrawals, therefore, the Historic P50 is estimated by subtracting the RLWR 50 from the HGL.

2. If the Current lake specific P10-P50 difference is less than the RLWR 50 then the Historic P50 is estimated by subtracting the Current lake specific P10-P50 difference from the HGL.

3. If Current data does not exist for the lake, the Historic P50 is estimated by subtracting the RLWR 50 from the HGL.

(6)(a) The High Minimum Lake Level (HML) is the elevation that a lake’s water levels are required to equal or exceed ten percent of the time (P10) on a Long-term basis. This level is used in the evaluation of compliance with Chapter 40D-2, F.A.C., Consumptive Use of Water, and Chapter 40D-4, F.A.C., Environmental Resource Permits. The High Minimum Lake Level is established to ensure that a lake reaches higher levels on a periodic basis.

(b) Lake levels are deemed to be below the High Minimum Lake Level when the Long-term P10 of the lake fails to equal or exceed the High Minimum Lake Level. If insufficient data exists to determine if the lake level is below the High Minimum Lake Level, the lake level can be determined to be below the High Minimum Lake Level based on a comparison with lakes that are hydrologically or hydrogeologically similar, or located in close proximity or in the same drainage basin, or by use of aerial photographs or by evaluation of hydrologic data or Hydrologic Indicators.

(7)(a) The Minimum Lake Level is the elevation that the lake’s water levels are required to equal or exceed fifty percent of the time on a Long-term basis. This level is used in the evaluation of applications for water use permits pursuant to Chapter 40D-2, F.A.C., and Environmental Resource Permits pursuant to Chapter 40D-4, F.A.C.

(b) Lake levels are deemed to be below the Minimum Lake Level when the Long-term P50 lake elevation is below the Minimum Lake Level. If insufficient data exists to determine if the lake level is below the Minimum Lake Level, the lake level can be determined to be below the Minimum Lake Level based on a comparison with lakes that are hydrologically or hydrogeologically similar or, located in close proximity or in the same drainage basin or, by use of aerial photographs or evaluation of Hydrologic data or Hydrologic Indicators.

(8) The method for establishing the High Minimum Lake Level and the Minimum Lake Level pursuant to subsections 40D-8.624(6) and (7), F.A.C., above, will depend on the category within which the subject lake falls, as follows:

(a) Category 1 Lakes – Those lakes with lake-fringing cypress swamp(s) greater than 0.5 acres in size where Structural Alterations have not prevented the Historic P50 from equaling or rising above an elevation that is 1.8 feet below the normal pool of the cypress swamp(s). Figure 8-3 depicts the method for calculating the Historic P50.

1. The High Minimum Lake Level for Category 1 lakes is established 0.4 feet below the normal pool elevation in the cypress swamp(s) contiguous with the lake.

2. The Minimum Lake Level for Category 1 lakes shall be 1.8 feet below the normal pool elevation in the cypress swamps contiguous with the lake. Figure 8-1 depicts the method for calculating the minimum lake levels for Category 1 lakes.

(b) Category 2 Lakes – Those lakes with lake-fringing cypress swamp(s) greater than 0.5 acres in size where Structural Alterations have prevented the Historic P50 from equaling or rising above an elevation that is equal to an elevation that is 1.8 feet below normal pool and the lake-fringing cypress swamp(s) remain viable and perform functions beneficial to the lake in spite of the Structural Alterations. Figure 8-3 depicts the method for calculating the Historic P50.

1. The High Minimum Lake Level shall be established at the HGL.

2. The Minimum Lake Level shall be established at the Historic P50 elevation. Figure 8-3 depicts the method for calculating the Historic P50. Figure 8-1 depicts the method for calculating the Minimum Lake Level for Category 2 lakes.

(c) Category 3 Lakes – Those lakes where there are no lake-fringing cypress swamp(s) greater than 0.5 acre in size.

1. High Minimum Lake Level. Once the Minimum Lake Level is identified as described in subparagraph 40D-8.624(8)(c)2., F.A.C., below, the High Minimum Lake Level may be established, using the region-specific reference lake water regime statistic, or Historic hydrologic data. If Historic data are available, the High Minimum Lake Level may be established at the elevation corresponding to the Minimum Lake Level plus the difference between the Historic P10 and the Historic P50. If Historic data are not available, the High Minimum Lake Level may be established at the elevation corresponding to the Minimum Lake Level plus the region-specific RLWR50 value.

2. The Minimum Lake Level for Category 3 lakes is established utilizing a process that considers, applying professional experience and judgment, multiple parameters including changes in lake mixing and susceptibility to sediment resuspension, changes in water depth associated with docks, change in basin connectivity, changes in species richness, change in coverage of herbaceous wetland vegetation, change in coverage of aquatic macrophytes, and change in cultural (aesthetic and recreational) values as described below.

a. Lake Mixing Standard and Information for Consideration.

(I) Step 1.

(A) Establish stage specific dynamic ratio values (square root of the lake surface area in square kilometers divided by the mean depth in meters).

(B) Establish the Mixing Standard at the highest elevation at or below the Historic P50 elevation where the dynamic ratio shifts from a value of <0.8 to a value >0.8 or from a value of >0.8 to a value of <0.8.

(II) Step 2. Develop water column depth profiles of water temperature, and dissolved oxygen concentration during summer months.

(III) Step 3. Based on professional experience and judgment, review all relevant information pertaining to water column mixing and stratification in the lake (including the Mixing Standard and stability of water column thermal stratification) for development of minimum levels.

b. Dock-Use Standard and Information for Consideration.

(I) Step 1. If boats or other watercraft are used on the lake, determine the elevation of sediments at the end of existing docks, and establish the elevation exceeded by ten percent of the sediment elevation values.

(II) Step 2.

(A) If Historic data are available, derive the Dock-Use Standard by adding 2 feet and the difference (in feet) between the Historic P50 and Historic P90 elevation to the elevation exceeded by ten percent of the sediment elevations at the end of existing docks.

(B) If Historic data are not available, derive the Dock-Use Standard by adding 2 feet and the region-specific RLWR5090 value (in feet) to the elevation exceeded by ten percent of the sediment elevations at the end of existing docks.

(III) Step 3. Based on professional experience and judgment, review relevant information pertaining to dock use and dock elevations at the lake (including the Dock-Use Standard) for development of minimum levels.

c. Basin Connectivity Standard and Information for Consideration.

(I) Step 1. Determine elevations for areas of potential surface water connectivity among sub-basins within the lake basin or between the lake and other lakes.

(II) Step 2. Identify the highest elevation (or other appropriate elevation in the areas of connectivity) as the critical high-spot elevation.

(III) Step 3.

(A) If powerboats are used at the lake and Historic data are available, derive the Basin Connectivity Standard by adding 2 feet and the difference (in feet) between the Historic P50 and Historic P90 elevations to the critical high spot elevation.

(B) If powerboats are used at the lake, and Historic data are not available, derive the Basin Connectivity Standard by adding 2 feet and the region-specific RLWR5090 value (in feet) to the critical high spot elevation.

(C) If powerboats are not used at the lake and Historic data are available, derive the Basin Connectivity Standard by adding 1 foot and the difference (in feet) between the Historic P50 and Historic P90 elevations to the critical high spot elevation.

(D) If powerboats are not used at the lake, and Historic data are not available, derive the Basin Connectivity Standard by adding 1 foot and the region-specific RLWR5090 value (in feet) to the critical high spot elevation.

(IV) Step 4. Based on professional experience and judgment, review relevant information pertaining to inter- and intra-basin connections for the lake basin (including the Basin Connectivity Standard) for development of minimum levels.

d. Species Richness Standard and Information for Consideration.

(I) Step 1. Determine the lake surface area associated with the Historic P50 elevation.

(II) Step 2. Establish the Species Richness Standard at an elevation corresponding to the lowest elevation associated with less than a 15% reduction in lake surface area relative to the area at the Historic P50 elevation.

(III) Step 3. Based on professional experience and judgment, review relevant information pertaining to biological diversity within the lake basin (including the Species Richness Standard) for development of minimum levels.

e. Herbaceous Wetland Information for Consideration.

(I) Step 1. Determine stage-specific potential wetland area values (i.e., lake area with a water depth less than or equal to 4 feet) for the lake basin.

(II) Step 2. Identify elevations at which change in lake stage would result in substantial change in potential wetland area within the lake basin.

(III) Step 3. Based on professional experience and judgment, review relevant information pertaining to herbaceous wetlands in the lake basin (including elevations at which change in lake stage would result in substantial change in potential wetland area, and elevations of connections between the lake basin and contiguous wetland areas) for development of minimum levels.

f. Submersed Aquatic Macrophyte Information for Consideration.

(I) Step 1. Determine lake-specific maximum depth of colonization values for submersed aquatic macrophytes using a representative, lake-specific Secchi Disk depth value and an empirically-derived relationship between Secchi Disk depth and maximum depth of macrophyte colonization.

(II) Step 2. Determine stage-specific lake area available for submersed aquatic macrophyte colonization using the lake-specific maximum depth of colonization value.

(III) Step 3. Identify elevations at which change in lake stage would result in substantial change in the area available for colonization by submersed aquatic macrophytes.

(IV) Step 4. Based on professional experience and judgment, review relevant information pertaining to aquatic macrophyte coverage in the lake basin (including elevations at which change in lake stage would result in substantial change in the area available for colonization by submersed aquatic macrophytes, and coverage that could hinder navigation) for development of minimum levels.

g. Aesthetics Standard and Information for Consideration.

(I) Step 1. Establish the Aesthetics Standard at the Low Guidance Level.

(II) Step 2. Based on professional experience and judgment, review relevant information pertaining to aesthetic values associated with the lake basin (including the Aesthetics Standard) for development of minimum levels.

h. Recreation/Ski Standard and Information for Consideration.

(I) Step 1. Determine whether the lake basin can contain a ski corridor delineated as a circular area with a radius of 418 feet or a rectangular or polygonal area 200 ft. in width and 2,000 feet in length.

(II) Step 2. If the lake basin can contain a ski corridor, identify the minimum elevation at which the lake basin can contain a ski corridor with a depth of five feet (Ski Elevation) by adding five feet to the elevation at which the basin can contain the ski corridor.

(III) Step 3.

(A) If Historic data are available, derive the Recreation/Ski Standard by adding the difference (in feet) between the Historic P50 and Historic P90 elevations to the Ski Elevation.

(B) If Historic data are not available, derive the Recreation/Ski Standard by adding the region specific RLWR5090 value (in feet) to the Ski Elevation.

(IV) Step 4.

(A) Compare the Recreation/Ski Standard to the Historic P50 elevation.

(B) If the Recreation/Ski Standard is less than or equal to the Historic P50 elevation, use of the standard for development of the Minimum Lake Level is appropriate.

(C) If the Recreation/Ski Standard is greater than the Historic P50 elevation, use of the standard for development of the Minimum Lake Level is not appropriate.

(V) Step 5. Based on professional experience and judgment, review relevant information pertaining to skiing and other recreational activities within the lake basin (including the Recreation/Ski Standard) for development of minimum levels.

i. Establish Minimum Lake Level – Following development of lake-specific standards (Mixing Standard, Dock-Use Standard, Basin Connectivity Standard, Species Richness Standard, Recreation/Ski Standard, Aesthetics Standard) and compilation of other relevant information, the Minimum Lake Level for Category 3 lakes shall be established at the elevation corresponding to the most conservative, i.e., the highest standard, except where that result is an elevation above the Historic P50, in which case the Minimum Level shall be established at the Historic P50 elevation.

(9) Where the Governing Board determines that there are unique factors to be addressed at a particular lake such that the Minimum Lake Level that would be established based upon subsection 40D-8.624(8), F.A.C., above is inappropriate, the Governing Board shall determine the High Minimum Lake Level and the Minimum Lake Level, as applicable, considering the appropriate category standards described above, and the lake's unique factors. Unique factors include:

(a) Elevations associated with residential dwellings, roads or other structures,

(b) Substantial changes in the coverage of herbaceous wetland vegetation or submersed aquatic macrophytes,

(c) Frequent submergence of dock platforms, and

(d) Information relating to protection of nonconsumptive uses, including lake vegetation maps, faunal surveys, bathymetric maps, aerial photographs, elevations of docks, seawalls, house slabs, other structures, typical uses of a lake (e.g., recreation, aesthetics, navigation, irrigation, and surrounding land uses), socio-economic effects of the minimum level and public health, safety and welfare matters.

(10)(a) Low Guidance Level – The Low Guidance Level is provided as an advisory guideline for water dependent structures, information for lake shore residents and operation of water management structures. The Low Guidance Level is the elevation that a lake’s water levels are expected to equal or exceed ninety percent of the time on a Long-term basis. Therefore, this is also the elevation that a lake’s water levels are expected to be below 10 percent of the time on a Long-term basis.

(b) The Low Guidance Level (LGL) is established as:

1. The Historic P90 elevation, if competent Historic data is available.

2. In those cases where competent Historic data are unavailable, but Current data are available, the Low Guidance Level is determined by first calculating the difference between the Current P10 and the Current P90 and comparing this to the region-specific Reference Lake Water Regime 90 (RLWR90), then:

a. If the Current lake specific P10-P90 difference is greater than the RLWR 90, it is assumed the lake is impacted by water withdrawals, therefore, the Low Guidance Level is estimated by subtracting the RLWR 90 from the HGL.

b. If the Current lake specific P10-P90 difference is less than the RLWR 90, then the Low Guidance Level is estimated by subtracting the Current lake specific P10-P90 difference from the HGL.

3. The HGL minus the region-specific RLWR 90 value if competent Historic or Current data are not available.

(11)(a) The Ten Year Flood Guidance Level is provided as an advisory guideline for lake shore development. The Ten Year Flood Guidance Level incorporates the level of flooding expected on a frequency of not less than the ten year recurring interval, or on a frequency of not greater than a ten percent probability of occurrence in any given year. The Ten Year Flood Guidance Level is established using methods that correspond to the hydrology and type of conveyance system of the lake being evaluated.

(b) Open Basin Lakes That Discharge –

1. Data Available – If lake stage records of sufficient quality and quantity are available, the Ten Year Flood Guidance Level for open-basin lakes will be established using statistics derived from frequency analysis of the stage records (statistical method). Annual peak stages will be ranked and fit to a distribution or plotted to estimate the ten year peak stage. At least thirty years of hydrologic data are preferred for establishment of the Ten Year Flood Guidance Level using the statistical method.

2. Data Not Available – Storm event modeling of open-basin lakes will be utilized when sufficient stage data for use of a statistical method are not available. Rainfall depths used for the process are taken from sources such as the National Weather Service Technical Paper 49, and Part D of the District’s Environmental Resource Permitting Information Manual described and incorporated by reference in Rule 40D-4.091, F.A.C. Runoff volumes used for the modeling are computed using conventional methods such as the Natural Resources Conservation Service (NCRS) curve number method, or with standard infiltration formulas (e.g., Horton’s Equation, Green-Ampt Equation). Runoff distributions are computed using conventional methods including the NRCS method or other unit hydrograph methods, or the kinematic wave overland flow method. Modeling programs that account for tailwater and compute backflow (dynamic models) are preferred for the hydraulic routing component of the stormwater event modeling. Methods described in subparagraph 40D-8.624(11)(c)2., F.A.C., “Closed Basin Lakes – Not Available” below can also be applied when the probability of the 10-year flood elevation being exceeded in any given year is influenced by serial correlation to annual peak elevations.

(c) Closed Basin Lakes –

1. Data Available – If lake stage records of sufficient quality and quantity are available, the Ten Year Flood Guidance Level for closed-basin lakes will be established using statistics derived from frequency analysis of the stage record (statistical method). Annual peak stages will be ranked and fit to a distribution or plotted to estimate the ten-year peak stage. As a general rule, at least thirty years of hydrologic data are preferred for establishment of the Ten Year Flood Guidance Level using the statistical method.

2. Data Not Available – Numerical or empirical modeling of closed-basin lakes will be utilized when sufficient stage data for use of a statistical method are not available. Simulation periods of thirty or more years are preferred for either numerical or empirical modeling techniques. A composite record of rainfall records from more than one rainfall station in the region in which the subject lake is located may be used for the process. Calibration of the simulation model shall be based on stage records, Hydrologic Indicators of water level, and eye-witness accounts of peak stages. Model simulations to determine the Ten Year Flood Guidance Level will exclude effects of water withdrawals.

(12) Posted Notice.

(a) Staff gauges will be installed in prominent locations on each lake for which Guidance Levels or Minimum Levels have been established. A notice shall be posted in the immediate proximity of the staff gauge indicating that Levels have been established.

(b) The notice shall indicate the elevations of the Guidance Level(s) and the established Minimum Level(s).

(13) Levels for lakes established during or after August 7, 2000, are set forth in the following table. After the High Minimum Lake Level and Minimum Lake Level elevation for each lake is a designation indicating the Method used, as described in subsection 40D-8.624(8), F.A.C., to establish the level. Compliance with the High Minimum and Minimun Lake Levels is determined pursuant to paragraphs (6)(b) and (7)(b) above. Guidance Levels established prior to August 7, 2000, are set forth in Table 8-3 in subsection 40D-8.624(14), F.A.C., below.

Table 8-2 Minimum and Guidance Levels Established During or After August 7, 2000. Levels are elevations,

in feet above the National Geodetic Vertical Datum of 1929.

|  | Location by |  | Name of Lake |  | Ten-Year |  |  | High |  |  |  |  | High Minimum |  | Minimum Lake |  |  | Low |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | County and |  | and Section, |  | Flood |  |  | Guidance |  |  |  |  | Lake Level |  | Level |  |  | Guidance |  |  |
|  | Basin |  | Township |  | Guidance |  |  | Level |  |  |  |  |  |  |  |  |  | Level |  |  |
|  |  |  | and Range Information |  | Level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (a) In Charlotte |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Peace River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (b) In Citrus County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Coastal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rivers Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (c) In Citrus County |  | Fort Cooper, Lake |  | 35.4' |  |  | 30.9' |  |  |  |  | 30.1' |  | 28.7' |  |  | 26.7' |  |  |
|  | Within the Withlacoochee |  | S-27, T-19S, R-20E |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Tsala Apopka, Lake (Lakes and wetlands comprising the Floral City Pool), S-14, T-20S, R-20E |  | 43.4' |  |  | 41.8' |  |  |  |  | 41.2' (CAT 1) |  | 39.8' (CAT 1) |  |  | 39.6' |  |  |
|  |  |  | Tsala Apopka, Lake (Lakes and wetlands comprising the Hernando Pool), S-26, T-18S, R-19E |  | 40.5' |  |  | 39.0' |  |  |  |  | 38.7' (CAT 1) |  | 37.3' (CAT 1) |  |  | 35.9' |  |  |
|  |  |  | Tsala Apopka, Lake (Lakes and wetlands comprising the Inverness Pool), S-08, T-19S, R-20E |  | 41.8' |  |  | 40.3' |  |  |  |  | 40.1' (CAT 1) |  | 38.7' (CAT 1) |  |  | 37.8' |  |  |
|  | (d) In DeSoto County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Peace |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (e) In Hardee County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Peace |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (f) In Hernando |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Coastal Rivers Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Hunters, Lake |  | 20.8' |  |  | 19.3' |  |  |  |  | 19.3' |  | 17.1' |  |  | 13.7' |  |  |
|  |  |  | S-32, T-23S, |  |  |  |  |  |  |  |  |  | (CAT3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-17E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Weekiwachee |  | 24.0' |  |  | 22.6' |  |  |  |  | 21.4' |  | 18.3' |  |  | 17.5' |  |  |
|  |  |  | Prairie, Lake |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | (Lake Theresa) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | S-23, T-23S, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | R-17E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (g) In Hernando |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hillsborough River |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (h) In Hernando |  | Lindsey, Lake |  | 72.4' |  |  | 68.5' |  |  |  |  | 68.5' |  | 66.1' |  |  | 63.9' |  |  |
|  | County Within the |  | S-25, T-21S, |  |  |  |  |  |  |  |  |  | (CAT 2) |  | (CAT 2) |  |  |  |  |  |
|  | Withlacoochee River |  | R-19E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Mountain, Lake |  | 105.0' |  |  | 102.8' |  |  |  |  | 102.8' |  | 99.5' |  |  | 96.3' |  |  |
|  |  |  | S-16, T-23S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-20E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Neff, Lake |  | 104.1' |  |  | 102.2' |  |  |  |  | 102.2' |  | 94.5' |  |  | 87.4' |  |  |
|  |  |  | S-20, T-23S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-20E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Spring, Lake |  | 183.3' |  |  | 181.6' |  |  |  |  | 180.1' |  | 179.0' |  |  | 179.0' |  |  |
|  |  |  | S-15, T23S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-20E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (i) In Highlands |  | Jackson, Lake |  | 104.1' |  |  | 102.6' |  |  |  |  | 102.4' |  | 101.3' |  |  | 100.2' |  |  |
|  | County Within the |  | S-30 T-34S |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  | Peace River Basin |  | R-29E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | June-in- |  | 75.5' |  |  | 74.7' |  |  |  |  | 74.5' |  | 74.0' |  |  | 73.2' |  |  |
|  |  |  | Winter, Lake |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | S-34, T-36S, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | R-29E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Letta, Lake |  | 100.5' |  |  | 99.5' |  |  |  |  | 99.5' |  | 98.4' |  |  | 97.1' |  |  |
|  |  |  | S-31, T-33S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-29E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Little Jackson, Lake |  | 104.1' |  |  | 102.6' |  |  |  |  | 102.4' |  | 101.3' |  |  | 100.2' |  |  |
|  |  |  | S-06, T-35S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-29E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Lotela, Lake |  | 108.5' |  |  | 107.5' |  |  |  |  | 106.8' |  | 105.7' |  |  | 105.0' |  |  |
|  |  |  | S-26, T-33S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  |  |  | R-28E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (j) In Hillsborough |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Alafia River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (k) In Hillsborough |  | Stemper, Lake |  | 62.6' |  |  | 61.2' |  |  |  |  | 60.8' |  | 59.4' |  |  | 59.1' |  |  |
|  | County Within the |  | S-13, T-27, R-18 |  |  |  |  |  |  |  |  |  | (CAT 1) |  | (CAT 1) |  |  |  |  |  |
|  | Hillsborough River |  | (Levels in feet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Basin |  | NGVD) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (l) In Hillsborough |  | Alice, Lake |  | 42.4' |  |  | 40.9' |  |  |  |  | 40.9' |  | 39.9' |  |  | 38.8' |  |  |
|  | County Within the |  | S-16, T-27, R-17 |  |  |  |  |  |  |  |  |  | (CAT 2) |  | (CAT 2) |  |  |  |  |  |
|  | Northwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hillsborough Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Allen, Lake S-10, T-27S, R-18E |  | 63.1' |  |  | 61.8' |  |  |  |  | 61.8' (CAT 1) |  | 60.7' (CAT 1) |  |  | 59.7' |  |  |
|  |  |  | Barbara, Lake S-19, T-27, R-18 |  | 54.96' |  |  | 53.15' |  |  |  |  | 53.15' (CAT 3) |  | 52.15' (CAT 3) |  |  | 51.05' |  |  |
|  |  |  | Bird, Lake S-26, T-27, R-18 |  | 53.0' |  |  | 49.6' |  |  |  |  | 49.6' (CAT 2) |  | 48.6' (CAT 2) |  |  | 47.5' |  |  |
|  |  |  | Brant, Lake S-23, T-27, R-18 |  | 60.5' |  |  | 58.0' |  |  |  |  | 58.0' (CAT 2) |  | 57.0' (CAT 2) |  |  | 55.9' |  |  |
|  |  |  | Calm, Lake S-14, T-27, R-17 |  | 51.02' |  |  | 49.41' |  |  |  |  | 49.41' (CAT 3) |  | 48.41' (CAT 3) |  |  | 47.31' |  |  |
|  |  |  | Charles, Lake S-23, T-27, R-18 |  | 56.2' |  |  | 54.2' |  |  |  |  | 53.8' (CAT 1) |  | 52.4' (CAT 1) |  |  | 52.1' |  |  |
|  |  |  | Church, Lake S-28, T-27, R-17 |  | 36.74' |  |  | 35.64' |  |  |  |  | 35.64' (CAT 3) |  | 34.64' (CAT 3) |  |  | 33.54' |  |  |
|  |  |  | Crenshaw, Lake S-22, T-27, R-18 |  | 57.64' |  |  | 55.5' |  |  |  |  | 54.45' (CAT 3) |  | 53.45' (CAT 3) |  |  | 53.4' |  |  |
|  |  |  | Crescent, Lake S-10, T-27S, R-17E |  | 43.4' |  |  | 41.9' |  |  |  |  | 41.3' (CAT 3) |  | 40.3' (CAT 3) |  |  | 39.8' |  |  |
|  |  |  | Crystal, Lake S-14, T-27, R-18 |  | 62.1' |  |  | 59.8' |  |  |  |  | 59.8' (CAT 2) |  | 58.8' (CAT 2) |  |  | 57.7' |  |  |
|  |  |  | Cypress, Lake S-24, T-27, R-17 |  | 50.86' |  |  | 48.89' |  |  |  |  | 48.89' (CAT 3) |  | 47.89' (CAT 3) |  |  | 46.79' |  |  |
|  |  |  | Dan, Lake S-6, T-27, R-17 |  | 34.9' |  |  | 32.5' |  |  |  |  | 31.9' (CAT 3) |  | 30.9' (CAT 3) |  |  | 30.4' |  |  |
|  |  |  | Deer, Lake |  | 70.0' |  |  | 66.5' |  |  |  |  | 66.5' |  | 65.5' |  |  | 64.4' |  |  |
|  |  |  | S-1, T-27, R-18 |  |  |  |  |  |  |  |  |  | (CAT 2) |  | (CAT 2) |  |  |  |  |  |
|  |  |  | Dosson, Lake |  | 55.1' |  |  | 53.4' |  |  |  |  | 53.4' |  | 52.4' |  |  | 51.3' |  |  |
|  |  |  | S-20, T-27, R-18 |  |  |  |  |  |  |  |  |  | (CAT 2) |  | (CAT 2) |  |  |  |  |  |
|  |  |  | Echo, Lake S-28, T-27, R-17 |  | 36.74' |  |  | 35.64' |  |  |  |  | 35.64' (CAT 3) |  | 34.64' (CAT 3) |  |  | 33.54' |  |  |
|  |  |  | Ellen, Lake S-19, T-27, R-18 |  | 54.96' |  |  | 53.15' |  |  |  |  | 53.15' (CAT 3) |  | 52.15' (CAT 3) |  |  | 51.05' |  |  |
|  |  |  | Fairy (Maurine), Lake S-34, T-27, R-17 |  | 34.51' |  |  | 33.51' |  |  |  |  | 33.41' (CAT 3) |  | 32.41' (CAT 3) |  |  | 31.31' |  |  |
|  |  |  | Garden, Lake S-17, T-27, R-17 |  | 33.9' |  |  | 31.5' |  |  |  |  | 30.5' (CAT 3) |  | 29.5' (CAT 3) |  |  | 29.4' |  |  |
|  |  |  | Halfmoon, Lake S-31, T-27, R-18 |  | 45.07' |  |  | 43.3' |  |  |  |  | 43.3' (CAT 2) |  | 42.3' (CAT 2) |  |  | 41.2' |  |  |
|  |  |  | Harvey, Lake S-3, T-27S, R-18E |  | 63.1' |  |  | 61.8' |  |  |  |  | 61.8' (CAT 1) |  | 60.7' (CAT 1) |  |  | 59.7' |  |  |
|  |  |  | Helen, Lake S-19, T-27, R-18 |  | 54.96' |  |  | 53.15' |  |  |  |  | 53.15' (CAT 3) |  | 52.15' (CAT 3) |  |  | 51.05' |  |  |
|  |  |  | Hobbs, Lake S-1, T-27, R-18 |  | 67.75' |  |  | 65.46' |  |  |  |  | 65.46' (CAT 2) |  | 64.46' (CAT 2) |  |  | 63.36' |  |  |
|  |  |  | Horse, Lake S-26, T-27, R-17 |  | 48.9' |  |  | 46.9' |  |  |  |  | 45.8' (CAT 3) |  | 44.8' (CAT 3) |  |  | 44.8' |  |  |
|  |  |  | Jackson, Lake S-17, T-27, R-17 |  | 34.7' |  |  | 33.0' |  |  |  |  | 33.0' (CAT 2) |  | 32.0' (CAT 2) |  |  | 30.9' |  |  |
|  |  |  | Juanita, Lake S-22, T-27, R-17 |  | 43.8' |  |  | 41.7' |  |  |  |  | 41.7' (CAT 2) |  | 40.7' (CAT 2) |  |  | 39.6' |  |  |
|  |  |  | Little Moon, Lake S-28, T-27, R-17 |  | 40.8' |  |  | 39.1' |  |  |  |  | 39.1' (CAT 2) |  | 38.1' (CAT 2) |  |  | 37.0' |  |  |
|  |  |  | Merrywater, Lake S-22, T-27, R-18 |  | 58.0' |  |  | 55.8' |  |  |  |  | 55.8' (CAT 2) |  | 54.8' (CAT 2) |  |  | 53.7' |  |  |
|  |  |  | Mound, Lake S-11, T-27, R-17 |  | 51.8' |  |  | 50.2' |  |  |  |  | 50.7' (CAT 1) |  | 49.3' (CAT 1) |  |  | 48.3' |  |  |
|  |  |  | Platt, Lake S-35, T-27, R-18 |  | 52.0' |  |  | 49.7' |  |  |  |  | 49.5' (CAT 1) |  | 48.1' (CAT 1) |  |  | 45.5' |  |  |
|  |  |  | Pretty, Lake S-26, T-27S, R-17E |  | 45.8' |  |  | 44.3' |  |  |  |  | 43.9' (CAT 1) |  | 42.5' (CAT 1) |  |  | 42.2' |  |  |
|  |  |  | Rainbow, Lake S-22, T-27, R-17 |  | 40.8' |  |  | 39.1' |  |  |  |  | 39.1' (CAT 2) |  | 38.1' (CAT 2) |  |  | 37.0' |  |  |
|  |  |  | Raleigh, Lake S-27, T-27, R-17 |  | RESERVED |  |  | RESERVED |  |  |  |  | RESERVED (CAT 3) |  | RESERVED (CAT 3) |  |  | RESERVED |  |  |
|  |  |  | Reinheimer, Lake S-15, T-27, R-18 |  | 60.8' |  |  | 58.6' |  |  |  |  | 58.9' (CAT 1) |  | 57.5' (CAT 1) |  |  | 56.5' |  |  |
|  |  |  | Rogers, Lake S-27, T-27, R-17 |  | RESERVED |  |  | RESERVED |  |  |  |  | RESERVED (CAT 3) |  | RESERVED (CAT 3) |  |  | RESERVED |  |  |
|  |  |  | Round, Lake S-22, T-27, R-18 |  | 56.49' |  |  | 55.6' |  |  |  |  | 54.5' (CAT 3) |  | 53.5' (CAT 3) |  |  | 53.5' |  |  |
|  |  |  | Saddleback, Lake S-22, T-27, R-18 |  | 56.11' |  |  | 54.58' |  |  |  |  | 54.58' (CAT 2) |  | 53.58' (CAT 2) |  |  | 52.48' |  |  |
|  |  |  | Sapphire, Lake S-14, T-27, R-18 |  | 64.1' |  |  | 63.4' |  |  |  |  | 63.0' (CAT 1) |  | 61.6' (CAT 1) |  |  | 61.3' |  |  |
|  |  |  | Starvation, Lake S-21, T-27, R-18 |  | RESERVED |  |  | RESERVED |  |  |  |  | RESERVED (CAT 3) |  | RESERVED (CAT 3) |  |  | RESERVED |  |  |
|  |  |  | Strawberry (North Crystal), Lake S-14, T-27, R-18 |  | 62.0' |  |  | 60.1' |  |  |  |  | 60.1' (CAT 3) |  | 59.1' (CAT 3) |  |  | 58.0' |  |  |
|  |  |  | Sunset, Lake S-17, T-27, R-17 |  | 35.0' |  |  | 34.8' |  |  |  |  | 34.4' (CAT 1) |  | 33.0' (CAT 1) |  |  | 32.7' |  |  |
|  |  |  | Sunshine, Lake S-20, T-27, R-18 |  | 55.1' |  |  | 53.4' |  |  |  |  | 53.4' (CAT 2) |  | 52.4' (CAT 2) |  |  | 51.3' |  |  |
|  |  |  | Taylor, Lake S-16, T-27S, R-17E |  | 40.1' |  |  | 38.2' |  |  |  |  | 38.2' (CAT 2) |  | 37.2' (CAT 2) |  |  | 36.1' |  |  |
|  |  |  | Virginia, Lake S-3, T-27S, R-18E |  | 63.1' |  |  | 61.8' |  |  |  |  | 61.8' (CAT 1) |  | 60.7' (CAT 1) |  |  | 59.7' |  |  |
|  | (m) In Lake County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Green |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Swamp Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (n) In Levy County Within the |  | Marion, Lake S-02, T-14S, R-17E |  | 56.6' |  |  | 55.3' |  |  |  |  | 54.6' (CAT 3) |  | 50.7' (CAT 3) |  |  | 47.7' |  |  |
|  | Withlacoochee |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (o) In Manatee County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Manasota River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (p) In Marion County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Withlacoochee |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (q) In Pasco County |  | Big Fish, Lake |  | 77.41' |  |  | 76.05' |  |  |  |  | 75.65' |  | 73.05' |  |  | 71.75' |  |  |
|  | Within the Coastal |  | S-21, T-24, R-19 |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  | Rivers Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Green, Lake S-16, T-26, R-18 |  | 75.5' |  |  | 74.4' |  |  |  |  | 74.2' (CAT 2) |  | 71.9' (CAT 2) |  |  | 70.0' |  |  |
|  |  |  | Moon, Lake S-28, T-25, R-17 |  | 41.7' |  |  | 39.9' |  |  |  |  | 39.9' (CAT 3) |  | 38.3' (CAT 3) |  |  | 36.2' |  |  |
|  | (r) In Pasco County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Green |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Swamp Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (s) In Pasco County |  | Bell, Lake |  | 73.3' |  |  | 71.6' |  |  |  |  | 70.8' |  | 69.4' |  |  | 69.2' |  |  |
|  | Within the |  | S-13, T-26, R-18 |  |  |  |  |  |  |  |  |  | (CAT 1) |  | (CAT 1) |  |  |  |  |  |
|  | Hillsborough |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Bird, Lake S-36, T-26, R-18 |  | 68.2' |  |  | 66.8' |  |  |  |  | 66.6' (CAT 1) |  | 65.2' (CAT 1) |  |  | 64.3' |  |  |
|  |  |  | Buddy, Lake S-17, T-25S, R-21E |  | 96.9' |  |  | 93.7' |  |  |  |  | 93.7' (CAT 3) |  | 87.3' (CAT 3) |  |  | 83.9' |  |  |
|  |  |  | Hancock, Lake S-5, T-24S, R-20E |  | 106.9' |  |  | 102.5' |  |  |  |  | 102.5' (CAT 3) |  | 100.2' (CAT 3) |  |  | 97.7' |  |  |
|  |  |  | Middle, Lake S-4, T-24S, R-20E |  | 106.9' |  |  | 103.1' |  |  |  |  | 102.5' (CAT 3) |  | 100.2' (CAT 3) |  |  | 99.2' |  |  |
|  |  |  | Padgett, Lake S-24, T-26, R-18 |  | 71.5' |  |  | 70.5' |  |  |  |  | 70.5' (CAT 1) |  | 69.5' (CAT 1) |  |  | 68.4' |  |  |
|  |  |  | Pasadena, Lake S-16, T-25S, R-21E |  | 96.9' |  |  | 93.7' |  |  |  |  | 93.7' (CAT 3) |  | 87.3' (CAT 3) |  |  | 83.9' |  |  |
|  | (t) In Pasco County |  | Camp, Lake |  | 64.3' |  |  | 63.8' |  |  |  |  | 63.4' |  | 62.0' |  |  | 61.3' |  |  |
|  | Within the Pinellas- |  | S-34, T-26, R-18 |  |  |  |  |  |  |  |  |  | (CAT 1) |  | (CAT 1) |  |  |  |  |  |
|  | Anclote River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Parker (Ann), Lake S-35, T-26, R-17 |  | 49.5' |  |  | 48.3' |  |  |  |  | 48.1' (CAT 1) |  | 46.7' (CAT 1) |  |  | 46.2' |  |  |
|  | (u) In Pasco County Within the |  | Clear, Lake S-1, T-25S, R-20E |  | 128.5' |  |  | 127.0' |  |  |  |  | 127.0' (CAT 3) |  | 125.7' (CAT 3) |  |  | 123.7' |  |  |
|  | Withlacoochee |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (v) In Pinellas County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Pinellas- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Anclote River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (w) In Polk County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Alafia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (x) In Polk County Within the Green |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Swamp Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (y) In Polk County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hillsborough |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (z) In Polk County |  | Annie, Lake |  |  |  |  | 116.0' |  |  |  |  | 115.2' |  | 112.8' |  |  | 111.7' |  |  |
|  | Within the Peace |  | S-3, T-29S, |  |  |  |  |  |  |  |  |  | (CAT 3) |  | (CAT 3) |  |  |  |  |  |
|  | River Basin |  | R-27E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bonnie, Lake S-31, T-29S, R-28E |  |  |  | 105.9' |  |  |  |  |  | 105.8' (CAT 3) |  | 102.1' (CAT 3) |  |  | 99.8' |  |  |  |
|  |  | Clinch Lake S-31, T-31S, R-28E |  | 107.4' |  | 105.5' |  |  |  |  |  | 105.5' (CAT 3) |  | 104.4' (CAT 3) |  |  | 103.1' |  |  |  |
|  |  | Dinner Lake S-15, T-29S, R-27E |  |  |  | 115.7' |  |  |  |  |  | 114.9' (CAT 3) |  | 112.7' (CAT 3) |  |  | 112.2' |  |  |  |
|  |  | Eagle Lake S-01, T-29S, R-25E |  | 131.3' |  | 129.6' |  |  |  |  |  | 129.0' (CAT 3) |  | 127.9' (CAT 3) |  |  | 127.2' |  |  |  |
|  |  | Lee, Lake S-10, T-29S, R-27E |  |  |  | 116.8' |  |  |  |  |  | 116.0' (CAT 3) |  | 113.9' (CAT 3) |  |  | 113.1' |  |  |  |
|  |  | Mabel, Lake S-11, T-29S, R-27E |  |  |  | 108.7' |  |  |  |  |  | 107.9' (CAT 3) |  | 105.9' (CAT 3) |  |  | 105.4' |  |  |  |
|  |  | McLeod Lake S-07, T-29S, R-26E |  | 133.3' |  | 129.4' |  |  |  |  |  | 129.4' (CAT 3) |  | 128.3' (CAT 3) |  |  | 127.0' |  |  |  |
|  |  | Parker, Lake S-8, T-28S, R-24E |  | 131.8' |  | 130.6' |  |  |  |  |  | 130.6' (CAT 3) |  | 129.6' (CAT 3) |  |  | 129.0' |  |  |  |
|  |  | Starr, Lake S-14, T-29S, R-27E |  |  |  | 106.9' |  |  |  |  |  | 106.1' (CAT 3) |  | 104.2' (CAT 3) |  |  | 103.8' |  |  |  |
|  |  | Venus Lake S-9, T-29S, R-27E |  |  |  | 121.2' |  |  |  |  |  | 120.4' (CAT 3) |  | 118.2' (CAT 3) |  |  | 117.4' |  |  |  |
|  |  | Wales Lake S-01, T-30S, R-27E |  | 114.1' |  | ND |  |  |  |  |  | 107.7' (CAT 3) |  | 106.6' (CAT 3) |  |  | ND |  |  |  |
|  | (aa) In Sarasota |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | County Within the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Manasota Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (bb) In Sumter County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Within the Green |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Swamp Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RESERVED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (cc) In Sumter County |  | Big Gant, Lake |  | 77.6' |  |  | 76.1' |  |  |  |  | 76.3' |  | 74.9' |  |  | 73.4' |  |  |
|  | Within the |  | S-14, T-22S, R-22E |  |  |  |  |  |  |  |  |  | (CAT 1) |  | (CAT 1) |  |  |  |  |  |
|  | Withlacoochee |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | River Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Black, Lake S-23, T-18S, R-23E |  | 57.5' |  | 54.7' |  |  |  | 53.9' (CAT 3) |  |  |  | 51.3' (CAT 3) |  |  | 49.6' |  |  |
|  |  |  | Deaton, Lake S-14, T-19S, R-23E |  | 65.8' |  | 65.2' |  |  |  | 64.8' (CAT 3) |  |  |  | 63.2' (CAT 3) |  |  | 62.2' |  |  |
|  |  |  | Miona, Lake S-27, T-18S, R-23E |  | 57.5' |  | 54.7' |  |  |  | 53.9' (CAT 3) |  |  |  | 51.3' (CAT 3) |  |  | 49.6' |  |  |
|  |  |  | Okahumpka, Lake S-21, T-19S, R-23E |  | 59.9' |  | 58.1' |  |  |  | 58.1' (CAT 1) |  |  |  | 56.7' (CAT 1) |  |  | 55.0' |  |  |
|  |  |  | Panasoffkee, Lake S-29, T-19S, R-22E |  | 42.8' |  | 40.5' |  |  |  | 40.8' (CAT 1) |  |  |  | 39.4' (CAT 1) |  |  | 38.9' |  |  |

(14) Guidance Levels established for lakes prior to August 7, 2000, are set forth in the following table:

Table 8-3 Guidance Water Levels adopted prior to August 7, 2000

| Location of | Ten-Year | High | Low | Extreme Low |
| --- | --- | --- | --- | --- |
| Impoundment by County | Flood | Level in Feet | Level in Feet Above | Level in |
| and Basin | Guidance | Above Mean | Mean Sea Level | Feet Above Mean |
|  | Level In Feet | Sea Level (msl) | (msl) | Sea Level (msl) |
|  | Above Mean Sea |  |  |  |
|  | Level (msl) |  |  |  |

(a) In Charlotte

County Within the

Peace River Basin

(b) In Citrus

County Within the

Coastal Rivers

Basin

(c) In Citrus

County Within the

Withlacoochee

River Basin

| Bradley, Lake | 43.40' | 42.50' | 40.25' | 38.25' |
| --- | --- | --- | --- | --- |
| S23, T20S, R20E |  |  |  |  |
| Cato, Lake | 38.82' | 38.00' | 35.50' | 34.00' |
| S5, T19, R20 |  |  |  |  |
| Connell, Lake | 35.68' | 35.50' | 32.00' | 30.00' |
| S6, T19, R20 |  |  |  |  |
| Cooter, Lake | 41.80' | 40.50' | 38.25' | 36.25' |
| S17, T19, R20 |  |  |  |  |
| Hog Pond, Lake |  |  |  |  |
| (Nina) | 35.92' | 34.50' | 32.00' | 30.00' |
| S2, T19, R19 |  |  |  |  |
| Holden, Lake |  |  |  |  |
| (Inverness) | 32.44' | 32.00' | 29.50' | 28.00' |
| S32, T19, R20 |  |  |  |  |
| Magnolia, Lake | 32.36' | 31.00' | 28.50' | 27.00' |
| S3, T20, R20 |  |  |  |  |
| Rush, Lake (Williams) | 36.20' | 35.75' | 33.25' | 31.25' |
| S03, T17, R18 |  |  |  |  |
| Little, Lake | 43.40' | 41.50' | 39.00' | 37.25' |
| (Consuella) |  |  |  |  |
| S15, T20S, R20E |  |  |  |  |
| Twin, Lake | 38.11' | 37.00' | 35.00' | 33.00' |
| S6, T19, R20 |  |  |  |  |

(d) In Desoto

County Within the

Peace River Basin

(e) In Hardee

County Within the

Peace River Basin

(f) In Hernando

County Within the

Coastal Rivers

Basin

(g) In Hernando

County Within the

Hillsborough River

Basin

LAKES

| Nicks, Lake S29, T23S, R20E | 114.46' | 113.00' | 110.50' | 108.50' |
| --- | --- | --- | --- | --- |

| St. Clair, Lake S33, T23S, R20E | 114.46' | 113.00' | 110.50' | 108.50' |
| --- | --- | --- | --- | --- |

(h) In Hernando

County Within the

Withlacoochee

River Basin

LAKES

| Elizabeth, Lake | 62.90' | 60.25' | 57.00' | 55.50' |
| --- | --- | --- | --- | --- |
| S11, T23S, R21E |  |  |  |  |
| Francis, Lake | 62.90' | 60.25' | 57.00' | 55.50' |
| 11, 23S, 21E |  |  |  |  |
| Geneva, Lake | 62.90' | 60.25' | 57.00' | 55.50' |
| 11, 23S, 21E |  |  |  |  |
| Sparkman, Lake | 94.40' | 91.50' | 89.00' | 88.00' |
| 24, 23S, 19E |  |  |  |  |

(i) In Highlands

County Within the

Peace River Basin

LAKES

Sec. Twsp. Rng.

| Adelaide, Lake | 110.00' | 106.50' | 104.00' | 102.50' |
| --- | --- | --- | --- | --- |
| 5, 33S, 28E |  |  |  |  |
| Angelo, Lake | 104.00' | 99.50' | 97.00' | 95.00' |
| 4, 27S, 25E |  |  |  |  |
| Anoka, Lake | 124.00' | 124.00' | 122.00' | 120.00' |
| 27, 33S, 28E |  |  |  |  |
| Apthorpe, Lake | 72.00' | 71.50' | 68.00' | 66.00' |
| 18, 36S, 30E |  |  |  |  |
| Blue, Lake | 78.70' | 77.50' | 75.00' | 73.50' |
| 30, 36S, 30E |  |  |  |  |
| Bonnet, Lake | 91.90' | 90.75' | 88.00' | 86.00' |
| 8, 34S, 29E |  |  |  |  |
| Brentwood, Lake | 102.30' | 102.75' | 99.50' | 98.00' |
| 10, 33S, 28E |  |  |  |  |
| Buck, Lake | 96.40' | 94.00' | 91.50' | 89.50' |
| 29, 37S, 30E |  |  |  |  |
| Byrd, Lake | 110.60' | 108.25' | 105.50' | 104.00' |
| 9, 33S, 28E |  |  |  |  |
| Carrie, Lake | 75.13' | 75.50' | 73.00' | 72.50' |
| 21, 36S, 29E |  |  |  |  |
| Charlotte, Lake | 93.90' | 93.75' | 91.25' | 89.75' |
| 17, 35S, 29E |  |  |  |  |
| Chilton, Lake | 116.30' | 114.00' | 111.00' | 109.50' |
| 7, 33S, 28E |  |  |  |  |
| Clay, Lake | 79.00' | 78.75' | 76.00' | 75.00' |
| 29, 36S, 30E |  |  |  |  |
| Crews, Lake 32, 36S, 29E | 120.50' | 119.50' | 117.00' | 115.50' |
| Damon, Lake | 102.30' | 101.00' | 98.00' | 95.00' |
| 3, 33S, 28E |  |  |  |  |
| Denton, Lake | 117.10' | 116.50' | 114.00' | 112.00' |
| 2, 34S, 28E |  |  |  |  |
| Dinner, Lake | 103.50' | 102.50' | 98.50' | 97.00' |
| 17, 34S, 29E |  |  |  |  |
| Francis, Lake | 70.80' | 70.50' | 67.50' | 66.50' |
| 22, 36S, 29E |  |  |  |  |
| Glenada, Lake | 118.40' | 120.00' | 117.00' | 115.50' |
| 34, 33S, 28E |  |  |  |  |
| Grassy, Lake | 92.10' | 91.50' | 88.50' | 87.50' |
| 17, 37S, 30E |  |  |  |  |
| Harry, Lake | 67.60' | 67.50' | 63.00' | 62.00' |
| 1, 36S, 29E |  |  |  |  |
| Henry, Lake | 75.13' | 75.50' | 73.00' | 72.50' |
| 25, 36S, 29E |  |  |  |  |
| Hill, Lake | 101.00' | 99.25' | 96.00' | 94.50' |
| 17, 36S, 29E |  |  |  |  |
| Huckleberry, Lake | 104.80' | 104.50' | 102.00' | 101.00' |
| 7, 35S, 29E |  |  |  |  |
| Huntley, Lake | 83.40' | 83.75' | 81.00' | 79.50' |
| 5, 37S, 30E |  |  |  |  |
| Josephine, Lake | 72.70' | 72.50' | 69.00' | 68.50' |
| 32, 35S, 29E |  |  |  |  |
| Lake Lachard | 79.56' | 78.50' | 76.00' | 74.00' |
| 36, 36S, 29E |  |  |  |  |
| Lelia, Lake | 113.00' | 114.50' | 112.50' | 110.50' |
| 34, 33S, 29E |  |  |  |  |
| Little Bonnet, Lake | 101.70' | 100.00' | 97.00' | 96.00' |
| 36, 33S, 28E |  |  |  |  |
| Little Red Water, Lake 14, 36S, 29E | 104.10' | 103.25' | 100.50' | 98.50' |
| Lost, Lake | 90.60' | 88.00' | 84.00' | 82.75' |
| 12, 37S, 29E |  |  |  |  |
| McCoy, Lake | 87.39' | 87.00' | 84.00' | 82.00' |
| 6, 37S, 30E |  |  |  |  |
| Mirror, Lake | 94.70' | 93.50' | 90.00' | 88.00' |
| 7, 37S, 30E |  |  |  |  |
| Center Nellie | 73.20' | 71.50' | 67.00' | 65.00' |
| 13, 36S, 29E |  |  |  |  |
| Nellie N.W., Lake | 73.20' | 71.50' | 67.00' | 65.00' |
| 13, 36S, 29E |  |  |  |  |
| Nellie S.E., Lake | 73.20' | 71.50' | 67.00' | 65.00' |
| 13, 36S, 29E |  |  |  |  |
| Olivia, Lake | 118.10' | 117.50' | 114.50' | 113.00' |
| 6, 33S, 28E |  |  |  |  |
| Pearl, Lake | 87.27' | 87.00' | 84.00' | 82.00' |
| 6, 37S, 30E |  |  |  |  |
| Persimmon, Lake | 69.30' | 68.25' | 65.00' | 63.50' |
| 10, 36S, 29E |  |  |  |  |
| Pioneer, Lake | 108.60' | 108.00' | 104.50' | 103.00' |
| 11, 33S, 28E |  |  |  |  |
| Placid, Lake | 94.70' | 94.50' | 91.50' | 90.00' |
| 30, 36S, 30E |  |  |  |  |
| Pythias, Lake | 101.20' | 101.00' | 98.00' | 95.00' |
| 2, 33S, 28E |  |  |  |  |
| Red Beach Lake | 76.80' | 76.50' | 73.75' | 72.75' |
| 15, 35S, 29E |  |  |  |  |
| Red Water Lake | 70.80' | 70.50' | 67.50' | 66.50' |
| 14, 36S, 29E |  |  |  |  |
| Ruth, Lake | 94.20' | 94.00' | 91.50' | 90.00' |
| 18, 35S, 29E |  |  |  |  |
| Saddlebags, Lake | 84.27' | 84.00' | 81.00' | 79.00' |
| 6, 37S, 30E |  |  |  |  |
| Sebring, Lake 14, 34S, 28E | 107.60' | 107.25' | 104.50' | 103.00' |
| Simmons, Lake | 74.30' | 72.50' | 68.00' | 66.50' |
| 24, 36S, 29E |  |  |  |  |
| Sirena, Lake | 87.27' | 87.00' | 84.00' | 82.00' |
| 1, 37S, 29E |  |  |  |  |
| Trout, Lake | 100.60' | 101.00' | 98.00' | 95.00' |
| 34, 32S, 28E |  |  |  |  |
| Tulane, Lake | 120.50' | 120.00' | 116.00' | 114.00' |
| 27, 33S, 28E |  |  |  |  |
| Unnamed Lake (B) | 92.10' | 91.50' | 88.50' | 87.50' |
| 20, 37S, 30E |  |  |  |  |
| Unnamed Lake (F) | 78.50' | 78.00' | 74.00' | 72.00' |
| 24, 36S, 29E |  |  |  |  |
| Verona, Lake | 123.00' | 119.00' | 115.25' | 113.00' |
| 23, 33S, 28E |  |  |  |  |
| Viola, Lake | 112.20' | 109.50' | 105.75' | 104.00' |
| 14, 33S, 28E |  |  |  |  |
| Wolf Lake | 93.80' | 92.50' | 90.00' | 88.00' |
| 24, 35S, 28E |  |  |  |  |
| (j) In Hillsborough |  |  |  |  |
| County Within the |  |  |  |  |
| Alafia River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Carlton, Lake | 93.60' | 93.50' | 90.50' | 88.00' |
| Edward Medard Reservoir | 67.10' | 62.75' | 57.50' | 56.00' |
| Grady, Lake | 40.70' | 39.00' | 36.00' | 34.00' |
| Hickory Hammock, Lake | 32.80' | 32.25' | 30.50' | 29.00' |
| Unnamed Lake # 2 | 56.00' | 56.00' | NO RECOMMENDATION |  |
| Wimauma, Lake | 87.20' | 86.75' | 83.00' | 81.00' |
| (k) In Hillsborough |  |  |  |  |
| County Within the |  |  |  |  |
| Hillsborough River |  |  |  |  |
| Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Bellows, Lake | 24.50' | 23.75' | 21.50' | 19.00' |
| (East Lake) |  |  |  |  |
| Burrell, Lake | 50.50' | 50.00' | 47.50' | 45.00' |
| Commiston, Lake | 64.20' | 63.00' | 60.50' | 59.00' |
| Eckles, Lake | 33.40' | 32.50' | 30.00' | 28.00' |
| Egypt, Lake | 38.50' | 37.50' | 35.00' | 32.50' |
| Gornto, Lake | 39.00' | 38.50' | 36.00' | 34.00' |
| Hanna, Lake | 63.50' | 62.50' | 59.50' | 58.25' |
| Hart, Lake | 67.20' | 66.00' | 64.00' | 63.00' |
| Hog Island, Lake | 67.00' | 66.00' | 64.00' | 61.00' |
| Hooker, Lake | 45.50' | 45.00' | 43.00' | 42.00' |
| Kathy, Lake | 45.50' | 43.50' | 42.50' | 42.00' |
| Keene, Lake | 63.90' | 63.00' | 60.50' | 59.00' |
| Kell, Lake | 67.30' | 66.00' | 63.50' | 62.50' |
| Long, Lake | 52.00' | 50.25' | 48.00' | 46.00' |
| Long Pond | 48.50' | 46.50' | 44.00' | 42.00' |
| Mud, Lake (Lake Walden) | 115.80' | 115.00' | 112.50' | 110.50' |
| Thonotosassa, Lake | 39.10' | 37.00' | 34.50' | 33.00' |
| Unnamed Lake | 63.40' | 63.00' | 60.50' | 59.00' |
| Unnamed Lake | 62.00' | 61.00' | 58.50' | 57.00' |
| Valrico, Lake | 48.50' | 45.00' | 42.50' | 41.00' |
| Weeks, Lake | 43.80' | 43.25' | 41.00' | 39.50' |
| (l) In Hillsborough County Within the Northwest Hillsborough Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Sec. Twsp. Rng. |  |  |  |  |
| SWEETWATER CREEK WATERSHED |  |  |  |  |
| Avis, Lake 15, 28S, 18E | 38.50' | 37.00' | 34.50' | 32.50' |
| Bay, Lake | 47.00' | 46.75' | 44.00' | 42.50' |
| Boat, Lake | 38.00' | 35.50' | 33.75' | 31.25' |
| Brooker, Lake | 66.00' | 64.25' | 61.00' | 59.00' |
| Carroll, Lake | 38.50' | 37.00' | 34.50' | 32.50' |
| Chapman, Lake | 52.30' | 52.25' | 49.50' | 48.00' |
| Cooper, Lake | 64.00' | 61.75' | 59.75' | 57.00' |
| Elaine, Lake | 38.50' | 37.00' | 34.50' | 32.50' |
| 15, 28S, 18E |  |  |  |  |
| Gass, Lake | 51.50' | 49.50' | 46.25' | 44.50' |
| George, Lake | 51.00' | 48.00' | 45.00' | 42.00' |
| Geraci, Lake | 65.00' | 63.50' | 61.50' | 59.50' |
| Halls, Lake | 50.70' | 50.00' | 47.50' | 46.00' |
| 3, 28S, 18E |  |  |  |  |
| Lipsey, Lake | 42.50' | 41.50' | 39.00' | 37.00' |
| Magdalene, Lake | 50.70' | 50.00' | 47.50' | 46.00' |
| Thomas, Lake | 64.00' | 63.50' | 61.25' | 59.25' |
| Twin, Lake | 34.00' | 32.00' | 30.00' | 27.75' |
| White Trout, Lake | 38.50' | 36.50' | 34.00' | 32.50' |
| ROCKY CREEK WATERSHED |  |  |  |  |
| Armistead, Lake | 46.50' | 44.00' | 40.50' | 39.00' |
| Browns, Lake | 63.70' | 63.50' | 60.75' | 59.00' |
| Josephine, Lake | 47.50' | 46.00' | 42.75' | 40.00' |
| LeClare, Lake | 53.50' | 52.00' | 49.50' | 47.00' |
| Rock, Lake | 48.00' | 46.00' | 42.75' | 40.00' |
| Starvation, Lake | 55.00' | 53.00' | 50.00' | 48.00' |
| Turkey Ford, Lake | 55.00' | 54.00' | 51.50' | 50.00' |

| DOUBLEBRANCH CREEK WATERSHED |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hixon, Lake | 37.00' | 36.50' | 33.25' | 31.00' |
| Unnamed Lake #1 | 38.90' | 37.50' | 35.00' | 34.50' |
| 2, 28S, 17E |  |  |  |  |
| Unnamed Lake #2 | 38.90' | 37.50' | 35.00' | 34.50' |
| 2, 28S, 17E |  |  |  |  |
|  | BROOKER CREEK WATERSHED |  |  |  |
| Artillery, Lake | 44.50' | 44.00' | 40.50' | 39.00' |
| 3, 27S, 17E |  |  |  |  |
| Buck, Lake | 35.50' | 35.00' | 32.00' | 29.50' |
| Elizabeth, Lake | 54.00' | 53.00' | 51.00' | 49.00' |
| Fern, Lake | 48.00' | 46.00' | 43.00' | 41.50' |
| Frances, Lake | 42.50' | 40.50' | 38.00' | 36.00' |
| Island Ford, Lake | 42.30' | 41.50' | 39.00' | 37.00' |
| James, Lake | 47.70' | 46.50' | 43.50' | 42.00' |
| 23, 27S, 17E |  |  |  |  |
| Keystone, Lake | 43.20' | 42.00' | 39.75' | 39.00' |
| Little, Lake | 47.70' | 46.50' | 43.50' | 42.00' |
| 23, 27S, 17E |  |  |  |  |
| Raleigh, Lake | 43.30' | 42.50' | 38.00' | 35.00' |
| Rogers, Lake |  | NO LEVELS RECOMMENDED |  |  |
| Velburton, Lake | 41.50' | 40.00' | 37.25' | 35.00' |
| ANCLOTE RIVER WATERSHED |  |  |  |  |
| Hiawatha, Lake | 52.50' | 50.50' | 48.00' | 45.00' |
| Osceola, Lake | 47.40' | 46.50' | 44.50' | 42.50' |
| (m) In Lake County Within the Green Swamp Basin |  |  |  |  |
| (n) In Levy County Within the Withlacoochee River Basin |  |  |  |  |
| (o) In Manatee County Within the Manasota Basin |  |  |  |  |
| (p) In Marion County Within the Withlacoochee River Basin |  |  |  |  |
| Bonable, Lake | 65.10' | 64.00' | 61.50' | 59.50' |
| S31, T15, R18 |  |  |  |  |
| Little Bonable, Lake | 59.80' | 58.00' | 55.50' | 53.50' |
| S30, T15, R18 |  |  |  |  |
| Tiger, Lake | 65.10' | 64.00' | 61.50' | 59.50' |
| S32, T15, R18 |  |  |  |  |
| (q) In Pasco County Within the Coastal Rivers Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Crews, Lake | 57.00' | 55.00' | 52.00' | 50.00' |
| S16, T24S, R18E |  |  |  |  |
| Garden, Lake | 22.10' | 19.00' | 16.25' | 15.75' |
| S16, T25S, R16E |  |  |  |  |
| Pasco, Lake S22, T24S, R18E | 67.50' | 67.00' | 64.00' | 62.00' |
| Pierce, Lake | 73.60' | 73.00' | 70.00' | 68.00' |
| S9, T25S, R18E |  |  |  |  |
| Richey, Lake | 14.10' | 13.00' | 10.00' | 8.00' |
| S3, T26S, R16E |  |  |  |  |
| Unnamed Lake No. 22 | 62.60' | 62.50' | 59.50' | 57.50' |
| S27, T24S, R18E |  |  |  |  |
| Worrell, Lake | 22.10' | 19.00' | 16.25' | 15.75' |
| (Bass Lake) |  |  |  |  |
| S26, T25S, R16E |  |  |  |  |
| (r) In Pasco County Within the Green Swamp Basin |  |  |  |  |
| (s) In Pasco County Within the Hillsborough River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Catfish, Lake | 68.72' | 68.00' | 65.50' | 63.50' |
| S30, T25, R19 |  |  |  |  |
| Cow (East), Lake | 78.63' | 78.50' | 76.00' | 75.00' |
| S19, T26S, R19E |  |  |  |  |
| Floyd, Lake | 68.41' | 68.50' | 66.00' | 64.00' |
| S36, T26, R19 |  |  |  |  |
| Gooseneck, Lake | 75.10' | 73.50' | 71.00' | 69.00' |
| S29, T26, R19 |  |  |  |  |
| Hog (Joyce), Lake | 76.66' | 76.50' | 73.50' | 72.50' |
| S19, T26S, R19E |  |  |  |  |
| Iola, Lake | 147.55' | 147.50' | 145.00' | 142.50' |
| S15, T24S, R20E |  |  |  |  |
| Jessamine, Lake | 144.18' | 142.00' | 138.00' | 136.00' |
| S11, T24S, R20E |  |  |  |  |
| JoAnn, Lake | 68.72' | 68.00' | 65.50' | 63.50' |
| S30, T26, R19 |  |  |  |  |
| King, Lake S7, T26S, R19E | 73.58' | 73.50' | 71.50' | 69.50' |
| King, Lake (East) | 105.49' | 105.25' | 102.50' | 100.00' |
| S22, T25S, R20E |  |  |  |  |
| Moody, Lake | 110.48' | 110.00' | 107.50' | 105.50' |
| S10, T24S, R20E |  |  |  |  |
| Myrtle, Lake | 68.72' | 68.00' | 65.50' | 63.50' |
| S30, T26, R19 |  |  |  |  |
| Saxon, Lake | 71.34' | 71.25' | 69.00' | 67.50' |
| S30, T26S, R19E |  |  |  |  |
| Tampa (Turtle), Lake | 66.00' | 65.50' | 63.00' | 61.00' |
| S32, T26, R19 |  |  |  |  |
| Toni, Lake | 68.72' | 68.00' | 65.50' | 63.50' |
| S30, T26, R19 |  |  |  |  |
| Twin, Lake | 68.35' | 67.50' | 65.00' | 63.00' |
| S28, T26, R19 |  |  |  |  |
| Unnamed Lake #26 | 68.75' | 68.00' | 65.50' | 63.50' |
| S25, T26, R18 |  |  |  |  |
| (t) In Pasco County Within the Pinellas- Anclote River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Bass, Lake (Holiday) | 48.80' | 48.75' | 45.75' | 45.00' |
| S34, T26E, R17S |  |  |  |  |
| Big Lake Vienna | 70.70' | 70.25' | 67.00' | 65.00' |
| 23, 26, 18 |  |  |  |  |
| Fishing, Lake | 48.80' | 48.75' | 45.75' | 45.00' |
| S34, T26E, R17S |  |  |  |  |
| Geneva, Lake (Mud) | 51.20' | 50.00' | 48.00' | 46.00' |
| S26, T26E, R17S |  |  |  |  |
| Linda, Lake | 67.30' | 66.75' | 64.00' | 62.00' |
| 26, 26, 18 |  |  |  |  |
| Little Moss (Como), Lake | 67.00' | 66.00' | 63.00' | 62.00' |
| S35, T26, R18 |  |  |  |  |
| Minniola, Lake S34, T26E, R17S | 51.20' | 50.00' | 48.00' | 46.00' |
| Moss, Lake | 65.00' | 64.00' | 61.50' | 59.00' |
| 35, 26, 18 |  |  |  |  |
| Seminole, Lake | 49.20' | 48.75' | 46.00' | 45.00' |
| S35, T26E, R17S |  |  |  |  |
| Thomas, Lake | 75.60' | 75.00' | 72.50' | 71.50' |
| 11, 26, 18 |  |  |  |  |
| Wistaria, Lake | 74.90' | 74.00' | 71.00' | 69.00' |
| S2, T26, R18 |  |  |  |  |
| (u) In Pasco County Within the Withlacoochee River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| (v) In Pinellas County Within the Pinellas- Anclote River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Lake Tarpon | 4.20' | 3.80' | 2.20' | 1.00' |
| Sec.Twsp.Rng. |  |  |  |  |
| 7, 27S, 16E |  |  |  |  |
| 8, 27S, 16E |  |  |  |  |
| 16, 27S, 16E |  |  |  |  |
| 17, 27S, 16E |  |  |  |  |
| 18, 27S, 16E |  |  |  |  |
| 19, 27S, 16E |  |  |  |  |
| 20, 27S, 16E |  |  |  |  |
| 21, 27S, 16E |  |  |  |  |
| 28, 27S, 16E |  |  |  |  |
| 29, 27S, 16E |  |  |  |  |
| 30, 27S, 16E |  |  |  |  |
| 32, 27S, 16E |  |  |  |  |
| 33, 27S, 16E |  |  |  |  |
| 4, 28S, 16E |  |  |  |  |
| (w) In Polk County Within the Alafia River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Scott, Lake | 168.60' | 168.00' | 165.00' | 164.25' |
| (x) In Polk County Within the Green Swamp Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Agnes, Lake | 135.20' | 135.75' | 134.75' | 130.75' |
| Alfred, Lake | 132.30' | 130.75' | 128.25' | 126.25' |
| S30, T27S, R26 |  |  |  |  |
| Arietta, Lake | 144.00' | 144.00' | 141.00' | 138.00' |
| Camp, Lake | 133.40' | 134.50' | 132.00' | 130.00' |
| Clearwater Lake | 146.20' | 143.50' | 141.00' | 139.00' |
| Cummings, Lake | 131.50' | 131.00' | 127.50' | 125.50' |
| S31, T27, R26 |  |  |  |  |
| Eva, Lake | 132.30' | 131.50' | 129.00' | 127.00' |
| S29, T27, R26 |  |  |  |  |
| Grassy, Lake (Big Glades) | 133.20' | 132.00' | 129.50' | 128.00' |
| Griffin, Lake | 132.30' | 131.50' | 129.00' | 127.00' |
| S30, T27, R26 |  |  |  |  |
| Gum, Lake | 132.60' | 131.00' | 128.50' | 126.00' |
| Haines, Lake | 129.70' | 128.75' | 126.50' | 124.50' |
| Helene, Lake | 144.80' | 144.00' | 141.00' | 139.00' |
| Juliana, Lake | 134.70' | 132.50' | 130.00' | 127.50' |
| Little Lake Agnes | 135.20' | 136.00' | 133.00' | 131.00' |
| Little Van, Lake | 141.40' | 139.00' | 136.50' | 135.50' |
| Mattie, Lake | 134.70' | 132.50' | 130.00' | 127.50' |
| Mud, Lake | 141.80' | 141.50' | 137.75' | 136.00' |
| Myrtle, Lake | 141.70' | 141.00' | 138.50' | 136.50' |
| Swoope, Lake | 133.00' | 132.50' | 130.00' | 128.00' |
| Tennessee, Lake | 134.70' | 134.00' | 130.00' | 128.00' |
| Van, Lake | 133.00' | 132.75' | 130.00' | 128.00' |
| Whistle, Lake | 140.90' | 137.50' | 135.00' | 133.75' |
| (y) In Polk County Within the Hillsborough River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Bonnet, Lake | 146.40' | 148.00' | 145.00' | 142.50' |
| Hunter, Lake | 162.30' | 162.75' | 160.25' | 159.00' |
| (z) In Polk County Within the Peace River Basin |  |  |  |  |
| Ada, Lake | 123.80' | 123.00' | 120.00' | 118.00' |
| S33, T28, R27 |  |  |  |  |
| Altamaha, Lake | 122.60' | 122.50' | 120.00' | 118.00' |
| S11, T30, R27 |  |  |  |  |
| Amoret, Lake | 115.50' | 115.25' | 113.00' | 111.00' |
| 24, 30, 27 |  |  |  |  |
| Ariana, Lake | 137.10' | 137.00' | 134.50' | 132.50' |
| 3, 28, 25E |  |  |  |  |
| Aurora, Lake | 103.30' | 100.00' | 97.00' | 95.00' |
| 13, 30, 28 |  |  |  |  |
| Banana, Lake | 106.75' | 106.50' | 103.50' | 102.00' |
| 10, 29, 24E |  |  |  |  |
| Belle, Lake | 123.60' | 120.00' | 117.00' | 115.00' |
| 11, 30, 27 |  |  |  |  |
| Bess, Lake | 125.50' | 125.25' | 123.00' | 121.00' |
| 18 29S 27E |  |  |  |  |
| Big Gum, Lake | 95.50' | 95.00' | 92.00' | 89.00' |
| 26, 29, R28 |  |  |  |  |
| Blue, Lake | 149.80' | 149.00' | 146.50' | 144.50' |
| S13, T28, R25 |  |  |  |  |
| Blue Lake 24, 30S, 27E | 118.00' | 117.00' | 114.00' | – |
| Bonny, Lake | 130.90' | 130.50' | 128.00' | 126.00' |
| 20, 28S, 24E |  |  |  |  |
| Buckeye, Lake | 130.10' | 129.00' | 126.00' | 124.50' |
| S22, T28S, R26E |  |  |  |  |
| Buffum, Lake | 132.75' | 132.25' | 129.25' | – |
| 12, 31S, 26E |  |  |  |  |
| Cannon, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 19, 28S, 26E |  |  |  |  |
| Conine, Lake | 129.70' | 128.75' | 126.50' | 124.50' |
| 9, 28S, 26E |  |  |  |  |
| Cooper (Worth) | 124.20' | 123.50' | 121.00' | 119.00' |
| S02, T30, R27 |  |  |  |  |
| Crooked, Lake | 122.60' | 122.00' | 118.50' | – |
| 1, 31S, 27E |  |  |  |  |
| Crystal, Lake | 121.40' | 121.25' | 118.00' | 115.00' |
| S02, T30, R27 |  |  |  |  |
| Crystal, Lake | 122.90' | 122.00' | 119.00' | 117.00' |
| S21, T28, R27 |  |  |  |  |
| Crystal, Lake | 130.00' | 129.50' | 127.00' | 125.00' |
| 23, 29S, 26E |  |  |  |  |
| Cypress, Lake | 100.20' | 98.50' | 95.00' | 93.00' |
| 36, 29, 28E |  |  |  |  |
| Lake Daisy | 130.90' | 130.00' | 127.00' | 126.00' |
| S6, T29, R27 |  |  |  |  |
| Lake Deer | 141.30' | 140.75' | 138.50' | 136.50' |
| 25, 28, 25E |  |  |  |  |
| Dell, Lake | 125.70' | 123.75' | 121.50' | 119.50' |
| S28, T28, R27 |  |  |  |  |
| Lake Dexter | 132.20' | 132.00' | 129.00' | 127.50' |
| S2, T29, R26 |  |  |  |  |
| Easy, Lake | 115.50' | 115.25' | 113.00' | 111.00' |
| 19, 30, 28 |  |  |  |  |
| Echo, Lake | 132.30' | 131.00' | 128.00' | 126.00' |
| S05, T28, R26 |  |  |  |  |
| Effie, Lake | 119.60' | 118.00' | 115.00' | 113.00' |
| 3, 30, 27 |  |  |  |  |
| Elbert, Lake | 137.50' | 135.50' | 133.00' | 131.50' |
| S22, T28, R26 |  |  |  |  |
| Eloise, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 3, 29S, 26E |  |  |  |  |
| Fannie, Lake | 127.00' | 125.75' | 123.50' | 120.00' |
| 11, 28S, 26E |  |  |  |  |
| Lake Florence | 128.80' | 128.75' | 127.00' | 125.00' |
| S35, T28, R26 |  |  |  |  |
| Lake Fox | 135.20' | 135.00' | 132.00' | 131.00' |
| S6, T29, R27 |  |  |  |  |
| Garfield, Lake | 105.70' | 104.75' | 101.00' | 100.00' |
| 5, 30, 26E |  |  |  |  |
| Gator, Lake | 133.60' | 133.00' | 130.75' | 128.50' |
| 26, 30S, 26E |  |  |  |  |
| George, Lake | 130.70' | 130.00' | 127.50' | 125.50' |
| S06, T28, R26 |  |  |  |  |
| Gibson, Lake | 144.20' | 143.50' | 141.50' | 141.50' |
| 25, 27S, 23E |  |  |  |  |
| Gordon, Lake | 121.30' | 119.00' | 116.00' | 114.00' |
| S16, T28, R27 |  |  |  |  |
| Lake Grassy | 134.80' | 129.00' | 126.50' | 125.50' |
| 2, 29, 25E |  |  |  |  |
| Lake Gross | 138.50' | 136.00' | 133.50' | 132.00' |
| (Grassy) |  |  |  |  |
| S14, T29, R26 |  |  |  |  |
| Hamilton, Lake | 122.50' | 121.50' | 119.00' | 117.25' |
| 18, 28S, 27E |  |  |  |  |
| Hancock, Lake | 102.40' | 99.00' | 96.00' | 94.00' |
| 8, 29S, 25E |  |  |  |  |
| Hart, Lake | 124.70' | 124.50' | 122.00' | 120.00' |
| 24, 29S, 26E |  |  |  |  |
| Hartridge, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 8, 28S, 26E |  |  |  |  |
| Henry, Lake | 160.10' | 159.00' | 156.00' | 154.00' |
| 16, 31S, 26E |  |  |  |  |
| Henry, Lake | 127.00' | 126.50' | 124.50' | 122.50' |
| 36, 27S, 26E |  |  |  |  |
| Hickory, Lake | 98.50' | 98.50' | 96.00' | 94.00' |
| 17, 32S, 28E |  |  |  |  |
| Howard, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 30, 28S, 26E |  |  |  |  |
| Ida, Lake | 80.00' | 79.00' | 76.50' | 75.00' |
| 28, 31S, 28E |  |  |  |  |
| Ida, Lake | 136.70' | 135.25' | 132.00' | 130.50' |
| S17, T28, R26 |  |  |  |  |
| Idyl, Lake | 134.90' | 134.00' | 131.50' | 130.00' |
| S16, T28, R26 |  |  |  |  |
| Idylwild, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 18, 28S, 26E |  |  |  |  |
| Jessie, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 12, 28S, 25E |  |  |  |  |
| Josephine, Lake | 121.30' | 120.00' | 116.50' | 114.50' |
| 13, 30, 27 |  |  |  |  |
| Josephine, Lake | 124.10' | 121.50' | 118.00' | 116.50' |
| S27, T28, R27 |  |  |  |  |
| Lee, Lake | 123.50' | 123.50' | 121.50' | 120.00' |
| S16, T28, R27 |  |  |  |  |
| Lena, Lake | 137.10' | 137.00' | 134.50' | 132.50' |
| 9, 28S, 25E |  |  |  |  |
| Leonore, Lake | 87.40' | 87.00' | 84.50' | 83.00' |
| 10, 31S, 28E |  |  |  |  |
| Link, Lake | 128.70' | 128.00' | 125.00' | 123.00' |
| 27, 28S, 26E |  |  |  |  |
| Little Aurora | 103.30' | 100.50' | 98.00' | 96.00' |
| Little Gum, Lake | 96.80' | 96.50' | 94.00' | 92.00' |
| 35, 29S, 28E |  |  |  |  |
| Little Lake Hamilton | 122.50' | 121.50' | 119.00' | 117.25' |
| 5, 28S, 27E |  |  |  |  |
| LuLu, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 4, 29S, 26E |  |  |  |  |
| Mariam, Lake | – | 124.75' | 122.75' | 121.00' |
| 27, 28S, 26E |  |  |  |  |
| Marie, Lake | 121.00' | 121.00' | 118.00' | 116.00' |
| S27, T28, R27 |  |  |  |  |
| Martha, Lake | 142.50' | 142.00' | 139.00' | 137.00' |
| S21, T28, R26 |  |  |  |  |
| Maude, Lake | 141.70' | 140.50' | 137.50' | 136.00' |
| S21, T28, R26 |  |  |  |  |
| May, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 29, 28S, 26E |  |  |  |  |
| Medora, Lake | 140.40' | 138.00' | 134.50' | 133.00' |
| S36, T27, R25 |  |  |  |  |
| Menzie, Lake | 127.00' | 122.00' | 120.00' | 118.00' |
| S28, T28, R27 |  |  |  |  |
| Middle Lake Hamilton | 122.50' | 121.50' | 119.00' | 117.25' |
| 7, 28S, 27E |  |  |  |  |
| Lake Millsite | 125.30' | 123.50' | 121.00' | 119.00' |
| 11, 29, 25E |  |  |  |  |
| Mirror, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 20, 28S, 27E |  |  |  |  |
| Moody, Lake | 92.80' | 93.50' | 91.00' | 89.00' |
| 17, 31S, R28E |  |  |  |  |
| Myrtle, Lake | 118.70' | 118.50' | 116.50' | 114.50' |
| 19, 29S, 27E |  |  |  |  |
| Lake Ned | 129.60' | 128.50' | 126.00' | 124.00' |
| S1, T29, R26 |  |  |  |  |
| North Lake Wales | 116.80' | 115.00' | 112.00' | 110.00' |
| S01, T30, R27 |  |  |  |  |
| Otis, Lake | 128.70' | 128.00' | 125.00' | 123.00' |
| 28, 28S, 25E |  |  |  |  |
| Pansy, Lake | 130.00' | 129.00' | 126.50' | 124.50' |
| S08, T28, R26 |  |  |  |  |
| Parker, Lake | 122.50' | 122.00' | 119.50' | 117.50' |
| 32, 29S, 27E |  |  |  |  |
| Parks, Lake | 104.50' | 102.50' | 100.00' | 98.00' |
| 36, 29S, 28E |  |  |  |  |
| Polecat, Lake | 142.40' | 142.00' | 139.50' | 137.50' |
| 27, 30S, 26E |  |  |  |  |
| Reedy, Lake | 80.00' | 79.75' | 77.25' | 75.25' |
| 35, 31S, 28E |  |  |  |  |
| Reeves, Lake | 125.10' | 124.50' | 122.00' | 120.00' |
| 13, 29S, 26E |  |  |  |  |
| Lake River | 141.60' | 139.50' | 136.00' | 134.00' |
| S1, T29, R26 |  |  |  |  |
| Rochelle, Lake | 129.70' | 128.75' | 126.50' | 124.50' |
| 4, 28S, 26E |  |  |  |  |
| Round, Lake | 129.40' | 129.25' | 126.50' | 124.50' |
| 13, 29S, 26E |  |  |  |  |
| Roy, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 34, 28S, 26E |  |  |  |  |
| Ruby, Lake | 125.50' | 125.25' | 123.00' | 121.00' |
| 12, 29S, 26E |  |  |  |  |
| Ruth, Lake | 123.50' | 121.50' | 117.50' | 115.50' |
| S28, T28, R27 |  |  |  |  |
| Saddlebag, Lake | 106.80' | 105.00' | 102.00' | 100.00' |
| 6, 30S, 29E |  |  |  |  |
| Saint Anne Lake | 97.50' | 96.00' | 93.00' | 91.00' |
| 14, 30, 28 |  |  |  |  |
| Sanitary (Mariana), Lake | 138.60' | 137.50' | 135.00' | 133.00' |
| S01, T28, R25 |  |  |  |  |
| Sara, Lake | 122.50' | 121.50' | 119.00' | 117.25' |
| S17, T28, R27 |  |  |  |  |
| Scott, Lake | 168.60' | 168.00' | 165.00' | 164.25' |
| 18, 29S, 24E |  |  |  |  |
| Lake Sears | 143.20' | 141.00' | 138.00' | 136.00' |
| 36, 28, 25E |  |  |  |  |
| Serena, Lake | 125.30' | 118.00' | 115.00' | 113.00' |
| S12, T30, R27 |  |  |  |  |
| Shipp, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 32, 28S, 26E |  |  |  |  |
| Silver, Lake | 105.00' | 103.00' | 100.50' | 98.50' |
| 5, 32S, 28E |  |  |  |  |
| Silver, Lake | 147.10' | 146.50' | 144.00' | 142.00' |
| S20, T28, R26 |  |  |  |  |
| Smart, Lake | 129.70' | 128.75' | 126.50' | 124.50' |
| 9, 28S, 26E |  |  |  |  |
| Lake Spirit | 134.10' | 131.50' | 129.00' | 127.00' |
| 35, 28, 25E |  |  |  |  |
| Spring, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 20, 28S, 27E |  |  |  |  |
| Streety, Lake | 108.70' | 105.50' | 102.50' | 101.00' |
| 24, 32S, 27E |  |  |  |  |
| Summit, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 34, 28S, 26E |  |  |  |  |
| Sunset, Lake | 101.10' | 98.00' | 95.50' | 93.50' |
| 10, 30, 28 |  |  |  |  |
| Surveyors, Lake | 133.60' | 133.00' | 130.75' | 128.50' |
| 26, 30S, 26E |  |  |  |  |
| Thomas, Lake | 104.20' | 99.50' | 97.00' | 95.00' |
| 1, 30E, 28E |  |  |  |  |
| Lake Thomas | 135.60' | 132.00' | 128.00' | 126.00' |
| 35, 28, 25E |  |  |  |  |
| Tractor, Lake | 125.00' | 123.25' | 121.00' | 119.00' |
| 14, 30, 27 |  |  |  |  |
| Trask, Lake | 114.90' | 113.00' | 108.00' | 106.00' |
| S22, T28, R27 |  |  |  |  |
| Trout, Lake | 100.60' | 101.00' | 98.00' | 95.00' |
| 34, 32S, 28E |  |  |  |  |
| Twin, Lakes | 124.10' | 123.75' | 120.00' | 118.00' |
| S11, T30, R27 |  |  |  |  |
| Walker, Lake | 143.00' | 141.00' | 137.00' | 135.00' |
| 21, 30S, 26E |  |  |  |  |
| Warren, Lake | 124.60' | 123.50' | 121.00' | 119.00' |
| S11, T30, R27 |  |  |  |  |
| Weader (Weaver), Lake | 122.00' | 121.75' | 119.00' | 117.00' |
| S03, T30, R27 |  |  |  |  |
| Winterset, Lake | 132.60' | 132.00' | 129.50' | 127.00' |
| 11, 29S, 26E |  |  |  |  |
| (aa) In Sarasota County Within |  |  |  |  |
| the Manasota Basin |  |  |  |  |
| (bb) In Sumter County Within the Green Swamp Basin |  |  |  |  |
| (cc) In Sumter County Within the Withlacoochee River Basin |  |  |  |  |
| LAKES |  |  |  |  |
| Cherry, Lake | 56.70' | 55.00' | 53.00' | 51.00' |
| S24, T18S, R23E |  |  |  |  |

Specific Authority 373.044, 373.113, 373.171 FS. Law Implemented 373.036, 373.0361, 373.0395, 373.042, 373.0421, 373.086 FS. History–New 6-7-78, Amended 1-22-79, 4-27-80, 10-21-80, 12-22-80, 3-23-81, 4-14-81, 6-4-81, 10-15-81, 11-23-81, 1-5-82, 3-11-82, 5-10-82, 7-4-82, 9-2-82, 11-8-82, 1-10-83, 4-3-83, 7-5-83, 9-5-83, 10-16-83, 12-12-83, 5-8-84, 7-8-84, 12-16-84, 2-7-85, 5-13-85, 6-26-85, 11-3-85, 3-5-86, 6-16-86, Formerly 16J-8.678, Amended 9-7-86, 2-12-87, 9-2-87, 2-18-88, 6-27-88, 2-22-89, 3-23-89, 9-26-89, 7-26-90, 10-30-90, 3-3-91, 9-30-91, 10-7-91, 7-26-92, 3-1-93, 5-11-94, 6-6-96, 2-23-97, 8-7-00, 1-8-04, 12-21-04 (13), 12-21-04 (13), 6-5-05, 5-2-06, 1-1-07, 2-12-07, 1-10-08.