Notice of Proposed Rule

DEPARTMENT OF BUSINESS AND PROFESSIONAL REGULATION
Board of Professional Surveyors and Mappers

RULE NO: RULE TITLE
61G17-6.003: General Survey, Map, and Report Content
61G17-6.004: Topographic Survey, Map, and Report
PURPOSE AND EFFECT: The Board proposes the rule amendment to review regulatory objectives and clarify minimum technical standards.
SUMMARY: The rule amendment will update regulatory objectives and clarify minimum technical standards.
SUMMARY OF STATEMENT OF ESTIMATED REGULATORY COSTS: No Statement of Estimated Regulatory Cost was prepared.
Any person who wishes to provide information regarding a statement of estimated regulatory costs, or provide a proposal for a lower cost regulatory alternative must do so in writing within 21 days of this notice.
SPECIFIC AUTHORITY: 472.008, 472.015, 472.027, 472.033(1)(h) FS.
LAW IMPLEMENTED: 472.015, 472.025, 472.027, 472.033(1)(h) FS.
IF REQUESTED WITHIN 21 DAYS OF THE DATE OF THIS NOTICE, A HEARING WILL BE SCHEDULED AND ANNOUNCED IN FAW.
THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE IS: Rick Morrison, Executive Director, Board of Professional Surveyors and Mappers, 1940 North Monroe Street, Tallahassee, Florida 32399-0750

THE FULL TEXT OF THE PROPOSED RULE IS:

61G17-6.003 General Survey, Map, and Report Content Requirement.

(1)(2)(b) Nothing in these rules shall preclude a surveyor and mapper from entering into a contract with a client which requires more stringent surveying standards than those set forth in this rule.

(2) Survey Data.

(a)(1) REGULATORY OBJECTIVE: The public must be able to rely on the accuracy of measurements and maps produced by a surveyor and mapper. In meeting this objective,

(b) Ssurveyors and mappers must achieve the following minimum standards of accuracy, completeness, and quality:

1.(a) The accuracy of the survey measurements shall be premised upon the type of survey and the expected use of the survey and map. All measurements must be in accordance with the United States standard, using either feet or meters.

2.(b) Records of these measurements shall be maintained for each survey by either the individual surveyor and mapper or the surveying and mapping business entity.

3.(c) Measurement and computation records must be dated and must contain sufficient data to substantiate the survey map and insure that the accuracy portion of these standards has been met.

(2) Other More Stringent Requirements:

(a) The Board is authorized to enforce through the disciplinary process survey standards that are more stringent than those set forth in this chapter that are required by federal, state, or local governmental agencies.

(3) Other Standards and/or Requirements that Apply to All Surveys, Maps, and/or Survey Products Content.:

(a) REGULATORY OBJECTIVE: In order to avoid misuse of a survey and map, the surveyor and mapper must adequately communicate the survey results to the public through a map, report, or report with an attached map. Any survey map or report must identify the responsible surveyor and mapper and contain standard content. In meeting this objective,

(b) Ssurveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.(b) Each survey map and report shall state the type of survey it depicts consistent with the types of surveys defined in paragraphs 61G17-6.002(10)(a)-(k), F.A.C. The purpose of a survey, as set out in paragraphs 61G17-6.002(10)(a)-(k), F.A.C., dictates the type of survey to be performed and depicted, and a licensee may not avoid the minimum standards required by rule of a particular survey type merely by changing the name of the survey type to conform with what standards or lack of them the licensee chooses to follow.

2.(c) All survey maps and reports must bear the name, certificate of authorization number, and street and mailing address of the business entity issuing the map and report, along with the name and license number of the surveyor and mapper in responsible charge. The name, license number, and street and mailing address of a surveyor and mapper practicing independent of any business entity must be shown on each survey map and report.

3.(d) All survey maps must reflect a survey date, which is the date of data acquisition. When the graphics of a map are revised, but the survey date stays the same, the map must list dates for all revisions.

4.(e) The survey map and report and the copies of the survey map and report, except those with electronic signature and electronic seal, must contain a statement indicating that the survey map and report or the copies thereof are not valid without the signature and the original raised seal of a Florida licensed surveyor and mapper.

5.(f) If either the business entity or the individual licensee does not possess professional liability insurance, then the map, report, and/or survey must contain the following printed statement in letters at least 1/4'' high: The survey depicted here is not covered by professional liability insurance.

6.(g) Additions or deletions to survey maps or reports by other than the signing party or parties is prohibited without written consent of the signing party or parties.

7.(h) All computed data or plotted features shown on survey maps must be supported by accurate survey measurements unless clearly stated otherwise.

8.(i) Bearings, distances, coordinates, and elevations shown on a survey map shall be substantiated by survey measurements unless clearly stated otherwise.

9.(j) A reference to all bearings shown on a survey map or report must be clearly stated, i.e., whether to “True North”; “Grid North as established by the NOS”; “Assumed North based on a bearing for a well defined line, such as the center line of a road or right of way, etc.”; “a Deed Call for a particular line”; or “the bearing of a particular line shown upon a plat." References to Magnetic North should be avoided except in the cases where a comparison is necessitated by a Deed Call. In all cases, the bearings used shall be referenced to some well-established and monumented line.

10.(k) A designated “north arrow” and either a stated scale or graphic scale of the map shall be prominently shown upon the survey map.

11.(l) Abbreviations generally used by the public or in proper names that do not relate to matters of survey are excluded from the legend requirement.

a. 1. Acceptable abbreviations on the face of survey maps are:

N = North

S = South

E = East

W = West

or any combination such as NE, SW, etc.

°= Degrees

' = Minutes when used in a bearing

'' = Seconds when used in a bearing

' = Feet when used in a distance

'' = Inches when used in a distance

AC = Acres

+/- = More or less (or Plus or Minus)

Metric notation

b.2. Any other abbreviations relating to survey matters must be clearly shown within a legend or notes appearing on the face of the map or report.

12.(m) When special conditions exist that effectively prevent the survey from meeting these minimum standards, the special conditions and any necessary deviation from the standards shall be noted upon the map or report.

13.(n) The responsibility for all mapped features must be clearly stated on any map or report signed by a Florida licensed surveyor and mapper. When mapped features surveyed by the signing surveyor and mapper have been integrated with mapped features surveyed by others, then the map or report shall clearly state the individual primarily responsible for the map or report.

14.(o) Report Items:

a.1. Report items are information, as required by other parts of this rule, such as: abbreviations, legends, accuracy statements, feature lists, datums used, and things done or not done as part of the survey and mapping process. In addition, the map or report shall contain other items necessary for an adequate communication of survey methods and results as judged by the surveyor and mapper such as: data sources, measurement methods, history and lineage of data, and limitations pertaining to the information presented.

b.2. Text Report items shall be displayed either through notes on the map, report, or in a text report delivered with the map. When the report is produced as a text document and a map is attached, the report shall be signed and sealed. When the map is delivered in digital form only, then a report is required. An attached map must clearly reference the report by title, date and subject; and the report must likewise clearly refer to the map by title, date, and subject. Statements must be made on the map and in the report that neither is full and complete without the other.

15.(p) Map Accuracy.

a.1. Vertical Feature Accuracy: Vertical Control: Field-measured control for elevation information shown upon survey maps or reports shall be based on a level loop or closure to a second benchmark. Closure in feet must be accurate to a standard of plus or minus .05 ft. times the square root of the distance in miles. All surveys and maps or reports with elevation data shall indicate the datum and a description of the benchmark(s) upon which the survey is based. Minor elevation data may be obtained on an assumed datum provided the base elevation of the datum is obviously different than the established datum.

b.2. Horizontal Feature Accuracy:

i.a. Horizontal Control: All surveys and maps or reports expressing or displaying features in a publicly published coordinate system shall indicate the coordinate datum and a description of the control points upon which the survey is based. Minor coordinate data may be obtained and used on an assumed datum provided the numerical basis of the datum is obviously different than a publicly published datum.

ii.b. The accuracy of control survey data shall be verified by redundant measurements or traverse closures. All control measurements shall achieve the following closures:

Commercial/High Risk Linear: 1 foot in 10,000 feet;

Suburban: Linear: 1 foot in 7,500 feet;

Rural: Linear: 1 foot in 5,000 feet;

iii.c. When statistical procedures are used to calculate survey accuracies, the maximum acceptable positional tolerance, based on the 95% confidence level, should meet the same equivalent relative distance standards as set forth in sub-subparagraphs 61G17-6.003(3)(p)2.c., F.A.C.

iv.d. Intended Display Scale: All maps or reports of surveys produced and delivered with digital coordinate files must contain a statement to the effect of: “This map is intended to be displayed at a scale of 1/__ or smaller”.

Specific Authority 472.008, 472.015, 472.027, 472.033(1)(h) FS. Law Implemented 472.015, 472.025, 472.027 FS. History–New 9-1-81, Amended 7-29-85, Formerly 21HH-6.03, Amended 12-18-88, 11-27-89, 5-26-91, Formerly 21HH-6.003, Amended 12-25-95, 5-13-96, 11-3-97, 5-25-99, 4-4-06, 8-31-06,             .

61G17-6.004 Specific Survey, Map, and Report Requirements.

(1) No change.

(2) Boundary Survey, Map, and Report:

(a) Boundaries of Real Property:

1. REGULATORY OBJECTIVE: The public must have confidence that boundaries of real property are located on the ground in an adequate and defensible manner. In meeting this objective, surveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.2. The surveyor and mapper shall make a determination of the position of the boundary of real property in complete accord with the real property description shown on or attached to the survey map or report.

3. through 9. renumbered 2. through 8. No change.

(b) Boundary Monuments:

1. REGULATORY OBJECTIVE: In order to prevent boundary conflicts, the public must have assurance that the corners of accurately located real property boundaries as found by a survey are durably marked with survey monuments that may be identified on the ground with the aid of the survey map. In meeting this objective, surveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.2. The surveyor and mapper shall set monuments as defined herein, unless monuments already exist or cannot be set due to physical obstructions at such corners or unless a water boundary has been located in approximate position. The survey map shall clearly label all approximate water boundaries with notes and these shall be mapped in a distinctly different graphic fashion from water boundaries located to full survey accuracy.

3. through 8. renumbered 2. through 7. No change.

(c) Boundary Inconsistencies:

1. REGULATORY OBJECTIVE: In order to protect and enhance stability of property location and title, the public must have assurance that potential boundary inconsistencies are adequately researched and disclosed. A survey map should present the factual basis of potential boundary inconsistencies in a clear fashion. In meeting this objective, surveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.2. Potential boundary inconsistencies that the survey process did not attempt to detect shall be clearly indicated and explained on the survey map or in the report. Where evidence of inconsistency is found, the nature of the inconsistency shall be shown upon the survey map, such as:

a. Overlapping descriptions or hiatuses;

b. Excess or deficiency;

c. Conflicting boundary lines or monuments; or

d. Doubt as to the location on the ground of survey lines or property rights.

3. through 5. renumbered 2. through 4. No change.

(d) Rights-of-Way, Easements, and Other Real Property Concerns:

1. REGULATORY OBJECTIVE: In order to provide assurance of the status of access and other real property rights, the public must be informed of the existence and location of rights-of-way and easements associated with property being surveyed. In meeting this objective, surveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.2. All recorded public and private rights-of-way shown on applicable recorded plats adjoining or across the land being surveyed shall be located and shown upon the map.

3. through 6. renumbered 2. through 5. No change.

(e) Real Property Improvements:

1. REGULATORY OBJECTIVE: The public must be informed of the existence and location of pertinent real property improvements and their relation to the boundary of real property. In meeting this objective, surveyors and mappers must meet the following minimum standards of accuracy, completeness, and quality:

1.2. Location of fixed improvements pertinent to the survey shall be graphically shown upon the map and their positions shall be dimensioned in reference to the boundaries, either directly or by offset lines.

3. through 5. renumbered  2. through 4. No change.

(3) through (8) No change.

(9) Raster Imagery:

(a) REGULATORY OBJECTIVE: The public must be able to rely on surveys and maps presented in image form, digital or graphical, where coordinate positions of mapped features on a recognized coordinate system may be extracted from the image.

(a)(b) The survey and report must contain a list of control points employed in geo-referencing the image along with the source of control positions used. The survey and report must contain a statement clearly stating that “This is not an ortho-image or ortho-photo.”

(b)(c) Feature accuracies shall be stated.

(10) through (11) No change.

(12) Topographic Survey:

(a) Topographic surveying and mapping by field methods shall meet general provisions applicable to all surveys and maps as set out in Rule 61G17-6.003, F.A.C. A minimum of two site benchmarks on or near the survey shall be indicated upon the survey map.

(b) Topographic Features.

1. REGULATORY OBJECTIVE: The public must be able to rely on topographic information contained on a survey map and must be able to correctly interpret the intended map coverage.

1.2. Intended Features. The surveyor and mapper shall devise a method of reporting which topographic features were intended to be surveyed and mapped, the style of cartographic representation employed for each, and the degree of intended completeness in the surveying and mapping of each feature. As with abbreviations, any symbols, line types, etc. shown on the survey map shall be explained and/or defined in a legend.

3. through 5. renumbered 2. through 4. No change.

Specific Authority 472.008, 472.027, 472.033(1)(h) FS. Law Implemented 472.027, 472.033(1)(h) FS. History–New 9-1-81, Formerly 21HH-6.04, Amended 12-18-88, Formerly 21HH-6.004, Amended 12-25-95, 5-13-96, 5-25-99, 4-4-06, 8-31-06,________.

NAME OF PERSON ORIGINATING PROPOSED RULE: Board of Professional Surveyors and Mappers
NAME OF SUPERVISOR OR PERSON WHO APPROVED THE PROPOSED RULE: Board of Professional Surveyors and Mappers
DATE PROPOSED RULE APPROVED BY AGENCY HEAD: April 9, 2008
DATE NOTICE OF PROPOSED RULE DEVELOPMENT PUBLISHED IN FAW: October 26, 2007