- 40D-40.040 General Environmental Resource Permits.

(1) Three types of General Environmental Resource Permits are issued pursuant to this chapter and Chapter 40D-4, F.A.C. They are:

(a) General Environmental Resource Permit for Minor Surface Water Management Systems. The conditions for issuance for this permit are contained within Rule 40D-40.301, F.A.C.;

(b) General Environmental Resource Permit for Surface Water Management Systems. The conditions for issuance of this permit are contained within Rule 40D-40.302, F.A.C.; and

(c) General Environmental Resource Permit for Site Conditions Assessment. The conditions for issuance of this permit are contained within Rule 40D-40.302, F.A.C.

(2) General Environmental Resource Construction and Operation Permits are required prior to the construction, alteration, removal, maintenance, operation or abandonment of certain surface water management systems.

(3) Site Conditions Assessment Permits are optional general permits that are issued as the first phase of construction permitting which identify and document the boundaries of certain existing site conditions found within a project area. At a minimum, all site conditions assessment permits shall evaluate, and identify if present, the landward extent of wetlands and other surface waters; the level and landward extent of the 100 year floodplain; seasonal high water levels; and existing watershed delineations. Additional site conditions boundaries may be requested by the applicant and evaluated for permitting by the District. A site conditions assessment permit does not authorize construction, alteration, operation, or abandonment of a surface water management system or establishment of a mitigation bank, but it can be formally modified by the permittee to apply for either an individual or a general construction and operation permit.

(4) An activity which requires both a general environmental resource permit or a permit under Sections 373.414(11)-(16), F.S., and a proprietary authorization under Chapter 253 or 258, F.S., shall be subject to the requirements and procedures in Section 373.427, F.S., Chapters 18-20 and 18-21 and Rules 62-312.065, 62-343.075 and 40D-1.603, F.A.C.

Rulemaking Authority 373.044, 373.113, 373.118, 373.421(2) FS. Law Implemented 373.413, 373.414, 373.416, 373.419, 373.427 FS. History–New 10-3-95, Amended 7-23-96, 10-16-96, 7-2-98, 9-26-02, 9-10-08.