62-204.200 Definitions.

The following words and phrases when used in this chapter, unless content clearly indicates otherwise, have the following meanings:

(1) “Actual Emissions” – The actual rate of emission of a pollutant from an emissions unit as determined in accordance with the following provisions.

(a) In general, actual emissions as of a particular date shall equal the average rate, in tons per year, at which the emissions unit actually emitted the pollutant during a consecutive 24-month period which precedes the particular date and which is representative of the normal operation of the emissions unit. The Department shall allow the use of a different time period upon a determination that it is more representative of the normal operation of the emissions unit. Actual emissions shall be calculated using the emissions unit’s actual operating hours, production rates and types of materials processed, stored, or combusted during the selected time period.

(b) The Department may presume that unit-specific allowable emissions for an emissions unit are equivalent to the actual emissions of the emissions unit provided that such unit-specific allowable emissions limits are federally enforceable.

(c) For any emissions unit that has not begun normal operations on a particular date, actual emissions shall equal the potential emissions of the emissions unit on that date.

(2) “Administrator” – The Administrator of the United States Environmental Protection Agency or the Administrator’s designee.

(3) “Air Pollutant” – Any substance (particulate, liquid, gaseous, organic or inorganic) which if released, allowed to escape, or emitted, whether intentionally or unintentionally, into the outdoor atmosphere may result in or contribute to air pollution.

(4) “Air Pollution” – The presence in the outdoor atmosphere of the state of any one or more substances or pollutants in quantities which are or may be harmful or injurious to human health or welfare, animal or plant life, or property, or unreasonably interfere with the enjoyment of life or property, including outdoor recreation.

(5) “Air Quality Control Region” – Any air quality control region designated pursuant to Section 107 of the Clean Air Act. The boundaries of the air quality control regions in Florida are set forth in 40 C.F.R. Part 81, Sections 81.49, 81.68, 81.91, 81.95, 81.96 and 81.97, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

(6) “Ambient Air Quality Standard” or “Ambient Standard” – A restriction established to limit the quantity or concentration of an air pollutant that may be allowed to exist in the ambient air for any specific period of time.

(a) “National Ambient Air Quality Standard” means an ambient standard established by EPA and specified at 40 C.F.R. Part 50, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

(b) “Primary Standard” means an ambient standard established to protect public health.

(c) “Secondary Standard” means an ambient standard established to protect the public welfare including the protection of animal and plant life, property, visibility and atmospheric clarity, and the enjoyment of life and property.

(d) “State Ambient Air Quality Standard” means an ambient standard established or adopted by the Department.

(7) “Baseline Area” – The area (and every part thereof) designated as a prevention of significant deterioration (PSD) area under Rule 62-204.360, F.A.C., in which the facility or major modification establishing the minor source baseline date would construct or in which the emissions of the facility (or the significant net increase in emissions for a major modification) would have a predicted air quality impact equal to or greater than one microgram per cubic meter (annual average) of the pollutant for which the minor source baseline date is established. For purposes of this definition, “major modification” has the meaning given at Rule 62-210.200, F.A.C.

(8) “Baseline Concentration” – The ambient concentration level that exists in the baseline area at the time of the applicable minor source baseline date. A baseline concentration is determined for each pollutant for which a minor source baseline date is established and for each averaging time for which a maximum allowable increase is established in Rule 62-204.260, F.A.C.

(a) The baseline concentration shall include the concentration attributable to:

1. The actual emissions representative of sources in existence on the applicable minor source baseline date, except as provided at paragraph 62-204.200(8)(b), F.A.C.; and

2. The federally enforceable allowable emissions of major stationary sources on which construction commenced on or before the major source baseline date but which were not in operation by the applicable minor source baseline date.

(b) The baseline concentration shall not include the concentration attributable to the following emissions; rather, such emissions shall affect the amount of any applicable maximum allowable increase remaining available:

1. The actual emissions from any major stationary source on which construction commenced after the major source baseline date; and

2. Any increase or decrease in the actual emissions of facilities occurring after the applicable minor source baseline date.

(c) For purposes of this definition, “construction” means any physical change or change in the method of operation (including fabrication, erection, installation, demolition, shutdown or modification of an emissions unit) that would result in a change in emissions; and “commence construction” has the meaning given at Rule 62-210.200, F.A.C., provided, however, that in the case of demolition or shutdown of an emissions unit, “commence construction” means that the owner or operator has permanently ceased all operations of the unit.

(d) Notwithstanding the provisions of paragraph (b) above:

1. The change in concentration attributable to any decrease in the actual emissions of a facility on which the Department has relied in demonstrating attainment, defining reasonable further progress, or issuing a permit under the provisions of Rule 17-2.17 (repealed), 17-2.510 (transferred), 17-2.650 (transferred), 62-212.500, 62-296.500 through 62-296.570, or 62-296.700 through 62-296.712, F.A.C., shall be included in the baseline concentration and not be considered in determining the amount of any maximum allowable increase remaining available; and

2. Concentrations of particulate matter attributable to the increase in emissions from construction or other temporary emission-related activities of new or modified facilities shall be excluded in determining compliance with any maximum allowable increase.

(9) “Cause or Contribute” – With respect to a violation of an ambient air quality standard, to have a significant impact on the ambient air concentration of a pollutant at any locality that does not or would not meet the applicable standard.

(10) “Clean Air Act (CAA)” or “Act” – The Federal Clean Air Act (42 U.S.C. s. 7401 et seq.)

(11) “Department” – The State of Florida Department of Environmental Protection.

(12) “Emission” – The discharge or release into the atmosphere of one or more air pollutants.

(13) “Emission Limiting Standard” or “Emission Standard” or “Emission Limitation” or “Performance Standard” – Any restriction established in or pursuant to a regulation adopted by the Department which limits the quantity, rate, concentration or opacity of any pollutant released, allowed to escape or emitted, whether intentionally or unintentionally, into the atmosphere, including any restriction which prescribes equipment, sets fuel specifications, or prescribes operation or maintenance procedures for an emissions unit to assure emission reduction or control.

(14) “Emissions Unit” – Any part or activity of a facility that emits or has the potential to emit any air pollutant.

(15) “Environmental Protection Agency” or “EPA” – The United States Environmental Protection Agency.

(16) “Facility” – All of the emissions units which are located on one or more contiguous or adjacent properties and which are under the control of the same person (or persons under common control).

(17) “Federal Land Manager” – With respect to any lands in the United States, the Secretary of the department with authority over such lands.

(18) “Indian Governing Body” – The governing body of any tribe, band, or group of Indians subject to the jurisdiction of the United States and recognized by the United States as possessing power of self-government.

(19) “Indian Reservation” – Any federally recognized reservation established by Treaty, Agreement, Executive Order, or Act of Congress.

(20) “Major Source Baseline Date” – Pursuant to 40 C.F.R. 51.166(b)(14)(i), adopted and incorporated by reference in Rule 62-204.800, F.A.C.;

(a) In the case of particulate matter and sulfur dioxide, January 6, 1975; and

(b) In the case of nitrogen dioxide, February 8, 1988.

(21) “Marginal Nonattainment Area for Ozone” – The lowest category of five classifications of nonattainment for the air pollutant ozone as defined in the Clean Air Act (42 U.S.C. s. 7511).

(22) “Minor Source Baseline Date” – Pursuant to 40 C.F.R. 51.166(b)(14)(ii), adopted and incorporated by reference in Rule 62-204.800, F.A.C., the minor source baseline date for each pollutant for which maximum allowable increases have been established under Rule 62-204.260, F.A.C., is the earliest date after August 7, 1977, for particulate matter and sulfur dioxide, and February 8, 1988, for nitrogen dioxide, that a facility or a modification subject to preconstruction review under 40 C.F.R. 52.21, Rule 17-2.500, F.A.C. (transferred), or Rule 62-212.400, F.A.C., submits a complete application for permit under such regulations provided that:

(a) On the date the complete application is filed, the area in which the facility or modification would be constructed is designated as attainment or unclassifiable for the applicable pollutant under 42 U.S.C. Section 7407(d)(1) of the Clean Air Act (if the application is filed under 40 C.F.R. 52.21), or as a PSD area under Rule 17-2.450 (transferred), 62-275.700 (repealed) or 62-204.360, F.A.C., (if the application is filed under Rule 17-2.500 (transferred) or 62-212.400, F.A.C.); and

(b) In the case of a facility, the emissions of the applicable pollutant would be equal to or greater than the significant emissions rate as defined at Rule 62-210.200, F.A.C., or, in the case of modification, there would be a significant net emissions increase of the pollutant.

(23) “Moderate Nonattainment Area for Ozone” – The second-lowest category of five classifications of nonattainment for the air pollutant ozone as defined in the Clean Air Act (42 U.S.C. s. 7511).

(24) “Modification” – Any physical change in, change in the method of operation of, or addition to a facility which would result in an increase in the actual emissions of any air pollutant subject to regulation under the Act, including any not previously emitted, from any emissions unit or facility.

(a) A physical change or change in the method of operation shall not include:

1. Routine maintenance, repair, or replacement of component parts of an emissions unit; or

2. A change in ownership of an emissions unit or facility.

(b) For any pollutant that is specifically regulated by the EPA under the Clean Air Act, a change in the method of operation shall not include an increase in the hours of operation or in the production rate, unless such change would be prohibited under any federally enforceable permit condition which was established after January 6, 1975.

(c) For any pollutant that is not specifically regulated by the EPA under the Clean Air Act, a change in the method of operation shall not include an increase in the hours of operation or in the production rate, unless such change would exceed any restriction on hours of operation or production rate included in any applicable Department air construction or air operation permit.

(25) “Nonattainment Area” – Any area not meeting ambient air quality standards and designated as a nonattainment area under Rule 62-204.340, F.A.C. Such an area may be designated as a particulate, sulfur dioxide, nitrogen dioxide, carbon monoxide, lead or ozone nonattainment area, depending on which ambient standard has been violated. An area may be designated as nonattainment for more than one air pollutant. Ozone nonattainment areas may be transitional, marginal, moderate, serious, severe, or extreme as classified in Rule 62-204.340, F.A.C.

(26) “Particulate Matter”.

(a) With respect to concentrations in the atmosphere, particulate matter means any airborne finely divided solid or liquid material.

(b) With respect to emissions, particulate matter means all finely divided solid or liquid material, other than uncombined water, emitted to the atmosphere as measured by applicable reference methods, or an equivalent or alternative method, specified in 40 C.F.R. Part 60, Appendix A, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

(27) “PM10”.

(a) With respect to concentrations in the atmosphere, PM10 means particulate matter with an aerodynamic diameter less than or equal to a nominal ten (10) micrometers as measured by a reference method based on 40 C.F.R. Part 50, Appendix J, adopted and incorporated by reference in Rule 62-204.800, F.A.C., and designated in accordance with 40 C.F.R. Part 53 or by an equivalent method designated in accordance with 40 C.F.R. Part 53, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

(b) With respect to emissions, PM10 means finely divided solid or liquid material, with an aerodynamic diameter less than or equal to a nominal ten (10) micrometers emitted to the atmosphere as measured by an applicable reference method or by an equivalent or alternative method specified in 40 C.F.R. Part 51, Appendix M, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

(28) “Redesignation of an Area” – A change in the designation or a redefinition of the boundaries of an area for any of the designations listed under Rule 62-204.340 or 62-204.360, F.A.C.

(29) “Significant Impact” – An impact of emissions on ambient air quality in excess of any of the following pollutant-specific concentration values:

(a) Sulfur Dioxide.

1. Maximum three-hour concentration not to be exceeded more than once per year – 25.0 micrograms per cubic meter.

2. Maximum 24-hour concentration not to be exceeded more than once per year – 1.0 microgram per cubic meter for Class I areas; 5.0 micrograms per cubic meter for all other areas.

3. Annual arithmetic mean – 1.0 microgram per cubic meter.

(b) PM10.

1. Maximum 24-hour concentration not to be exceeded more than once per year – 1.0 microgram per cubic meter for Class I areas; 5.0 micrograms per cubic meter for all other areas.

2. Annual arithmetic mean – 1.0 microgram per cubic meter.

(c) Nitrogen Dioxide. Annual arithmetic mean – 1.0 microgram per cubic meter.

(d) Carbon Monoxide.

1. Maximum one-hour concentration not to be exceeded more than once per year – 2.0 milligrams per cubic meter.

2. Maximum eight-hour concentration not to be exceeded more than once per year – 0.5 milligram per cubic meter.

(e) Lead. Maximum quarterly arithmetic mean – 0.03 microgram per cubic meter.

(30) “State Implementation Plan (SIP)” or “Implementation Plan” – The EPA approved plan which Section 110 of the Clean Air Act requires a state to submit to the Administrator. The State Implementation Plan for the State of Florida, as approved by the U.S. Environmental Protection Agency, is identified in 40 C.F.R. Part 52, Subpart K, adopted and incorporated by reference in Rule 62-204.800, F.A.C.

Specific Authority 403.061, 403.8055 FS. Law Implemented 403.031, 403.061, 403.8055 FS. History–New 11-30-94, Amended 3-13-96, 2-12-06, 10-6-08.