# 62-304.300 St. Marks River Basin TMDLs.

# Munson Slough TMDLs. Munson Slough TMDL for Fecal Coliform. The Total Maximum Daily Load for Munson Slough is 400 counts/100mL for fecal coliform, and is allocated as follows:

(1) The Wasteload Allocation (WLA) for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2006 period, will require a 31.6 percent reduction at sources contributing to exceedances of the criteria at Roberts Ave., and for the 2006 period, will require a 96.9 percent reduction at sources contributing to exceedances of the criteria at Springhill Road, and for the 1992 to 2007 period, will require a 91.5 percent reduction at sources contributing to exceedances of the criteria at Capital Circle S. W.

(2) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2006 period, will require a 31.6 percent reduction at sources contributing to exceedances of the criteria at Roberts Ave., and for the 2006 period, will require a 96.9 percent reduction at sources contributing to exceedances of the criteria at Springhill Road, and for the 1992 to 2007 period, will require a 91.5 percent reduction at sources contributing to exceedances of the criteria at Capital Circle S. W.

(3)The Margin of Safety is implicit.

(4) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Specific Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 10-21-08.