64J-1.006 Neonatal Interfacility Transfers.

(1) Neonates requiring critical care interfacility transport to a Level II or Level III Neonatal Intensive Care Unit shall be transported in either a neonatal ambulance or a permitted ALS or BLS transport ambulance or aircraft.

(a) A neonatal ambulance shall meet the requirements listed in Table V, paragraphs 64J-1.006(1)(c) and (d) and subsections 64J-1.006(2) and (3), F.A.C., and shall be exempt from meeting the equipment and medical supplies listed in Rule 64J-1.002, Table I, F.A.C., and in Rule 64J-1.003, Table II, F.A.C.

(b) When a permitted BLS or ALS ambulance is used to transport a neonate, the sending neonatologist or physician and EMS provider’s medical director shall ensure that the level of care, staffing, and equipment is commensurate to the needs of the neonate being transported.

(c) The neonatal ambulance as defined in subsection 64J-1.001(14), F.A.C., shall have exterior wording or marking which identifies that the ambulance is only for neonatal transport. The wording shall be such that the public cannot mistake a neonatal vehicle as an ambulance for general patient care.

(d) Any EMS provider operating a permitted neonatal ambulance for transporting neonates to a Level II or Level III Neonatal Intensive Care Unit shall contract with a neonatologist or have the hospital’s staff neonatologist assigned as the provider’s medical director.

| TABLE V (Reference Section 64J-1.006, F.A.C.) Neonatal Interfacility Transfers |  |  |  |
| --- | --- | --- | --- |
| ITEM |  |  | QTY. |
| 1. Direct two-way communications with the |  |  |  |
| designated neonatologist or attending |  |  |  |
| physician and or receiving ICU. |  |  |  |
| 2. A standby or backup power source |  | One. |  |
| other than the one contained in |  |  |  |
| the isolette. |  |  |  |
| 3. A source of electrical power |  | One. |  |
| sufficient to operate the isolette |  |  |  |
| and ancillary electrically powered |  |  |  |
| equipment. |  |  |  |
| 4. A transport incubator with portable |  | One. |  |
| power supply, portable oxygen tanks |  |  |  |
| or liquid oxygen, and a source of |  |  |  |
| compressed air, including appropriate |  |  |  |
| valves, meters, and fittings. |  |  |  |
| 5. Portable heart rate monitor with |  | One per patient. |  |
| visual or audible display and alarm |  |  |  |
| system. |  |  |  |
| 6. Portable blood pressure monitor |  | One each. |  |
| with assortment of cuff sizes suitable |  |  |  |
| for infants. |  |  |  |
| 7. Battery powered mechanical I.V. pumps |  | Two. |  |
| capable of delivering as low as 1 cc. |  |  |  |
| increments for I.V. fluids. |  |  |  |
| 8. Battery or self-powered oxygen sensor |  | One. |  |
| and transcutaneous oxygen monitor or |  |  |  |
| oxygen saturation monitor. |  |  |  |
| 9. Oxygen delivery device and tubing |  | One. |  |
| capable of administering high |  |  |  |
| concentrations of oxygen. |  |  |  |
| 10. Temperature monitoring device. |  |  | One. |
| 11. Portable ventilator appropriate |  |  | One. |
| for neonatal patients. |  |  |  |
| 12. Anesthesia and/or self-inflating bag |  |  |  |
| with oxygen reservoir less than |  |  |  |
| 750 ml and manometer (pressure gauge); |  |  |  |
| premature, newborn and infant size |  |  |  |
| clear masks. |  |  |  |
| 13. Laryngoscope handle. |  |  | One. |
| 14. Blades. |  |  | Miller 00, Miller 0. |
| 15. Bulbs and batteries. |  |  | Two each. |
| 16. Endotracheal tubes. |  |  | 2.0, 2.5, 3.0, 3.5, |
|  |  |  | 4.0. |
| 17. Stylet. |  |  | Two each. |
| 18. Adapters. |  |  | Assortment of sizes. |
| 19. Oral Airways. |  |  | Assortment of sizes. |
| 20. Suction equipment with low suction |  |  | One. |
| capabilities of less than 80 mm of hg. |  |  |  |
| 21. Sterile Gloves assorted sizes. |  |  | Sufficient quantity |
| for all crew |  |  |  |
| members. |  |  |  |
| 22. Suction catheters. |  | Size 5.0, 6.0, | Two each. |
|  |  |  | 8, & 10. |
| 23. Syringes sizes 1 cc. through 60 cc. |  |  | Assortment of sizes. |
| 24. Medication access device. |  |  | Two each. |
| 25. Vascular access devices 23-27 gauge. |  |  | Assortment of sizes. |
| 26. I.V. extension tubing. |  |  | Sufficient length to |
|  |  |  | administer I.V. |
| 27. Securing device. |  |  | Assorted sizes. |
| 28. I.V. filters. |  |  | Two. |
| 29. Umbilical catheters. |  | Size 3.5 & 5 | Two. |
| 30. Antiseptic solution. |  |  | Ten. |
| 31. Blood sugar device. |  |  | One. |
| 32. Lancets. |  |  | Five. |
| 33. Neonatal stethoscope. |  |  | One. |
| 34. Flashlight. |  |  | One. |
| 35. Gauze pads. |  |  | Assortment of sizes. |
| 36. No. 5 & No. 8 French feeding tubes. |  |  | One each. |
| 37. High intensity light capable of |  |  | One. |
| transillumination. |  |  |  |
| 38. Approved biomedical waste plastic |  |  | One each. |
| bag or impervious container and |  |  |  |
| used sharps container per Chapter |  |  |  |
| 64E-16, F.A.C. |  |  |  |
| 39. Gloves – latex or other suitable |  |  | Sufficient quantity |
| materials. |  |  | for all crew members. |
| 40. Respiratory face masks. |  |  | Sufficient quantity |
|  |  |  | for all crew members. |
| 41. Special procedure tray or instruments |  |  | One. |
| with capability for performing |  |  |  |
| umbilical catheterization, venous |  |  |  |
| cutdown and thoracostomy. |  |  |  |
| 42. Bulb syringe. (Additional to OB kit) |  |  | One. |
| 43. Cord clamp. |  |  | One. |
| 44. Chest tube evacuation device. |  |  | One. |
| 45. Needle aspiration device or |  |  | Appropriate sizes |
| chest tubes. |  |  | for neonate. |
| MEDICATION |  | WT/VOL | QTY. |
| 1. Atropine Sulfate. |  | 1 mg./10 ml. | One. |
| 2. Injectable Vitamin K. |  | 1 mg./0.5 ml. | One. |
| 3. Antibiotics, to be |  |  |  |
| determined by medical |  |  |  |
| director. |  |  |  |
| 4. Calcium Gluconate. |  | 10% - 10- ml. | One. |
| 5. Digoxin ped. |  | 0.1 mg./ml. | One. |
| 6. Anticonvulsant as |  |  |  |
| required by medical |  |  |  |
| director. |  |  |  |
| 7. Dextrose. |  | 50% 50 cc. | One. |
| 8. Dopamine or |  | Depends on | One. |
| dobutamine. |  | medication |  |
| 9. Epinephrine. |  | 1:10,000 | One. |
| 10. Eye prophylaxis. |  |  | One. |
| 11. Furosemide (Lasix). |  | 20 mg./2 ml. | One. |
| 12. Heparin. |  |  | One. |
| 13. Lidocaine. |  | 1%/2 mg. | One. |
| 14. Naloxone (Narcan). |  | 1.0 mg./ml or | One. |
|  |  | .4 mg./ml. |  |
| 15. Paralyzing agent. |  |  | One. |
| 16. Phenobarbital. |  |  | One. |
| 17. Prostin VR. |  | 500 mcg/ml. | One. |
| (available for |  |  |  |
| transport) |  |  |  |
| 18. Sodium Bicarbonate. |  | 4.2% soln. | One. |
| 19. Sedative as |  |  | One. |
| determined by the |  |  |  |
| medical director. |  |  |  |
| 20. Volume expander. |  |  | One. |
| 21. I.V. fluid. |  | Bags of | One each. |
|  |  | D5W and D10W |  |
| 22. Injectable |  |  | One. |
| non-preservative |  |  |  |
| sterile water. |  |  |  |
| 23. Injectable |  |  | One. |
| non-preservative normal saline. |  |  |  |

(2) Each permitted ambulance or neonate ambulance when transporting a neonate to a Level II or Level III Neonatal Intensive Care Unit shall be staffed with a minimum of two persons. One person shall be a registered neonatal nurse, the second person shall be either a neonatal registered respiratory therapist (RT), or a paramedic or a registered neonatal nurse. The staffing for each neonate transport shall be determined by the licensee’s medical director in conjunction with the attending physician and the neonatologist. A physician can be substituted for any team member.

(a) The provider shall assure the RN is licensed in accordance with Chapter 464, F.S.; have a minimum of 4,000 hours RN experience, which includes 2,000 hours of Level II or Level III Neonatal Intensive Care Unit (NICU) nursing experience; and have American Heart Association (AHA) Neonatal Resuscitation Program (NRP) Certification or an equivalent certification and successfully complete a neonatal transport stabilization program within 2 years prior to application to neonatal transport, approved in writing by a medical director; and, accompany a minimum of six neonatal transports prior to staffing a neonatal transport as the only RN in attendance.

(b) The provider shall assure the RT is registered by the National Board of Respiratory Care with a minimum of 2,000 hours of Level II or Level III NICU experience or be certified as a RT with a minimum of 3,000 hours of Level II or Level III NICU experience; and have AHA NRP Certification or an equivalent certification and successfully complete a neonatal transport stabilization program within 2 years prior to application to neonatal transport, approved in writing by a medical director; and, accompany a minimum of six neonatal transports prior to staffing a transport as the only RT in attendance.

(c) The provider shall assure the paramedic is a Florida licensed paramedic with a minimum of 2,000 hours of Level II or Level III Neonatal Intensive Care unit experience; or be a Florida paramedic with a minimum of 3,000 hours experience; and have AHA NRP Certification or an equivalent certification and successfully complete a neonatal transport stabilization program within 2 years prior to application to neonatal transport, approved in writing by a medical director; and, accompany a minimum of six neonatal transports prior to staffing a neonatal transport.

(d) A neonatologist or a licensee’s medical director may make medical staff substitutions with individuals of comparable skills when the condition of the neonate warrants such substitution.

(3) Treatment protocols for the management of the neonatal patient from the responsible neonatologist shall accompany each neonatal transport.

Specific Authority 381.0011, 383.19, 395.405, 401.251(6), 401.35 FS. Law Implemented 381.001, 383.15, 395.405, 401.24, 401.25, 401.251, 401.252, 401.26, 401.265, 401.27, 401.30, 401.31, 401.35, 401.41, 401.411, 401.414, 401.421 FS. History–New 11-30-93, Amended 1-26-97, Formerly 10D-66.0525, Amended 8-4-98, 9-3-00, 12-18-06, Formerly 64E-2.006.