61G15-33.007 Design of Lightning Protection Systems.

(1) Lightning Protection Systems are passive systems used to protect building and structures from damage caused by lightning and static discharges. Items to be considered in the design or analysis of this system include the requirements of NFPA-780.

(2) Electrical Engineering documents for lightning protection systems shall indicate:

(a) Lightning Risk Assessment.

(b) Air terminals height and spacing.

(c) Corrosion protection measures.

(d) Arrangement of Main and Down conductors.

(e) Grounding points and spacing.

(f) Conductor type and size.

(g) Legend.

(h) Testing requirements of grounds.

Specific Authority 471.008, 471.033(2) FS. Law Implemented 471.033 FS. History–New 5-19-93, Formerly 21H-33.007, Amended 11-13-08.