61G15-34.007 Design of Plumbing Systems.

(1) Plumbing systems are those systems within a building that convey fluids and gases generally as required by building codes.

(2) Mechanical Engineering Documents applicable to Plumbing Systems shall when applicable, include but are not limited to the following:

(a) Equipment schedules for all plumbing fixtures, water heaters, boilers, pumps, grease traps, septic tanks, storage tanks, expansion tanks, compression tanks and roof and floor drains.

(b) Floor plans, site plans, and building and plumbing system elevations are appropriate.

(c) Potable Water isometric diagrams with pipe sizes and total water fixture units.

(d) Sanitary riser diagrams with pipe sizes and total sanitary waste fixture units.

(e) Storm riser diagrams with pipe sizes and cumulative drain area square footages.

(f) Cold water, hot water, sanitary, and storm drainage piping layouts.

(g) System isometrics and flow diagrams of other fluids and gases.

(h) Design data for septic tank, grease trap(s), drain field sizing, when applicable.

(i) List of ASHRAE, ASME, ASPE, ANSI and other applicable codes, design standards and requirements.

(j) Design shall be in accordance with handicap requirements adopted by the authority having jurisdiction.

(k) Instrumentation and Control Diagrams and sequence of operation.

(l) All plumbing fixtures, valves, pumps, tanks, accessories, specialties, enclosures, and such equipment shall be described and located on the drawings.

(m) Materials for all plumbing systems shall be specified.

Specific Authority 471.008, 471.033(2) FS. Law Implemented 471.033 FS. History–New 11-16-94, Amended 11-13-08.