62-160.240 Record Keeping and Reporting Requirements for Field Procedures.

(1) The record keeping requirements for entities that conduct or support field activities and field measurements are specified in DEP-SOP-001/01 FD 1000 (March 31, 2008). The specified records shall contain sufficient information to allow independent reconstruction of all activities related to generating data that are submitted to the Department. These records shall be kept by the generator of the records for a minimum of five years after the date of project completion or permit cycle unless otherwise specified in a Department contract, order, permit or Title 62 rules.

(2) When requested by the Department, the following field sampling information shall be provided to the Department for each site/facility and sampling location, as applicable:

(a) Project information including:

1. Project and/or program identification or name; and

2. Site and/or facility name, address and phone number.

(b) Site and/or facility locational information to include:

1. Latitude measure in degrees-minutes-seconds (seconds may contain up to four decimal places);

2. Longitude measure in degrees-minutes-seconds (seconds may contain up to four decimal places);

3. Datum – the horizontal reference for measuring locations on the Earth’s surface;

4. Spheroid – the ellipsoid used as a model for the surface of the Earth; and

5. Geolocational collection information:

a. Collection method – the method or mechanism used to derive the measurements;

b. Collector name – name of individual who collected the locational data;

c. Collector affiliation – collector’s agency or entity affiliation;

d. Collection date – date locational data were collected;

e. Relationship of point to feature – the type of the feature for which the measurement is being made;

f. Coordinate accuracy level – the measured, estimated or deduced degree of correctness of the measurement; and

g. Verification information including name of the person verifying the measurement, the date and the time when verification was performed.

(c) Information about the collected samples:

1. Name(s) and affiliation of individual(s) collecting samples;

2. Sampling method(s) used;

3. Sample description such as sample type, sample matrix, and sample treatments (preservation, filtration, etc.);

4. Client or field identification number for each sample;

5. Date and time of sample collection, including date and time sample collection ended (if collecting a composite sample);

6. Sample collection depth;

7. Unambiguous identification of all field-generated quality control samples such as field or equipment blanks, replicate samples or split samples; and

8. Any additional information from the field documentation records specified in DEP-SOP-001/01 (March 31, 2008).

(d) Information about field measurement activities:

1. Method(s) used to make field measurement;

2. Name of field parameter;

3. Result, result units and associated data qualifier code(s); and

4. Any additional information from the field documentation records specified in DEP-SOP-001/01 (March 31, 2008).

(e) Information about site conditions:

1. Weather;

2. Flow (including units); and

3. Any additional information from the field documentation records specified in DEP-SOP-001/001/01 (March 31, 2008).

(f) Any additional information specified by the Department in contracts, orders, permits or Title 62 rules.

(3) Field sampling data issued to a client(s) for Department-related work or directly to the Department shall be provided to the Department in an electronic format consistent with requirements for importing into Department databases, as specified by the Department in applicable contracts, orders, permits or Title 62 rules. In addition, certain Department programs specify the submission of paper reports. Field sampling information may be incorporated into laboratory reports specified in Rule 62-160.340, F.A.C. Specific electronic and paper report format requirements shall be as specified by the Department in the applicable contract, order, permit or Title 62 rule.

Specific Authority 403.061, 403.0623 FS. Law Implemented 373.026, 373.309, 373.409, 373.413, 373.414, 373.416, 373.4592, 376.303, 376.305, 376.3071, 403.0623, 403.0625, 403.087, 403.088, 403.0881, 403.504, 403.704, 403.707, 403.722, 403.853 FS. History–New 4-9-02, Amended 6-8-04, 12-3-08.