- 62-304.410 Santa Fe Basin TMDLs.

Santa Fe River Planning Unit.

(1) Santa Fe River TMDLs. Santa Fe River TMDL for nutrient and dissolved oxygen impairments: The Total Maximum Daily Load for nutrients in the Santa Fe River (below river rise) is to achieve a monthly average of 0.35 mg/L nitrate-N, and is allocated as follows:

(a) The Wasteload Allocation (WLA) for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program are to meet a monthly average in-stream ambient water quality target of 0.35 mg/L nitrate-N. The range of percent reduction necessary to achieve the LA is estimated between 13 and 35 percent depending on the month and location within the basin. Achievement of the TMDL constitutes meeting the water quality target;

(c) The Load Allocations (LA) for nonpoint sources are to meet a monthly average of 0.35 mg/L nitrate-N. The range of percent reduction necessary to achieve the LA is estimated between 13 and 35 percent depending on the month and location within the basin. Achievement of the TMDL constitutes meeting the water quality target; and

(d) The Margin of Safety is implicit.

(2) New River TMDLs.

(a) New River TMDL for fecal coliform impairment. The Total Maximum Daily Load for New River 400 counts/100mL for fecal coliform, and is allocated as follows:

1. The WLA for wastewater point sources is not applicable;

2. The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable;

3. The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1995 to 2007 period, will require a 45 percent reduction at sources contributing to exceedances of the criteria; and

4. The Margin of Safety is implicit.

5. While the LA for fecal coliform has been expressed as the percent reductions needed to attain the applicable Class III criteria, it is not the intent of the TMDL to abate natural background conditions.

(b) New River TMDL for dissolved oxygen impairment. The Total Maximum Daily Load for the New River is allocated as follows:

1. The WLA for wastewater point sources is not applicable;

2. The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable;

3. The LA for nonpoint sources is to address anthropogenic sources in the basin such that there is a 38 percent reduction of current anthropogenic total nitrogen (TN) loading to the upper portion of the New River, a 13 percent reduction of current anthropogenic total nitrogen (TN) loading to the lower portion of the New River, and a 38 percent reduction of current anthropogenic total phosphorus (TP) loading to the lower portion of the New River, based on measured concentrations from the 1995 to 2006 period.

(3) Alligator Lake TMDLs. Alligator Lake TMDL for nutrient and dissolved oxygen impairments. The TMDL for nutrients in Alligator Lake is 42,595 pounds/year of total nitrogen (a 28.4 percent reduction) and 3,050 pounds/year of total phosphorus (a 61.2 percent reduction) and is allocated as follows:

(a) The WLA for wastewater sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is not applicable,

(c) The LAs for nonpoint sources are 42,595 pounds/year of total nitrogen and 3,050 pounds/year of total phosphorus, and

(d) The Margin of safety is implicit.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 11-16-08, Amended 12-7-08.