- 14-86.003 Permit, Assurance Requirements, and Exceptions.

(1) Permit.

(a) No permits are required for properties without improvements on or after November 12, 1986.

(b) All improvements on or after November 12, 1986, require a Drainage Connection Permit, Form 850-040-06 (10/08), whether or not the work is done in conjunction with a driveway connection, and whether or not the improvement retains stormwater runoff on the adjacent property up to and including the 100 year event of critical duration.

(2) Assurance Requirements.

(a) The applicant for a drainage connection permit shall provide reasonable assurances that:

1. The peak discharge rates and total volumes of stormwater discharging from the adjacent property to the Department’s right of way are those provided for in an approved stormwater management plan or master drainage plan; otherwise the post-improvement stormwater runoff discharging from the adjacent property to the Department’s right of way shall not exceed the more stringent of the following:

a. The peak discharge rates and total volumes allowed by applicable local regulation; or

b. The improvement shall not increase stormwater discharge rate above the pre-improvement discharge rate, and in watersheds which do not have a positive outlet, the post-improvement total volume of stormwater runoff shall not be increased beyond the pre-improvement volume considering worst case storms for up to the frequencies and durations contained in paragraph 14-86.003(2)(c), F.A.C.

2. Any discharge pipe establishing or constituting a drainage connection to the Department’s right of way is limited in size based on the pre-improvement discharge rate, downstream conveyance limitations, downstream tailwater influences, and design capacity restrictions imposed by other governmental entities.

3. If the improvement changes the inflow pattern of stormwater or method of drainage connection to the Department’s right-of-way, post-improvement discharge will not exceed the pre-improvement discharge to the Department’s right of way, any new drainage connection will not threaten the safety or integrity of the Department’s right-of-way, and will not increase maintenance costs to the Department. At a minimum pavement hydraulics, ditch hydraulics, storm drain hydraulics, cross drain hydraulics, and stormwater management facilities shall be analyzed. The analysis must follow the methodology used in the design of the Department’s facilities receiving the discharge and meet the criteria in chapters 2, 3, 4, and 6 of the Department’s Drainage Manual, Topic Number 625-040-002-c, May 2008, incorporated herein by reference. The Drainage Manual is available from the Department at: http://www.dot.state.fl.us/rddesign/dr/Manualsandhandbooks.shtm.

4. The quality of water conveyed by the connection meets all applicable water quality standards, and such assurance shall be certified in writing. In the event the discharge is identified causing or contributing to a violation of applicable water quality standards, the permittee will be required to incorporate such abatement as necessary to bring the permittee’s discharge into compliance with applicable standards.

(b) If the requirements set forth in paragraph 14-86.003(2)(a), F.A.C., cannot be fully complied with, the applicant may submit alternative drainage connection designs. The analysis supporting the proposed alternative connection must follow the methodology used in the design of the Department’s facilities receiving the proposed alternative drainage connection and meet the criteria in chapters 2, 3, 4, and 6 of the Department’s Drainage Manual. Deviation from a standard in the Drainage Manual must be approved by the District Drainage Engineer. Acceptance of any alternative design must serve the purpose of this rule chapter and shall be based upon consideration of the following:

1. The type of stormwater management practice proposed;

2. The efficacy and costs of alternative controls;

3. The impact upon the operation and maintenance of the Department’s facilities; and,

4. The public interest served by the drainage connection.

(c) In providing reasonable assurances, the applicant shall:

1. Use a methodology which is compatible with the methodology employed in the design of the Department’s facilities receiving the stormwater;

2. Determine the peak discharge rates considering various rainfall event frequencies up to and including a 100 year event of critical duration of up to three days; and,

3. In watersheds without a positive outlet, determine the stormwater runoff total volumes considering various rainfall amounts up to a 100 year rainfall frequency of critical durations of up to ten days. The pond retention volume must recover at a rate such that one-half of the volume is available in seven days with the total volume available in 30 days, with a sufficient amount recovered within the time necessary to satisfy applicable water treatment requirements.

(3) Exceptions. The following exceptions do not require a Drainage Connection Permit:

(a) Improvements to adjacent properties not draining to the Department’s right-of-way in the pre-improvement and post-improvement condition.

(b) Single-family residential improvements which are not part of a larger common plan of improvement or larger common plan of sale.

(c) Agricultural and silvicultural improvements that:

1. Are subject to regulation by the Department of Environmental Protection or regional Water Management Districts;

2. Are exempt under the provisions of Section 373.406, F.S.; or

3. Are implementing applicable best management practices adopted by the Florida Department of Agriculture and Consumer Services in Rule Chapter 5M or 5I-6, F.A.C.

(d) Any other improvement, provided that all of the following apply:

1. The total impervious area, after improvement, is less than 5,000 square feet of cumulative impervious area and is less than 40% of that portion of the property that naturally drained to the Department’s right-of-way;

2. The improvement does not create or alter a drainage connection;

3. The improvement does not change flow patterns of stormwater to the Department’s right of way, and does not increase the surface area draining to the Department’s right-of-way;

4. The property is located in a watershed which has a positive outlet; and,

5. The site or improvement is not part of a larger common plan of improvement or larger common plan of sale.

(4) An exception provided in subsection 14-86.003(3), F.A.C., shall not apply if any drainage connection from the adjacent property threatens the safety and integrity of the Department’s facilities or creates an unreasonable burden on lower properties, including violations of applicable water quality standards.

Rulemaking Authority 334.044(2), (15) FS. Law Implemented 334.044(15) FS. History–New 11-12-86, Amended 1-20-09.