- 14-97.004 Interim Access Management Standards.

(1) Interim access management standards shall be applied to unclassified roadways on the SHS, such as when a local government transfers a roadway to the Department to become part of the SHS. After a roadway has been classified pursuant to this rule chapter, the access management standards associated with the designated access control classification shall supersede these interim standards.

| Table 3 INTERIM ACCESS MANAGEMENT STANDARDS |  |  |  |
| --- | --- | --- | --- |
| Posted Speed (MPH) | Connection Spacing Standard (Feet) | Median Opening Spacing Standard (Feet) |  |
|  |  | Full | Directional |
| 35 or less | 245 | 1,320 | 660 |
| 36 to 45 | 440 | 1,320 | 660 |
| Over 45 | 660 | 1,320 | 1,320 |

(2) The assignment of an access control classification to all segments of the SHS shall be the responsibility of the Department. The Department shall provide notice by publication in a newspaper of general circulation of proposed access control classification and shall coordinate with and consider the comments of the affected governmental entities before assigning a final access control classification to a roadway segment. The Department will hold public meetings, if requested to seek comment before final access classification of a roadway segment. Upon assignment of access control classification, the Department will provide notice to affected governmental entities.

Rulemaking Authority 334.044(2), 335.182, 335.184, 335.188 FS. Law Implemented 334.044(10)(a), 335.188 FS. History–New 2-13-91, Amended 10-7-09.