- 14-61.0017 Other Regulations.

(1) Driver Requirements.

(a) All drivers of Turnpike Tandem trailer must have a current Commercial Driver’s License (CDL) with an endorsement for double trailers consistent with the provisions of section 322.57, F.S.

(b) All drivers of Turnpike Tandems and all other individuals or companies operating turnpike tandems must comply with section 316.302(1), F.S., except that driver exemptions as set forth in 49 C.F.R. 391.21 and 391.67 shall not apply to drivers of turnpike tandems.

(c) All drivers of Turnpike Tandems must have a minimum of five years’ experience driving truck tractor semi-trailer combinations.

(d) A driver of a Turnpike Tandem must have had no suspension or revocation of driving privileges in any state or province during the past three years where such suspension arose out of operations of a commercial motor vehicle.

(2) Overall Length, Height, and Width.

(a) All overdimensional rules of the Turnpike Enterprise shall apply to Turnpike Tandems unless specifically excluded under the provisions of this rule chapter.

(b) The overall cargo carrying length of a Turnpike Tandem, as measured from the front of the first trailer to the rear of the second trailer including, the interval between the two trailers, shall not exceed 106 feet.

(c) Turnpike Tandems shall not exceed 13 feet 6 inches in height or 8 feet 6 inches in width.

(3) Weight and Axle Requirements.

(a) All overweight rules of section 316.515, F.S., shall apply to Turnpike Tandems unless specifically excluded under the provisions of this rule chapter.

(b) The maximum gross weight of the truck tractor and the first semitrailer of a Turnpike Tandem shall not exceed 80,000 pounds.

(c) The maximum gross weight of the unit of dolly and second trailer of a Turnpike Tandem shall not exceed the lesser of:

1. 67,000 pounds, or

2. The weight provisions of the State’s outer bridge formula set forth in section 316.535(5), F.S., as measured between the center of the foremost axle of the dolly and the rearmost axle of the trailer.

(d) In the event that a Turnpike Tandem is composed of trailers of unequal gross weight, the heavier of the two shall be used as the lead trailer.

(e) The gross weight limits described in chapter 316, F.S., may be exceeded with valid oversize/overweight permit issued by the Road Use Permits Office for a maximum gross weight not to exceed 147,000 pounds.

(f) A minimum of five load bearing axles are required unless stated otherwise in a valid oversize/overweight permit issued by the Road Use Permits Office.

Rulemaking Authority 334.044(2), 338.239 FS. Law Implemented 316.515(12), 322.57, 322.61, 338.239 FS. History–New 11-5-09.