- 14-61.0020 Brake Regulations.

(1) Every Turnpike Tandem shall be equipped with full air brakes or air activated hydraulic brakes on the tractor and either air or electric brakes on the dolly and trailers. All brakes shall equal or exceed both the equipment requirements and the performance standards cited in chapter 316, F.S., and 49 C.F.R., 393.40 through 393.52, hereby incorporated by reference.

(2) The brakes on any vehicle or combination of vehicles, used in the Turnpike Tandem operations shall be adequate to control the movement of, and to stop and hold, such vehicle, or combination of vehicles, and meet the general requirements of the provisions of the Florida Uniform Traffic Control Law, section 316.262, F.S.

(3) The Permittee shall certify to the Turnpike Enterprise on Form 800-040-04, General Certification Covering Turnpike Tandem Operations by Permittee, that the brakes on any vehicle or combination of vehicles used as a Turnpike Tandem meet the specific requirements specified in this rule.

Rulemaking Authority 334.044(2), 338.239 FS. Law Implemented 316.261-.263, 338.239 FS. History–New 11-5-09.