- 14-61.0026 Coupling Devices/Hitch Connections.

(1) All coupling devices shall equal or exceed both the equipment requirements and the performance standards established in 49, C.F.R. 393.70, as required by section 316.302(1), F.S.

(2) Vehicles in a Turnpike Tandem shall be designed, constructed, and connected as to ensure that shifting or swerving from side to side will not exceed two inches to each side of the path of the towing vehicle when it is moving in a straight line.

(3) All coupling devices/hitch connections shall be of a no-slack type which must be visible and operating. All drawbars, pickup plates, and fifth wheels must be rated to exceed the weight carried. Any kingpin must be rated to exceed the weight carried. Any kingpin must be solid and must be permanently fastened.

Rulemaking Authority 334.044(2), 338.239 FS. Law Implemented 338.239 FS. History–New 11-5-09.