- 14-61.0028 Speed Limits, Minimum Distances, Passing, and Operations under Hazardous Conditions.

(1) Speed Limits. When the speed of a Turnpike Tandem drops to 20 mph less than the posted maximum speed limit, the driver must use emergency flashers to notify the passing traffic that they are approaching a vehicle traveling substantially slower than the rest of the traffic.

(2) Minimum Distances. A minimum distance of 100 feet for each 10 mph of speed shall be maintained between a Turnpike Tandem trailer and another vehicle traveling in front of it in the same travel lane, unless weather or other roadway conditions do not permit such distance.

(3) Passing. A Turnpike Tandem may pass another vehicle traveling in the same direction only if the speed differential will allow the Turnpike Tandem to complete the maneuver and return to the normal driving lane within a distance of one mile and be performed within the posted speed limit. Turnpike Tandems must stay in the right lane, or those lanes designated for travel by posted signs, unless they are in the act of passing.

(4) Operations under Hazardous Conditions.

(a) Drivers of Turnpike Tandems shall exercise extreme caution when hazardous conditions exist, such as fog, smoke, dust, mist, or rain. Speed shall be reduced when such conditions exist.

(b) When hazardous weather conditions become dangerous, the driver or Permittee shall discontinue operations, and such operations shall not be resumed until the vehicle can be safely operated. The Turnpike Enterprise, Motor Carrier Compliance Office, Florida Highway Patrol, or their respective staffs may restrict or prohibit operations during periods when traffic, weather, or other safety conditions make such operations unsafe or inadvisable.

Rulemaking Authority 334.044(2), 338.239 FS. Law Implemented 316.55, 338.239 FS. History–New 11-5-09.