- 62-304.620 Little Manatee River Basin TMDLs.

(1) Little Manatee River. The fecal coliform Total Maximum Daily Load (TMDL) for Little Manatee River is 400 counts/100mL, and is allocated as follows:

(a) The Wasteload Allocation (WLA) for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2008 period, will require a 79 percent reduction of sources contributing to exceedances of the criteria;

(c) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2008 period, will require a 79 percent reduction of sources contributing to exceedances of the criteria;

(d) The Margin of Safety is implicit;

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(2) South Fork Little Manatee River. The fecal coliform TMDL for South Fork Little Manatee River is 400 counts/100mL, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2008 period, will require a 43 percent reduction of sources contributing to exceedances of the criteria;

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2001 to 2008 period, will require a 43 percent reduction of sources contributing to exceedances of the criteria;

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 11-2-09.