- 62-701.710 Waste Processing Facilities.

(1) Applicability.

(a) This section applies to solid waste management facilities that process but do not dispose of solid waste on-site. This includes materials recovery facilities and transfer stations, but does not include used oil processing facilities, waste tire processing facilities, soil treatment facilities, yard trash processing facilities that meet the registration requirements of Rule 62-709.320, F.A.C., or solid waste composting facilities, each of which is regulated under separate rules. Solid waste combustors will require permits under this section for any storage, processing, or disposal operations that are not directly addressed in another Department permit or certification as specified in paragraphs 62-701.320(14)(a) and (b), F.A.C. However, in accordance with paragraph 62-701.320(5)(c), F.A.C., owners or operators of facilities which manage several different types of wastes, including used oil, waste tires, contaminated soil, or compost, may apply for a single permit which addresses all applicable requirements.

(b) No person shall construct or operate a waste processing facility without a permit issued by the Department. All modifications or renewals of existing permits, and all new construction or operation permits issued on or after May 27, 2001, for waste processing facilities, shall comply with this rule.

(c) For facilities operating under a general permit, a timely and sufficient application for an individual permit will be considered a renewal application for purposes of Section 120.60(4), F.S.

(d) A waste processing facility which ceases accepting waste prior to the expiration of its permit shall close in accordance with the provisions of that permit.

(e) The following types of facilities are not subject to the requirements of this section; however, these facilities shall be operated to minimize the discharge of leachate to the environment and to control objectionable odors, litter, dust, and other fugitive particulates:

1. Facilities comprised solely of green boxes, compactor units, permanent dumpsters, and other containers from which wastes are transported to a landfill or other solid waste management facility, which do not accept waste from commercial waste haulers that accept waste from multiple generators;

2. Facilities owned or operated by local governments which serve as drop-off points for household waste, provided:

a. The facility accepts only household waste, which may include yard trash;

b. All putrescible waste, household garbage, yard trash, or other waste which may produce leachate is containerized; and

c. The facility does not accept waste from commercial waste haulers that collect municipal solid waste from multiple generators;

3. Household hazardous waste collection centers operated by or exclusively on behalf of a local government;

4. Facilities at industrial operations where waste is stored prior to shipment to a solid waste management facility, or where industrial byproducts are segregated and managed, provided that the industrial operation is regulated under another Department permit or certification.

5. Facilities used solely for the temporary storage of road maintenance byproducts, which include street sweepings, ditch scrapings, shoulder scrapings, and catch basin sediments, provided:

a. Materials that require screening, as well as separated Class I wastes, shall be stored so that leachate and litter are controlled. Examples would include storage in covered roll-offs, storage on an impervious surface and under roof, or storage indoors;

b. Unscreened materials that will not be beneficially used shall be disposed of as soon as practical but shall be stored for no longer than three months unless a longer storage time is approved by the Department;

c. Class I wastes that are separated from the material shall be disposed of at a permitted facility at least weekly; and

d. Screened materials, or materials that do not require screening, shall be stored for no longer than six months unless a longer storage time is approved by the Department.

(2) Application. A permit application for a waste processing facility shall be submitted on Form 62-701.900(4), Application to Construct, Operate, or Modify a Waste Processing Facility, effective date January 6, 2010, hereby adopted and incorporated by reference. Copies of this form are available from a local District Office or by writing to the Department of Environmental Protection, Solid Waste Section, MS 4565, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400. The form shall indicate whether the facility will operate as a materials recovery facility, transfer station, some other type of processing facility, or some combination thereof, shall be signed and sealed by a professional engineer, and shall include the information required in subsections 62-701.320(5), (6), (7), and paragraph (8)(a), F.A.C., specifically including:

(a) A description of the solid waste that is proposed to be collected, stored, processed or disposed of by the facility, a projection of those waste types and quantities expected in future years, and the assumptions used to make the projections;

(b) A site plan, of a scale not greater than 200 feet to the inch, which shows the facility location, total acreage of the site, and any other relevant features such as water bodies or wetlands on or within 200 feet of the site and potable water wells or within 500 feet of the site;

(c) A description of the operation and functions of all processing equipment that will be used, with design criteria and expected performance. The description shall show the flow of solid waste and associated operations in detail, and shall include:

1. Regular facility operations as they are expected to occur;

2. Procedures for start up operations, and scheduled and unscheduled shut down operations; and

3. Potential safety hazards and control methods, including fire detection and control;

(d) A description of loading, unloading, storage, and processing areas;

(e) Identification and capacity of any on-site storage areas for recyclable materials, non-processable wastes, unauthorized wastes, and residues;

(f) A plan for disposal of unmarketable recyclable materials and residue, and for waste handling capability in the event of breakdowns in the operations or equipment;

(g) A boundary survey, legal description, and topographic survey of the property;

(h) An operation plan which describes how the applicant will comply with subsection 62-701.710(4), F.A.C.;

(i) A closure plan which describes generally how the applicant will comply with subsection 62-701.710(6), F.A.C.; and

(j) The financial assurance documentation required by subsection 62-701.710(7), F.A.C.

(3) Design requirements. Minimum design requirements for waste processing facilities are as follows:

(a) Tipping, processing, sorting, storage and compaction areas that are in an enclosed building or covered area shall have ventilation systems. The areas that are not enclosed shall be equipped with litter control devices.

(b) The facility shall be designed with a leachate control system to prevent discharge of leachate and avoid mixing of leachate with stormwater, and to minimize the presence of standing water.

(c) Provisions shall be made for evaluating the quantity of all incoming solid waste and recovered materials. Storage areas shall be designed to hold the expected volume of materials until they are transferred for disposal or recycling.

(4) Operational requirements.

(a) A permit application for a waste processing facility shall include the following operational requirements:

1. An operation and maintenance manual describing the facility operations, the persons responsible for the operations, and types of equipment that will be used. All activities at the facility shall be performed in accordance with the manual and plans for the facility. Manuals and plans shall be updated as operations change but no less frequently than upon renewal of the operation permit;

2. A plan to inspect the wastes received by the facility, that specifies inspection procedures and procedures to handle unauthorized wastes; and

3. A contingency plan to cover operational interruptions and emergencies such as fires, explosions, or natural disasters.

(b) Stored putrescible wastes shall not be allowed to remain unprocessed for more than 48 hours; however, if the operation plan includes provisions to control vectors and odors, putrescible wastes may be stored for up to seven days. Any other unauthorized waste received by the facility shall be segregated and transported to an authorized disposal or recycling facility within 30 days of receipt. Areas where putrescible waste is stored or processed shall be cleaned at least weekly to prevent odor or vector problems, and all drains and leachate conveyances shall be kept clean so that leachate flow is not impeded.

(c) Operators and spotters shall be trained in accordance with subsection 62-701.320(15), F.A.C.

1. A trained operator shall be on duty whenever the facility is operating. Operating hours shall be posted at the facility.

2. At least one trained spotter shall be on duty at all times that waste is received at the site to inspect the incoming waste. Any unauthorized waste shall be removed from the waste stream and placed into appropriate containers for disposal at a permitted facility in accordance with a schedule submitted as part of the operation plan.

(d) The facility shall be operated to control objectionable odors in accordance with subsection 62-296.320(2), F.A.C.

(e) Adequate fire protection shall be available at all times.

(f) Access to the facility shall be controlled during the design period of the facility by fencing or other effective barriers to prevent disposal of unauthorized solid waste.

(g) If any regulated hazardous wastes are discovered to be improperly deposited at the facility, the facility operator shall promptly notify the Department, the person responsible for shipping the wastes to the facility, and the generator of the wastes, if known. The area where the wastes are deposited shall immediately be cordoned off from public access. If the generator or hauler cannot be identified, the facility operator shall assure the cleanup, transportation, and disposal of the waste at a permitted hazardous waste management facility.

(5) Certification. Certification of construction completion shall be done in accordance with paragraph 62-701.320(9)(b), F.A.C.

(6) Closure requirements.

(a) The permit application shall include a closure plan that identifies the steps needed to close the facility.

(b) The owner or operator shall notify the Department in writing prior to ceasing operations, and shall specify a closing date. No waste shall be received by the facility after the closing date.

(c) Within 30 days after receiving the final solid waste shipment, the owner or operator shall remove or otherwise dispose of all solid waste or residue in accordance with the approved closure plan. Stored putrescible wastes shall continue to be managed in accordance with paragraph 62-701.710(4)(b), F.A.C.

(d) Closure must be completed within 180 days after receiving the final solid waste shipment. Closure will include removal of all recovered materials from the site, as well as performing any contamination evaluation required by paragraph 62-701.710(10)(b), F.A.C. When closure is completed, the owner or operator shall certify in writing to the Department that closure is complete. The Department will make an inspection within 30 days to verify the closure and advise the owner or operator of the closure status.

(7) Financial assurance.

(a) The owner or operator of a waste processing facility shall provide the Department with proof of financial assurance issued in favor of the State of Florida in the amount of the closing cost estimates for the facility. This proof, along with the closing cost estimates, shall be submitted to the Department as part of the permit application for the facility. Proof of financial assurance shall consist of one or more of the following financial instruments which comply with the requirements of subsection 62-701.630(6), F.A.C.: trust fund; surety bond guaranteeing payment; surety bond guaranteeing performance; irrevocable letter of credit; insurance; and financial test and corporate guarantee. If the owner or operator of the facility is a local government, an escrow account which complies with the requirements of subsection 62-701.630(5), F.A.C., may be used to provide proof of financial assurance. Financial documents shall be submitted on Form 62-701.900(5)(a), (b), (c), (d), (e), (f), (g), or (h), as appropriate.

(b) Closure cost estimates and annual updates thereof shall comply with the provisions of subsections 62-701.630(3) and (4), F.A.C., except that long-term care costs need not be included, and the costs shall be based upon compliance with this section.

(c) If a local government requires financial assurance for closure, which is at least as stringent as that required by this rule, the Department will attempt to establish a cooperative mechanism with the local government and thereby avoid duplicative financial requirements.

(8) Stormwater. Stormwater shall be controlled in accordance with Part IV of Chapter 373, F.S., and the rules promulgated thereunder. A copy of any permit for stormwater control issued by the Department, or documentation that no such permit is required, shall be submitted to the Department and construction authorized by that permit shall be completed before the facility receives waste. Applicants should be aware that other government agencies may also regulate stormwater management and may require separate permits.

(9) Recordkeeping.

(a) Operational records shall be maintained to include a daily log of the quantity of solid waste received, processed, stored, and removed from the site for recycling or disposal, and the county of origin of the waste, if known. These records shall include each type of solid waste, recovered materials, residuals, and unacceptable waste which is processed, recycled, and disposed. Such records shall be compiled on a monthly basis and shall be available for inspection by the Department. Records shall be retained at the facility for three years.

(b) The owner or operator of any facility which recycles construction and demolition debris shall submit an annual report to the Department on Form 62-701.900(7), Annual Report for a Construction and Demolition Debris Facility, effective date January 6, 2010, hereby adopted and incorporated by reference. Copies of this form are available from a local District Office or by writing to the Department of Environmental Protection, Solid Waste Section, MS 4565, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400. This report shall include a summary of the amounts and types of wastes disposed of or recycled. The county of origin of materials which are recycled, or a statement that the county of origin is unknown, shall be included in the report. The report shall be submitted no later than April 1 of each year beginning in 1998, and shall cover the preceding calendar year.

(10) Special requirements for facility types.

(a) Transfer stations that accept primarily household waste, commercial waste, recovered materials, or construction and demolition debris, that manage waste on a first-in, first-out basis, and that store waste for no greater than 7 days are exempt from the requirement to provide financial assurance set forth in subsection (7) of this section.

(b) Waste processing facilities that accept only construction and demolition debris are exempt from the requirement to provide a leachate control system set forth in paragraph (3)(b) of this section, provided that all areas where waste is stored or processed are covered by a ground water monitoring system which meets the requirements of paragraph 62-701.730(4)(b), F.A.C. A facility that operates without a leachate control system shall perform a contamination evaluation as part of its closure activities, and shall continue to operate the ground water monitoring system if the evaluation indicates the potential for ground water contamination.

(11) Alternate procedures. The owner or operator of a facility may request alternate procedures and requirements in accordance with Rule 62-701.310, F.A.C. However, if such request is based upon the nature of the waste accepted at the facility (for example, if a facility accepts only segregated wastes which are expected to have a minimal environmental impact), the request will be submitted to and acted on by the appropriate District office of the Department as part of a permit application or modification, and need not be accompanied by any additional fee.

Rulemaking Authority 403.061, 403.704 FS. Law Implemented 403.702, 403.704, 403.707 FS. History–New 5-27-01, Amended 1-6-10.