- 62B-34.060 Non-Habitable Major Structures and Associated Minor Structures or Activities.

(1) Applicability. The following activities shall be eligible for a General Permit subject to the criteria and conditions set forth in this rule:

(a) New construction of non-habitable major structures. Swimming pools are not eligible for a General Permit pursuant to Section 161.053(18), F.S.

(b) Additions to an existing non-habitable major structure.

(c) Repair or rebuilding of an existing non-habitable major structure.

(d) Minor structures and activities associated with a proposed non-habitable major structure.

(2) Seaward Limit of Construction.

(a) The General Permit Line shall establish the seaward limit of construction if the line is defined in Part III of this rule chapter.

(b) If the General Permit Line is not defined in Part III of this rule chapter, the proposed construction activities shall be situated landward of a major road or the second line of construction.

(c) If the General Permit Line is not defined in Part III of this rule chapter and the proposed construction activities are not situated landward of a major road or the second line of construction, the seaward limit of construction activities allowed under paragraphs 62B-34.060(1)(a) through (d), F.A.C., shall be determined by the following criteria:

1. Construction shall remain a minimum of 25 feet landward of the primary dune feature.

2. Construction shall remain a minimum of 50 feet landward of the top of the bluff, with a bluff height greater than 15 feet.

3. Construction shall remain a minimum of 100 feet landward of the vegetation line, where the profile does not have certified armoring, a primary dune, or a bluff greater than 15 feet high.

4. Construction shall remain a minimum of 250 feet landward of the Erosion Control Line or a minimum of 250 feet landward of the Mean High Water Line, whichever is greater, where conditions contained in this rule, subparagraphs 62B-34.060(2)(a)1., 2., and 3., F.A.C., do not exist.

(d) No construction shall extend seaward of the General Permit Line or the first line of construction, except for elevated walkovers that provide access to the beach and that meet the design criteria contained in subsection 62B-34.050(19), F.A.C. If no General Permit Line has been established and no line of construction exists or can be determined, the proposed activity does not qualify for a General Permit.

(e) No construction shall extend seaward of the thirty-year erosion projection, except for elevated walkovers. The thirty-year erosion projection shall be made in accordance with the procedures stipulated in Rule 62B-33.024, F.A.C.

(f) No construction shall extend seaward of a point 25 feet landward of the existing armoring, except for elevated walkovers.

(3) Prohibitions and Limitations. Activities in paragraphs 62B-34.060(1)(a) through (d), F.A.C., shall be subject to the following prohibitions and limitations:

(a) Excavation shall be limited to the installation of, or repairs to, subgrade residential service utilities (e.g., water and sewer lines, septic tanks and drainfields, gas lines, and electrical, cable television and telephone cables). Minor and temporary excavation shall be allowed under this General Permit that would not result in the lowering of existing general ground elevations. This does not preclude temporary excavation for the installation of foundations.

(b) New construction of decks and additions to decks shall be limited to a total area of 1,500 square feet in size, except for elevated walkovers providing access to the beach. New or expanded decks shall be sited at or landward of the seaward limit of construction established by subsection 62B-34.060(2), F.A.C.

(c) Except where situated landward of a major road or landward of the second line of construction, new non-habitable major structures, and the combined dimensions of any existing single family dwellings, additions, decks, and other non-habitable major structures shall be limited to the following:

1. A total footprint of 6,300 square feet; and,

2. A total shore-parallel coverage of 65 percent of the shore-parallel dimension of the parcel measured at the maximum shore-parallel coverage of the construction authorized under this rule chapter.

(d) All work authorized under this General Permit shall comply with the turtle protection requirements of subsection 62B-34.070(4), F.A.C.

(e) All landscape work authorized under this General Permit shall comply with the native vegetation protection requirements of subsection 62B-34.070(5), F.A.C.

Rulemaking Authority 161.053(18), (20) FS. Law Implemented 161.052, 161.053(4), (18), (21) FS. History–New 3-27-03, Amended 11-21-05, 4-7-10.