- 40D-4.051 Exemptions.

The District will exempt from regulation under Section 373, Part IV, F.S., those activities that the District determines will have only minimal or insignificant individual or cumulative adverse impacts on the water resources of the District.

The following activities are exempt from permitting under this chapter:

(1) The activities specified in Section 373.406, F.S.

(2) The operation and maintenance of a surface water management system which:

(a) Was constructed before October 1, 1984; or

(b) Was constructed or was being constructed on or before December 9, 1999 and was not required to obtain a Disteict permit under exemptions existing at the time.

(3) The construction, alteration, or operation of a surface water management system for agricultural or silvicultural activities which satisfies the following requirements:

(a) The total land area does not equal or exceed 10 acres;

(b) The area of impervious surface will not equal or exceed 2 acres;

(c) The activities will not be conducted in wetlands;

(d) The activities will not be conducted in existing lakes, streams, or other watercourses;

(e) The surface water management system will not utilize drainage pumps or operable discharge structures;

(f) The activities will not utilize storm drainage facilities larger than one 24-inch diameter pipe, or its hydraulic equivalent;

(g) Discharges from the site will meet applicable state water quality standards, as set forth in Chapter 62-302 and Rule 62-4.242, F.A.C.;

(h) The activities are part of a conservation plan prepared or approved by a local Soil and Water Conservation District Board organized pursuant to Chapter 582, F.S., (S. C. S.). If the S. C. S. conservation plan is not implemented according to its terms, the exemption created in this subsection does not apply;

(i) The activities can otherwise reasonably be expected not to have significant adverse water resource impacts; and

(j) The surface water management system can be effectively maintained.

(4) All normal and necessary farming and forestry operations as are customary for the area, which can be conducted without the construction or alteration of a surface water management system. In order to qualify for this exemption, such operations and facilities shall not impede or divert the flow of surface waters entering or leaving the operation or intrude into or otherwise substantially and adversely impact significant wetlands.

(5) Phosphate mining, phosphate mining related surface water management systems, and reclamation and restoration conducted in accordance with Chapter 62C-16, F.A.C., are exempt from the requirements of this chapter, provided that all conditions for exemption in Rule 40D-4.053, F.A.C., are met.

(6) Any system for a mining or mining related activity which has a valid permit issued by the District or the Department pursuant to Rule 40D-45.041, F.A.C. This exemption shall be for the plans, terms and conditions approved in the permit issued pursuant to Chapter 40D-45, F.A.C. If an operator of a system previously permitted under Chapter 40D-45, F.A.C., proposes an “alteration” as the term is defined in subsection 40D-4.021(1), F.A.C., such system shall be reviewed under the provisions of Chapter 40D-4, F.A.C.

(7) Construction or private use of a single family dwelling unit, duplex, triplex or quadruplex that is not part of a larger common plan of development or sale and does not involve wetlands or other surface waters.

(8) Routine maintenance of a surface water management system; however, maintenance of surface water management systems will be considered in conjunction with the applications for construction, alteration, or operation.

(9) In accordance with the provisions of Section 403.813(2), F.S., no permit shall be required under Chapter 40D-4, 40D-40 or 40D-400, F.A.C., for the following activities:

(a) The repair or replacement of existing functional pipes or culverts the purpose of which is the discharge or conveyance of stormwater. In all cases, the invert elevation, the diameter, and the length of the culvert shall not be changed. However, the material used for the culvert may be different from the original material. This exemption does not authorize the repair, replacement, or alteration of dams, spillways or appurtenant works, nor construction activities or procedures that cause a violation of water quality standards as set forth in Chapter 62-302 and Rule 62-4.242, F.A.C.

(b) The installation, replacement or repair of mooring pilings and dolphins associated with private docking facilities or piers.

(c) The installation and repair of private docks, piers and recreational docking facilities, or piers and recreational docking facilities of local government entities when the local governmental entity’s activities will not take place in any manatee habitat, which structures have 1000 square feet or less of surface area over wetlands or other surface waters or 500 square feet or less of surface area over wetlands or other surface waters which are Outstanding Florida Waters. This exemption shall include the construction of structures above the dock or pier area, such as gazebos and boat shelters, provided such structures are not enclosed with walls and doors, are not used for living, commercial purposes, or storage of materials other than those associated with recreational use and provided the structures do not exceed, together with the docking facility or pier, the total area limitations above. To qualify for this exemption, any such structure:

1. Shall be used for recreational, non-commercial activities; and

2. Shall be constructed or held in place by pilings, including floating docks, so as not to involve filling or dredging other than necessary to install the pilings; and

3. Shall not substantially impede the flow of water or create a navigational hazard; and

4. Shall be the sole dock or pier constructed pursuant to this exemption as measured along the shoreline for a minimum distance of 65 feet, unless the parcel of land or individual lot as platted is less than 65 feet in length along the shoreline, in which case there may be one exempt dock allowed per parcel or lot. For the purposes of this rule, multi-family living complexes and other types of complexes or facilities associated with the proposed private dock shall be treated as one parcel of property regardless of the legal division of ownership or control of the associated property. Construction of a dock under this exemption does not obligate the District to issue a subsequent permit to construct a channel to provide navigational access to the dock. Activities associated with a dock shall include the construction of the structures attached to the dock which are only suitable for the mooring or storage of boats (i.e., boatlifts). Nothing in this paragraph shall prohibit the Department from taking appropriate enforcement action pursuant to Chapter 403, F.S., to abate or prohibit any activity otherwise exempt from permitting pursuant to this paragraph if the Department can demonstrate that the exempted activity has caused water pollution in violation of Chapter 403, F.S.

(d) The performance of maintenance dredging of existing manmade canals, channels, basins, berths, intake and discharge structures, and previously dredged portions of natural water bodies within drainage rights-of-way or drainage easements which have been recorded in the public records of the county, where the spoil material is to be removed and deposited on a self-contained, upland spoil site which will prevent the escape of the spoil material into waters of the state, provided that no more dredging is performed than is necessary to restore the canals, channels, basins, berths, and intake and discharge structures, and previously dredged portions of natural water bodies, to original design specifications, provided that the work is conducted in compliance with Section 370.12(2)(d), F.S., provided that no significant impacts occur to previously undisturbed natural areas, and provided that control devices for return flow and best management practices of erosion and sediment control are utilized to prevent bank erosion and scouring and to prevent turbidity, dredged material, and toxic or deleterious substances from discharging into adjacent waters during maintenance dredging. Further, for maintenance dredging of previously dredged portions of natural water bodies within recorded drainage rights-of-way or drainage easements, an entity that seeks an exemption must notify the department or water management district, as applicable, at least 30 days prior to dredging and provide documentation of original design specifications or configurations where such exist. This exemption shall apply to all canals and previously dredged portions of natural water bodies within recorded drainage rights-of-way or drainage easements constructed before April 3, 1970, and to those canals and previously dredged portions of natural water bodies constructed on or after April 3, 1970, pursuant to all necessary state permits. This exemption shall not apply to the removal of a natural or manmade barrier separating a canal or canal system from adjacent wetlands or other surface waters. Where no previous permit has been issued by the Board of Trustees of the Internal Improvement Trust Fund, the Department, the District or the United States Army Corps of Engineers for construction or maintenance dredging of the existing manmade canal, channel, basin, berth or intake or discharge structure, such maintenance dredging shall be limited to a depth of no more than 5 feet below mean low water.

(e) The installation and maintenance to design specifications of boat ramps on artificial bodies of water where navigational access to the proposed ramp exists, or the installation and maintenance to design specifications of boat ramps open to the public in any wetlands or other surface waters where navigational access to the proposed ramp exists and where the construction of the proposed ramp will be less than 30 feet wide and will involve the removal of less than 25 cubic yards of material from the wetlands or other surface waters and the installation of docks with an area of 500 square feet or less over wetlands or other surface waters that are associated with and adjoining the boat ramps constructed pursuant to this exemption. All material removed shall be placed upon a self-contained upland site so as to prevent the escape of the spoil material and return water from the spoil site into the wetlands or other surface waters. For the purpose of this exemption artificial bodies of water shall include, residential canal systems, canals permitted by a District created under Section 373.069, F.S., and artificially created portions of the Florida Intracoastal Waterway.

(f) Construction of seawalls or riprap, including only that backfilling needed to level the land behind seawalls or riprap, in artificially created waterways where such construction will not violate existing water quality standards, impede navigation or adversely affect flood control. An artificially created waterway is defined as a body of water that has been totally dredged or excavated and which does not overlap natural wetlands or other surface waters. For the purpose of this exemption, artificially created waterways shall also include existing residential canal systems. This exemption does not apply to the construction of vertical seawalls in estuaries or lagoons unless the proposed construction is within an existing man-made canal where the shoreline is currently occupied in whole or in part by vertical seawalls.

(g) Construction of private docks in artificially created waterways where construction will not violate water quality standards, impede navigation, or adversely affect flood control.

(h) The replacement or repair of existing docks and mooring pilings provided that no fill material other than the piles is to be used, and provided that the replacement or the repaired dock or mooring pile is in the same location and of the same configuration and dimensions as the dock or mooring pile being replaced or repaired. A dock or mooring pile must be functional and able to provide access to boats moored at the dock or pile before this exemption may be used unless the dock or mooring pile has been rendered nonfunctional by a discrete event such as a storm, collision or fire.

(i) The restoration of a seawall or riprap at its previous location, upland of its previous location, or within 18 inches waterward of its previous location, as measured from the face of the existing seawall slab to the face of restored seawall slab or from the front slope of the existing riprap to the front slope of the restored riprap. No filling can be performed except in the actual restoration of the seawall or riprap. No construction shall be undertaken without necessary title or leasehold interest, especially where private and public ownership boundaries have changed as a result of natural occurrences such as accretion, reliction and natural erosion. This exemption shall be limited to functioning seawalls or riprap. This exemption shall not affect the permitting requirements of Chapter 161, F.S.

(j) The maintenance of functioning insect control structures, and the maintenance of functioning dikes and functioning irrigation and drainage ditches, including roadway ditches, provided that the spoil material is deposited on a self-contained upland spoil site which will prevent the escape of the spoil material and return water into wetlands or other surface waters. In the case of insect control structures, if the cost of using a self-contained upland spoil site is so excessive as determined by the Department of Agriculture and Consumer Services, pursuant to Section 403.088(1), F.S., that it will inhibit the proposed insect control, existing spoil sites or dikes may be used, upon notification to the District. In the case of insect control where upland spoil sites are not used pursuant to this exemption, turbidity control devices shall be used to confine the spoil material discharge to that area previously disturbed when the receiving body of water is used as a potable water supply, is designated as approved, conditionally approved, restricted or conditionally restricted waters for shellfish harvesting by the Department, or functions as a habitat for commercially or recreationally important shellfish or finfish. In all cases, no more dredging is to be performed than is necessary to restore the dike or irrigation or drainage ditch to its original design specifications. This exemption shall apply to man-made trenches dug for the purpose of draining water from the land or for transporting water for use on the land and which are not built for navigational purposes.

(k) The restoration of less than 100 feet in length of existing insect control impoundment dikes and the connection of such impoundments to tidally influenced waters. Such impoundments shall be connected to tidally influenced waters for at least 6 months each year, beginning September 1, and ending February 28. The connection shall be of sufficient cross-sectional area to allow beneficial tidal influence. Restoration shall involve no more dredging than needed to restore the dike to original design specifications, and such that the final elevation of the dredge area shall be within 2 feet of immediately adjacent bottom elevations. For the purposes of this paragraph, restoration shall not include maintenance of impoundment dikes of insect control impoundments.

(l) The installation of subaqueous transmission and distribution lines laid on, or embedded in, the bottoms of wetlands or other surface waters, except in Class I and Class II waters and aquatic preserves, provided that no dredging or filling is necessary.

(m) The replacement or repair of subaqueous transmission and distribution lines laid on or embedded in, the bottoms of wetlands or other surface waters.

(n) The construction of seawalls or riprap in wetlands or other surface waters, where such construction is between and adjoins at both ends existing seawalls or riprap, follows a continuous and uniform construction line with the existing seawalls or riprap, is no more than 150 feet in length, and does not violate existing water quality standards, impede navigation adversely or affect flood control. However, this shall not affect the permitting requirements of Chapter 161, F.S. In estuaries and lagoons construction of vertical seawalls is limited to the circumstances and purposes stated in Sections 373.414(5)(b)1.-4., F.S.

(o) The replacement or repair of existing open-trestle foot bridges and vehicular bridges that are 100 feet or less in length and two lanes or less in width, provided that no more dredging or filling in wetlands or other surface waters is performed than that necessary to replace or repair pilings and that the structure to be replaced or repaired is the same length, the same configuration, and in the same location as the original bridge. No debris from the original bridge shall be allowed to remain in wetlands or other surface waters.

(10) The following activities shall not be required to obtain a permit under Chapter 40D-4, 40D-40, or 40D-400, F.A.C.:

(a) Activities necessary to preserve, restore, repair, remove, or replace an existing communication or power pole or line, provided that work does not involve dredge or fill activities other than the removal of the existing structure and the installation of the new structure, and in the case of a power pole or line, the activity does not increase the voltage of existing power lines. An activity does not qualify to use this exemption if it results in relocation of an existing structure or facility more than 10 feet in any direction from its original location, or if it involves the construction of new power or telephone lines or the repair and replacement of existing structures that require dredge or fill activities to provide access to the site.

(b) The installation, removal, and replacement of utility poles that support telephone or communication cable lines, or electric distribution lines of 35kV or less, together with the bases and anchoring devices to support those poles, as specified below. For the purpose of this exemption, “anchoring device” shall mean steel guy wires fastened to the ground and “base” shall mean a concrete or steel foundation not exceeding four feet in radius, used to support a utility pole. This exemption shall be subject to the following conditions:

1. No more than 15 utility poles may be installed, removed, or replaced in wetlands;

2. This exemption shall not apply in surface waters other than wetlands;

3. The temporary disturbance to wetlands shall be limited to a length of 0.5 miles, an areal extent of 0.5 acre, and a width of 30 feet to access the site to actually install, remove, or replace the utility poles; thereafter, maintenance of the utility right of way in wetlands shall be limited to a cleared corridor that does not exceed a total width of 15 feet and a total area of 0.25 ac.;

4. This exemption shall not apply in forested wetlands located within 550 feet from the mean or ordinary high water line of a named waterbody that is designated as an Outstanding Florida Water or an Outstanding National Resource Water, or to activities in any aquatic preserves;

5. There shall be no permanent placement of fill other than utility poles and anchoring devices;

6. There shall be no dredging or filling of fill pads or access roads except for temporary mats, which may be used to access pole installation sites, and all temporary mats shall be removed within thirty days after the installation, removal or replacement of the utility poles, associated bases, and anchoring devices;

7. The installation of the utility pole(s) and associated bases and anchoring devices shall not interfere with navigation or impede water flow in wetlands;

8. Turbidity, sedimentation, and erosion shall be controlled during and after construction to prevent violations of state water quality standards due to construction related activities;

9. Except for the permitted structures, pre-construction ground elevations and the contours of all soils that are disturbed by construction activities, including vehicle ruts in wetlands, shall be restored within 30 days of completion of the installation of the utility line or cable, and restored grades shall be stabilized within 72 hours following completion of elevation and contour restoration to minimize erosion;

10. Vehicle usage in wetlands shall be conducted so as to minimize tire rutting and erosion impacts;

11. Water jets shall not be used except for those which are a pre-engineered part of the pole, and provided that the water for the jets is either recirculated on site or is discharged in a self-contained upland disposal site;

12. Vehicular access in wetlands shall be limited to existing roads, trails, rights-of-way or easements, and to other previously disturbed corridors where they exist; and

13. The permittee shall provide an annual report to the District which summarizes the activities conducted under this exemption for the period from January 1 to December 31 of each year, including: the acreage of temporary impacts in wetlands resulting from the use of temporary mats and the clearing of wetland vegetation; the extent of permanent impacts to wetlands including the number of poles and structures in wetlands and the acreage of clearing in wetlands; the voltage of all electric lines that are installed; the number of times this exemption is used; the specific location of each line that is installed (including the county, the section, township, and range, and the identity of permanent landmarks such as roads and named wetlands and other surface waters within or adjacent to the work location), and the number of times and locations where water jets are used.

(c) The installation of aids to navigation, including bridge fender piles, “No Wake” and similar regulatory signs, and buoys associated with such aids, provided that the devices are marked in accordance with Section 327.40, F.S.

(d) Maintenance of minor silvicultural surface water management systems as described in subsection 40D-400.500(4), F.A.C., which were permitted under Part IV of Chapter 373, F.S., or were constructed prior to the requirements for a permit under this part, provided such maintenance is conducted in accordance with the performance standards set forth in subsection 40D-400.500(5), F.A.C.

(e) The construction or maintenance of culverted driveway or roadway crossings and bridges of artificial waterways, subject to the following provisions:

1. This exemption shall apply only to wholly artificial, non-navigable drainage conveyances.

2. The construction project area shall not exceed one acre, and the construction shall be for a discrete project that is not part of a larger plan of development which requires permitting under Chapters 40D-40 and 40D-400, F.A.C., or this chapter.

3. The artificial waterway in existing condition shall be not more than 4 feet deep, measured from the top of bank to the bottom of the artificial waterway.

4. The person performing the exempt activity shall ensure that the size and capacity of the culvert(s) will be adequate to pass normal high water stages of the artificial waterway without causing adverse impacts to upstream or downstream property, but the culvert(s) shall not be larger than one 24 inch diameter pipe, or its equivalent; and in no instance shall the culvert(s) provide a smaller cross-sectional area or discharge capacity than any upstream culvert.

5. The elevation of the culvert invert shall be at the existing bottom grade of the artificial waterway.

6. The length of the driveway or roadway crossing the waterway shall not exceed 30 feet from top of bank to top of bank.

7. The top width of the driveway or roadway shall not exceed 20 feet, the toe to toe width shall not exceed 40 feet, and side slopes shall be no steeper than 3 feet horizontal to 1 foot vertical.

8. Clean fill used for the crossing shall be obtained from an upland borrow pit or from a dredge site that is in compliance with the regulatory requirements of Part IV, Chapter 373, F.S., either through a permit, or an exemption.

9. There shall be no additional dredging, filling, or construction activities within the artificial waterway or project area, except those directly involved in the construction or operation and maintenance of the culverted crossing and those exempted from regulation under Part IV, Chapter 373, F.S.

10. All temporary fill in construction areas shall be removed and the area regraded to original elevations and revegetated.

11. The person performing the exempt activity shall implement measures for erosion and pollution control using best management practices, including turbidity curtains or similar devices and other site specific practices, in strict adherence to the Florida Department of Transportation’s “Standard Specifications for Road and Bridge Construction,” and Chapter 6 of the Department’s “Florida Development Manual,” to prevent violations of state water quality standards. Temporary erosion controls shall be implemented prior to and during construction, and permanent erosion control measures for all exposed soils shall be completed within seven calendar days of the most recent construction activity.

12. Any spoil material from construction or maintenance shall be used or disposed of on an upland portion of the property or shall be transported off site and deposited on a self-contained upland spoil site that is in compliance with the permitting requirements of Chapters 40D-4 and 40D-40, F.A.C., as applicable.

13. If dewatering is performed, all temporary fill dikes and dewatering discharges shall be installed and constructed so that no upstream flooding or impoundment occurs and to prevent siltation, erosion or turbid discharges into waters of the State in violation of state water quality standards. Any temporary works shall be completely removed, and all areas upstream and downstream from the crossing shall be restored to grades, elevations and conditions which existed before the construction.

14. This exemption shall apply only to a maximum of 2 crossings on any total land area of property with a minimum distance of 500 feet between crossings.

15. This exemption shall not apply to activities involving relocation or other alteration of all or part of the artificial waterway, or construction for other than the proposed culvert crossing.

(f) Construction of freshwater fish attractors by Florida Fish and Wildlife Conservation Commission, U.S. Forest Service, and county and municipal governments, provided that the material to be used shall be clean concrete, rock, brush, logs, or trees, and shall be free of soils, preservatives, oil, grease, debris, litter, putrescible substances, “white goods,” asphalt material, tires, or other pollutants, and shall be firmly anchored to the bottom of the waterbody. The size of an individual fish attractor shall not exceed one quarter acre in area. The material shall be placed so that the top of the fish attractor is at least three (3) feet below the surface of the water at ordinary low water and shall be outside any posted navigational channels. No fish attractor material shall be placed on or in areas vegetated by native aquatic vegetation. The site shall be marked with a buoy or buoys to ensure that no material is deposited outside of the site.

(g) Installation of piling support structures associated with water testing or monitoring equipment by the Department or the Districts, provided that flow or navigation are not impeded.

(11) Surface Waters or Wetlands Created by Mosquito Control Activities. Construction, alteration, operation, maintenance, removal, and abandonment of stormwater management systems, dams, impoundments, reservoirs, appurtenant works, or works, in, on, or over lands that have become surface waters or wetlands solely because of mosquito control activities undertaken as a part of a governmental mosquito control program, and which lands were neither wetlands nor other surface water before such activities, shall be exempt from the provisions in this chapter adopted by the District to implement Sections 373.414(1) through (6); 373.414(7), F.S., regarding any authority granted pursuant to Sections 373.414, F.S. (1991); 373.414(8) and 373.414(10), F.S.

(12) Exemptions for Treatment or Disposal Systems.

(a) Alteration and maintenance of the following shall be exempt from the provisions in Chapter 40D-4, F.A.C., implementing Sections 373.414(1) through 373.414(6), 373.414(8), and 373.414(10), F.S.; and Section 373.414(7), F.S., regarding any authority to apply state water quality standards within any works, impoundments, reservoirs, and other watercourses described in this subsection and any authority granted pursuant to Section 373.414, F.S. (1991):

1. Works, impoundments, reservoirs, and other watercourses constructed and operated solely for wastewater treatment or disposal in accordance with a valid permit reviewed or issued under Rules 62-28.700, 62-302.520 or Chapters 62-17, 62-600, 62-610, 62-640, 62-650, 62-660, 62-670, 62-671, 62-673, 62-701, F.A.C., or Section 403.0885, F.S., or rules implementing Section 403.0885, F.S., except for treatment wetlands or receiving wetlands permitted to receive wastewater pursuant to Chapter 62-611, F.A.C., or Section 403.0885, F.S., or its implementing rules.

2. Works, impoundments, reservoirs, and other watercourses constructed solely for wastewater treatment or disposal before a construction permit was required under Chapter 403, F.S., and operated solely for wastewater treatment or disposal in accordance with a valid permit reviewed or issued under Rules 62-28.700, 62-302.520 or Chapter 62-17, 62-600, 62-610, 62-640, 62-650, 62-660, 62-670, 62-671, 62-673, or 62-701, F.A.C.; or Section 403.0885, F.S., or rules implementing Section 403.0885, F.S., except for treatment wetlands or receiving wetlands permitted to receive wastewater pursuant to Chapter 62-611, F.A.C., or Section 403.0885, F.S., or its implementing rules;

3. Works, impoundments, reservoirs, and other watercourses of less than 0.5 acres in combined area on a project-wide basis, constructed and operated solely for stormwater treatment in accordance with a noticed exemption under Chapter 62-25, F.A.C., or a valid permit issued under Chapters 40D-4, 62-25 (excluding Rule 62-25.042, F.A.C.), Chapter 62-330, F.A.C., except those permitted as wetland stormwater treatment systems.

4. Works, impoundments, reservoirs, and other watercourses of less than 0.5 acres in combined area on a project-wide basis, constructed and operated solely for stormwater treatment before a permit being required under Chapters 62-25, 40D-4, F.A.C.

(b) Alteration and maintenance of the following shall be exempt from the provisions in Chapter 40D-4, F.A.C., adopted to implement Sections 373.414(1), 373.414(2)(a), 373.414(8), and 373.414(10), F.S.; and Sections 373.414(3) through 373.414(6), F.S.; and Section 373.414(7), F.S., regarding any authority to apply state water quality standards within any works, impoundments, reservoirs, and other watercourses described in this subsection and any authority granted pursuant to Section 373.414, F.S. (1991), except for authority to protect threatened and endangered species in isolated wetlands:

1. Works, impoundments, reservoirs, and other watercourses of 0.5 acre or greater in combined area on a project-wide basis, constructed and operated solely for stormwater treatment in accordance with a noticed exemption under Chapter 62-25, F.A.C., or a valid permit issued under Chapters 40D-4, 62-25 (excluding Rule 62-25.042, F.A.C.), Chapter 62-330, F.A.C., except those permitted as wetland stormwater treatment systems.

2. Works, impoundments, reservoirs, and other watercourses of 0.5 acres or greater in combined area on a project-wide basis, constructed and operated solely for stormwater treatment before a permit was required under Chapters 40D-4, 62-25, F.A.C.

(c) The exemptions in paragraphs (a) and (b) above shall not apply to works, impoundments, reservoirs or other watercourses that are:

1. Currently wetlands which existed before construction of the stormwater treatment system and were incorporated in it;

2. Being altered through expansion into wetlands or other surface waters; or

3. Wetlands created, enhanced or restored as mitigation for wetland or surface water impacts under a permit issued by the Department or the District.

(d) Alterations and maintenance of works, impoundments, reservoirs and other watercourses exempt under this subsection shall not be considered in determining whether the wetland permitting thresholds in Chapters 40D-4 or 40D-40, F.A.C., are met or exceeded.

(e) Works, impoundments, reservoirs and other watercourses exempt under this subsection, other than isolated wetlands in systems described in paragraph (b) above, shall not be delineated under Section 373.421, F.S.

(f) This exemption shall not affect the application of state water quality standards, including those applicable to Outstanding Florida Waters, at the point of discharge to waters as defined in Section 403.031(13), F.S.

(g) As used in this subsection, “solely for” means the reason for which a work, impoundment, reservoir, or other watercourse is constructed and operated, and such construction and operation would not have occurred but for the purposes identified in paragraphs (a) and (b) above. Furthermore, the phrase does not refer to a work, impoundment, reservoir, or other watercourse constructed or operated for multiple purposes. Uses, such as occasional recreational uses, will not render the exemption inapplicable, so long as the incidental uses are not part of the original planned purpose of the work, impoundment, reservoir or other watercourse. However, for those works, impoundments, reservoirs, or other watercourses described in subparagraphs (a)3. and (b)1., use of the system for flood attenuation, whether originally planned or unplanned, shall be considered an incidental use so long as the works, impoundments, reservoirs, and other watercourses are no more than two acres larger than the minimum area required to comply with the applicable stormwater treatment requirements of Chapters 62-25, 62-330, 40D-4, F.A.C. For the purposes of this subsection, reuse from a work, impoundment, reservoir, or other watercourse is part of treatment or disposal.

(13) Minor Roadway Safety Projects. The construction of the following minor roadway safety projects, provided that the capacity of existing swales, ditches or other stormwater management systems is not reduced; the projects are not located within wetlands or other surface waters; and the projects include best management practices during construction to prevent secondary impacts in adjacent wetlands or other surface waters due to erosion and sedimentation:

(a) Sidewalks adjacent to roadways that have a width of six feet or less and do not obstruct or impound surface waters;

(b) Turnlanes less than 0.25 mile in length and other intersection improvements; and

(c) Road widening and shoulder paving projects which do not result in the creation of additional traffic lanes.

(14) Recreational Paths. Recreational paths adjacent to roadways, provided that the recreational paths are not located within wetlands or other surface waters; do not obstruct or impound surface waters; best management practices are used during construction to prevent secondary impacts in adjacent wetlands or other surface waters due to erosion or sedimentation; do not exceed eight feet in width for unidirectional paths and 12 feet in width for bidirectional paths; and do not allow motorized vehicles powered by internal combustion engines except for maintenance and emergency vehicles.

(15) Construction, alteration, operation or abandonment of a stormwater management system meeting the following criteria:

(a) The proposed activities are not conducted within twenty-five feet of wetlands or other surface waters;

(b) The proposed activities include no more than 4,000 square feet of impervious or semi-impervious surface area subject to vehicular traffic, such as roads, parking lots, driveways, and loading zones;

(c) The proposed activities include no more than 9,000 square feet of total impervious or semi-impervious area;

(d) The proposed activities, including placement of fill, encompass no more than one-half acre of total project area;

(e) The proposed activities do not include filling in floodways, 100-year floodplains or water conveyance features such as ditches or swales and do not adversely affect offsite properties by altering overland sheetflow;

(f) The system does not directly discharge into an Outstanding Florida Water, as listed in Rule 62-302-700, F.A.C.;

(g) The proposed activities are not conducted as part of a larger common plan of development or sale regulated under Part IV of Chapter 373, F.S.;

(h) The proposed activities are not conducted within the geographic limits of an existing permit issued pursuant to Part IV of Chapter 373, F.S.; and

(i) The proposed activities, considered separately or in combination with other activities conducted pursuant to this exemption, do not cumulatively exceed any of the thresholds indicated above.

(16) The performance of activities pursuant to the provisions of the exemptions described above does not relieve the person or persons who are using the exemption or who are constructing or otherwise implementing the activity from meeting the permitting or performance requirements of other District rules.

Rulemaking Authority 373.044, 373.113, 373.149, 373.171, 373.414(9) FS. Law Implemented 373.406, 373.413, 373.414(9), 373.416 FS. History–Readopted 10-5-74, Formerly 16J-4.05, Amended 10-1-84, 10-1-86, 3-1-88, 1-24-90, 10-3-95, 4-18-01, 5-17-01, 4-9-02, 2-19-04, 6-30-05, 11-26-07, 9-29-08, 8-1-10.