- 62-304.805 Charlotte Harbor Basin TMDLs.

(1) Coral Creek – East Branch. The Total Maximum Daily Loads (TMDLs) to address the low dissolved oxygen condition for Coral Creek – East Branch are 0.74 mg/L total nitrogen (TN), 0.044 mg/L total phosphorus (TP), and 2.0 mg/L five-day biochemical oxygen demand (BOD5), and are allocated as follows:

(a) The Wasteload Allocation (WLA) for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Stormwater Permitting Program to address anthropogenic sources in the basin such that in-stream concentrations meet the dissolved oxygen criteria, based on the measured concentrations from the January 2007 to December 2007 period, will require a 31 percent reduction of TN, a 36 percent reduction of TP, and a 33 percent reduction of BOD5 at sources contributing to exceedances of the criteria;

(c) The Load Allocation (LA) for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the dissolved oxygen criteria which, based on the measured concentrations from the January 2007 to December 2007 period, will require a 31 percent reduction of TN, a 36 percent reduction of TP, and a 33 percent reduction of BOD5 at sources contributing to exceedances of the criteria; and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP have been expressed as the percent reduction needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream TN and TP concentrations. However, it is not the intent of the TMDL to abate natural background conditions.

(2) Gottfried Creek. The TMDL for Gottfried Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable.

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2005 to 2008 period, will require a 74 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2005 to 2008 period, will require a 74 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

(3) North Prong of Alligator Creek. The TMDL for the North Prong of Alligator Creek is 400 counts/100mL for fecal coliform, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable.

(b) The WLA for discharges subject to the Department’s NPDES Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2005 and 2007 period, will require a 72 percent reduction of sources contributing to exceedances of the criteria,

(c) The LA for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 2005 and 2007 period, will require a 72 percent reduction of sources contributing to exceedances of the criteria, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for fecal coliform have been expressed as the percent reductions needed to attain the applicable Class I criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal concentration. However, it is not the intent of the TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 10-15-09, Amended 8-26-10.