- 62-346.050 Permits Required.

(1) An individual permit under this chapter must be obtained from the Department prior to the construction, alteration, operation, maintenance or repair (excluding routine custodial maintenance), abandonment, or removal of any surface water management system, unless the activity:

(a) Qualifies for an exemption under Section 373.406, 373.4145(3), or 403.813(1), F.S., or Rule 62-346.051, F.A.C., or a noticed general permit under Chapter 62-341, F.A.C. This includes all applications for conceptual approval permits. However, the establishment of a mitigation bank under Chapter 62-342, F.A.C., shall not require a permit under this chapter;

(b) Meets the following thresholds and criteria and is not part of a larger common plan of development or ownership:

1. The system is not located in whole or in part in, on, or over wetlands or other surface waters;

2. The system consists of less than or equal to 4,000 square feet of impervious or semi-impervious surface area subject to vehicular traffic, such as roads, parking lots, driveways, and loading zones;

3. The system consists of less than or equal to 5,000 square feet of building area or other impervious area not subject to vehicular traffic;

4. The system has less than or equal to one acre total project area;

5. The system does not have a direct discharge into an Outstanding Florida Water, as listed in Rule 62-302.700, F.A.C.;

6. The system does not cause any of the following:

a. Adverse water quantity impacts to receiving waters and adjacent lands;

b. Adverse flooding to on-site or off-site property;

c. Adverse impacts to existing surface water storage and conveyance capabilities;

d. A violation of the water quality standards set forth in Chapters 62-4, 62-302, 62-520, 62-522 and 62-550, F.A.C., including the provisions of Rules 62-4.243, 62-4.244 and 62-4.246, F.A.C., the antidegradation provisions of paragraphs 62-4.242(1)(a) and (b), F.A.C., subsections 62-4.242(2) and (3), F.A.C., and Rule 62-302.300, F.A.C., and any special standards for Outstanding Florida Waters and Outstanding National Resource Waters set forth in subsections 62-4.242(2) and (3), F.A.C.;

e. Adverse secondary or cumulative impacts to the water resources;

7. The new work, by itself, or in combination with a system that has come into existence since the effective date of this chapter, cannot cumulatively exceed any of the thresholds in subparagraphs 62-346.050(1)(b)1. through 5., F.A.C., or violate the criteria of subparagraph 62-346.050(1)(b)6., F.A.C.

(c) Is an agricultural or forestry surface water management system regulated under Rule 40A-44.041, 40A-44.051, or 40A-44.052, F.A.C., which will continue to be regulated under Chapter 40A-44, F.A.C.; or

(d) Meets the provisions of Section 373.4145(6), F.S.

(2) Notwithstanding the provisions of this section, an individual permit under this chapter is required for the construction, alteration, operation, maintenance, abandonment, or removal of any dry storage facility for 10 or more vessels that is functionally associated with a boat launching area, including when the dry storage facility does not involve any work within the landward extent of wetlands or other surface waters.

(3) Applications for individual permits will be reviewed and acted upon in accordance with subsection 62-346.090(2), F.A.C. Noticed general permits will be reviewed and acted upon in accordance with subsection 62-346.090(1), F.A.C.

(4) Permits for construction or alteration of activities regulated under this chapter must be converted to an operation and maintenance phase in accordance with Rule 62-346.095, F.A.C.

Rulemaking Authority 373.026(7), 373.043, 373.118, 373.4145, 373.418, 403.805(1) FS. Law Implemented 373.117, 373.118, 373.409, 373.413, 373.4132, 373.4145, 373.416, 373.426, 403.0877 FS. History–New 10-1-07, Amended 11-1-10.