40E-63.438 Early Implementation of Water Quality Improvement Activities.

An applicant may request approval for early implementation by opting to submit a proposal for voluntary implementation of additional BMPs (early BMPs), or a voluntary demonstration project that includes a BMP performance verification plan. Applicants electing these approaches must submit for District review the following:

(1) Either proposal shall be submitted together with an application for a new permit, permit renewal, or as a Letter Modification.

(a) For optional early BMPs the application shall provide information for meeting the criteria below:

1. A description of the BMP or group of BMPs (early BMPs) that are proposed in addition to those required by rule at the time of application (Rule 40E-63.435 or subsection 40E-63.461(3), F.A.C., as applicable). The proposal shall include the specific methods for implementation and maintenance of the early BMPs.

2. The proposal shall provide reasonable assurance through technical documentation, and the requirements indicated in subsections 40E-63.435(4) and (5), F.A.C., that the combined effect of the optional early BMPs and rule-required BMPs will ensure a phosphorus loading reduction for the identified permit basin or parcels sufficient for the C-139 Basin to consistently achieve the performance measurer’s target, as described in Appendix B2 (incorporated by reference in subsection 40E-63.446(1), F.A.C.). The District will review whether the proposed loading reduction levels would be conducive to meeting the target Unit Area Load (UAL) based on the most recent five years of water quality data.

3. The proposal shall include an implementation schedule.

(b) For voluntary demonstration projects, the application shall propose a BMP or water quality improvement measure demonstration project that meets the following:

1. Complies with the criteria described under paragraph 40E-63.437(3)(a), F.A.C.,

2. Projects estimated phosphorus reductions based on available technical references, and

3. Proposes a verification plan through a Permit Discharge Monitoring Program to confirm and quantify the estimated phosphorus reductions. The verification plan shall meet the criteria described in subsection 40E-63.461(4), F.A.C.

(2) Upon District approval of the voluntary early BMP implementation project or demonstration project with a verification plan, the permittee will be subject to the BMP reporting and verification requirements of this chapter for those voluntary initiatives, as described in permit conditions. Permittees cannot be deemed out of compliance solely for failure to implement the early initiatives, however, the permittee cannot qualify with the conditions of paragraphs 40E-63.446(2)(b) and (c), F.A.C. unless:

(a) The early BMP’s are implemented.

(b) Reporting and verification requirements for the voluntary early implementation projects are met, as determined by the District; and

(c) The permittee is in compliance with the BMP Plan required by the permit.

Rulemaking Authority 373.044, 373.113, 373.4592 FS. Law Implemented 373.016, 373.085, 373.086, 373.451, 373.453, 373.4592(4)(f) FS. History–New 11-9-10.