## Pt. 273

Wyoming Hazardous Waste Management Rules, as amended effective March 18, 2015, Chapter 1, General Provisions: Sections 2(a) and (b); 3; 4; 124(a)(v); 260 (except 260(b)(ii));

261;  262;  263;  264(a)  through  264(d),  264(e)(i)

(except the citation ‘‘W.S. 35–11–1607’’  and the phrase ‘‘or a signed remedy agreement pursuant to W.S. 35–11–1607’’ in the first sen- tence), 264(e)(iii)(A) and (B), 264(f) through 264(m);  265;  266;  267;  268;  270(a)  through

270(m); 273; and 279.

Copies of the Wyoming regulations  that are incorporated by reference are available from Wyoming Secretary of State’s Office, The Capitol Building, Room B–10, 200 West 24th Street, Cheyenne, Wyoming 82002–0020, (Phone: (307) 777–5407).

[58 FR 3500, Jan. 11, 1993]

EDITORIAL NOTE: For FEDERAL REGISTER ci-

tations affecting appendix A to part 272, see the List of CFR Sections Affected, which ap- pears in the Finding Aids section of the printed volume and at www.govinfo.gov.

# PART 273—STANDARDS FOR UNIVERSAL WASTE MANAGEMENT

### Subpart A—General

Sec.

  - Scope.

  - Applicability—batteries.

  - Applicability—pesticides.

  - Applicability—Mercury-containing equipment.

  - Applicability—lamps. 273.6–273.7  [Reserved]

  - Applicability—household	and	very small quantity generator waste.

  - Definitions.

### Subpart B—Standards for Small Quantity Handlers of Universal Waste

  - Applicability.

  - Prohibitions.

  - Notification.

  - Waste management.

  - Labeling/marking.

  - Accumulation time limits.

  - Employee training.

  - Response to releases.

  - Off-site shipments.

  - Tracking universal waste shipments.

  - Exports.

### Subpart C—Standards for Large Quantity Handlers of Universal Waste

  - Applicability.

  - Prohibitions.

  - Notification.

  - Waste management.

  - Labeling/marking.

  - Accumulation time limits.

40 CFR Ch. I (7–1–19 Edition)

  - Employee training.

  - Response to releases.

  - Off-site shipments.

  - Tracking universal waste shipments.

  - Exports.

### Subpart D—Standards for Universal Waste Transporters

  - Applicability.

  - Prohibitions.

  - Waste management.

  - Storage time limits.

  - Response to releases.

  - Off-site shipments.

  - Exports.

### Subpart E—Standards for Destination Facilities

  - Applicability.

  - Off-site shipments.

  - Tracking universal waste shipments.

### Subpart F—Import Requirements

273.70   Imports.

### Subpart G—Petitions To Include Other Wastes Under 40 CFR Part 273

  - General.

  - Factors for petitions  to include other wastes under 40 CFR part 273.

AUTHORITY:  42  U.S.C.  6922,  6923,  6924,  6925,

6930, and 6937.

SOURCE: 60 FR 25542, May 11, 1995, unless

otherwise noted.

# Subpart A—General

### § 273.1   Scope.

    - This  part  establishes  require- ments for managing the following:

- Batteries  as  described  in  40  CFR 273.2;

- Pesticides as described in § 273.3;

- Mercury-containing  equipment  as described in § 273.4; and

- Lamps as described in § 273.5.

    - This part  provides an  alternative set of management standards in lieu of regulation under 40 CFR parts 260 through 272.

[60 FR 25542, May 11, 1995, as amended at 64

FR 36488, July 6, 1999; 70 FR 45520, Aug. 5,

2005]

### § 273.2   Applicability—batteries.

- Batteries covered under 40 CFR part

- (1) The requirements of this part apply to persons managing batteries,

Environmental Protection Agency	§ 273.3

as described in § 273.9, except those list- ed in paragraph (b) of this section.

(2) Spent lead-acid batteries which are not managed under 40 CFR part 266, subpart G, are subject to management under this part.

- Batteries not covered under 40 CFR part 273. The requirements of this part do not apply to persons managing the following  batteries:

  - Spent lead-acid batteries that are managed under 40 CFR part 266, sub- part G.

  - Batteries, as described in § 273.9, that are not yet wastes under part 261 of this chapter, including those that do not meet the criteria for waste genera- tion in paragraph (c) of this section.

  - Batteries, as described in § 273.9, that are not hazardous waste. A bat- tery is a hazardous waste if it exhibits one or more of the characteristics iden- tified in part 261, subpart C of this chapter.

- Generation of waste batteries. (1) A used battery becomes a waste on the date it is discarded (e.g., when sent for reclamation).

- An unused battery becomes a waste on the date the handler decides to discard it.

[60 FR 25542, May 11, 1995, as amended at 64

FR 36488, July 6, 1999]

### § 273.3   Applicability—pesticides.

  - Pesticides covered   under  this  part

  - The requirements of this part apply to persons managing pesticides, as described in § 273.9, meeting the fol- lowing conditions, except those listed in paragraph (b) of this section:

    - Recalled pesticides that are:

      - Stocks of a suspended and can- celed pesticide that are part of a vol- untary or mandatory recall under FIFRA Section 19(b), including, but not limited to those owned by the reg- istrant responsible for conducting the recall; or

      - Stocks of a suspended or can- celled pesticide, or a pesticide that is not in compliance with FIFRA,  that are part of a voluntary recall by the registrant.

    - Stocks of other unused pesticide products that are collected and man- aged as part of a waste pesticide collec- tion program.

  - Pesticides not covered under 40 CFR part 273. The requirements of this part do not apply to persons managing the following  pesticides:

    - Recalled pesticides described in paragraph (a)(1) of this section, and un- used pesticide products described in paragraph (a)(2) of this section, that are managed by farmers in compliance with 40 CFR 262.70. (40 CFR 262.70 ad- dresses pesticides disposed of on the farmer’s own farm in a manner con- sistent with the disposal  instructions on the pesticide label, providing the container is triple rinsed in accordance with 40 CFR 261.7(b)(3));

    - Pesticides not meeting the condi- tions set forth in paragraph (a) of this section. These pesticides must be man- aged in compliance with the hazardous waste regulations in 40 CFR parts 260 through 272;

    - Pesticides that are not wastes under part 261 of this chapter, includ- ing those that do not meet the criteria for waste generation in paragraph (c) of this section or those that are not wastes as described in paragraph (d) of this section; and

    - Pesticides that are not hazardous waste. A pesticide is a hazardous waste if it is listed in 40 CFR part 261, subpart D or if it exhibits one or more of the characteristics identified in 40 CFR part 261, subpart C.

  - When  a  pesticide  becomes  a  waste.

- A recalled pesticide described in paragraph (a)(1) of this section be- comes a waste on the first date on which both of the following conditions apply:

  - The generator of the recalled pes- ticide agrees to participate in the re- call; and

  - The person conducting the recall decides to discard (e.g., burn the pes- ticide for energy recovery).

- An unused pesticide product de- scribed in paragraph (a)(2) of this sec- tion becomes a waste on the date the generator decides to discard it.

  - Pesticides that are not wastes. The following pesticides are not wastes:

    - Recalled pesticides described in paragraph (a)(1) of this section, pro- vided that the person conducting the recall:

## § 273.4

      - Has not made a decision to discard (e.g., burn for energy recovery) the pes- ticide. Until such a decision is made, the pesticide does not meet the defini- tion of ‘‘solid waste’’ under 40 CFR 261.2; thus the pesticide is not a haz- ardous waste and is not subject to haz- ardous waste requirements, including this part 273. This pesticide remains subject to the requirements of FIFRA; or

      - Has made a decision to use a management option that, under 40 CFR 261.2, does not cause the pesticide to be a solid waste (i.e., the selected option is use (other than use constituting dis- posal) or reuse (other than burning for energy recovery),  or  reclamation). Such a pesticide is not a solid waste and therefore is not a hazardous waste, and is not subject to the hazardous waste requirements including this part

273. This pesticide, including a recalled pesticide that is exported to a foreign destination for use or reuse, remains subject to the requirements of FIFRA.

    - Unused pesticide products de- scribed in paragraph (a)(2) of this sec- tion, if the generator of the unused pes- ticide product has not decided to dis- card (e.g., burn for energy recovery) them. These pesticides remain subject to the requirements of FIFRA.

[60 FR 25542, May 11, 1995, as amended at 64

FR 36488, July 6, 1999]

### § 273.4   Applicability—Mercury-con- taining equipment.

- Mercury-containing equipment cov- ered under this part 273. The require- ments of this part apply to persons managing mercury-containing equip- ment, as described in § 273.9, except those listed in paragraph (b) of this section.

- Mercury-containing equipment not covered under this part 273. The require- ments of this part do not apply to per- sons managing the following mercury- containing  equipment:

- Mercury-containing equipment that is not yet a waste under part 261 of this chapter. Paragraph (c) of this section describes when mercury-con- taining equipment becomes a waste;

- Mercury-containing equipment that is not a hazardous waste. Mer- cury-containing equipment is a haz- ardous waste if it exhibits one or more

## 40 CFR Ch. I (7–1–19 Edition)

of the characteristics identified in part 261, subpart C of this chapter or is list- ed in part 261, subpart D of this chap- ter; and

- Equipment and devices from which the mercury-containing compo- nents have been removed.

- Generation of waste mercury-con- taining equipment. (1) Used mercury- containing equipment becomes a waste on the date it is discarded.

- Unused mercury-containing equip- ment becomes a waste on the date the handler decides to discard it.

[70 FR 45520, Aug. 5, 2005]

### § 273.5   Applicability—lamps.

  - Lamps covered under this part 273. The requirements of this part apply to persons managing lamps  as  described in § 273.9, except those listed in para- graph (b) of this section.

  - Lamps  not  covered  under  this  part

- The requirements of this part do not apply to persons managing the fol- lowing lamps:

  - Lamps that are not yet wastes under part 261 of this chapter as pro- vided in paragraph (c) of this section.

  - Lamps that are not hazardous waste. A lamp is a hazardous waste if it exhibits one or more of the characteris- tics identified in part 261, subpart C of this chapter.

  - Generation of waste lamps. (1)  A used lamp becomes a waste on the date it is discarded.

- An unused lamp becomes a waste on the date the handler decides to dis- card it.

[64 FR 36488, July 6, 1999]

### §§ 273.6–273.7   [Reserved]

§ 273.8 Applicability—household and very small quantity  generator waste.

  - Persons managing the wastes list- ed below may, at their option, manage them under the requirements of this part:

    - Household wastes that are exempt under § 261.4(b)(1) of this chapter and are also of the same type as the uni- versal wastes defined at § 273.9; and/or

    - Very small quantity generator wastes that are exempt under § 262.14 of this chapter and are also of the same

## Environmental Protection Agency	§ 273.9

type as the universal wastes defined at

§ 273.9.

  - Persons who commingle the wastes described in paragraphs (a)(1) and (a)(2) of this section together with universal waste regulated under this part must manage the commingled waste under the requirements of this part.

[64 FR 36488, July 6, 1999, as amended at 81

FR 85828, Nov. 28, 2016]

### § 273.9   Definitions.

Ampule means an airtight vial made of glass, plastic, metal, or any com- bination of these materials.

Battery means a device consisting of one or more electrically connected electrochemical cells which is designed to receive, store, and deliver electric energy. An electrochemical cell is a system consisting of an anode, cathode, and an electrolyte, plus such connec- tions (electrical and mechanical) as may be needed to allow the cell to de- liver or receive electrical energy. The term battery also includes an intact, unbroken battery from which the elec- trolyte has been removed.

Destination facility means a facility that treats, disposes of, or recycles a particular category of universal waste, except those management activities described  in  § 273.13  (a)  and  (c)  and

§ 273.33 (a) and (c). A facility at which a particular category of universal waste is only accumulated, is not a destina- tion facility for purposes of managing that category of universal waste.

FIFRA means the Federal Insecti- cide, Fungicide, and Rodenticide Act (7

U.S.C. 136–136y).

Generator means any person, by site, whose act or process produces haz- ardous waste identified or listed in part

261 of this chapter or whose act first causes a hazardous waste to become subject to regulation.

Lamp, also referred to as ‘‘universal waste lamp’’ is defined as the bulb or tube portion of an electric lighting de- vice. A lamp is specifically designed to produce radiant energy, most often in the ultraviolet, visible, and infra-red regions of the electromagnetic spec- trum. Examples of common universal waste electric lamps include, but are not limited to, fluorescent, high inten- sity  discharge,  neon,  mercury  vapor,

high pressure sodium, and metal halide lamps.

Large Quantity Handler of Universal Waste means a universal waste handler (as defined in this section) who accu- mulates 5,000 kilograms or more total of universal waste (batteries, pes- ticides, mercury-containing equip- ment, or lamps, calculated collec- tively) at any time. This designation as a large quantity handler of universal waste is retained through the end of the calendar year in which the 5,000 kilogram limit is met or exceeded.

Mercury-containing equipment means a device or part of a device (including thermostats, but excluding batteries and lamps) that contains elemental mercury integral to its function.

On-site means the same or geographi- cally contiguous property which  may be divided by public or private right-of- way, provided that  the entrance and exit between the properties is at a cross-roads intersection, and access is by crossing as opposed to going along the right of way. Non-contiguous prop- erties owned by the same person but connected by a right-of-way which he controls and to which the public does not have access, are also considered on- site property.

Pesticide means any substance or mixture of substances intended for pre- venting, destroying, repelling, or miti- gating any pest, or intended for use as a plant regulator, defoliant, or des- iccant, other than any article that:

- Is a new animal drug under FFDCA section 201(w), or

- Is an animal drug that has been determined by regulation of the Sec- retary of Health and Human Services not to be a new animal drug, or

- Is an animal feed under FFDCA section 201(x) that bears or contains any substances described by paragraph

- or (b) of this section.

Small Quantity Handler of Universal Waste means a universal waste handler (as defined in this section) who does not accumulate 5,000 kilograms or more of universal waste (batteries, pes- ticides, mercury-containing equip- ment, or lamps, calculated collec- tively) at any time.

## § 273.10

Thermostat means a temperature con- trol device that contains metallic mer- cury in an ampule attached to a bi- metal sensing element, and mercury- containing ampules that have been re- moved from these temperature control devices in compliance with the require- ments of 40 CFR 273.13(c)(2) or 273.33(c)(2).

Universal Waste means any of the fol- lowing hazardous wastes that are sub- ject to the universal waste require- ments of this part 273:

  - Batteries as described in § 273.2;

  - Pesticides as described in § 273.3;

  - Mercury-containing equipment as described in § 273.4; and

  - Lamps as described in § 273.5.

Universal Waste Handler:

    - Means:

      - A generator (as defined in this section) of universal waste; or

      - The owner or operator of a facil- ity, including all contiguous property, that receives universal waste from other universal waste handlers, accu- mulates universal waste, and sends universal waste to another universal waste handler, to a destination facil- ity, or to a foreign destination.

    - Does not mean:

      - A person who treats (except under the provisions of 40 CFR 273.13 (a) or (c), or 273.33 (a) or (c)), disposes of, or recycles universal waste; or

      - A person engaged in the off-site transportation of universal waste by air, rail, highway, or water, including a universal waste transfer facility.

Universal Waste Transfer Facility means any transportation-related fa- cility including loading docks, parking areas, storage areas and other similar areas where shipments of universal waste are held during the normal course of transportation for ten days or less.

Universal Waste Transporter means a person engaged in the off-site transpor- tation of universal waste by air, rail, highway, or water.

[60 FR 25542, May 11, 1995, as amended at 63

FR  71230,  Dec.  24,  1998.  Redesignated  and

amended at 64 FR 36488, 36489, July 6, 1999; 70

FR 45521, Aug. 5, 2005; 71 FR 40280, July 14,

2006]

## 40 CFR Ch. I (7–1–19 Edition)

Subpart B—Standards for Small Quantity Handlers of Universal Waste

### § 273.10   Applicability.

This subpart applies to small quan- tity handlers of universal waste (as de- fined in 40 CFR 273.9).

[64 FR 36489, July 6, 1999]

### § 273.11   Prohibitions.

A small quantity handler of universal waste is:

- Prohibited from disposing of uni- versal waste; and

- Prohibited from diluting or treat- ing universal waste, except by respond- ing to releases as provided in 40 CFR 273.17; or by managing specific wastes as provided in 40 CFR 273.13.

### § 273.12   Notification.

A small quantity handler of universal waste is not required to notify EPA of universal waste handling activities.

### § 273.13   Waste management.

- Universal waste batteries. A small quantity handler of universal waste must manage universal waste batteries in a way that prevents releases of any universal waste or component of a uni- versal waste to the environment, as follows:

  - A small quantity handler of uni- versal waste must contain any uni- versal waste battery that shows evi- dence of leakage, spillage, or damage that could cause leakage under reason- ably foreseeable conditions in a con- tainer. The container must be closed, structurally sound, compatible with the contents of the battery, and must lack evidence of leakage, spillage, or damage that could cause leakage under reasonably foreseeable conditions.

  - A small quantity handler of uni- versal waste may conduct the following activities as long as the casing of each individual battery cell is not breached and remains intact and closed (except that cells may be opened to remove electrolyte but must be immediately closed after removal):

    - Sorting batteries by type;

    - Mixing battery types in one con- tainer;