- Pay the appropriate per manifest fee to EPA for each manifest submitted to the e-Manifest system, subject to the fee determination methodology, payment methods, dispute procedures, sanctions, and other fee requirements specified in subpart FF of part 264 of this chapter.

[83 FR 451, Jan. 3, 2018]

# § 260.5 Applicability of electronic manifest system and user fee re- quirements to facilities receiving state-only regulated waste ship- ments.

  - For purposes of this section, ‘‘state-only regulated waste’’ means:

    - A non-RCRA waste that a state regulates more broadly under its state regulatory program, or

    - A RCRA hazardous waste that is federally exempt from manifest re- quirements, but not exempt from mani- fest requirements under state law.

  - In any case in which a state re- quires a RCRA manifest to be used under state law to track the shipment and transportation of a state-only reg- ulated waste to a receiving facility, the facility receiving such a waste ship- ment for management shall:

    - Comply with the provisions of

§§ 264.71 (use of the manifest) and 264.72 (manifest discrepancies) of this chap- ter; and

    - Pay the appropriate per manifest fee to EPA for each manifest submitted to the e-Manifest system, subject to the fee determination methodology, payment methods, dispute procedures, sanctions, and other fee requirements specified in subpart FF of part 264 of this chapter.

[83 FR 451, Jan. 3, 2018]

# Subpart B—Definitions

# § 260.10  Definitions.

When used in parts 260 through 273 of this chapter, the following terms have the meanings given below:

Above ground tank means a device meeting the definition of ‘‘tank’’ in

§ 260.10 and that is situated in such a way that the entire surface area of the tank is completely above the plane of the adjacent surrounding surface and the entire surface area of the tank (in-

cluding the tank bottom) is able to be visually inspected.

Act or RCRA means the Solid Waste Disposal Act, as amended by the Re- source Conservation and Recovery Act of 1976, as amended, 42 U.S.C. section 6901 et seq.

Active life of a facility means the pe- riod from the initial receipt of haz- ardous waste at the facility until the Regional Administrator receives cer- tification of final closure.

Active portion means that portion of a facility where treatment, storage, or disposal operations are being or have been conducted after the effective date of part 261 of this chapter and which is not a closed portion. (See also ‘‘closed portion’’ and ‘‘inactive portion’’.)

Acute hazardous waste means haz- ardous wastes that meet the listing cri- teria in § 261.11(a)(2) and therefore are either listed in § 261.31 of this chapter with the assigned hazard code of (H) or are listed in § 261.33(e) of this chapter.

Administrator means the Adminis- trator of the Environmental Protection Agency, or his designee.

Aerosol can means a non-refillable re- ceptacle containing a gas compressed, liquefied, or dissolved under pressure, the sole purpose of which is to expel a liquid, paste, or powder and fitted with a self-closing release device allowing the contents to be ejected by the gas.

AES filing compliance date means the date that EPA announces in the FED- ERAL REGISTER, on or after which ex- porters of hazardous waste and export- ers of cathode ray tubes for recycling are required to file EPA information in the Automated Export System or its successor system, under the Inter- national Trade Data System (ITDS) platform.

Airbag waste means any hazardous waste airbag modules or hazardous waste airbag inflators.

Airbag waste collection facility means any facility that receives airbag waste from airbag handlers subject to regula- tion under § 261.4(j) of this chapter, and accumulates the waste for more than ten days.

Airbag waste handler means any per- son, by site, who generates airbag waste that is subject to regulation under this chapter.

Ancillary equipment means any device including, but not limited to, such devices as piping, fittings, flanges, valves, and pumps, that is used to dis- tribute, meter, or control the flow of hazardous waste from its point of gen- eration to a storage or treatment tank(s), between hazardous waste stor- age and treatment tanks to a point of disposal onsite, or to a point of ship- ment for disposal off-site.

Aquifer means a geologic formation, group of formations, or part of a forma- tion capable of yielding a significant amount of ground water to wells or springs.

Authorized representative means the person responsible for the overall oper- ation of a facility or an operational unit (i.e., part of a facility), e.g., the plant manager, superintendent or per- son of equivalent responsibility.

Battery means a device consisting of one or more electrically connected electrochemical cells which is designed to receive, store, and deliver electric energy. An electrochemical cell is a system consisting of an anode, cathode, and an electrolyte, plus such connec- tions (electrical and mechanical) as may be needed to allow the cell to de- liver or receive electrical energy. The term battery also includes an intact, unbroken battery from which the elec- trolyte has been removed.

Boiler means an enclosed device using controlled flame combustion and hav- ing the following characteristics:

(1)(i) The unit must have physical provisions for recovering and exporting thermal energy in the form of steam, heated fluids, or heated gases; and

- The unit’s combustion chamber and primary energy recovery sec- tions(s) must be of integral design. To be of integral design, the combustion chamber and the primary energy recov- ery section(s) (such as waterwalls and superheaters) must be physically formed into one manufactured or as- sembled unit. A unit in which the com- bustion chamber and the primary en- ergy recovery section(s) are joined only by ducts or connections carrying flue gas is not integrally designed; however, secondary energy recovery equipment (such as economizers or air preheaters) need not be physically formed into the same unit as the combustion chamber

and the primary energy recovery sec- tion. The following units are not pre- cluded from being boilers solely be- cause they are not of integral design: process heaters (units that transfer en- ergy directly to a process stream), and fluidized bed combustion units; and

- While in operation, the unit must maintain a thermal energy recov- ery efficiency of at least 60 percent, calculated in terms of the recovered energy compared with the thermal value of the fuel; and

- The unit must export and utilize at least 75 percent of the recovered en- ergy, calculated on an annual basis. In this calculation, no credit shall be given for recovered heat used inter- nally in the same unit. (Examples of internal use are the preheating of fuel or combustion air, and the driving of induced or forced draft fans or feedwater pumps); or

- The unit is one which the Re- gional Administrator has determined, on a case-by-case basis, to be a boiler, after considering the standards in

§ 260.32.

Carbon dioxide stream means carbon dioxide that has been captured from an emission source (e.g., power plant), plus incidental associated substances derived from the source materials and the capture process, and any sub- stances added to the stream to enable or improve the injection process.

Carbon regeneration unit means any enclosed thermal treatment device used to regenerate spent activated car- bon.

Cathode ray tube or CRT means a vac- uum tube, composed primarily of glass, which is the visual or video display component of an electronic device. A used, intact CRT means a CRT whose vacuum has not been released. A used, broken CRT means glass removed from its housing or casing whose vacuum has been released.

Central accumulation area means any on-site hazardous waste accumulation area with hazardous waste accumu- lating in units subject to either § 262.16 (for  small  quantity  generators)  or

§ 262.17 of this chapter (for large quan- tity generators). A central accumula- tion area at an eligible academic enti- ty that chooses to operate under 40 CFR part 262 subpart K is also subject

to § 262.211 when accumulating un- wanted material and/or hazardous waste.

Certification means a statement of professional opinion based upon knowl- edge and belief.

Closed portion means that portion of a facility which an owner or operator has closed in accordance with the approved facility closure plan and all applicable closure requirements. (See also ‘‘active portion’’ and ‘‘inactive portion’’.)

Component means either the tank or ancillary equipment of a tank system. Confined aquifer means an aquifer bounded above and below by imper- meable beds or by beds of distinctly lower permeability than that of the aq- uifer itself; an aquifer containing con-

fined ground water.

Contained means held in a unit (in- cluding a land-based unit as defined in this subpart) that meets the following criteria:

  - The unit is in good condition, with no leaks or other continuing or intermittent unpermitted releases of the hazardous secondary materials to the environment, and is designed, as appropriate for the hazardous sec- ondary materials, to prevent releases of hazardous secondary materials to the environment. Unpermitted releases are releases that are not covered by a permit (such as a permit to discharge to water or air) and may include, but are not limited to, releases through surface transport by precipitation run- off, releases to soil and groundwater, wind-blown dust, fugitive air emis- sions, and catastrophic unit failures;

  - The unit is properly labeled or otherwise has a system (such as a log) to immediately identify the hazardous secondary materials in the unit; and

  - The unit holds hazardous sec- ondary materials that are compatible with other hazardous secondary mate- rials placed in the unit and is compat- ible with the materials used to con- struct the unit and addresses any po- tential risks of fires or explosions.

  - Hazardous secondary materials in units that meet the applicable require- ments of 40 CFR parts 264 or 265 are presumptively contained.

Container means any portable device in which a material is stored, trans-

ported, treated, disposed of, or other- wise handled.

Containment building means a haz- ardous waste management unit that is used to store or treat hazardous waste under the provisions of subpart DD of parts 264 or 265 of this chapter.

Contingency plan means a document setting out an organized, planned, and coordinated course of action to be fol- lowed in case of a fire, explosion, or re- lease of hazardous waste or hazardous waste constituents which could threat- en human health or the environment.

Corrosion expert means a person who, by reason of his knowledge of the phys- ical sciences and the principles of engi- neering and mathematics, acquired by a professional education and related practical experience, is qualified to en- gage in the practice of corrosion con- trol on buried or submerged metal pip- ing systems and metal tanks. Such a person must be certified as being quali- fied by the National Association of Corrosion Engineers (NACE) or be a registered professional engineer who has certification or licensing that in- cludes education and experience in cor- rosion control on buried or submerged metal piping systems and metal tanks. CRT collector means a person who re- ceives used, intact CRTs for recycling,

repair, resale, or donation.

CRT exporter means any person in the United States who initiates a trans- action to send used CRTs outside the United States or its territories for re- cycling or reuse, or any intermediary in the United States arranging for such export.

CRT glass manufacturer means an op- eration or part of an operation that uses a furnace to manufacture CRT glass.

CRT processing means conducting all of the following activities:

- Receiving broken or intact CRTs; and

- Intentionally breaking intact CRTs or further breaking or separating broken CRTs; and

- Sorting or otherwise managing glass removed from CRT monitors.

Designated facility means:

- A hazardous waste treatment, storage, or disposal facility which:

  - Has received a permit (or interim status) in accordance with the require- ments of parts 270 and 124 of this chap- ter;

  - Has received a permit (or interim status) from a State authorized in ac- cordance with part 271 of this chapter; or

  - Is regulated under § 261.6(c)(2) or subpart F of part 266 of this chapter; and

  - That has been designated on the manifest by the generator pursuant to

§ 262.20.

- Designated facility also means a generator site designated on the mani- fest to receive its waste as a return shipment from a facility that has re- jected the waste in accordance with

§ 264.72(f) or § 265.72(f) of this chapter.

- If a waste is destined to a facility in an authorized State which has not yet obtained authorization to regulate that particular waste as hazardous, then the designated facility must be a facility allowed by the receiving State to accept such waste.

Destination facility means a facility that treats, disposes of, or recycles a particular category of universal waste, except those management activities described in paragraphs (a) and (c) of

§§ 273.13 and 273.33 of this chapter. A fa- cility at which a particular category of universal waste is only accumulated, is not a destination facility for purposes of managing that category of universal waste.

Dike means an embankment or ridge of either natural or man-made mate- rials used to prevent the movement of liquids, sludges, solids, or other mate- rials.

Dioxins and furans (D/F) means tetra, penta,	hexa,	hepta,	and	octa- chlorinated dibenzo dioxins and furans. Discharge or hazardous waste discharge means the accidental or intentional spilling, leaking, pumping, pouring, emitting, emptying, or dumping of haz- ardous waste into or on any land or

water.

Disposal means the discharge, de- posit, injection, dumping, spilling, leaking, or placing of any solid waste or hazardous waste into or on any land or water so that such solid waste or hazardous waste or any constituent thereof may enter the environment or

be emitted into the air or discharged into any waters, including ground waters.

Disposal facility means a facility or part of a facility at which hazardous waste is intentionally placed into or on any land or water, and at which waste will remain after closure. The term dis- posal facility does not include a correc- tive action management unit into which remediation wastes are placed.

Drip pad is an engineered structure consisting of a curbed, free-draining base, constructed of non-earthen mate- rials and designed to convey preserva- tive kick-back or drippage from treat- ed wood, precipitation, and surface water run-on to an associated collec- tion system at wood preserving plants.

Electronic import-export reporting com- pliance date means the date that EPA announces in the FEDERAL REGISTER, on or after which exporters, importers, and receiving facilities are required to submit certain export and import re- lated documents to EPA using EPA’s Waste Import Export Tracking System, or its successor system.

Electronic manifest (or e-Manifest) means the electronic format of the haz- ardous waste manifest that is obtained from EPA’s national e-Manifest system and transmitted electronically to the system, and that is the legal equiva- lent of EPA Forms 8700–22 (Manifest) and 8700–22A (Continuation Sheet).

Electronic Manifest System (or e-Mani- fest System) means EPA’s national in- formation technology system through which the electronic manifest may be obtained, completed, transmitted, and distributed to users of the electronic manifest and to regulatory agencies.

Elementary neutralization unit means a device which:

- Is used for neutralizing wastes that are hazardous only because they exhibit the corrosivity characteristic defined in § 261.22 of this chapter, or they are listed in subpart D of part 261 of the chapter only for this reason; and

- Meets the definition of tank, tank system, container, transport vehicle, or vessel in § 260.10 of this chapter.

EPA hazardous waste number means the number assigned by EPA to each hazardous waste listed in part 261, sub- part D, of this chapter and to each

characteristic identified in part 261, subpart C, of this chapter.

EPA identification number means the number assigned by EPA to each gener- ator, transporter, and treatment, stor- age, or disposal facility.

EPA region means the states and ter- ritories found in any one of the fol- lowing ten regions:

Region I—Maine, Vermont, New Hampshire, Massachusetts, Connecticut, and Rhode Is- land.

Region II—New York, New Jersey, Common- wealth of Puerto Rico, and the U.S. Virgin Islands.

Region III—Pennsylvania, Delaware, Mary- land, West Virginia, Virginia, and the Dis- trict of Columbia.

Region IV—Kentucky, Tennessee, North Carolina, Mississippi, Alabama, Georgia, South Carolina, and Florida.

Region V—Minnesota, Wisconsin, Illinois, Michigan, Indiana and Ohio.

Region VI—New Mexico, Oklahoma, Arkan- sas, Louisiana, and Texas.

Region VII—Nebraska, Kansas, Missouri, and Iowa.

Region VIII—Montana, Wyoming, North Da- kota, South Dakota, Utah, and Colorado.

Region IX—California, Nevada, Arizona, Ha- waii, Guam, American Samoa, Common- wealth of the Northern Mariana Islands.

Region X—Washington, Oregon, Idaho, and Alaska.

Equivalent method means any testing or analytical method approved by the Administrator under §§ 260.20 and 260.21. Existing hazardous waste management (HWM) facility or existing facility means a facility which was in operation or for which construction commenced on or before November 19, 1980. A facility has

commenced construction if:

(1) The owner or operator has ob- tained the Federal, State and local ap- provals or permits necessary to begin physical construction; and either

(2)(i) A continuous on-site, physical construction program has begun; or

(ii) The owner or operator has en- tered into contractual obligations— which cannot be cancelled or modified without substantial loss—for physical construction of the facility to be com- pleted within a reasonable time.

Existing portion means that land sur- face area of an existing waste manage- ment unit, included in the original Part A permit application, on which wastes have been placed prior to the issuance of a permit.

Existing tank system or existing compo- nent means a tank system or compo- nent that is used for the storage or treatment of hazardous waste and that is in operation, or for which installa- tion has commenced on or prior to July 14, 1986. Installation will be considered to have commenced if the owner or op- erator has obtained all Federal, State, and local approvals or permits nec- essary to begin physical construction of the site or installation of the tank system and if either (1) a continuous on-site physical construction or instal- lation program has begun, or (2) the owner or operator has entered into con- tractual obligations—which cannot be canceled or modified without substan- tial loss—for physical construction of the site or installation of the tank sys- tem to be completed within a reason- able time.

Explosives or munitions emergency means a situation involving the sus- pected or detected presence of unexploded ordnance (UXO), damaged or deteriorated explosives or muni- tions, an improvised explosive device (IED), other potentially explosive ma- terial or device, or other potentially harmful military chemical munitions or device, that creates an actual or po- tential imminent threat to human health, including safety, or the envi- ronment, including property, as deter- mined by an explosives or munitions emergency response specialist. Such situations may require immediate and expeditious action by an explosives or munitions emergency response spe- cialist to control, mitigate, or elimi- nate the threat.

Explosives or munitions emergency re- sponse means all immediate response activities by an explosives and muni- tions emergency response specialist to control, mitigate, or eliminate the ac- tual or potential threat encountered during an explosives or munitions emergency. An explosives or munitions emergency response may include in- place render-safe procedures, treat- ment or destruction of the explosives or munitions and/or transporting those items to another location to be ren- dered safe, treated, or destroyed. Any reasonable delay in the completion of an explosives or munitions emergency

response caused by a necessary, unfore- seen, or uncontrollable circumstance will not terminate the explosives or munitions emergency. Explosives and munitions emergency responses can occur on either public or private lands and are not limited to responses at RCRA facilities.

Explosives or munitions emergency re- sponse specialist means an individual trained in chemical or conventional munitions or explosives handling, transportation, render-safe procedures, or destruction techniques. Explosives or munitions emergency response spe- cialists include Department of Defense (DOD) emergency explosive ordnance disposal (EOD), technical escort unit (TEU), and DOD-certified civilian or contractor personnel; and other Fed- eral, State, or local government, or ci- vilian personnel similarly trained in explosives or munitions emergency re- sponses.

Facility means:

- All contiguous land, and struc- tures, other appurtenances, and im- provements on the land, used for treat- ing, storing, or disposing of hazardous waste, or for managing hazardous sec- ondary materials prior to reclamation. A facility may consist of several treat- ment, storage, or disposal operational units (e.g., one or more landfills, sur- face impoundments, or combinations of them).

- For the purpose of implementing corrective action under 40 CFR 264.101 or 267.101, all contiguous property under the control of the owner or oper- ator seeking a permit under Subtitle C of RCRA. This definition also applies to facilities implementing corrective action under RCRA Section 3008(h).

- Notwithstanding paragraph (2) of this definition, a remediation waste management site is not a facility that is subject to 40 CFR 264.101, but is sub- ject to corrective action requirements if the site is located within such a fa- cility.

Federal agency means any depart- ment, agency, or other instrumentality of the Federal Government, any inde- pendent agency or establishment of the Federal Government including any Government corporation, and the Gov- ernment Printing Office.

Federal, State and local approvals or permits necessary to begin physical con- struction means permits and approvals required under Federal, State or local hazardous waste control statutes, regu- lations or ordinances.

Final closure means the closure of all hazardous waste management units at the facility in accordance with all ap- plicable closure requirements so that hazardous waste management activi- ties under parts 264 and 265 of this chapter are no longer conducted at the facility unless subject to the provisions in §§ 262.16 and 262.17.

Food-chain crops means tobacco, crops grown for human consumption, and crops grown for feed for animals whose products are consumed by hu- mans.

Free liquids means liquids which read- ily separate from the solid portion of a waste under ambient temperature and pressure.

Freeboard means the vertical distance between the top of a tank or surface impoundment dike, and the surface of the waste contained therein.

Generator means any person, by site, whose act or process produces haz- ardous waste identified or listed in part

261 of this chapter or whose act first causes a hazardous waste to become subject to regulation.

Ground water means water below the land surface in a zone of saturation.

Hazardous secondary material means a secondary material (e.g., spent mate- rial, by-product, or sludge) that, when discarded, would be identified as haz- ardous waste under part 261 of this chapter.

Hazardous secondary material gener- ator means any person whose act or process produces hazardous secondary materials at the generating facility. For purposes of this paragraph, ‘‘gen- erating facility’’ means all contiguous property owned, leased, or otherwise controlled by the hazardous secondary material generator. For the purposes of

§ 261.2(a)(2)(ii) and § 261.4(a)(23), a facil- ity that collects hazardous secondary materials from other persons is not the hazardous secondary material gener- ator.

Hazardous waste means a hazardous waste as defined in § 261.3 of this chap- ter.

Hazardous waste constituent means a constituent that caused the Adminis- trator to list the hazardous waste in part 261, subpart D, of this chapter, or a constituent listed in table 1 of § 261.24 of this chapter.

Hazardous waste management unit is a contiguous area of land on or in which hazardous waste is placed, or the larg- est area in which there is significant likelihood of mixing hazardous waste constituents in the same area. Exam- ples of hazardous waste management units include a surface impoundment, a waste pile, a land treatment area, a landfill cell, an incinerator, a tank and its associated piping and underlying containment system and a container storage area. A container alone does not constitute a unit; the unit includes containers and the land or pad upon which they are placed.

In operation refers to a facility which

is treating, storing, or disposing of haz- ardous waste.

Inactive portion means that portion of a facility which is not operated after the effective date of part 261 of this chapter. (See also ‘‘active portion’’ and ‘‘closed portion’’.)

Incinerator means any enclosed de- vice that:

- Uses controlled flame combustion and neither meets the criteria for clas- sification as a boiler, sludge dryer, or carbon regeneration unit, nor is listed as an industrial furnace; or

- Meets the definition of infrared incinerator or plasma arc incinerator.

Incompatible waste means a hazardous waste which is unsuitable for:

- Placement in a particular device or facility because it may cause corro- sion or decay of containment materials (e.g., container inner liners or tank walls); or

- Commingling with another waste or material under uncontrolled condi- tions because the commingling might produce heat or pressure, fire or explo- sion, violent reaction, toxic dusts, mists, fumes, or gases, or flammable fumes or gases.

(See appendix V of parts 264 and 265 of this chapter for examples.)

Individual generation site means the contiguous site at or on which one or more hazardous wastes are generated. An individual generation site, such as a

large manufacturing plant, may have one or more sources of hazardous waste but is considered a single or individual generation site if the site or property is contiguous.

Industrial furnace means any of the following enclosed devices that are in- tegral components of manufacturing processes and that use thermal treat- ment to accomplish recovery of mate- rials or energy:

- Cement kilns

- Lime kilns

- Aggregate kilns

- Phosphate kilns

- Coke ovens

- Blast furnaces

- Smelting, melting and refining furnaces (including pyrometallurgical devices such as cupolas, reverberator furnaces, sintering machine, roasters, and foundry furnaces)

- Titanium dioxide chloride process oxidation reactors

- Methane reforming furnaces

- Pulping liquor recovery furnaces

- Combustion devices used in the recovery of sulfur values from spent sulfuric acid

- Halogen acid furnaces (HAFs) for the production of acid from halo- genated hazardous waste generated by chemical production facilities where the furnace is located on the site of a chemical production facility, the acid product has a halogen acid content of at least 3%, the acid product is used in a manufacturing process, and, except for hazardous waste burned as fuel, hazardous waste fed to the furnace has a minimum halogen content of 20% as- generated.

- Such other devices as the Admin- istrator may, after notice and com- ment, add to this list on the basis of one or more of the following factors:

  - The design and use of the device primarily to accomplish recovery of material products;

  - The use of the device to burn or reduce raw materials to make a mate- rial product;

  - The use of the device to burn or reduce secondary materials as effective substitutes for raw materials, in proc- esses using raw materials as principal feedstocks;

  - The use of the device to burn or reduce secondary materials as ingredi- ents in an industrial process to make a material product;

  - The use of the device in common industrial practice to produce a mate- rial product; and

  - Other factors, as appropriate.

Infrared incinerator means any en- closed device that uses electric pow- ered resistance heaters as a source of radiant heat followed by an afterburner using controlled flame combustion and which is not listed as an industrial fur- nace.

Inground tank means a device meet- ing the definition of ‘‘tank’’ in § 260.10 whereby a portion of the tank wall is situated to any degree within the ground, thereby preventing visual in- spection of that external surface area of the tank that is in the ground.

Injection well means a well into which fluids are injected. (See also ‘‘under- ground injection’’.)

Inner liner means a continuous layer of material placed inside a tank or con- tainer which protects the construction materials of the tank or container from the contained waste or reagents used to treat the waste.

Installation inspector means a person who, by reason of his knowledge of the physical sciences and the principles of engineering, acquired by a professional education and related practical experi- ence, is qualified to supervise the in- stallation of tank systems.

Intermediate facility means any facil- ity that stores hazardous secondary materials for more than 10 days, other than a hazardous secondary material generator or reclaimer of such mate- rial.

International shipment means the transportation of hazardous waste into or out of the jurisdiction of the United States.

Lamp, also referred to as ‘‘universal waste lamp’’, is defined as the bulb or tube portion of an electric lighting de- vice. A lamp is specifically designed to produce radiant energy, most often in the ultraviolet, visible, and infra-red regions of the electromagnetic spec- trum. Examples of common universal waste electric lamps include, but are not limited to, fluorescent, high inten- sity discharge, neon, mercury vapor,

high pressure sodium, and metal halide lamps.

Land-based unit means an area where hazardous secondary materials are placed in or on the land before recy- cling. This definition does not include land-based production units.

Landfill means a disposal facility or part of a facility where hazardous waste is placed in or on land and which is not a pile, a land treatment facility, a surface impoundment, an under- ground injection well, a salt dome for- mation, a salt bed formation, an under- ground mine, a cave, or a corrective ac- tion management unit.

Landfill cell means a discrete volume of a hazardous waste landfill which uses a liner to provide isolation of wastes from adjacent cells or wastes. Examples of landfill cells are trenches and pits.

Land treatment facility means a facil- ity or part of a facility at which haz- ardous waste is applied onto or incor- porated into the soil surface; such fa- cilities are disposal facilities if the waste will remain after closure.

Large quantity generator is a gener- ator who generates any of the fol- lowing amounts in a calendar month:

- Greater than or equal to 1,000 kilograms (2200 lbs) of non-acute haz- ardous waste; or

- Greater than 1 kilogram (2.2 lbs) of acute hazardous waste listed in

§ 261.31 or § 261.33(e) of this chapter; or

- Greater than 100 kilograms (220 lbs) of any residue or contaminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute haz- ardous  waste  listed  in  § 261.31  or

§ 261.33(e) of this chapter.

Leachate means any liquid, including any suspended components in the liq- uid, that has percolated through or drained from hazardous waste.

Leak-detection system means a system capable of detecting the failure of ei- ther the primary or secondary contain- ment structure or the presence of a re- lease of hazardous waste or accumu- lated liquid in the secondary contain- ment structure. Such a system must employ operational controls (e.g., daily visual inspections for releases into the secondary  containment  system  of

aboveground tanks) or consist of an in- terstitial monitoring device designed to detect continuously and automati- cally the failure of the primary or sec- ondary containment structure or the presence of a release of hazardous waste into the secondary containment structure.

Liner means a continuous layer of natural or man-made materials, be- neath or on the sides of a surface im- poundment, landfill, or landfill cell, which restricts the downward or lat- eral escape of hazardous waste, haz- ardous waste constituents, or leachate. Management or hazardous waste man- agement means the systematic control of the collection, source separation, storage,	transportation,	processing, treatment, recovery, and disposal of

hazardous waste.

Manifest means the shipping docu- ment EPA Form 8700–22 (including, if necessary, EPA Form 8700–22A), or the electronic manifest, originated and signed in accordance with the applica- ble requirements of parts 262 through 265 of this chapter.

Manifest tracking number means: The alphanumeric identification number (i.e., a unique three letter suffix pre- ceded by nine numerical digits), which is pre-printed in Item 4 of the Manifest by a registered source.

Mercury-containing equipment means a device or part of a device (including thermostats, but excluding batteries and lamps) that contains elemental mercury integral to its function.

Military munitions means all ammuni- tion products and components pro- duced or used by or for the U.S. De- partment of Defense or the U.S. Armed Services for national defense and secu- rity, including military munitions under the control of the Department of Defense, the U.S. Coast Guard, the U.S. Department of Energy (DOE), and Na- tional Guard personnel. The term mili- tary munitions includes: confined gas- eous, liquid, and solid propellants, ex- plosives, pyrotechnics, chemical and riot control agents, smokes, and incen- diaries used by DOD components, in- cluding bulk explosives and chemical warfare agents, chemical munitions, rockets, guided and ballistic missiles, bombs, warheads, mortar rounds, artil- lery ammunition, small arms ammuni-

tion, grenades, mines, torpedoes, depth charges, cluster munitions and dis- pensers, demolition charges, and de- vices and components thereof. Military munitions do not include wholly inert items, improvised explosive devices, and nuclear weapons, nuclear devices, and nuclear components thereof. How- ever, the term does include non-nuclear components of nuclear devices, man- aged under DOE’s nuclear weapons pro- gram after all required sanitization op- erations under the Atomic Energy Act of 1954, as amended, have been com- pleted.

Mining overburden returned to the mine site means any material overlying an economic mineral deposit which is re- moved to gain access to that deposit and is then used for reclamation of a surface mine.

Miscellaneous unit means a hazardous waste management unit where haz- ardous waste is treated, stored, or dis- posed of and that is not a container, tank, surface impoundment, pile, land treatment unit, landfill, incinerator, boiler, industrial furnace, underground injection well with appropriate tech- nical standards under part 146 of this chapter, containment building, correc- tive action management unit, unit eli- gible for a research, development, and demonstration permit under 40 CFR 270.65, or staging pile.

Movement  means  that  hazardous

waste transported to a facility in an in- dividual vehicle.

New hazardous waste management fa- cility or new facility means a facility which began operation, or for which construction commenced after Novem- ber 19, 1980. (See also ‘‘Existing haz- ardous waste management facility’’.)

New tank system or new tank compo- nent means a tank system or compo- nent that will be used for the storage or treatment of hazardous waste and for which installation has commenced after July 14, 1986; except, however, for purposes   of   § 264.193(g)(2)   and

§ 265.193(g)(2), a new tank system is one for which construction commences after July 14, 1986. (See also ‘‘existing tank system.’’)

No free liquids, as used in 40 CFR 261.4(a)(26)  and  40  CFR  261.4(b)(18),

means that solvent-contaminated wipes may not contain free liquids as

determined by Method 9095B (Paint Fil- ter Liquids Test), included in ‘‘Test Methods for Evaluating Solid Waste, Physical/Chemical Methods’’ (EPA Publication SW–846), which is incor- porated by reference, and that there is no free liquid in the container holding the wipes. No free liquids may also be determined using another standard or test method as defined by an author- ized state.

Non-acute hazardous waste means all hazardous wastes that are not acute hazardous waste, as defined in this sec- tion.

On ground tank means a device meet- ing the definition of ‘‘tank’’ in § 260.10 and that is situated in such a way that the bottom of the tank is on the same level as the adjacent surrounding sur- face so that the external tank bottom cannot be visually inspected.

On-site means the same or geographi- cally contiguous property which may be divided by public or private right-of- way, provided the entrance and exit be- tween the properties is at a cross-roads intersection, and access is by crossing as opposed to going along, the right-of- way. Non-contiguous properties owned by the same person but connected by a right-of-way which he controls and to which the public does not have access, is also considered on-site property.

Open burning means the combustion of any material without the following characteristics:

- Control of combustion air to maintain adequate temperature for ef- ficient combustion,

- Containment of the combustion- reaction in an enclosed device to pro- vide sufficient residence time and mix- ing for complete combustion, and

- Control of emission of the gaseous combustion products.

(See also ‘‘incineration’’ and ‘‘ther- mal treatment’’.)

Operator means the person respon- sible for the overall operation of a fa- cility.

Owner means the person who owns a facility or part of a facility.

Partial closure means the closure of a hazardous waste management unit in accordance with the applicable closure requirements of parts 264 and 265 of this chapter at a facility that contains other active hazardous waste manage-

ment units. For example, partial clo- sure may include the closure of a tank (including its associated piping and un- derlying containment systems), land- fill cell, surface impoundment, waste pile, or other hazardous waste manage- ment unit, while other units of the same facility continue to operate.

Person means an individual, trust, firm, joint stock company, Federal Agency, corporation (including a gov- ernment corporation), partnership, as- sociation, State, municipality, com- mission, political subdivision of a State, or any interstate body.

Personnel or facility personnel means all persons who work at, or oversee the operations of, a hazardous waste facil- ity, and whose actions or failure to act may result in noncompliance with the requirements of part 264 or 265 of this chapter.

Pesticide means any substance or mixture of substances intended for pre- venting, destroying, repelling, or miti- gating any pest, or intended for use as a plant regulator, defoliant, or des- iccant, other than any article that:

- Is a new animal drug under FFDCA section 201(w), or

- Is an animal drug that has been determined by regulation of the Sec- retary of Health and Human Services not to be a new animal drug, or

- Is an animal feed under FFDCA section 201(x) that bears or contains any substances described by paragraph

(1) or (2) of this definition.

Pile means any non-containerized ac- cumulation of solid, nonflowing haz- ardous waste that is used for treatment or storage and that is not a contain- ment building.

Plasma arc incinerator means any en- closed device using a high intensity electrical discharge or arc as a source of heat followed by an afterburner using controlled flame combustion and which is not listed as an industrial fur- nace.

Point source means any discernible, confined, and discrete conveyance, in- cluding, but not limited to any pipe, ditch, channel, tunnel, conduit, well, discrete fissure, container, rolling stock, concentrated animal feeding op- eration, or vessel or other floating craft, from which pollutants are or may be discharged. This term does not

include return flows from irrigated ag- riculture.

Publicly owned treatment works or POTW means any device or system used in the treatment (including recy- cling and reclamation) of municipal sewage or industrial wastes of a liquid nature which is owned by a ‘‘State’’ or ‘‘municipality’’ (as defined by section 502(4) of the CWA). This definition in- cludes sewers, pipes, or other convey- ances only if they convey wastewater to a POTW providing treatment.

Qualified Ground-Water Scientist means a scientist or engineer who has received a baccalaureate or post-grad- uate degree in the natural sciences or engineering, and has sufficient training and experience in ground-water hydrol- ogy and related fields as may be dem- onstrated by state registration, profes- sional certifications, or completion of accredited university courses that en- able that individual to make sound professional judgements regarding ground-water monitoring and contami- nant fate and transport.

Recognized  trader  means  a  person

domiciled in the United States, by site of business, who acts to arrange and fa- cilitate transboundary movements of wastes destined for recovery or dis- posal operations, either by purchasing from	and	subsequently	selling	to United States and foreign facilities, or by acting under arrangements with a United States waste facility to arrange for the export or import of the wastes. Regional Administrator means the Re- gional Administrator for the EPA Re- gion in which the facility is located, or

his designee.

Remanufacturing means processing a higher-value hazardous secondary ma- terial in order to manufacture a prod- uct that serves a similar functional purpose as the original commercial- grade material. For the purpose of this definition, a hazardous secondary ma- terial is considered higher-value if it was generated from the use of a com- mercial-grade material in a manufac- turing process and can be remanufac- tured into a similar commercial-grade material.

Remediation waste means all solid and hazardous wastes, and all media (in- cluding ground water, surface water, soils, and sediments) and debris, that

are managed for implementing clean- up.

Remediation waste management site means a facility where an owner or op- erator is or will be treating, storing or disposing of hazardous remediation wastes. A remediation waste manage- ment site is not a facility that is sub- ject to corrective action under 40 CFR 264.101, but is subject to corrective ac- tion requirements if the site is located in such a facility.

Replacement unit means a landfill, surface impoundment, or waste pile unit (1) from which all or substantially all of the waste is removed, and (2) that is subsequently reused to treat, store, or dispose of hazardous waste. ‘‘Re- placement unit’’ does not apply to a unit from which waste is removed dur- ing closure, if the subsequent reuse solely involves the disposal of waste from that unit and other closing units or corrective action areas at the facil- ity, in accordance with an approved closure plan or EPA or State approved corrective action.

Representative sample means a sample

of a universe or whole (e.g., waste pile, lagoon, ground water) which can be ex- pected to exhibit the average prop- erties of the universe or whole.

Run-off means any rainwater, leach- ate, or other liquid that drains over land from any part of a facility.

Run-on means any rainwater, leach- ate, or other liquid that drains over land onto any part of a facility.

Saturated zone or zone of saturation means that part of the earth’s crust in which all voids are filled with water.

Sludge means any solid, semi-solid, or liquid waste generated from a munic- ipal, commercial, or industrial waste- water treatment plant, water supply treatment plant, or air pollution con- trol facility exclusive of the treated ef- fluent from a wastewater treatment plant.

Sludge dryer means any enclosed ther- mal treatment device that is used to dehydrate sludge and that has a max- imum total thermal input, excluding the heating value of the sludge itself, of 2,500 Btu/lb of sludge treated on a wet-weight basis.

Small quantity generator is a generator who generates the following amounts in a calendar month:

- Greater than 100 kilograms (220 lbs) but less than 1,000 kilograms (2200 lbs) of non-acute hazardous waste; and

- Less than or equal to 1 kilogram (2.2 lbs) of acute hazardous waste listed in § 261.31 or § 261.33(e) of this chapter; and

- Less than or equal to 100 kilo- grams (220 lbs) of any residue or con- taminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute hazardous waste listed in § 261.31 or § 261.33(e) of this chapter.

Solid waste means a solid waste as de- fined in § 261.2 of this chapter.

Solvent-contaminated wipe means—

- A wipe that, after use or after cleaning up a spill, either:

  - Contains one or more of the F001 through F005 solvents listed in 40 CFR

261.31 or the corresponding P- or U- listed solvents found in 40 CFR 261.33;

  - Exhibits a hazardous char- acteristic found in 40 CFR part 261 sub- part C when that characteristic results from a solvent listed in 40 CFR part 261; and/or

  - Exhibits only the hazardous waste characteristic of ignitability found in 40 CFR 261.21 due to the pres- ence of one or more solvents that are not listed in 40 CFR part 261.

- Solvent-contaminated wipes that contain listed hazardous waste other than solvents, or exhibit the char- acteristic of toxicity, corrosivity, or reactivity due to contaminants other than solvents, are not eligible for the exclusions at 40 CFR 261.4(a)(26) and 40 CFR 261.4(b)(18).

Sorbent means a material that is used to soak up free liquids by either ad- sorption or absorption, or both. Sorb means to either adsorb or absorb, or both.

Staging pile means an accumulation of solid, non-flowing remediation waste (as defined in this section) that is not a containment building and that is used only during remedial operations for temporary storage at a facility. Staging piles must be designated by the Director according to the require- ments of 40 CFR 264.554.

State means any of the several States, the District of Columbia, the Commonwealth of Puerto Rico, the Virgin  Islands,  Guam,  American

Samoa, and the Commonwealth of the Northern Mariana Islands.

Storage means the holding of haz- ardous waste for a temporary period, at the end of which the hazardous waste is treated, disposed of, or stored else- where.

Sump means any pit or reservoir that meets the definition of tank and those troughs/trenches connected to it that serve to collect hazardous waste for transport to hazardous waste storage, treatment, or disposal facilities; except that as used in the landfill, surface im- poundment, and waste pile rules, ‘‘sump’’ means any lined pit or res- ervoir that serves to collect liquids drained from a leachate collection and removal system or leak detection sys- tem for subsequent removal from the system.

Surface impoundment or impoundment

means a facility or part of a facility which is a natural topographic depres- sion, man-made excavation, or diked area formed primarily of earthen mate- rials (although it may be lined with man-made materials), which is de- signed to hold an accumulation of liq- uid wastes or wastes containing free liquids, and which is not an injection well. Examples of surface impound- ments are holding, storage, settling, and aeration pits, ponds, and lagoons.

Tank means a stationary device, de- signed to contain an accumulation of hazardous waste which is constructed primarily of non-earthen materials (e.g., wood, concrete, steel, plastic) which provide structural support.

Tank system means a hazardous waste storage or treatment tank and its asso- ciated ancillary equipment and con- tainment system.

TEQ means toxicity equivalence, the international method of relating the toxicity of various dioxin/furan congeners to the toxicity of 2,3,7,8- tetrachlorodibenzo-p-dioxin.

Thermal treatment means the treat- ment of hazardous waste in a device which uses elevated temperatures as the primary means to change the chemical, physical, or biological char- acter or composition of the hazardous waste. Examples of thermal treatment processes are incineration, molten salt, pyrolysis, calcination, wet air oxida- tion, and microwave discharge. (See

also ‘‘incinerator’’ and ‘‘open burn- ing’’.)

Thermostat means a temperature con- trol device that contains metallic mer- cury in an ampule attached to a bi- metal sensing element, and mercury- containing ampules that have been re- moved from these temperature control devices in compliance with the require- ments of 40 CFR 273.13(c)(2) or 273.33(c)(2).

Totally enclosed treatment facility means a facility for the treatment of hazardous waste which is directly con- nected to an industrial production process and which is constructed and operated in a manner which prevents the release of any hazardous waste or any constituent thereof into the envi- ronment during treatment. An example is a pipe in which waste acid is neutral- ized.

Transfer facility means any transpor- tation-related facility, including load- ing docks, parking areas, storage areas and other similar areas where ship- ments of hazardous waste or hazardous secondary materials are held during the normal course of transportation.

Transport vehicle means a motor vehi- cle or rail car used for the transpor- tation of cargo by any mode. Each cargo-carrying body (trailer, railroad freight car, etc.) is a separate transport vehicle.

Transportation means the movement of hazardous waste by air, rail, high- way, or water.

Transporter means a person engaged in the offsite transportation of haz- ardous waste by air, rail, highway, or water.

Treatability study means a study in which a hazardous waste is subjected to a treatment process to determine:

(1) Whether the waste is amenable to the treatment process, (2) what pretreatment (if any) is required, (3) the optimal process conditions needed to achieve the desired treatment, (4) the efficiency of a treatment process for a specific waste or wastes, or (5) the characteristics and volumes of residu- als from a particular treatment proc- ess. Also included in this definition for the purpose of the § 261.4 (e) and (f) ex- emptions are liner compatibility, cor- rosion, and other material compat- ibility studies and toxicological and

health effects studies. A ‘‘treatability study’’ is not a means to commercially treat or dispose of hazardous waste.

Treatment means any method, tech- nique, or process, including neutraliza- tion, designed to change the physical, chemical, or biological character or composition of any hazardous waste so as to neutralize such waste, or so as to recover energy or material resources from the waste, or so as to render such waste non-hazardous, or less hazardous; safer to transport, store, or dispose of; or amenable for recovery, amenable for storage, or reduced in volume.

Treatment zone means a soil area of the unsaturated zone of a land treat- ment unit within which hazardous con- stituents are degraded, transformed, or immobilized.

Underground injection means the sub- surface emplacement of fluids through a bored, drilled or driven well; or through a dug well, where the depth of the dug well is greater than the largest surface dimension. (See also ‘‘injection well’’.)

Underground tank means a device meeting the definition of ‘‘tank’’ in

§ 260.10 whose entire surface area is to- tally below the surface of and covered by the ground.

Unfit-for use tank system means a tank system that has been determined through an integrity assessment or other inspection to be no longer capa- ble of storing or treating hazardous waste without posing a threat of re- lease of hazardous waste to the envi- ronment.

United States means the 50 States, the District of Columbia, the Common- wealth of Puerto Rico, the U.S. Virgin Islands, Guam, American Samoa, and the Commonwealth of the Northern Mariana Islands.

Universal waste means any of the fol- lowing hazardous wastes that are man- aged under the universal waste require- ments of part 273 of this chapter:

- Batteries as described in § 273.2 of this chapter;

- Pesticides as described in § 273.3 of this chapter;

- Mercury-containing equipment as described in § 273.4 of this chapter;

- Lamps as described in § 273.5 of this chapter; and

- Aerosol cans as described in § 273.6 of this chapter.

Universal waste handler:

- Means:

  - A generator (as defined in this sec- tion) of universal waste; or

  - The owner or operator of a facil- ity, including all contiguous property, that receives universal waste from other universal waste handlers, accu- mulates universal waste, and sends universal waste to another universal waste handler, to a destination facil- ity, or to a foreign destination.

- Does not mean:

  - A person who treats (except under the provisions of 40 CFR 273.13(a) or (c), or 40 CFR 273.33(a) or (c)), disposes of, or recycles (except under the provi- sions of 40 CFR 273.13(e) or 40 CFR 273.33(e)) universal waste; or

  - A person engaged in the off-site transportation of universal waste by air, rail, highway, or water, including a universal waste transfer facility.

Universal waste transporter means a person engaged in the off-site transpor- tation of universal waste by air, rail, highway, or water.

Unsaturated zone or zone of aeration means the zone between the land sur- face and the water table.

Uppermost aquifer means the geologic formation nearest the natural ground surface that is an aquifer, as well as lower aquifers that are hydraulically interconnected with this aquifer within the facility’s property boundary.

Used oil means any oil that has been refined from crude oil, or any synthetic oil, that has been used and as a result of such use is contaminated by phys- ical or chemical impurities.

User of the electronic manifest system means a hazardous waste generator, a hazardous waste transporter, an owner or operator of a hazardous waste treat- ment, storage, recycling, or disposal fa- cility, or any other person that:

- Is required to use a manifest to comply with:

  - Any federal or state requirement to track the shipment, transportation, and receipt of hazardous waste or other waste material that is shipped from the site of generation to an off-site des- ignated facility for treatment, storage, recycling, or disposal; or

  - Any federal or state requirement to track the shipment, transportation, and receipt of rejected wastes or regu- lated container residues that are shipped from a designated facility to an alternative facility, or returned to the generator; and

- Elects to use the system to ob- tain, complete and transmit an elec- tronic manifest format supplied by the EPA electronic manifest system, or

- Elects to use the paper manifest form and submits to the system for data processing purposes a paper copy of the manifest (or data from such a paper  copy),  in  accordance  with

§ 264.71(a)(2)(v) or § 265.71(a)(2)(v) of this chapter. These paper copies are sub- mitted for data exchange purposes only and are not the official copies of record for legal purposes.

Very small quantity generator is a gen- erator who generates less than or equal to the following amounts in a calendar month:

- 100 kilograms (220 lbs) of non- acute hazardous waste; and

- 1 kilogram (2.2 lbs) of acute haz- ardous waste listed in § 261.31 or

§ 261.33(e) of this chapter; and

- 100 kilograms (220 lbs) of any res- idue or contaminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute hazardous waste listed in

§ 261.31 or § 261.33(e) of this chapter.

Vessel includes every description of watercraft, used or capable of being used as a means of transportation on the water.

Wastewater treatment unit means a de- vice which:

- Is part of a wastewater treatment facility that is subject to regulation under either section 402 or 307(b) of the Clean Water Act; and

- Receives and treats or stores an influent wastewater that is a haz- ardous waste as defined in § 261.3 of this chapter, or that generates and accumu- lates a wastewater treatment sludge that is a hazardous waste as defined in

§ 261.3 of this chapter, or treats or stores a wastewater treatment sludge which is a hazardous waste as defined in § 261.3 of this Chapter; and

- Meets the definition of tank or tank system in § 260.10 of this chapter.

Water (bulk shipment) means the bulk transportation of hazardous waste which is loaded or carried on board a vessel without containers or labels.

Well means any shaft or pit dug or bored into the earth, generally of a cy- lindrical form, and often walled with bricks or tubing to prevent the earth from caving in.

Well injection: (See ‘‘underground in- jection’’.)

Zone of engineering control means an area under the control of the owner/op- erator that, upon detection of a haz- ardous waste release, can be readily cleaned up prior to the release of haz- ardous waste or hazardous constituents to ground water or surface water.

Wipe means a woven or non-woven shop towel, rag, pad, or swab made of wood pulp, fabric, cotton, polyester blends, or other material.

[45 FR 33073, May 19, 1980]

EDITORIAL NOTE: For FEDERAL REGISTER ci-

tations affecting § 260.10, see the List of CFR Sections Affected, which appears in the Finding Aids section of the printed volume and at www.govinfo.gov.

# § 260.11  Incorporation by reference.

When used in parts 260 through 268 of this chapter, the following materials are incorporated by reference with the approval of the Director of the Federal Register under 5 U.S.C. 552(a) and 1 CFR part 51. All approved materials are available for inspection at the OLEM Docket in the Environmental Protection	Agency	Docket		Center (EPA/DC), West William Jefferson Clin- ton Bldg., Rm. 3334, 1301 Constitution Ave. NW, Washington, DC. The EPA/DC Public Reading Room hours of oper- ation are 8:30 a.m. to 4:30 p.m., Monday through Friday, excluding legal holi- days. The telephone number of the EPA/DC Public Reading room is (202) 566–1744, and the telephone number for the OLEM Docket is (202) 566–0270. These approved materials are also available for inspection at the National Archives and Records Administration (NARA). For information on the avail- ability of this material at NARA, email fedreg.legal@nara.gov		or	go	to www.archives.gov/federal-register/cfr/ibr- locations.html. In addition, these mate- rials are available from the following sources:

- American Petroleum Institute (API). 1220 L Street Northwest, Washington, DC 20005, (855) 999–9870, www.api.org.

  - API Publication 2517, Third Edi- tion, February 1989, ‘‘Evaporative Loss from External Floating-Roof Tanks,’’ IBR approved for § 265.1084.

  - [Reserved]

- ASTM International (ASTM). 100 Barr Harbor Drive, P.O. Box C700, West Conshohocken, PA 19428–2959, (877) 909– ASTM, www.astm.org.

  - ASTM D93–79, ‘‘Standard Test Methods for Flash Point by Pensky- Martens Closed Cup Tester,’’ IBR ap- proved for § 261.21(a).

  - ASTM D93–80, ‘‘Standard Test Methods for Flash Point by Pensky- Martens Closed Cup Tester,’’ IBR ap- proved for § 261.21(a).

  - ASTM D1946–82, ‘‘Standard Meth- od for Analysis of Reformed Gas by Gas Chromatography,’’ IBR approved for

§§ 264.1033 and 265.1033.

  - ASTM D2267–88, ‘‘Standard Test Method for Aromatics in Light Naph- thas and Aviation Gasolines by Gas Chromatography,’’ IBR approved for

§ 264.1063.

  - ASTM D2382–83, ‘‘Standard Test Method for Heat of Combustion of Hy- drocarbon Fuels by Bomb Calorimeter (High-Precision Method),’’ IBR ap- proved for §§ 264.1033 and 265.1033.

  - ASTM D2879–92, ‘‘Standard Test Method for Vapor Pressure—Tempera- ture Relationship and Initial Decompo- sition Temperature of Liquids by Isoteniscope,’’  IBR  approved  for

§ 265.1084.

  - ASTM D3278–78, ‘‘Standard Test Methods for Flash Point for Liquids by Setaflash Closed Tester,’’ IBR approved for § 261.21(a).

  - ASTM D8174–18 ‘‘Standard Test Method for Finite Flash Point Deter- mination of Liquid Wastes by Small Scale Closed Cup Tester.’’ Approved March  15,  2018,  IBR  approved  for

§ 261.21(a).

  - ASTM D8175–18 ‘‘Standard Test Method for Finite Flash Point Deter- mination of Liquid Wastes by Pensky- Martens Closed Cup Tester.’’ Approved March  15,  2018,  IBR  approved  for

§ 261.21(a).