## Environmental Protection Agency	§ 263.10

hazardous waste at the generator’s fa- cility. These measures must include, where applicable, stopping processes and operations, collecting and con- taining released hazardous waste, and removing or isolating containers.

- If the generator stops operations in response to a fire, explosion or re- lease, the emergency coordinator must monitor for leaks, pressure buildup, gas generation, or ruptures in valves, pipes, or other equipment, wherever this is appropriate.

- Immediately after an emergency, the emergency coordinator must pro- vide for treating, storing, or disposing of recovered waste, contaminated soil or surface water, or any other material that results from a release, fire, or ex- plosion at the facility. Unless the gen- erator can demonstrate, in accordance with § 261.3(c) or (d) of this chapter, that the recovered material is not a hazardous waste, then it is a newly generated hazardous waste that must be managed in accordance with all the applicable requirements and conditions for exemption in parts 262, 263, and 265 of this chapter.

- The emergency coordinator must

ensure that, in the affected area(s) of the facility:

- No hazardous waste that may be incompatible with the released mate- rial is treated, stored, or disposed of until cleanup procedures are com- pleted; and

- All emergency equipment listed in the contingency plan is cleaned and fit for its intended use before operations are resumed.

  - The generator must note in the operating record the time, date, and details of any incident that requires implementing the contingency plan. Within 15 days after the incident, the generator must submit a written re- port on the incident to the Regional Administrator. The report must in- clude:

    - Name, address, and telephone number of the generator;

    - Date, time, and type of incident (e.g., fire, explosion);

    - Name and quantity of material(s) involved;

    - The extent of injuries, if any;

    - An assessment of actual or poten- tial hazards to human health or the en-

vironment, where this is applicable; and

    - Estimated quantity and disposi- tion of recovered material that re- sulted from the incident.

# PART 263—STANDARDS APPLICA- BLE TO TRANSPORTERS OF HAZ- ARDOUS WASTE

### Subpart A—General

Sec.

  - Scope.

  - EPA identification number.

  - Transfer facility requirements.

### Subpart B—Compliance With the Manifest System and Recordkeeping

  - The manifest system.

  - Compliance with the manifest.

  - Recordkeeping.

263.25 Electronic manifest signatures.

### Subpart C—Hazardous Waste Discharges

  - Immediate action.

  - Discharge clean up.

AUTHORITY: 42 U.S.C. 6906, 6912, 6922–6925,

6937, 6938, and 6939g.

SOURCE: 45 FR 33151, May 19, 1980, unless

otherwise noted.

# Subpart A—General

### § 263.10  Scope.

    - These regulations establish stand- ards which apply to persons trans- porting hazardous waste within the United States if the transportation re- quires a manifest under 40 CFR part 262.

NOTE: The regulations set forth in parts 262 and 263 establish the responsibilities of gen- erators and transporters of hazardous waste in the handling, transportation, and manage- ment of that waste. In these regulations, EPA has expressly adopted certain regula- tions of the Department of Transportation (DOT) governing the transportation of haz- ardous materials. These regulations concern, among other things, labeling, marking, placarding, using proper containers, and re- porting discharges. EPA has expressly adopt- ed these regulations in order to satisfy its statutory obligation to promulgate regula- tions which are necessary to protect human health and the environment in the transpor- tation of hazardous waste. EPA’s adoption of these DOT regulations ensures consistency with the requirements of DOT and thus

## § 263.11

avoids the establishment of duplicative or conflicting requirements with respect to these matters. These EPA regulations which apply to both interstate and intrastate transportation of hazardous waste are en- forceable by EPA.

DOT has revised its hazardous materials transportation regulations in order to en- compass the transportation of hazardous waste and to regulate intrastate, as well as interstate, transportation of hazardous waste. Transporters of hazardous waste are cautioned that DOT’s regulations are fully applicable to their activities and enforceable by DOT. These DOT regulations are codified in title 49, Code of Federal Regulations, sub- chapter C.

    - These regulations do not apply to on-site transportation of hazardous waste by generators or by owners or operators of permitted hazardous waste management facilities.

    - A transporter of hazardous waste must also comply with 40 CFR part 262, Standards Applicable to Generators of Hazardous Waste, if he:

      - Transports hazardous waste into the United States from abroad; or

      - Mixes hazardous wastes of dif- ferent DOT shipping descriptions by placing them into a single container.

    - A transporter of hazardous waste that is being imported from or ex- ported to any other country for pur- poses of recovery or disposal is subject to this Subpart and to all other rel- evant requirements of subpart H of 40 CFR part 262, including, but not lim- ited to, 40 CFR 262.83(d) and 262.84(d) for movement documents.

    - The regulations in this part do not apply to transportation during an explosives or munitions emergency re- sponse, conducted in accordance with

40  CFR  264.1(g)(8)(i)(D)  or  (iv)  or

265.1(c)(11)(i)(D)   or   (iv),   and

270.1(c)(3)(i)(D) or (iii).

    - Section 266.203 of this chapter identifies how the requirements of this part apply to military munitions clas- sified as solid waste under 40 CFR 266.202.

[45 FR 33151, May 19, 1980, as amended at 45

FR 86968, Dec. 31, 1980; 61 FR 16314, Apr. 12,

1996; 62 FR 6651, Feb. 12, 1997; 75 FR 1259, Jan.

8, 2010; 81 FR 85724, Nov. 28, 2016]

### § 263.11  EPA identification number.

- A transporter must not transport hazardous wastes without having re-

## 40 CFR Ch. I (7–1–24 Edition)

ceived an EPA identification number from the Administrator.

- A transporter who has not re- ceived an EPA identification number may obtain one by applying to the Ad- ministrator using EPA Form 8700–12. Upon receiving the request, the Admin- istrator will assign an EPA identifica- tion number to the transporter.

### § 263.12  Transfer facility requirements.

- A transporter who stores mani- fested shipments of hazardous waste in containers meeting the independent re- quirements of § 262.30 of this chapter at a transfer facility for a period of ten

(10) days or less is not subject to regu- lation under parts 264, 265, 267, 268, and 270 of this chapter with respect to the storage of those wastes.

- When consolidating the contents of two or more containers with the same hazardous waste into a new con- tainer, or when combining and consoli- dating two different hazardous wastes that are compatible with each other, the transporter must mark its con- tainers of 119 gallons or less with the following information:

  - The words ‘‘Hazardous Waste’’ and

  - The applicable EPA hazardous waste number(s) (EPA hazardous waste codes) in subparts C and D of part 261 of this  chapter,  or  in  compliance  with

§ 262.32(c).

[81 FR 85825, Nov. 28, 2016]

# Subpart B—Compliance With the Manifest System and Record- keeping

### § 263.20  The manifest system.

(a)(1) Manifest requirement. A trans- porter may not accept hazardous waste from a generator unless the trans- porter is also provided with a manifest form (EPA Form 8700–22, and if nec- essary, EPA Form 8700–22A) signed in accordance  with  the  requirement  of

§ 262.23, or is provided with an elec- tronic manifest that is obtained, com- pleted, and transmitted in accordance with § 262.20(a)(3) of this chapter, and signed with a valid and enforceable electronic signature as described in 40 CFR 262.25.

(2) Exports. For exports of hazardous waste subject to the requirements of

## Environmental Protection Agency	§ 263.20

subpart H of 40 CFR part 262, a trans- porter may not accept hazardous waste without a manifest signed by the gen- erator in accordance with this section, as appropriate, and for exports occur- ring under the terms of a consent issued by EPA on or after December 31, 2016, a movement document that in- cludes all information required by 40 CFR 262.83(d).

- Compliance date for form revisions. The revised Manifest form and proce- dures in 40 CFR 260.10, 261.7, 263.20, and

263.21, had an effective date of Sep- tember 5, 2006. The Manifest form and procedures in 40 CFR 260.10, 261.7,

263.20, and 263.21, contained in the 40 CFR, parts 260 to 265, edition revised as of July 1, 2004, were applicable until September 5, 2006.

- Use of electronic manifest—legal equivalence to paper forms for partici- pating transporters. Electronic mani- fests that are obtained, completed, and transmitted   in   accordance   with

§ 262.20(a)(3) of this chapter, and used in accordance with this section in lieu of EPA Forms 8700–22 and 8700–22A, are the legal equivalent of paper manifest forms bearing handwritten signatures, and satisfy for all purposes any re- quirement in these regulations to ob- tain, complete, sign, carry, provide, give, use, or retain a manifest.

  - Any requirement in these regula- tions to sign a manifest or manifest certification by hand, or to obtain a handwritten signature, is satisfied by signing with or obtaining a valid and enforceable electronic signature within the meaning of 40 CFR 262.25.

  - Any requirement in these regula- tions to give, provide, send, forward, or return to another person a copy of the manifest is satisfied when a copy of an electronic manifest is transmitted to the other person by submission to the system.

  - Any requirement in these regula- tions for a manifest to accompany a hazardous waste shipment is satisfied when a copy of an electronic manifest is accessible during transportation and forwarded to the person or persons who are scheduled to receive delivery of the waste shipment, except that to the ex- tent that the Hazardous Materials reg- ulation on shipping papers for carriage by  public  highway  requires  trans-

porters of hazardous materials to carry a paper document to comply with 49 CFR 177.817, a hazardous waste trans- porter must carry one printed copy of the electronic manifest on the trans- port vehicle.

  - Any requirement in these regula- tions for a transporter to keep or re- tain a copy of a manifest is satisfied by the retention of an electronic manifest in the transporter’s account on the e- Manifest system, provided that such copies are readily available for viewing and production if requested by any EPA or authorized state inspector.

  - No transporter may be held liable for the inability to produce an elec- tronic manifest for inspection under this section if that transporter can demonstrate that the inability to produce the electronic manifest is ex- clusively due to a technical difficulty with the EPA system for which the transporter bears no responsibility.

- A transporter may participate in the electronic manifest system either by accessing the electronic manifest system from the transporter’s own electronic equipment, or by accessing the electronic manifest system from the equipment provided by a partici- pating generator, by another trans- porter, or by a designated facility.

- Special procedures when electronic manifest is not available. If after a mani- fest has been originated electronically and signed electronically by the initial transporter, and the electronic mani- fest system should become unavailable for any reason, then:

  - The transporter in possession of the hazardous waste when the elec- tronic manifest becomes unavailable shall reproduce sufficient copies of the printed manifest that is carried on the transport vehicle pursuant to para- graph (a)(4)(iii)(A) of this section, or obtain and complete another paper manifest for this purpose. The trans- porter shall reproduce sufficient copies to provide the transporter and all sub- sequent waste handlers with a copy for their files, plus two additional copies that will be delivered to the designated facility with the hazardous waste.

  - On each printed copy, the trans-

porter shall include a notation in the Special Handling and Additional De- scription space (Item 14) that the paper

## § 263.20

manifest is a replacement manifest for a manifest originated in the electronic manifest system, shall include (if not pre-printed on the replacement mani- fest) the manifest tracking number of the electronic manifest that is replaced by the paper manifest, and shall also include a brief explanation why the electronic manifest was not available for completing the tracking of the shipment electronically.

  - A transporter signing a replace- ment manifest to acknowledge receipt of the hazardous waste must ensure that each paper copy is individually signed and that a legible handwritten signature appears on each copy.

  - From the point at which the elec- tronic manifest is no longer available for tracking the waste shipment, the paper replacement manifest copies shall be carried, signed, retained as records, and given to a subsequent transporter or to the designated facil- ity, following the instructions, proce- dures, and requirements that apply to the use of all other paper manifests.

- Special procedures for electronic sig- nature methods undergoing tests. If a transporter using an electronic mani- fest signs this manifest electronically using an electronic signature method which is undergoing pilot or dem- onstration tests aimed at dem- onstrating the practicality or legal de- pendability of the signature method, then the transporter shall sign the electronic manifest electronically and also sign with an ink signature the transporter acknowledgement of re- ceipt of materials on the printed copy of the manifest that is carried on the vehicle in accordance with paragraph (a)(4)(iii)(A) of this section. This print- ed copy bearing the generator’s and transporter’s ink signatures shall also be presented by the transporter to the designated facility to sign in ink to in- dicate the receipt of the waste mate- rials or to indicate discrepancies. After the owner/operator of the designated facility has signed this printed mani- fest copy with its ink signature, the printed manifest copy shall be deliv- ered to the designated facility with the waste materials.

- [Reserved]

- Post-receipt manifest data correc- tions. After facilities have certified to

## 40 CFR Ch. I (7–1–24 Edition)

the receipt of hazardous wastes by signing Item 20 of the manifest, any post-receipt data corrections may be submitted at any time by any inter- ested person (e.g., waste handler) named on the manifest. Transporters may participate electronically in the post-receipt data corrections process by following the process described in

§ 264.71(l) of this chapter, which applies to corrections made to either paper or electronic manifest records.

- Before transporting the hazardous waste, the transporter must sign and date the manifest acknowledging ac- ceptance of the hazardous waste from the generator. The transporter must return a signed copy to the generator before leaving the generator’s prop- erty.

- The transporter must ensure that the manifest accompanies the haz- ardous waste. In the case of exports oc- curring under the terms of a consent issued by EPA to the exporter on or after December 31, 2016, the transporter must ensure that a movement docu- ment that includes all information re- quired by 40 CFR 262.83(d) also accom- panies the hazardous waste. In the case of imports occurring under the terms of a consent issued by EPA to the country of export or the importer on or after December 31, 2016, the transporter must ensure that a movement docu- ment that includes all information re- quired by 40 CFR 262.84(d) also accom- panies the hazardous waste.

- A transporter who delivers a haz- ardous waste to another transporter or to the designated facility must:

  - Obtain the date of delivery and the handwritten signature of that transporter or of the owner or operator of the designated facility on the mani- fest; and

  - Retain one copy of the manifest in accordance with § 263.22; and

  - Give the remaining copies of the manifest to the accepting transporter or designated facility.

- The requirements of paragraphs (c), (d) and (f) of this section do not apply to water (bulk shipment) trans- porters if:

  - The hazardous waste is delivered by water (bulk shipment) to the des- ignated facility; and

## Environmental Protection Agency	§ 263.20

  - A shipping paper containing all the information required on the mani- fest (excluding the EPA identification numbers, generator certification, and signatures) and, for exports or imports occurring under the terms of a consent issued by EPA on or after December 31, 2016, a movement document that in- cludes all information required by 40 CFR 262.83(d) or 262.84(d) accompanies the hazardous waste; and

  - The delivering transporter obtains the date of delivery and handwritten signature of the owner or operator of the designated facility on either the manifest or the shipping paper; and

  - The person delivering the haz- ardous waste to the initial water (bulk shipment) transporter obtains the date of delivery and signature of the water (bulk shipment) transporter on the manifest and forwards it to the des- ignated facility; and

  - A copy of the shipping paper or manifest is retained by each water (bulk shipment) transporter in accord- ance with § 263.22.

- For shipments involving rail transportation, the requirements of paragraphs (c), (d) and (e) do not apply and the following requirements do apply:

  - When accepting hazardous waste from a non-rail transporter, the initial rail transporter must:

    - Sign and date the manifest ac- knowledging acceptance of the haz- ardous waste;

    - Return a signed copy of the mani- fest to the non-rail transporter;

    - Forward at least three copies of the manifest to:

      - The next non-rail transporter, if any; or,

      - The designated facility, if the shipment is delivered to that facility by rail; or

      - The last rail transporter des- ignated to handle the waste in the United States;

    - Retain one copy of the manifest and rail shipping paper in accordance with § 263.22.

  - Rail transporters must ensure that a shipping paper containing all the information required on the mani- fest (excluding the EPA identification numbers, generator certification, and signatures) and, for exports or imports

occurring under the terms of a consent issued by EPA on or after December 31, 2016, a movement document that in- cludes all information required by 40 CFR 262.83(d) or 262.84(d) accompanies the hazardous waste at all times.

NOTE TO PARAGRAPH (f)(2): Intermediate rail transporters are not required to sign the manifest, movement document, or shipping paper.

  - When delivering hazardous waste to the designated facility, a rail trans- porter must:

    - Obtain the date of delivery and handwritten signature of the owner or operator of the designated facility on the manifest or the shipping paper (if the manifest has not been received by the facility); and

    - Retain a copy of the manifest or signed shipping paper in accordance with § 263.22.

  - When delivering hazardous waste to a non-rail transporter a rail trans- porter must:

    - Obtain the date of delivery and the handwritten signature of the next non-rail transporter on the manifest; and

    - Retain a copy of the manifest in accordance with § 263.22.

  - Before accepting hazardous waste from a rail transporter, a non-rail transporter must sign and date the manifest and provide a copy to the rail transporter.

- Transporters who transport haz- ardous waste out of the United States must:

  - Sign and date the manifest in the International Shipments block to indi- cate the date that the shipment left the United States;

  - Retain one copy in accordance with § 263.22(d);

  - Return a signed copy of the mani- fest to the generator; and

  - For paper manifests only,

    - Send a copy of the manifest to the e-Manifest system in accordance with the allowable methods specified in 40 CFR 264.71(a)(2)(v); and

    - For shipments initiated prior to the AES filing compliance date, when instructed by the exporter to do so, give a copy of the manifest to a U.S. Customs official at the point of depar- ture from the United States.

## § 263.21

- A transporter transporting haz- ardous waste from a generator who generates greater than 100 kilograms but less than 1000 kilograms of haz- ardous waste in a calendar month need not comply with the requirements of this section or those of § 263.22 provided that:

  - The waste is being transported pursuant to a reclamation agreement as provided for in § 262.20(e);

  - The transporter records, on a log or shipping paper, the following infor- mation for each shipment:

    - The name, address, and U.S. EPA Identification Number of the generator of the waste;

    - The quantity of waste accepted;

    - All DOT-required shipping infor- mation;

    - The date the waste is accepted; and

  - The transporter carries this record when transporting waste to the reclamation facility; and

  - The transporter retains these records for a period of at least three years after termination or expiration of the agreement.

[45 FR 33151, May 19, 1980, as amended at 45

FR 86973, Dec. 31, 1980; 51 FR 10176, Mar. 24,

1986; 51 FR 28685, Aug. 8, 1986; 61 FR 16315,

Apr. 12, 1996; 70 FR 10821, Mar. 4, 2005; 79 FR

7558, Feb. 7, 2014; 81 FR 85724, Nov. 28, 2016; 83

FR 452, Jan. 3, 2018]

### § 263.21  Compliance with the manifest.

- Except as provided in paragraph

- of this section, the transporter must deliver the entire quantity of hazardous waste which he or she has accepted from a generator or a trans- porter to:

  - The designated facility listed on the manifest; or

  - The alternate designated facility, if the hazardous waste cannot be deliv- ered to the designated facility because an emergency prevents delivery; or

  - The next designated transporter; or

  - The place outside the United States designated by the generator.

(b)(1) Emergency condition. If the haz- ardous waste cannot be delivered in ac- cordance with paragraph (a)(1), (2), or

(4) of this section because of an emer- gency condition other than rejection of the waste by the designated facility or

## 40 CFR Ch. I (7–1–24 Edition)

alternate designated facility, then the transporter must contact the generator for further instructions and must re- vise the manifest according to the gen- erator’s instructions.

- Transporters without agency au- thority. If the hazardous waste is not delivered to the next designated trans- porter in accordance with paragraph (a)(3) of this section, and the current transporter is without contractual au- thorization from the generator to act as the generator’s agent with respect to transporter additions or substi- tutions, then the current transporter must contact the generator for further instructions prior to making any revi- sions to the transporter designations on the manifest. The current trans- porter may thereafter make such revi- sions if:

- The hazardous waste is not deliv-

ered in accordance with paragraph (a)(3) of this section because of an emergency condition; or

- The current transporter proposes to change the transporter(s) designated on the manifest by the generator, or to add a new transporter during transpor- tation, to respond to an emergency, or for purposes of transportation effi- ciency, convenience, or safety; and

- The generator authorizes the re- vision.

- Transporters with agency authority. If the hazardous waste is not delivered to the next designated transporter in accordance with paragraph (a)(3) of this section, and the current trans- porter has authorization from the gen- erator to act as the generator’s agent, then the current transporter may change the transporter(s) designated on the manifest, or add a new trans- porter, during transportation without the generator’s prior, explicit approval, provided that:

  - The current transporter is author-

ized by a contractual provision that provides explicit agency authority for the transporter to make such trans- porter changes on behalf of the gener- ator;

  - The transporter enters in Item 14 of each manifest for which such a change is made, the following state- ment of its agency authority: ‘‘Con- tract retained by generator confers agency authority on initial transporter

## Environmental Protection Agency	§ 263.22

to add or substitute additional trans- porters on generator’s behalf;’’ and

  - The change in designated trans- porters is necessary to respond to an emergency, or for purposes of transpor- tation efficiency, convenience, or safe- ty.

- Generator liability. The grant by a generator of authority to a transporter to act as the agent of the generator with respect to changes to transporter designations under paragraph (b)(3) of this section does not affect the genera- tor’s liability or responsibility for complying with any applicable require- ment under this chapter, or grant any additional authority to the transporter to act on behalf of the generator.

- If hazardous waste is rejected by the designated facility while the trans- porter is on the facility’s premises, then the transporter must obtain the following:

  - For a partial load rejection or for regulated quantities of container resi- dues, a copy of the original manifest that includes the facility’s date and signature, and the Manifest Tracking Number of the new manifest that will accompany the shipment, and a de- scription of the partial rejection or container residue in the discrepancy block of the original manifest. The transporter must retain a copy of this manifest in accordance with § 263.22, and give the remaining copies of the original manifest to the rejecting des- ignated facility. If the transporter is forwarding the rejected part of the shipment or a regulated container res- idue to an alternate facility or return- ing it to the generator, the transporter must obtain a new manifest to accom- pany the shipment, and the new mani- fest must include all of the information required in 40 CFR 264.72(e)(1) through

(6) or (f)(1) through (6) or 40 CFR

265.72(e)(1) through (6) or (f)(1) through (6).

  - For a full load rejection that will be taken back by the transporter, a copy of the original manifest that in- cludes the rejecting facility’s signature and date attesting to the rejection, the description of the rejection in the dis- crepancy block of the manifest, and the name, address, phone number, and Identification Number for the alternate facility or generator to whom the ship-

ment must be delivered. The trans- porter must retain a copy of the mani- fest in accordance with § 263.22, and give a copy of the manifest containing this information to the rejecting des- ignated facility. If the original mani- fest is not used, then the transporter must obtain a new manifest for the shipment and comply with 40 CFR 264.72(e)(1)  through  (6)  or  40  CFR

265.72(e)(1) through (6).

[83 FR 452, Jan. 3, 2018]

### § 263.22  Recordkeeping.

  - A transporter of hazardous waste must keep a copy of the manifest signed by the generator, himself, and the next designated transporter or the owner or operator of the designated fa- cility for a period of three years from the date the hazardous waste was ac- cepted by the initial transporter.

  - For shipments delivered to the

designated facility by water (bulk ship- ment), each water (bulk shipment) transporter must retain a copy of the shipping paper containing all the infor- mation required in § 263.20(e)(2) for a period of three years from the date the hazardous waste was accepted by the initial transporter.

  - For shipments of hazardous waste

by rail within the United States:

- The initial rail transporter must keep a copy of the manifest and ship- ping paper with all the information re- quired in § 263.20(f)(2) for a period of three years from the date the haz- ardous waste was accepted by the ini- tial transporter; and

- The final rail transporter must keep a copy of the signed manifest (or the shipping paper if signed by the des- ignated facility in lieu of the manifest) for a period of three years from the date the hazardous waste was accepted by the initial transporter.

NOTE: Intermediate rail transporters are not required to keep records pursuant to these regulations.

  - A transporter who transports haz- ardous waste out of the United States must keep a copy of the manifest indi- cating that the hazardous waste left the United States for a period of three years from the date the hazardous waste was accepted by the initial transporter.

## § 263.25

  - The periods of retention referred to in this Section are extended auto- matically during the course of any un- resolved enforcement action regarding the regulated activity or as requested by the Administrator.

[45 FR 33151, May 19, 1980, as amended at 45

FR 86973, Dec. 31, 1980]

### § 263.25 Electronic	manifest	signa- tures.

- Electronic manifest signatures shall meet the criteria described in

§ 262.25 of this chapter.

- [Reserved]

[79 FR 7560, Feb. 7, 2014]

## 40 CFR Ch. I (7–1–24 Edition)

### § 263.31  Discharge clean up.

A transporter must clean up any haz- ardous waste discharge that occurs during transportation or take such ac- tion as may be required or approved by Federal, State, or local officials so that the hazardous waste discharge no longer presents a hazard to human health or the environment.

# PART 264—STANDARDS FOR OWN- ERS AND OPERATORS OF HAZ- ARDOUS WASTE TREATMENT, STORAGE, AND DISPOSAL FA- CILITIES

Subpart A—General

# Subpart C—Hazardous Waste Discharges

### § 263.30  Immediate action.

- In the event of a discharge of haz- ardous waste during transportation, the transporter must take appropriate immediate action to protect human health and the environment (e.g., no- tify local authorities, dike the dis- charge area).

- If a discharge of hazardous waste occurs during transportation and an of- ficial (State or local government or a Federal Agency) acting within the scope of his official responsibilities de- termines that immediate removal of the waste is necessary to protect human health or the environment, that official may authorize the removal of the waste by transporters who do not have EPA identification numbers and without the preparation of a manifest.

- An air, rail, highway, or water transporter who has discharged haz- ardous waste must:

  - Give notice, if required by 49 CFR 171.15, to the National Response Center (800–424–8802 or 202–426–2675); and

  - Report in writing as required by 49 CFR 171.16 to the Director, Office of Hazardous Materials Regulations, Ma- terials Transportation Bureau, Depart- ment of Transportation, Washington, DC 20590.

- A water (bulk shipment) trans- porter who has discharged hazardous waste must give the same notice as re- quired by 33 CFR 153.203 for oil and haz- ardous substances.

Sec.

  - Purpose, scope and applicability.

  - [Reserved]

  - Relationship to interim status stand- ards.

  - Imminent hazard action.

### Subpart B—General Facility Standards

  - Applicability.

  - Identification number.

  - Required notices.

  - General waste analysis.

  - Security.

  - General inspection requirements.

  - Personnel training.

  - General requirements for ignitable, reactive, or incompatible wastes.

  - Location standards.

  - Construction quality assurance pro- gram.

### Subpart C—Preparedness and Prevention

  - Applicability.

  - Design and operation of facility.

  - Required equipment.

  - Testing and maintenance of equip- ment.

  - Access to communications or alarm system.

  - Required aisle space.

  - [Reserved]

  - Arrangements with local authorities.

### Subpart D—Contingency Plan and Emergency Procedures

  - Applicability.

  - Purpose and implementation of con- tingency plan.

  - Content of contingency plan.

  - Copies of contingency plan.

  - Amendment of contingency plan.

  - Emergency coordinator.

  - Emergency procedures.