## Pt. 270	40 CFR Ch. I (7–1–24 Edition)

Metal Bearing Wastes Prohibited From Dilution in a Combustion Unit According to 40

CFR 268.3(c) 1—Continued

| Waste code | Waste description |
| --- | --- |
| K007 ............................... K008 ............................... K061 ............................... K069 ............................... K071 ............................... | Wastewater treatment sludge from the production of iron blue pigments. Oven residue from the production of chrome oxide green pigments. Emission control dust/sludge from the primary production of steel in electric furnaces. Emission control dust/sludge from secondary lead smelting. Brine purification muds from the mercury cell processes in chlorine production, where separately prepurified brine is not used. Waste leaching solution from acid leaching of emission control dust/sludge from secondary lead smelting. Sludges from the mercury cell processes for making chlorine. Arsenic acid H3 AsO4 Arsenic oxide As2 O5 Arsenic trioxide Barium cyanide Beryllium Copper cyanide Cu(CN) Nickel cyanide Ni(CN)2 Osmium tetroxide Potassium silver cyanide Silver cyanide Thallic oxide Thallium (l) selenite Thallium (l) sulfate Ammonium vanadate Vanadium oxide V2 O5 Zinc cyanide. Calcium chromate. Lead phosphate. Mercury. Selenious acid. Selenium disulfide. Thallium (I) chloride. Thallium (I) nitrate. |
| K100 ............................... |  |
| K106 ............................... P010 ............................... P011 ............................... P012 ............................... P013 ............................... P015 ............................... P029 ............................... P074 ............................... P087 ............................... P099 ............................... P104 ............................... P113 ............................... P114 ............................... P115 ............................... P119 ............................... P120 ............................... P121 ............................... U032 ............................... U145 ............................... U151 ............................... U204 ............................... U205 ............................... U216 ............................... U217 ............................... |  |

1 A combustion unit is defined as any thermal technology subject to 40 CFR part 264, subpart O; Part 265, subpart O; and/or 266, subpart H.

[61 FR 15658, Apr. 8, 1996]

# PART 270—EPA ADMINISTERED PER- MIT PROGRAMS: THE HAZ- ARDOUS WASTE PERMIT PRO- GRAM

### Subpart A—General Information

Sec.

  - Purpose and scope of the regulations in this part.

  - Definitions.

  - Considerations under Federal law.

  - Effect of a permit.

  - Noncompliance and program reporting by the Director.

  - References.

### Subpart B—Permit Application

  - General application requirements.

  - Signatories to permit applications and reports.

  - Confidentiality of information.

  - Contents of part A of the permit ap- plication.

  - Contents of part B: General require- ments.

  - Specific part B information require- ments for containers.

  - Specific part B information require- ments for tank systems.

  - Specific part B information require- ments for surface impoundments.

  - Specific part B information require- ments for waste piles.

  - Specific part B information require- ments for incinerators.

  - Specific part B information require- ments for land treatment facilities.

  - Specific part B information require- ments for landfills.

  - Specific part B information require- ments for boilers and industrial furnaces burning hazardous waste.

  - Specific part B information require- ments for miscellaneous units.

  - Specific part B information require- ments for process vents.

  - Specific part B information require- ments for equipment.

  - Special part B information require- ments for drip pads.

## Environmental Protection Agency	Pt. 270

  - Specific part B information require- ments for air emission controls for tanks, surface impoundments, and con- tainers.

  - Part B information requirements for post-closure permits.

  - Permit denial.

### Subpart C—Permit Conditions

  - Conditions applicable to all permits.

  - Requirements for recording and re- porting of monitoring results.

  - Establishing permit conditions.

  - Schedules of compliance.

### Subpart D—Changes to Permits

  - Transfer of permits.

  - Modification	or	revocation	and reissuance of permits.

  - Permit modification at the request of the permittee.

  - Termination of permits.

### Subpart E—Expiration and Continuation of Permits

  - Duration of permits.

  - Continuation of expiring permits.

### Subpart F—Special Forms of Permits

  - Permits by rule.

  - Emergency permits.

  - Hazardous waste incinerator permits.

  - Permits for land treatment dem- onstrations using field test or laboratory analyses.

  - Interim permits for UIC wells.

  - Research, development, and dem- onstration permits.

  - Permits for boilers and industrial fur- naces burning hazardous waste.

  - RCRA standardized permits for stor- age and treatment units.

  - Remedial Action Plans (RAPs).

### Subpart G—Interim Status

  - Qualifying for interim status.

  - Operation during interim status.

  - Changes during interim status.

  - Termination of interim status.

### Subpart H—Remedial Action Plans (RAPs)

  - Why is this subpart written in a spe- cial format?

General Information

  - What is a RAP?

270.85 When do I need a RAP?

270.90 Does my RAP grant me any rights or relieve me of any obligations?

Applying for a RAP

270.95 How do I apply for a RAP?

270.100 Who must obtain a RAP?

270.105 Who must sign the application and any required reports for a RAP?

270.110 What must I include in my applica- tion for a RAP?

270.115 What if I want to keep this informa- tion confidential?

270.120 To whom must I submit my RAP ap- plication?

270.125 If I submit my RAP application as part of another document, what must I do?

Getting a RAP Approved

270.130 What is the process for approving or denying my application for a RAP?

270.135 What must the Director include in a draft RAP?

270.140 What else must the Director prepare in addition to the draft RAP or notice of intent to deny?

270.145 What are the procedures for public comment on the draft RAP or notice of intent to deny?

270.150 How will the Director make a final decision on my RAP application?

270.155 May the decision to approve or deny my RAP application be administratively appealed?

270.160 When does my RAP become effec- tive?

270.165 When may I begin physical construc- tion of new units permitted under the RAP?

How May My RAP Be Modified, Revoked and Reissued, or Terminated?

270.170 After my RAP is issued, how may it be modified, revoked and reissued, or ter- minated?

270.175 For what reasons may the Director choose to modify my final RAP?

270.180 For what reasons may the Director choose to revoke and reissue my final RAP?

270.185 For what reasons may the Director choose to terminate my final RAP, or deny my renewal application?

270.190 May the decision to approve or deny a modification, revocation and reissuance, or termination of my RAP be administratively appealed?

270.195 When will my RAP expire?

270.200 How may I renew my RAP if it is ex- piring?

270.205 What happens if I have applied cor- rectly for a RAP renewal but have not re- ceived approval by the time my old RAP expires?

Operating Under Your RAP

270.210 What records must I maintain con- cerning my RAP?

270.215 How are time periods in the require- ments in this subpart and my RAP com- puted?

## § 270.1

270.220 How may I transfer my RAP to a new owner or operator?

270.225 What must the State or EPA Region report about noncompliance with RAPs?

Obtaining a RAP for an Off-Site Location

270.230 May I perform remediation waste management activities under a RAP at a location removed from the area where the remediation wastes originated?

### Subpart I—Integration with Maximum Achievable Control Technology (MACT) Standards

270.235 Options for incinerators, cement kilns, lightweight aggregate kilns, solid fuel boilers, liquid fuel boilers and hydro- chloric acid production furnaces to mini- mize emissions from startup, shutdown, and malfunction events.

### Subpart J—RCRA Standardized Permits for Storage and Treatment Units

General Information About Standardized Permits

270.250 What is a RCRA standardized per- mit?

270.255 Who is eligible for a standardized permit?

270.260 What requirements of Part 270 apply to a standardized permit?

Applying for a Standardized Permit

270.270 How do I apply for a standardized permit?

270.275 What information must I submit to the permitting agency to support my standardized permit application?

270.280 What are the certification require- ments?

Information That Must Be Kept at Your Facility

270.290 What general types of information must I keep at my facility?

270.300 What container information must I keep at my facility?

270.305 What tank information must I keep at my facility?

270.310 What equipment information must I keep at my facility?

270.315 What air emissions control informa- tion must I keep at my facility?

Modifying a Standardized Permit

270.320 How do I modify my RCRA standard- ized permit?

AUTHORITY: 42 U.S.C. 6905, 6912, 6924, 6925,

6927, 6939, and 6974.

SOURCE: 48 FR 14228, Apr. 1, 1983, unless

otherwise noted.

40 CFR Ch. I (7–1–24 Edition)

# Subpart A—General Information

### § 270.1 Purpose and scope of the regu- lations in this part.

- Coverage. (1) These permit regula- tions establish provisions for the Haz- ardous Waste Permit Program under Subtitle C of the Solid Waste Disposal Act, as amended by the Resource Con- servation and Recovery Act of 1976, as amended (RCRA), (Pub. L. 94–580, as amended by Pub. L. 95–609 and by Pub.

L. 96–482; 42 U.S.C. 6091 et seq.). They

apply to EPA and to approved States to the extent provided in part 271.

- The regulations in this part cover basic EPA permitting requirements, such as application requirements, standard permit conditions, and moni- toring and reporting requirements. These regulations are part of a regu- latory scheme implementing RCRA set forth in different parts of the Code of Federal Regulations. The following chart indicates where the regulations implementing RCRA appear in the Code of Federal Regulations.

| Section of RCRA | Coverage | Final regulation |
| --- | --- | --- |
| Subtitle C. 3001 .... 3002 .... 3003 .... 3004 .... 3005 .... 3006 .... 3010 .... | Overview and definitions ... Identification and listing of hazardous waste. Generators of hazardous waste. Transporters of hazardous waste. Standards for HWM facili- ties. Permit requirements for HWM facilities. Guidelines for State pro- grams. Preliminary notification of HWM activity. | 40 CFR part 260 40 CFR part 261 40 CFR part 262 40 CFR part 263 40 CFR parts 264, 265, 266, and 267 40 CFR parts 270 and 124 40 CFR part 271 (public notice) 45 FR 12746 February 26, 1980 |

- Technical regulations. The RCRA permit program has separate addi- tional regulations that contain tech- nical requirements. These separate reg- ulations are used by permit issuing au- thorities to determine what require- ments must be placed in permits if they are issued. These separate regula- tions are located in 40 CFR parts 264, 266, 267, and 268.

- Overview of the RCRA Permit Pro- gram. Not later than 90 days after the promulgation or revision of regulations

## Environmental Protection Agency	§ 270.1

in 40 CFR part 261 (identifying and list- ing hazardous wastes) generators and transporters of hazardous waste, and owners or operators of hazardous waste treatment, storage, or disposal facili- ties may be required to file a notifica- tion of that activity under section 3010. Six months after the initial promulga- tion of the part 261 regulations, treat- ment, storage, or disposal of hazardous waste by any person who has not ap- plied for or received a RCRA permit is prohibited. Treatment, storage, and disposal facilities (TSDs) that are oth- erwise subject to permitting under RCRA and that meet the criteria in paragraph (b)(1), or paragraph (b)(2) of this section, may be eligible for a standardized permit under subpart J of this part. A RCRA permit application consists  of  two  parts,  part  A  (see

§ 270.13) and part B (see § 270.14 and ap-

plicable sections in §§ 270.15 through 270.29). For ‘‘existing HWM facilities,’’ the requirement to submit an applica- tion is satisfied by submitting only part A of the permit application until the date the Director sets for submit- ting part B of the application. (Part A consists of Forms 1 and 3 of the Con- solidated Permit Application Forms.) Timely submission of both notification under section 3010 and part A qualifies owners and operators of existing HWM facilities (who are required to have a permit) for interim status under sec- tion 3005(e) of RCRA. Facility owners and operators with interim status are treated as having been issued a permit until EPA or a State with interim au- thorization for Phase II or final au- thorization under part 271 makes a final determination on the permit ap- plication. Facility owners and opera- tors with interim status must comply with interim status standards set forth at 40 CFR part 265 and 266 or with the analogous provisions of a State pro- gram which has received interim or final authorization under part 271. Fa- cility owners and operators with in- terim status are not relieved from com- plying with other State requirements. For existing HWM facilities, the Direc- tor shall set a date, giving at least six months notice, for submission of part B of the application. There is no form for part B of the application; rather, part B must be submitted in narrative form

and contain the information set forth in the applicable sections of §§ 270.14 through 270.29. Owners or operators of new HWM facilities must submit parts A and B of the permit application at least 180 days before physical construc- tion is expected to commence.

- The facility generates hazardous waste and then non-thermally treats or stores hazardous waste on-site in tanks, containers, or containment buildings; or

- The facility receives hazardous waste generated off-site by a generator under the same ownership as the re- ceiving facility, and then stores or non- thermally treats the hazardous waste in containers, tanks, or containment buildings.

- Scope of the RCRA permit require- ment. RCRA requires a permit for the ‘‘treatment,’’ ‘‘storage,’’ and ‘‘dis- posal’’ of any ‘‘hazardous waste’’ as identified or listed in 40 CFR part 261. The terms ‘‘treatment,’’ ‘‘storage,’’ ‘‘disposal,’’ and ‘‘hazardous waste’’ are defined in § 270.2. Owners and operators of hazardous waste management units must have permits during the active life (including the closure period) of the unit. Owners and operators of sur- face impoundments, landfills, land treatment units, and waste pile units that received waste after July 26, 1982, or that certified closure (according to

§ 265.115 of this chapter) after January

26, 1983, must have post-closure per- mits, unless they demonstrate closure by removal or decontamination as pro- vided under § 270.1(c)(5) and (6), or ob- tain an enforceable document in lieu of a post-closure permit, as provided under paragraph (c)(7) of this section. If a post-closure permit is required, the permit must address applicable 40 CFR part 264 groundwater monitoring, un- saturated zone monitoring, corrective action, and post-closure care require- ments of this chapter. The denial of a permit for the active life of a haz- ardous waste management facility or unit does not affect the requirement to obtain a post-closure permit under this section.

- Specific inclusions. Owners and op- erators of certain facilities require RCRA permits as well as permits under other programs for certain aspects of

## § 270.1

the facility operation. RCRA permits are required for:

  - Injection wells that dispose of haz- ardous waste, and associated surface facilities that treat, store or dispose of hazardous waste, (See § 270.64). How- ever, the owner and operator with a UIC permit in a State with an approved or promulgated UIC program, will be deemed to have a RCRA permit for the injection well itself if they comply with the requirements of § 270.60(b) (permit-by-rule for injection wells).

  - Treatment, storage, or disposal of hazardous waste at facilities requiring an NPDES permit. However, the owner and operator of a publicly owned treat- ment works receiving hazardous waste will be deemed to have a RCRA permit for that waste if they comply with the requirements of § 270.60(c) (permit-by- rule for POTWs).

  - Barges or vessels that dispose of hazardous waste by ocean disposal and onshore hazardous waste treatment or storage facilities associated with an ocean disposal operation. However, the owner and operator will be deemed to have a RCRA permit for ocean disposal from the barge or vessel itself if they comply  with  the  requirements  of

§ 270.60(a) (permit-by-rule for ocean dis- posal barges and vessels).

- Specific exclusions and exemptions. The following persons are among those who are not required to obtain a RCRA permit:

  - Generators who accumulate haz- ardous waste on site in compliance with all of the conditions for exemp- tion provided in 40 CFR 262.14, 262.15,

262.16, and 262.17.

  - Farmers who dispose of hazardous waste pesticides from their own use as provided in § 262.70 of this chapter;

  - Persons who own or operate fa- cilities solely for the treatment, stor- age, or disposal of hazardous waste ex- cluded from regulations under this part by 40 CFR 261.4 or 262.14 (very small quantity generator exemption).

  - Owners or operators of totally enclosed treatment facilities as defined in 40 CFR 260.10.

  - Owners and operators of elemen- tary neutralization units or waste- water treatment units as defined in 40 CFR 260.10.

## 40 CFR Ch. I (7–1–24 Edition)

  - Transporters storing manifested shipments of hazardous waste in con- tainers meeting the requirements of 40 CFR 262.30 at a transfer facility for a period of ten days or less.

  - Persons adding absorbent mate- rial to waste in a container (as defined in § 260.10 of this chapter) and persons adding waste to absorbent material in a container, provided that these ac- tions occur at the time waste is first placed in the container; and §§ 264.17(b), 264.171, and 264.172 of this chapter are complied with.

  - Universal waste handlers and universal waste transporters (as de- fined in 40 CFR 260.10) managing the wastes listed below. These handlers are subject to regulation under 40 part CFR 273.

    - Batteries as described in 40 CFR 273.2;

    - Pesticides as described in § 273.3 of this chapter;

    - Mercury-containing equipment as described in § 273.4 of this chapter;

    - Lamps as described in § 273.5 of this chapter; and

    - Aerosol cans as described in § 273.6 of this chapter.

  - [Reserved]

  - Reverse distributors accumu- lating potentially creditable hazardous waste pharmaceuticals and evaluated hazardous waste pharmaceuticals, as defined in § 266.500. Reverse distributors are subject to regulation under 40 CFR part 266 subpart P for the accumula- tion of potentially creditable haz- ardous waste pharmaceuticals and evaluated hazardous waste pharma- ceuticals.

- Further exclusions. (i) A person is not required to obtain a RCRA permit for treatment or containment activi- ties taken during immediate response to any of the following situations:

  - A discharge of a hazardous waste;

  - An imminent and substantial threat of a discharge of hazardous waste;

  - A discharge of a material which, when discharged, becomes a hazardous waste.

  - An immediate threat to human health, public safety, property, or the environment from the known or sus- pected presence of military munitions,

## Environmental Protection Agency	§ 270.1

other explosive material, or an explo- sive device, as determined by an explo- sive or munitions emergency response specialist as defined in 40 CFR 260.10.

    - Any person who continues or ini- tiates hazardous waste treatment or containment activities after the imme- diate response is over is subject to all applicable requirements of this part for those activities.

    - In the case of emergency re- sponses involving military munitions, the responding military emergency re- sponse specialist’s organizational unit must retain records for three years identifying the dates of the response, the responsible persons responding, the type and description of material ad- dressed, and its disposition.

- Permits for less than an entire facil- ity. EPA may issue or deny a permit for one or more units at a facility without simultaneously issuing or denying a permit to all of the units at the facil- ity. The interim status of any unit for which a permit has not been issued or denied is not affected by the issuance or denial of a permit to any other unit at the facility.

- Closure by removal. Owners/opera- tors of surface impoundments, land treatment units, and waste piles clos- ing by removal or decontamination under part 265 standards must obtain a post-closure permit unless they can demonstrate to the Regional Adminis- trator that the closure met the stand- ards for closure by removal or decon- tamination in § 264.228, § 264.280(e), or

§ 264.258, respectively. The demonstra- tion may be made in the following ways:

- If the owner/operator has sub- mitted a part B application for a post- closure permit, the owner/operator may request a determination, based on information contained in the applica- tion, that section 264 closure by re- moval standards were met. If the Re- gional Administrator believes that § 264 standards were met, he/she will notify the public of this proposed decision, allow for public comment, and reach a final determination according to the procedures in paragraph (c)(6) of this section.

- If the owner/operator has not sub-

mitted a part B application for a post- closure  permit,  the  owner/operator

may petition the Regional Adminis- trator for a determination that a post- closure permit is not required because the closure met the applicable part 264 closure standards.

- The petition must include data demonstrating that closure by removal or decontamination standards were met, or it must demonstrate that the unit closed under State requirements that met or exceeded the applicable 264 closure-by-removal standard.

- The Regional Administrator shall approve or deny the petition according to the procedures outlined in para- graph (c)(6) of this section.

- Procedures for closure equivalency determination. (i) If a facility owner/op- erator seeks an equivalency dem- onstration under § 270.1(c)(5), the Re- gional Administrator will provide the public, through a newspaper notice, the opportunity to submit written com- ments on the information submitted by the owner/operator within 30 days from the date of the notice. The Regional Administrator will also, in response to a request or at his/her own discretion, hold a public hearing whenever such a hearing might clarify one or more issues concerning the equivalence of the part 265 closure to a part 264 clo- sure. The Regional Administrator will give public notice of the hearing at least 30 days before it occurs. (Public notice of the hearing may be given at the same time as notice of the oppor- tunity for the public to submit written comments, and the two notices may be combined.)

(ii) The Regional Administrator will determine whether the part 265 closure met 264 closure by removal or decon- tamination requirements within 90 days of its receipt. If the Regional Ad- ministrator finds that the closure did not meet the applicable part 264 stand- ards, he/she will provide the owner/op- erator with a written statement of the reasons why the closure failed to meet part 264 standards. The owner/operator may submit additional information in support of an equivalency demonstra- tion within 30 days after receiving such written statement. The Regional Ad- ministrator will review any additional information submitted and make a final determination within 60 days.

## § 270.2

- If the Regional Administrator determines that the facility did not close in accordance with part 264 clo- sure by removal standards, the facility is subject to post-closure permitting requirements.

- Enforceable documents for post-clo- sure care. At the discretion of the Re- gional Administrator, an owner or op- erator may obtain, in lieu of a post-clo- sure permit, an enforceable document imposing the requirements of 40 CFR

265.121. ‘‘Enforceable document’’ means an order, a plan, or other document issued by EPA or by an authorized State under an authority that meets the requirements of 40 CFR 271.16(e) in- cluding, but not limited to, a correc- tive action order issued by EPA under section 3008(h), a CERCLA remedial ac- tion, or a closure or post-closure plan.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30113, June 30, 1983; 51 FR 10176, Mar. 24,

1986; 52 FR 45798, Dec. 1, 1987; 53 FR 27165,

July 19, 1988; 54 FR 9607, Mar. 7, 1989; 56 FR

32692, July 17, 1991; 60 FR 25542, May 11, 1995;

62 FR 6656, Feb. 12, 1996; 63 FR 56735, Oct. 22,

1998; 64 FR 36488, July 6, 1999; 64 FR 37638,

July 12, 1999; 70 FR 45520, Aug. 5, 2005; 70 FR

53474, Sept. 8, 2005; 71 FR 40279, July 14, 2006;

81 FR 85828, Nov. 28, 2016; 84 FR 5950, Feb. 22,

2019; 84 FR 67217, Dec. 9, 2019; 88 FR 54115,

Aug. 9, 2023]

### § 270.2  Definitions.

The following definitions apply to parts 270, 271 and 124. Terms not de- fined in this section have the meaning given by RCRA.

Administrator means the Adminis- trator of the United States Environ- mental Protection Agency, or an au- thorized representative.

Application means the EPA standard national forms for applying for a per- mit, including any additions, revisions or modifications to the forms; or forms approved by EPA for use in approved States, including any approved modi- fications or revisions. Application also includes the information required by the Director under §§ 270.14 through

270.29 (contents of part B of the RCRA application).

Approved program or approved State means a State which has been approved or authorized by EPA under part 271.

Aquifer means a geological forma- tion, group of formations, or part of a formation that is capable of yielding a

## 40 CFR Ch. I (7–1–24 Edition)

significant amount of water to a well or spring.

Closure means the act of securing a Hazardous Waste Management facility pursuant to the requirements of 40 CFR part 264.

Component means any constituent part of a unit or any group of con- stituent parts of a unit which are as- sembled to perform a specific function (e.g., a pump seal, pump, kiln liner, kiln thermocouple).

Corrective Action Management Unit or CAMU means an area within a facility that is designated by the Regional Ad- ministrator under part 264 subpart S, for the purpose of implementing cor- rective  action  requirements  under

§ 264.101 and RCRA section 3008(h). A CAMU shall only be used for the man- agement of remediation wastes pursu- ant to implementing such corrective action requirements at the facility.

CWA means the Clean Water Act (for- merly referred to as the Federal Water Pollution Control Act or Federal Water Pollution Control Act amendments of 1972) Pub. L. 92–500, as amended by Pub.

L. 92–217 and Pub. L. 95–576; 33 U.S.C.

1251 et seq.

Director means the Regional Adminis- trator or the State Director, as the context requires, or an authorized rep- resentative. When there is no approved State program, and there is an EPA ad- ministered program, Director means the Regional Administrator. When there is an approved State program, Director normally means the State Di- rector. In some circumstances, how- ever, EPA retains the authority to take certain actions even when there is an approved State program. In such cases, the term Director means the Re- gional Administrator and not the State Director.

Disposal means the discharge, de- posit, injection, dumping, spilling, leaking, or placing of any hazardous waste into or on any land or water so that such hazardous waste or any con- stituent thereof may enter the environ- ment or be emitted into the air or dis- charged into any waters, including ground water.

Disposal facility means a facility or part of a facility at which hazardous waste is intentionally placed into or on

## Environmental Protection Agency	§ 270.2

the land or water, and at which haz- ardous waste will remain after closure. The term disposal facility does not in- clude a corrective action management unit into which remediation wastes are placed.

Draft permit means a document pre- pared under § 124.6 indicating the Direc- tor’s tentative decision to issue or deny, modify, revoke and reissue, ter- minate, or reissue a permit. A notice of intent to terminate a permit, and a no- tice of intent to deny a permit, as dis- cussed in § 124.5, are types of draft per- mits. A denial of a request for modi- fication, revocation and reissuance, or termination, as discussed in § 124.5 is not a ‘‘draft permit.’’ A proposed per- mit is not a draft permit.

Elementary neutralization unit means a device which:

- Is used for neutralizing wastes only because they exhibit the corrosivity  characteristic  defined  in

§ 261.22 of this chapter, or are listed in subpart D of part 261 of this chapter only for this reason; and

- Meets the definition of tank, tank system, container, transport vehicle, or vessel in § 260.10 of this chapter.

Emergency permit means a RCRA per- mit issued in accordance with § 270.61.

Environmental Protection Agency (EPA) means the United States Envi- ronmental Protection Agency.

EPA means the United States Envi- ronmental Protection Agency.

Existing hazardous waste management (HWM) facility or existing facility means a facility which was in operation or for which construction commenced on or before November 19, 1980. A facility has commenced construction if:

- The owner or operator has ob- tained the Federal, State and local ap- provals or permits necessary to begin physical construction; and either

(b)(1) A continuous on-site, physical construction program has begun; or

(2) The owner or operator has entered into contractual obligations which can- not be cancelled or modified without substantial loss—for physical construc- tion of the facility to be completed within a reasonable time.

Facility mailing list means the mailing list for a facility maintained by EPA in accordance with 40 CFR 124.10(c)(1)(ix).

Facility or activity means any HWM facility or any other facility or activ- ity (including land or appurtenances thereto) that is subject to regulation under the RCRA program.

Federal, State and local approvals or permits necessary to begin physical con- struction means permits and approvals required under Federal, State or local hazardous waste control statutes, regu- lations or ordinances.

Final authorization means approval by EPA of a State program which has met the requirements of section 3006(b) of RCRA and the applicable requirements of part 271, subpart A.

Functionally equivalent component means a component which performs the same function or measurement and which meets or exceeds the perform- ance specifications of another compo- nent.

Generator means any person, by site location, whose act, or process pro- duces ‘‘hazardous waste’’ identified or listed in 40 CFR part 261.

Ground water means water below the land surface in a zone of saturation.

Hazardous waste means a hazardous waste as defined in 40 CFR 261.3.

Hazardous Waste Management facility (HWM facility) means all contiguous land, and structures, other appur- tenances, and improvements on the land, used for treating, storing, or dis- posing of hazardous waste. A facility may consist of several treatment, stor- age, or disposal operational units (for example, one or more landfills, surface impoundments, or combinations of them).

HWM facility means Hazardous Waste Management facility.

Injection well means a well into which fluids are being injected.

In operation means a facility which is treating, storing, or disposing of haz- ardous waste.

Interim authorization means approval by EPA of a State hazardous waste pro- gram which has met the requirements of section 3006(g)(2) of RCRA and appli- cable requirements of part 271, subpart B.

Major facility means any facility or activity classified as such by the Re- gional Administrator, or, in the case of approved State programs, the Regional

## § 270.2

Administrator in conjunction with the State Director.

Manifest means the shipping docu- ment originated and signed by the gen- erator which contains the information required by subpart B of 40 CFR part 262.

National Pollutant Discharge Elimi- nation System means the national pro- gram for issuing, modifying, revoking and reissuing, terminating, monitoring and enforcing permits, and imposing and enforcing pretreatment require- ments, under sections 307, 402, 318, and

405 of the CWA. The term includes an approved program.

NPDES means National Pollutant Discharge Elimination System.

New HWM facility means a Hazardous Waste Management facility which began operation or for which construc- tion commenced after November 19, 1980.

Off-site means any site which is not on-site.

On-site means on the same or geo- graphically contiguous property which may be divided by public or private right(s)-of-way, provided the entrance and exit between the properties is at a cross-roads intersection, and access is by crossing as opposed to going along, the right(s)-of-way. Non-contiguous properties owned by the same person but connected by a right-of-way which the person controls and to which the public does not have access, is also con- sidered on-site property.

Owner or operator means the owner or operator of any facility or activity sub- ject to regulation under RCRA.

Permit means an authorization, li- cense, or equivalent control document issued by EPA or an approved State to implement the requirements of this part and parts 271 and 124 of this chap- ter. Permit includes permit by rule (§ 270.60), emergency permit (§ 270.61) and standardized permit (subpart J of this part). Permit does not include RCRA interim status (subpart G of this part), or any permit which has not been the subject of final agency action, such as a draft permit or a proposed permit.

Permit-by-rule means a provision of these regulations stating that a facil- ity or activity is deemed to have a

## 40 CFR Ch. I (7–1–24 Edition)

RCRA permit if it meets the require- ments of the provision.

Person means an individual, associa- tion, partnership, corporation, munici- pality, State or Federal agency, or an agent or employee thereof.

Physical construction means exca- vation, movement of earth, erection of forms or structures, or similar activity to prepare an HWM facility to accept hazardous waste.

POTW means publicly owned treat- ment works.

Publicly owned treatment works (POTW) means any device or system used in the treatment (including recy- cling and reclamation) of municipal sewage or industrial wastes of a liquid nature which is owned by a State or municipality. This definition includes sewers, pipes, or other conveyances only if they convey wastewater to a POTW providing treatment.

RCRA means the Solid Waste Dis- posal Act as amended by the Resource Conservation and Recovery Act of 1976 (Pub. L. 94–580, as amended by Pub. L. 95–609 and Pub. L. 96–482, 42 U.S.C. 6901

et seq.)

Regional Administrator means the Re- gional Administrator of the appro- priate Regional Office of the Environ- mental Protection Agency or the au- thorized representative of the Regional Administrator.

Remedial Action Plan (RAP) means a special form of RCRA permit that a fa- cility owner or operator may obtain in- stead of a permit issued under §§ 270.3 through 270.66, to authorize the treat- ment, storage or disposal of hazardous remediation waste (as defined in § 260.10 of this chapter) at a remediation waste management site.

Schedule of compliance means a sched- ule of remedial measures included in a permit, including an enforceable se- quence of interim requirements (for ex- ample, actions, operations, or mile- stone events) leading to compliance with the Act and regulations.

SDWA means the Safe Drinking Water Act (Pub. L. 95–523, as amended by Pub. L. 95–1900; 42 U.S.C. 3001 et

seq.).

Site means the land or water area where any facility or activity is phys- ically located or conducted, including

## Environmental Protection Agency	§ 270.3

adjacent land used in connection with the facility or activity.

Standardized permit means a RCRA permit issued under part 124, subpart G of this chapter and subpart J of this part authorizing the facility owner or operator to manage hazardous waste. The standardized permit may have two parts: A uniform portion issued in all cases and a supplemental portion issued at the Director’s discretion.

State means any of the 50 States, the District of Columbia, Guam, the Com- monwealth of Puerto Rico, the Virgin Islands, American Samoa, and the Commonwealth of the Northern Mar- iana Islands.

State Director means the chief admin- istrative officer of any State agency operating an approved program, or the delegated representative of the State Director. If responsibility is divided among two or more State agencies, State Director means the chief admin- istrative officer of the State agency authorized to perform the particular procedure or function to which ref- erence is made.

State/EPA Agreement means an agree- ment between the Regional Adminis- trator and the State which coordinates EPA and State activities, responsibil- ities and programs.

Storage means the holding of haz- ardous waste for a temporary period, at the end of which the hazardous waste is treated, disposed, or stored elsewhere.

Transfer facility means any transpor- tation-related facility including load- ing docks, parking areas, storage areas and other similar areas where ship- ments of hazardous waste are held dur- ing the normal course of transpor- tation.

Transporter means a person engaged in the off-site transportation of haz- ardous waste by air, rail, highway or water.

Treatment means any method, tech- nique, or process, including neutraliza- tion, designed to change the physical, chemical, or biological character or composition of any hazardous waste so as to neutralize such wastes, or so as to recover energy or material resources from the waste, or so as to render such waste non-hazardous, or less hazardous; safer to transport, store, or dispose of;

or amenable for recovery, amenable for storage, or reduced in volume.

UIC means the Underground Injec- tion Control Program under part C of the Safe Drinking Water Act, including an approved program.

Underground injection means a well injection.

Underground source of drinking water (USDW) means an aquifer or its por- tion:

(a)(1) Which supplies any public water system; or

- Which contains a sufficient quan- tity of ground water to supply a public water system; and

  - Currently supplies drinking water for human consumption; or

  - Contains fewer than 10,000 mg/l total dissolved solids; and

- Which is not an exempted aquifer.

USDW means underground source of drinking water.

Wastewater treatment unit means a de- vice which:

- Is part of a wastewater treatment facility which is subject to regulation under either section 402 or 307(b) of the Clean Water Act; and

- Receives and treats or stores an influent wastewater which is a haz- ardous waste as defined in § 261.3 of this chapter, or generates and accumulates a wastewater treatment sludge which is a hazardous waste as defined in

§ 261.3 of this chapter, or treats or stores a wastewater treatment sludge which is a hazardous waste as defined in § 261.3 of this chapter; and

- Meets the definition of tank or tank system in § 260.10 of this chapter.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30113, June 30, 1983; 53 FR 34087, Sept. 2,

1988; 53 FR 37935, Sept. 28, 1988; 58 FR 8685,

Feb. 16, 1993; 60 FR 33914, June 29, 1995; 60 FR

63433, Dec. 11, 1995; 63 FR 65941, Nov. 30, 1998;

70 FR 53474, Sept. 8, 2005; 71 FR 40279, July 14,

2006]

### § 270.3 Considerations under Federal law.

The following is a list of Federal laws that may apply to the issuance of per- mits under these rules. When any of these laws is applicable, its procedures must be followed. When the applicable law requires consideration or adoption of particular permit conditions or re- quires the denial of a permit, those re- quirements also must be followed.

## § 270.4

  - The Wild and Scenic Rivers Act. 16

U.S.C. 1273 et seq. Section 7 of the Act prohibits the Regional Administrator from assisting by license or otherwise the construction of any water re- sources project that would have a di- rect, adverse effect on the values for which a national wild and scenic river was established.

  - The National Historic Preservation Act of 1966. 16 U.S.C. 470 et seq. Section 106 of the Act and implementing regu- lations (36 CFR part 800) require the Regional Administrator, before issuing a license, to adopt measures when fea- sible to mitigate potential adverse ef- fects of the licensed activity and prop- erties listed or eligible for listing in the National Register of Historic Places. The Act’s requirements are to be implemented in cooperation with State Historic Preservation Officers and upon notice to, and when appro- priate, in consultation with the Advi- sory Council on Historic Preservation.

  - The  Endangered  Species  Act.  16

U.S.C. 1531 et seq. Section 7 of the Act and implementing regulations (50 CFR part 402) require the Regional Adminis- trator to ensure, in consultation with the Secretary of the Interior or Com- merce, that any action authorized by EPA is not likely to jeopardize the con- tinued existence of any endangered or threatened species or adversely affect its critical habitat.

  - The Coastal Zone Management Act.

16 U.S.C. 1451 et seq. Section 307(c) of the Act and implementing regulations (15 CFR part 930) prohibit EPA from issuing a permit for an activity affect- ing land or water use in the coastal zone until the applicant certifies that the proposed activity complies with the State Coastal Zone Management program, and the State or its des- ignated agency concurs with the cer- tification (or the Secretary of Com- merce overrides the State’s nonconcur- rence).

  - The Fish and Wildlife Coordination Act. 16 U.S.C. 661 et seq. requires that the Regional Administrator, before issuing a permit proposing or author- izing the impoundment (with certain exemptions), diversion, or other con- trol or modification of any body of

water, consult with the appropriate State  agency  exercising  jurisdiction

## 40 CFR Ch. I (7–1–24 Edition)

over wildlife resources to conserve those resources.

  - Executive orders. [Reserved]

(Clean Water Act (33 U.S.C. 1251 et seq.), Safe Drinking Water Act (42 U.S.C. 300f et seq.), Clean Air Act (42 U.S.C. 7401 et seq.), Re- source Conservation and Recovery Act (42 U.S.C. 6901 et seq.))

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 39622, Sept. 1, 1983]

### § 270.4  Effect of a permit.

(a)(1) Compliance with a RCRA per- mit during its term constitutes compli- ance, for purposes of enforcement, with subtitle C of RCRA except for those re- quirements not included in the permit which:

    - Become effective by statute;

    - Are promulgated under part 268 of this chapter restricting the placement of hazardous wastes in or on the land;

    - Are promulgated under part 264 of this chapter regarding leak detec- tion systems for new and replacement surface impoundment, waste pile, and landfill units, and lateral expansions of surface impoundment, waste pile, and landfill units. The leak detection sys- tem requirements include double lin- ers, CQA programs, monitoring, action leakage rates, and response action plans, and will be implemented through the procedures of § 270.42 Class 1 permit modifications; or

    - Are promulgated under subparts AA, BB, or CC of part 265 of this chap- ter limiting air emissions.

(2) A permit may be modified, re- voked and reissued, or terminated dur- ing its term for cause as set forth in

§§ 270.41 and 270.43, or the permit may be modified upon the request of the permittee as set forth in § 270.42.

- The issuance of a permit does not convey any property rights of any sort, or any exclusive privilege.

- The issuance of a permit does not authorize any injury to persons or property or invasion of other private rights, or any infringement of State or local law or regulations.

[48 FR 14228, Apr. 1, 1983, as amended at 57

FR 3495, Jan. 29, 1992; 59 FR 62952, Dec. 6,

1994; 75 FR 13009, Mar. 18, 2010]

## Environmental Protection Agency	§ 270.5

### § 270.5 Noncompliance and program reporting by the Director.

The Director shall prepare quarterly and annual reports as detailed below. When the State is the permit-issuing authority, the State Director shall sub- mit any reports required under this section to the Regional Administrator. When EPA is the permit-issuing au- thority, the Regional Administrator shall submit any report required under this section to EPA Headquarters. For purposes of this section only, RCRA permittees shall include RCRA interim status facilities, when appropriate.

  - Quarterly reports. The Director shall submit quarterly narrative re- ports for major facilities as follows:

    - Format. The report shall use the following format:

      - Information on noncompliance for each facility;

      - Alphabetize by permittee name. When two or more permittees have the same name, the lowest permit number shall be entered first; and

      - For each entry on the list, in- clude the following information in the following order:

        - Name, location, and permit num- ber of the noncomplying permittee.

        - A brief description and date of each instance of noncompliance for that permittee. Instances of non- compliance may include one or more of the kinds set forth in paragraph (a)(2) of this section. When a permittee has noncompliance of more than one kind, combine the information into a single entry for each such permittee.

        - The date(s) and a brief descrip-

tion of the action(s) taken by the Di- rector to ensure compliance.

        - Status of the instance(s) of non- compliance with the date of the review of the status or the date of resolution.

        - Any details which tend to explain or mitigate the instance(s) of non- compliance.

    - Instances of noncompliance to be re- ported. Any instances of noncompliance within the following categories shall be reported in successive reports until the noncompliance is reported as resolved. Once noncompliance is reported as re- solved it need not appear in subsequent reports.

      - Failure to complete construction ele-

ments. When the permittee has failed to

complete, by the date specified in the permit, an element of a compliance schedule involving either planning for construction (for example, award of a contract, preliminary plans), or a con- struction step (for example, begin con- struction, attain operation level); and the permittee has not returned to com- pliance by accomplishing the required element of the schedule within 30 days from the date a compliance schedule report is due under the permit.

      - Modifications to schedules of com- pliance. When a schedule of compliance in the permit has been modified under

§ 270.41 or § 270.42 because of the permit- tee’s noncompliance.

      - Failure to complete or provide com- pliance schedule or monitoring reports. When the permittee has failed to com- plete or provide a report required in a permit compliance schedule (for exam- ple, progress report or notice of non- compliance or compliance) or a moni- toring report; and the permittee has not submitted the complete report within 30 days from the date it is due under the permit for compliance sched- ules, or from the date specified in the permit for monitoring reports.

      - Deficient reports. When the re- quired reports provided by the per- mittee are so deficient as to cause mis- understanding by the Director and thus impede the review of the status of com- pliance.

      - Noncompliance with other permit re- quirements. Noncompliance shall be re- ported in the following circumstances:

- Whenever the permittee has vio- lated a permit requirement (other than reported under paragraph (a)(2)(i) or

(ii) of this section), and has not re- turned to compliance within 45 days from the date reporting of noncompli- ance was due under the permit; or

- When the Director determines that a pattern of noncompliance exists for a major facility permittee over the most recent four consecutive reporting periods. This pattern includes any vio- lation of the same requirement in two consecutive reporting periods, and any violation of one or more requirements in each of four consecutive reporting periods; or

- When the Director determines significant permit non-compliance or other significant event has occurred

## § 270.6

such as a fire or explosion or migration of fluids into a USDW.

      - All other. Statistical information shall be reported quarterly on all other instances of noncompliance by major facilities with permit requirements not otherwise reported under paragraph (a) of this section.

  - Annual reports—(1) Annual non- compliance report. Statistical reports shall be submitted by the Director on nonmajor RCRA permittees indicating the total number reviewed, the number of noncomplying nonmajor permittees, the number of enforcement actions, and number of permit modifications extending compliance deadlines. The statistical information shall be orga- nized to follow the types of noncompli- ance listed in paragraph (a) of this sec- tion.

(2) In addition to the annual non- compliance report, the Director shall prepare a ‘‘program report’’ which con- tains information (in a manner and form prescribed by the Administrator) on generators and transporters and the permit status of regulated facilities. The Director shall also include, on a bi- ennial basis, summary information on the quantities and types of hazardous wastes generated, transported, treated, stored and disposed during the pre- ceding odd-numbered year. This sum- mary information shall be reported in a manner and form prescribed by the Administrator and shall be reported ac- cording to EPA characteristics and lists of hazardous wastes at 40 CFR part 261.

  - Schedule. (1) For all quarterly re-

ports. On the last working day of May, August, November, and February, the State Director shall submit to the Re- gional Administrator information con- cerning noncompliance with RCRA per- mit requirements by major facilities in the State in accordance with the fol- lowing schedule. The Regional Admin- istrator shall prepare and submit infor- mation for EPA-issued permits to EPA Headquarters in accordance with the same schedule.

Quarters Covered by Reports on

Noncompliance by Major Dischargers

[Date for completion of reports]

January, February, and March ... 1 May 31 April, May, and June	1 August 31

July, August, and September	1 November 30

## 40 CFR Ch. I (7–1–24 Edition)

Quarters Covered by Reports on Non-

COMPLIANCE BY MAJOR DISCHARGERS—Con-

tinued

[Date for completion of reports]

October, November, and Decem- 1 February 28 ber.

1 Reports must be made available to the public for inspec- tion and copying on this date.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30113, June 30, 1983]

### § 270.6  References.

  - When used in part 270 of this chapter, the following publications are incorporated		by	reference.	These incorporations by reference were ap- proved by the Director of the Federal Register pursuant to 5 U.S.C. 552(a) and 1 CFR part 51. These materials are in- corporated as they exist on the date of approval and a notice of any change in these materials will be published in the FEDERAL REGISTER. Copies may be in- spected at the Library, U.S. Environ- mental Protection Agency, 1200 Penn- sylvania Ave., NW., (3403T), Wash- ington, DC 20460, libraryhq@epa.gov; or at the National Archives and Records Administration (NARA). For informa- tion on the availability of this mate- rial at NARA, call 202–741–6030, or go to:		http://www.archives.gov/fed- eralregister/codeoffederalregulations/ ibrlocations.html.

  - The following materials are avail- able for purchase from the National Technical Information Service (NTIS), 5285 Port Royal Road, Springfield, VA 22161, (703) 605–6000 or (800) 553–6847; or

for purchase from the Superintendent of Documents, U.S. Government Print- ing Office, Washington, DC 20402, (202)

512–1800:

- ‘‘APTI Course 415: Control of Gas- eous Emissions,’’ EPA Publication EPA–450/2–81–005, December 1981, IBR

approved for §§ 270.24 and 270.25.

- [Reserved]

[70 FR 59576, Oct. 12, 2005]

# Subpart B—Permit Application

### § 270.10 General application require- ments.

- Applying for a permit. Below is information on how to obtain a permit and where to find requirements for spe- cific permits:

## Environmental Protection Agency	§ 270.10

  - If you are covered by RCRA per- mits by rule (§ 270.60), you need not apply.

  - If you currently have interim sta- tus, you must apply for permits when required by the Director.

  - If you are required to have a per- mit (including new applicants and per- mittees with expiring permits), you must complete, sign, and submit an ap- plication to the Director, as described in this section and §§ 270.70 through 270.73.

  - If you are seeking an emergency permit, the procedures for application, issuance, and administration are found exclusively in § 270.61.

  - If you are seeking a research, de- velopment, and demonstration permit, the procedures for application, issuance, and administration are found exclusively in § 270.65.

  - If you are seeking a standardized permit, the procedures for application and issuance are found in part 124, sub- part G of this chapter and subpart J of this part.

- Who applies? When a facility or ac- tivity is owned by one person but is op- erated by another person, it is the op- erator’s duty to obtain a permit, ex- cept that the owner must also sign the permit application.

- Completeness. The Director shall not issue a permit before receiving a complete application for a permit ex- cept for permits by rule, or emergency permits. An application for a permit is complete when the Director receives an application form and any supplemental information which are completed to his satisfaction. An application for a per- mit is complete notwithstanding the failure of the owner or operator to sub- mit the exposure information described in paragraph (j) of this section. The Di- rector may deny a permit for the ac- tive life of a hazardous waste manage- ment facility or unit before receiving a complete application for a permit.

- Information requirements. All appli-

cants for RCRA permits shall provide information set forth in § 270.13 and ap- plicable sections in §§ 270.14 through

270.29 to the Director, using the appli- cation form provided by the Director.

- Existing HWM facilities and interim status qualifications. (1) Owners and op- erators  of  existing  hazardous  waste

management facilities or of hazardous waste management facilities in exist- ence on the effective date of statutory or regulatory amendments under the act that render the facility subject to the requirement to have a RCRA per- mit must submit part A of their permit application no later than:

  - Six months after the date of publi- cation of regulations which first re- quire them to comply with the stand- ards set forth in 40 CFR part 265 or 266, or

  - Thirty days after the date they first become subject to the standards set forth in 40 CFR part 265 or 266, whichever first occurs.

  - For generators generating great- er than 100 kilograms but less than 1000 kilograms of hazardous waste in a cal- endar month and treats, stores, or dis- poses of these wastes on-site, by March 24, 1987.

NOTE: For facilities which must comply with part 265 because they handle a waste listed in EPA’s May 19, 1980, part 261 regula- tions (45 FR 33006 et seq.), the deadline for submitting an application is November 19, 1980. Where other existing facilities must begin in complying with part 265 or 266 at a later date because of revisions to part 260, 261, 265, or 266, the Administrator will specify in the preamble to those revisions when those facilities must submit a permit appli- cation.

- The Administrator may by publi- cation in the FEDERAL REGISTER extend the date by which owners and opera- tors of specified classes of existing haz- ardous waste management facilities must submit part A of their permit ap- plication if he finds that (i) there has been substantial confusion as to wheth- er the owners and operators of such fa- cilities were required to file a permit application and (ii) such confusion is attributed to ambiguities in EPA’s parts 260, 261, 265, or 266 regulations.

- The Administrator may by com-

pliance order issued under section 3008 of RCRA extend the date by which the owner and operator of an existing haz- ardous waste management facility must submit part A of their permit ap- plication.

- The owner or operator of an exist- ing hazardous waste management facil- ity may be required to submit part B of their permit application. The State Di- rector may require submission of part

## § 270.10

B (or equivalent completion of the State RCRA application process) if the State in which the facility is located has received interim or final authoriza- tion; if not, the Regional Adminis- trator may require submission of Part

B. Any owner or operator shall be al- lowed at least six months from the date of request to submit part B of the application. Any owner or operator of an existing hazardous waste manage- ment facility may voluntarily submit part B of the application at any time. Notwithstanding the above, any owner or operator of an existing hazardous waste management facility must sub- mit a part B permit application in ac- cordance with the dates specified in

§ 270.73. Any owner or operator of a land disposal facility in existence on the ef- fective date of statutory or regulatory amendments under this Act that render the facility subject to the re- quirement to have a RCRA permit must submit a part B application in ac- cordance with the dates specified in

§ 270.73.

- Failure to furnish a requested part B application on time, or to fur- nish in full the information required by the part B application, is grounds for termination of interim status under part 124.

- New HWM facilities. (1) Except as provided in paragraph (f)(3) of this sec- tion, no person shall begin physical construction of a new HWM facility without having submitted parts A and B of the permit application and having received a finally effective RCRA per- mit.

(2) An application for a permit for a new hazardous waste management fa- cility (including both Parts A and B) may be filed any time after promulga- tion of those standards in part 264, sub- part I et seq. applicable to such facility. The application shall be filed with the Regional Administrator if at the time of application the State in which the new hazardous waste management fa- cility is proposed to be located has not received interim or final authorization for permitting such facility; otherwise it shall be filed with the State Direc- tor. Except as provided in paragraph (f)(3) of this section, all applications must be submitted at least 180 days be-

## 40 CFR Ch. I (7–1–24 Edition)

fore physical construction is expected to commence.

- Notwithstanding paragraph (f)(1) of this section, a person may construct a facility for the incineration of poly- chlorinated biphenyls pursuant to an approval issued by the Administrator under section (6)(e) of the Toxic Sub- stances Control Act and any person owning or operating such a facility may, at any time after construction or operation of such facility has begun, file an application for a RCRA permit to incinerate hazardous waste author- izing such facility to incinerate waste identified or listed under Subtitle C of RCRA.

  - Updating permit applications. (1) If

any owner or operator of a hazardous waste management facility has filed Part A of a permit application and has not yet filed part B, the owner or oper- ator shall file an amended part A appli- cation:

    - With the Regional Administrator if the facility is located in a State which has not obtained interim author- ization or final authorization, within six months after the promulgation of revised regulations under part 261 list- ing or identifying additional hazardous wastes, if the facility is treating, stor- ing or disposing of any of those newly listed or identified wastes.

    - With the State Director, if the fa- cility is located in a State which has obtained interim authorization or final authorization, no later than the effec- tive date of regulatory provisions list- ing or designating wastes as hazardous in that State in addition to those listed or designated under the previously ap- proved State program, if the facility is treating, storing or disposing of any of those newly listed or designated wastes; or

    - As necessary to comply with pro- visions of § 270.72 for changes during in- terim status or with the analogous pro- visions of a State program approved for final authorization or interim author- ization. Revised Part A applications necessary to comply with the provi- sions of § 270.72 shall be filed with the Regional Administrator if the State in which the facility in question is lo- cated does not have interim authoriza- tion or final authorization; otherwise it shall be filed with the State Director

## Environmental Protection Agency	§ 270.10

(if the State has an analogous provi- sion).

(2) The owner or operator of a facility who fails to comply with the updating requirements of paragraph (g)(1) of this section does not receive interim status as to the wastes not covered by duly filed part A applications.

  - Reapplying for a permit. If you have an effective permit and you want to reapply for a new one, you have two options:

- You may submit a new applica- tion at least 180 days before the expira- tion date of the effective permit, unless the Director allows a later date; or

- If you intend to be covered by a standardized permit, you may submit a Notice  of  Intent  as  described  in

§ 270.51(e)(1) at least 180 days before the expiration date of the effective permit, unless the Director allows a later date. The Director may not allow you to sub- mit applications or Notices of Intent later than the expiration date of the existing permit, except as allowed by

§ 270.51(e)(2).

  - Recordkeeping. Applicants shall keep records of all data used to com- plete permit applications and any sup- plemental information submitted under §§ 270.10(d), 270.13, 270.14 through

270.21 for a period of at least 3 years from the date the application is signed.

  - Exposure information. (1) After Au- gust 8, 1985, any part B permit applica- tion submitted by an owner or operator of a facility that stores, treats, or dis- poses of hazardous waste in a surface impoundment or a landfill must be ac- companied by information, reasonably ascertainable by the owner or operator, on the potential for the public to be ex- posed to hazardous wastes or hazardous constituents through releases related to the unit. At a minimum, such infor- mation must address:

    - Reasonably foreseeable potential releases from both normal operations and accidents at the unit, including re- leases associated with transportation to or from the unit;

    - The potential pathways of human exposure to hazardous wastes or con- stituents resulting from the releases described under paragraph (j)(1)(i) of this section; and

    - The potential magnitude and na- ture of the human exposure resulting from such releases.

(2) By August 8, 1985, owners and op- erators of a landfill or a surface im- poundment who have already sub- mitted a part B application must sub- mit the exposure information required in paragraph (j)(1) of this section.

  - The Director may require a per- mittee or an applicant to submit infor- mation in order to establish permit conditions under §§ 270.32(b)(2) and 270.50(d) of this chapter.

  - If the Director concludes, based on one or more of the factors listed in paragraph (l)(1) of this section that compliance with the standards of 40 CFR part 63, subpart EEE alone may not be protective of human health or the environment, the Director shall re- quire the additional information or as- sessment(s) necessary to determine whether additional controls are nec- essary to ensure protection of human health and the environment. This in- cludes information necessary to evalu- ate the potential risk to human health and/or the environment resulting from both direct and indirect exposure path- ways. The Director may also require a permittee or applicant to provide infor- mation necessary to determine wheth- er such an assessment(s) should be re- quired.

- The Director shall base the eval-

uation of whether compliance with the standards of 40 CFR part 63, subpart EEE alone is protective of human health or the environment on factors relevant to the potential risk from a hazardous waste combustion unit, in- cluding, as appropriate, any of the fol- lowing factors:

  - Particular site-specific consider- ations such as proximity to receptors (such as schools, hospitals, nursing homes, day care centers, parks, com- munity activity centers, or other po- tentially sensitive receptors), unique dispersion patterns, etc.;

  - Identities and quantities of emis- sions of persistent, bioaccumulative or toxic pollutants considering enforce- able controls in place to limit those pollutants;

  - Identities and quantities of nondioxin products of incomplete com- bustion most likely to be emitted and

## § 270.11

to pose significant risk based on known toxicities (confirmation of which should be made through emissions test- ing);

  - Identities and quantities of other off-site sources of pollutants in prox- imity of the facility that significantly influence interpretation of a facility- specific risk assessment;

  - Presence of significant ecological considerations, such as the proximity of a particularly sensitive ecological area;

  - Volume and types of wastes, for example wastes containing highly toxic constituents;

  - Other on-site sources of haz- ardous air pollutants that significantly influence interpretation of the risk posed by the operation of the source in question;

  - Adequacy of any previously conducted risk assessment, given any subsequent changes in conditions like- ly to affect risk; and

  - Such other factors as may be ap- propriate.

- [Reserved]

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983, as amended at 50 FR 28751, July 15, 1985;

51 FR 10176, Mar. 24, 1986; 52 FR 45799, Dec. 1,

1987; 54 FR 9607, Mar. 7, 1989; 60 FR 33914,

June 29, 1995; 70 FR 53474, Sept. 8, 2005; 70 FR

59576, Oct. 12, 2005; 71 FR 40279, July 14, 2006]

### § 270.11 Signatories to permit applica- tions and reports.

- Applications. All permit applica- tions shall be signed as follows:

  - For a corporation: By a responsible corporate officer. For the purpose of this section, a responsible corporate of- ficer means (i) A president, secretary, treasurer, or vice-president of the cor- poration in charge of a principal busi- ness function, or any other person who performs similar policy- or decision- making functions for the corporation, or (ii) the manager of one or more man- ufacturing, production or operating fa- cilities employing more than 250 per- sons or having gross annual sales or ex- penditures exceeding $25 million (in second-quarter 1980 dollars), if author- ity to sign documents has been as- signed or delegated to the manager in accordance with corporate procedures.

NOTE: EPA does not require specific assign- ments or delegations of authority to respon-

## 40 CFR Ch. I (7–1–24 Edition)

sible  corporate  officers  identified  in

§ 270.11(a)(1)(i). The Agency will presume that these responsible corporate officers have the requisite authority to sign permit applica- tions unless the corporation has notified the Director to the contrary. Corporate proce- dures governing authority to sign permit ap- plications may provide for assignment or delegation to applicable corporate positions under § 270.11(a)(1)(ii) rather than to specific individuals.

  - For a partnership or sole proprietor- ship; by a general partner or the propri- etor, respectively; or

  - For a municipality, State, Federal, or other public agency: by either a prin- cipal executive officer or ranking elect- ed official. For purposes of this section, a principal executive officer of a Fed- eral agency includes: (i) The chief exec- utive officer of the agency, or (ii) a senior executive officer having respon- sibility for the overall operations of a principal geographic unit of the agency (e.g., Regional Administrators of EPA).

- Reports. All reports required by permits and other information re- quested by the Director shall be signed by a person described in paragraph (a) of this section, or by a duly authorized representative of that person. A person is a duly authorized representative only if:

  - The authorization is made in writ- ing by a person described in paragraph

(a) of this section;

  - The authorization specifies either an individual or a position having re- sponsibility for overall operation of the regulated facility or activity such as the position of plant manager, operator of a well or a well field, super- intendent, or position of equivalent re- sponsibility. (A duly authorized rep- resentative may thus be either a named individual or any individual oc- cupying a named position); and

  - The written authorization is sub- mitted to the Director.

- Changes to authorization. If an au- thorization under paragraph (b) of this section is no longer accurate because a different individual or position has re- sponsibility for the overall operation of the facility, a new authorization satis- fying the requirements of paragraph (b) of this section must be submitted to the Director prior to or together with

## Environmental Protection Agency	§ 270.13

any reports, information, or applica- tions to be signed by an authorized rep- resentative.

(d)(1) Any person signing a document under paragraph (a) or (b) of this sec- tion must make the following certifi- cation:

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision according to a system designed to assure that qualified personnel properly gather and evaluate the information submitted. Based on my inquiry of the person or persons who manage the sys- tem, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

- For remedial action plans (RAPs) under subpart H of this part, if the op- erator certifies according to paragraph (d)(1) of this section, then the owner may choose to make the following cer- tification instead of the certification in paragraph (d)(1) of this section:

Based on my knowledge of the conditions of the property described in the RAP and my inquiry of the person or persons who manage the system referenced in the operator’s cer- tification, or those persons directly respon- sible for gathering the information, the in- formation submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are signifi- cant penalties for submitting false informa- tion, including the possibility of fine and im- prisonment for knowing violations.

(Clean Water Act (33 U.S.C. 1251 et seq.), Safe Drinking Water Act (42 U.S.C. 300f et seq.), Clean Air Act (42 U.S.C. 7401 et seq.), Re- source Conservation and Recovery Act (42 U.S.C. 6901 et seq.))

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 39622, Sept. 1, 1983; 63 FR 65941, Nov. 30,

1998; 71 FR 40279, July 14, 2006]

### § 270.12  Confidentiality of information.

  - In accordance with 40 CFR part 2, any information submitted to EPA pursuant to these regulations may be claimed as confidential by the sub- mitter. Any such claim must be as- serted at the time of submission in the manner prescribed on the application form or instructions or, in the case of other submissions, by stamping the words ‘‘confidential business informa-

tion’’ on each page containing such in- formation. If no claim is made at the time of submission, EPA may make the information available to the public without further notice. If a claim is as- serted, the information will be treated in accordance with the procedures in 40 CFR part 2 (Public Information).

  - Claims of confidentiality for the name and address of any permit appli- cant or permittee will be denied.

### § 270.13 Contents of part A of the per- mit application.

Part A of the RCRA application shall include the following information:

- The activities conducted by the applicant which require it to obtain a permit under RCRA.

- Name, mailing address, and loca- tion, including latitude and longitude of the facility for which the applica- tion is submitted.

- Up to four SIC codes which best reflect the principal products or serv- ices provided by the facility.

- The operator’s name, address, telephone number, ownership status, and status as Federal, State, private, public, or other entity.

- The name, address, and phone number of the owner of the facility.

- Whether the facility is located on Indian lands.

- An indication of whether the fa- cility is new or existing and whether it is a first or revised application.

- For existing facilities, (1) a scale drawing of the facility showing the lo- cation of all past, present, and future treatment, storage, and disposal areas; and (2) photographs of the facility clearly delineating all existing struc- tures; existing treatment, storage, and disposal areas; and sites of future treatment, storage, and disposal areas.

- A description of the processes to be used for treating, storing, and dis- posing of hazardous waste, and the de- sign capacity of these items.

- A specification of the hazardous wastes listed or designated under 40 CFR part 261 to be treated, stored, or disposed of at the facility, an estimate of the quantity of such wastes to be treated, stored, or disposed annually, and a general description of the proc- esses to be used for such wastes.

## § 270.14

- A listing of all permits or con- struction approvals received or applied for under any of the following pro- grams:

  - Hazardous Waste Management program under RCRA.

  - UIC program under the SWDA.

  - NPDES program under the CWA.

  - Prevention of Significant Deterio- ration (PSD) program under the Clean Air Act.

  - Nonattainment program under the Clean Air Act.

  - National Emission Standards for Hazardous Pollutants (NESHAPS) preconstruction approval under the Clean Air Act.

  - Ocean dumping permits under the Marine Protection Research and Sanc- tuaries Act.

  - Dredge or fill permits under sec- tion 404 of the CWA.

  - Other relevant environmental per- mits, including State permits.

- A topographic map (or other map if a topographic map is unavailable) ex- tending one mile beyond the property boundaries of the source, depicting the facility and each of its intake and dis- charge structures; each of its haz- ardous waste treatment, storage, or disposal facilities; each well where fluids from the facility are injected un- derground; and those wells, springs, other surface water bodies, and drink- ing water wells listed in public records or otherwise known to the applicant within 1⁄4 mile of the facility property boundary.

- A brief description of the nature

of the business.

- For hazardous debris, a descrip- tion of the debris category(ies) and contaminant category(ies) to be treat- ed, stored, or disposed of at the facil- ity.

[48 FR 14228, Apr. 1, 1983, as amended at 57

FR 37281, Aug. 18, 1992; 71 FR 40279, July 14,

2006]

### § 270.14 Contents of part B: General requirements.

- Part B of the permit application consists of the general information re- quirements of this section, and the spe- cific  information  requirements  in

§§ 0.14 through 270.29 applicable to the facility. The part B information re- quirements  presented  in  §§ 270.14

## 40 CFR Ch. I (7–1–24 Edition)

through 270.29 reflect the standards promulgated in 40 CFR part 264. These information requirements are nec- essary in order for EPA to determine compliance with the part 264 standards. If owners and operators of HWM facili- ties can demonstrate that the informa- tion prescribed in part B can not be provided to the extent required, the Di- rector may make allowance for submis- sion of such information on a case-by- case basis. Information required in part B shall be submitted to the Director and signed in accordance with the re- quirements in § 270.11. Certain tech- nical data, such as design drawings and specifications, and engineering studies shall be certified by a qualified Profes- sional Engineer. For post-closure per- mits, only the information specified in

§ 270.28 is required in part B of the per-

mit application.

- General information requirements. The following information is required for all HWM facilities, except as § 264.1 provides otherwise:

  - A general description of the facil- ity.

  - Chemical and physical analyses of the hazardous waste and hazardous de- bris to be handled at the facility. At a minimum, these analyses shall contain all the information which must be known to treat, store, or dispose of the wastes properly in accordance with part 264 of this chapter.

  - A copy of the waste analysis plan required by § 264.13(b) and, if applicable

§ 264.13(c).

  - A description of the security pro- cedures and equipment required by

§ 264.14, or a justification dem- onstrating the reasons for requesting a waiver of this requirement.

  - A copy of the general inspection schedule required by § 264.15(b) of this part. Include where applicable, as part of the inspection schedule, specific re- quirements in §§ 264.174, 264.193(i), 264.195, 264.226, 264.254, 264.273, 264.303,

264.602,	264.1033,	264.1052,	264.1053,

264.1058, 264.1084, 264.1085, 264.1086, and

264.1088 of this part.

  - A justification of any request for a waiver(s) of the preparedness and pre- vention requirements of part 264, sub- part C.

## Environmental Protection Agency	§ 270.14

  - A copy of the contingency plan re- quired by part 264, subpart D. Note: In- clude, where applicable, as part of the contingency plan, specific require- ments in §§ 264.227, 264.255, and 264.200.

  - A description of procedures, struc- tures, or equipment used at the facility to:

    - Prevent hazards in unloading op- erations (for example, ramps, special forklifts);

    - Prevent runoff from hazardous waste handling areas to other areas of the facility or environment, or to pre- vent flooding (for example, berms, dikes, trenches);

    - Prevent contamination of water supplies;

    - Mitigate effects of equipment failure and power outages;

    - Prevent undue exposure of per- sonnel to hazardous waste (for exam- ple, protective clothing); and

    - Prevent releases to atmosphere.

  - A description of precautions to prevent accidental ignition or reaction of ignitable, reactive, or incompatible wastes as required to demonstrate compliance with § 264.17 including docu- mentation demonstrating compliance with § 264.17(c).

  - Traffic pattern, estimated vol- ume (number, types of vehicles) and control (for example, show turns across traffic lanes, and stacking lanes (if ap- propriate); describe access road sur- facing and load bearing capacity; show traffic control signals).

  - Facility location information;

    - In order to determine the applica- bility  of  the  seismic  standard [§ 264.18(a)] the owner or operator of a new facility must identify the political jurisdiction (e.g., county, township, or election district) in which the facility is proposed to be located.

[Comment: If the county or election district is not listed in appendix VI of part 264, no further information is required to dem- onstrate compliance with § 264.18(a).]

    - If the facility is proposed to be lo- cated in an area listed in appendix VI of part 264, the owner or operator shall demonstrate compliance with the seis- mic standard. This demonstration may be made using either published geo- logic data or data obtained from field investigations carried out by the appli- cant. The information provided must

be of such quality to be acceptable to geologists experienced in identifying and evaluating seismic activity. The information submitted must show that either:

- No faults which have had dis- placement in Holocene time are present, or no lineations which suggest the presence of a fault (which have dis- placement in Holocene time) within 3,000 feet of a facility are present, based on data from:

  - Published geologic studies,

  - Aerial reconnaissance of the area within a five-mile radius from the fa- cility.

  - An analysis of aerial photographs covering a 3,000 foot radius of the facil- ity, and

  - If needed to clarify the above data, a reconnaissance based on walk- ing portions of the area within 3,000 feet of the facility, or

- If faults (to include lineations) which have had displacement in Holo- cene time are present within 3,000 feet of a facility, no faults pass within 200 feet of the portions of the facility where treatment, storage, or disposal of hazardous waste will be conducted, based on data from a comprehensive geologic analysis of the site. Unless a site analysis is otherwise conclusive concerning the absence of faults within 200 feet of such portions of the facility data shall be obtained from a sub- surface exploration (trenching) of the area within a distance no less than 200 feet from portions of the facility where treatment, storage, or disposal of haz- ardous waste will be conducted. Such trenching shall be performed in a direc- tion that is perpendicular to known faults (which have had displacement in Holocene time) passing within 3,000 feet of the portions of the facility where treatment, storage, or disposal of hazardous waste will be conducted. Such investigation shall document with supporting maps and other anal- yses, the location of faults found.

[Comment: The Guidance Manual for the Lo- cation Standards provides greater detail on the content of each type of seismic inves- tigation and the appropriate conditions under which each approach or a combination of approaches would be used.]

    - Owners and operators of all fa- cilities shall provide an identification

## § 270.14

of whether the facility is located with- in a 100-year floodplain. This identi- fication must indicate the source of data for such determination and in- clude a copy of the relevant Federal In- surance Administration (FIA) flood map, if used, or the calculations and maps used where an FIA map is not available. Information shall also be provided identifying the 100-year flood level and any other special flooding factors (e.g., wave action) which must be considered in designing, con- structing, operating, or maintaining the facility to withstand washout from a 100-year flood.

[Comment: Where maps for the National Flood Insurance Program produced by the Federal Insurance Administration (FIA) of the Federal Emergency Management Agency are available, they will normally be deter- minative of whether a facility is located within or outside of the 100-year floodplain. However, where the FIA map excludes an area (usually areas of the floodplain less than 200 feet in width), these areas must be considered and a determination made as to whether they are in the 100-year floodplain. Where FIA maps are not available for a pro- posed facility location, the owner or oper- ator must use equivalent mapping tech- niques to determine whether the facility is within the 100-year floodplain, and if so lo- cated, what the 100-year flood elevation would be.]

    - Owners and operators of facilities located in the 100-year floodplain must provide the following information:

      - Engineering analysis to indicate the various hydrodynamic and hydro- static forces expected to result at the site as consequence of a 100-year flood.

      - Structural or other engineering studies showing the design of oper- ational units (e.g., tanks, incinerators) and flood protection devices (e.g., floodwalls, dikes) at the facility and how these will prevent washout.

      - If applicable, and in lieu of para- graphs (b)(11)(iv) (A) and (B) of this section, a detailed description of proce- dures to be followed to remove haz- ardous waste to safety before the facil- ity is flooded, including:

        - Timing of such movement relative to flood levels, including estimated time to move the waste, to show that such movement can be completed be- fore floodwaters reach the facility.

## 40 CFR Ch. I (7–1–24 Edition)

        - A description of the location(s) to which the waste will be moved and demonstration that those facilities will be eligible to receive hazardous waste in accordance with the regulations under parts 270, 271, 124, and 264 through 266 of this chapter.

        - The planned procedures, equip- ment, and personnel to be used and the means to ensure that such resources will be available in time for use.

        - The potential for accidental dis- charges of the waste during movement.

    - Existing facilities NOT in compli- ance with § 264.18(b) shall provide a plan showing how the facility will be brought into compliance and a sched- ule for compliance.

  - An outline of both the introduc- tory and continuing training programs by owners or operators to prepare per- sons to operate or maintain the HWM facility in a safe manner as required to demonstrate compliance with § 264.16. A brief description of how training will be designed to meet actual job tasks in accordance  with  requirements  in

§ 264.16(a)(3).

  - A copy of the closure plan and, where applicable, the post-closure plan required by  §§ 264.112,  264.118,  and

264.197. Include, where applicable, as part of the plans, specific requirements in §§ 264.178, 264.197, 264.228, 264.258,

264.280, 264.310, 264.351, 264.601, and

264.603.

  - For hazardous waste disposal units that have been closed, docu- mentation that notices required under

§ 264.119 have been filed.

  - The most recent closure cost es- timate for the facility prepared in ac- cordance with § 264.142 and a copy of the documentation required to dem- onstrate  financial  assurance  under

§ 264.143. For a new facility, a copy of the required documentation may be submitted 60 days prior to the initial receipt of hazardous wastes, if that is later than the submission of the part B.

  - Where applicable, the most re- cent post-closure cost estimate for the facility prepared in accordance with

§ 264.144 plus a copy of the documenta- tion required to demonstrate financial assurance under § 264.145. For a new fa- cility, a copy of the required docu- mentation may be submitted 60 days

## Environmental Protection Agency	§ 270.14

prior to the initial receipt of hazardous wastes, if that is later than the submis- sion of the part B.

  - Where applicable, a copy of the insurance policy or other documenta- tion which comprises compliance with the requirements of § 264.147. For a new facility, documentation showing the amount of insurance meeting the speci- fication of § 264.147(a) and, if applicable,

§ 264.147(b), that the owner or operator plans to have in effect before initial re- ceipt of hazardous waste for treatment, storage, or disposal. A request for a variance in the amount of required cov- erage, for a new or existing facility, may  be  submitted  as  specified  in

§ 264.147(c).

  - Where appropriate, proof of cov- erage by a State financial mechanism in compliance with § 264.149 or § 264.150.

  - A topographic map showing a dis- tance of 1,000 feet around the facility at a scale of 2.5 centimeters (1 inch) equal to not more than 61.0 meters (200 feet). Contours must be shown on the map. The contour interval must be suf- ficient to clearly show the pattern of surface water flow in the vicinity of and from each operational unit of the facility. For example, contours with an interval of 1.5 meters (5 feet), if relief is greater than 6.1 meters (20 feet), or an interval of 0.6 meters (2 feet), if re- lief is less than 6.1 meters (20 feet). Owners and operators of HWM facilities located in mountainous areas should use large contour intervals to ade- quately show topographic profiles of fa- cilities. The map shall clearly show the following:

    - Map scale and date.

    - 100-year floodplain area.

    - Surface waters including inter- mittent streams.

    - Surrounding land uses (residen- tial, commercial, agricultural, rec- reational).

    - A wind rose (i.e., prevailing wind- speed and direction).

    - Orientation of the map (north arrow).

    - Legal boundaries of the HWM fa- cility site.

    - Access control (fences, gates).

    - Injection and withdrawal wells both on-site and off-site.

    - Buildings; treatment, storage, or disposal operations; or other structure

(recreation areas, runoff control sys- tems, access and internal roads, storm, sanitary, and process sewerage sys- tems, loading and unloading areas, fire control facilities, etc.)

    - Barriers for drainage or flood control.

    - Location of operational units within the HWM facility site, where hazardous waste is (or will be) treated, stored, or disposed (include equipment cleanup areas).

NOTE: For large HWM facilities the Agency will allow the use of other scales on a case- by-case basis.

  - Applicants may be required to submit such information as may be necessary to enable the Regional Ad- ministrator to carry out his duties under other Federal laws as required in

§ 270.3 of this part.

  - For land disposal facilities, if a case-by-case extension has been ap- proved under § 268.5 or a petition has been approved under § 268.6, a copy of the notice of approval for the extension or petition is required.

  - A summary of the pre-applica- tion meeting, along with a list of attendees and their addresses, and cop- ies of any written comments or mate- rials submitted at the meeting, as re- quired under § 124.31(c).

- Additional information require- ments. The following additional infor- mation regarding protection of ground- water is required from owners or opera- tors of hazardous waste facilities con- taining a regulated unit except as pro- vided in § 264.90(b) of this chapter:

  - A summary of the ground-water monitoring data obtained during the interim status period under §§ 265.90 through 265.94, where applicable.

  - Identification of the uppermost aquifer and aquifers hydraulically interconnected beneath the facility property, including ground-water flow direction and rate, and the basis for such identification (i.e., the informa- tion obtained from hydrogeologic in- vestigations of the facility area).

  - On the topographic map required under paragraph (b)(19) of this section, a delineation of the waste management area, the property boundary, the pro- posed ‘‘point of compliance’’ as defined under § 264.95, the proposed location of

## § 270.14

ground-water monitoring wells as re- quired under § 264.97, and, to the extent possible, the information required in paragraph (c)(2) of this section.

  - A description of any plume of con- tamination that has entered the ground water from a regulated unit at the time that the application was sub- mitted that:

    - Delineates the extent of the plume on the topographic map required under paragraph (b)(19) of this section;

    - Identifies the concentration of each appendix IX, of part 264 of this chapter, constituent throughout the plume or identifies the maximum con- centrations of each appendix IX con- stituent in the plume.

  - Detailed plans and an engineering report describing the proposed ground water monitoring program to be imple- mented to meet the requirements of

§ 264.97.

  - If the presence of hazardous con- stituents has not been detected in the ground water at the time of permit ap- plication, the owner or operator must submit sufficient information, sup- porting data, and analyses to establish a detection monitoring program which meets the requirements of § 264.98. This submission must address the following items specified under § 264.98:

    - A proposed list of indicator pa- rameters, waste constituents, or reac- tion products that can provide a reli- able indication of the presence of haz- ardous constituents in the ground water;

    - A proposed ground-water moni- toring system;

    - Background values for each pro- posed monitoring parameter or con- stituent, or procedures to calculate such values; and

    - A description of proposed sam- pling, analysis and statistical compari- son procedures to be utilized in evalu- ating ground-water monitoring data.

  - If the presence of hazardous con- stituents has been detected in the ground water at the point of compli- ance at the time of the permit applica- tion, the owner or operator must sub- mit sufficient information, supporting data, and analyses to establish a com- pliance monitoring program which meets the requirements of § 264.99. Ex- cept as provided in § 264.98(h)(5), the

## 40 CFR Ch. I (7–1–24 Edition)

owner or operator must also submit an engineering feasibility plan for a cor- rective action program necessary to meet the requirements of § 264.100, un- less the owner or operator obtains written authorization in advance from the Regional Administrator to submit a proposed permit schedule for sub- mittal of such a plan. To demonstrate compliance with § 264.99, the owner or operator must address the following items:

- A description of the wastes pre- viously handled at the facility;

- A characterization of the con- taminated ground water, including concentrations of hazardous constitu- ents;

- A list of hazardous constituents for which compliance monitoring will be undertaken in accordance with

§§ 264.97 and 264.99;

- Proposed concentration limits for each hazardous constituent, based on the criteria set forth in § 264.94(a), including a justification for estab- lishing any alternate concentration limits;

- Detailed plans and an engineering report describing the proposed ground- water monitoring system, in accord- ance with the requirements of § 264.97; and

- A description of proposed sam- pling, analysis and statistical compari- son procedures to be utilized in evalu- ating ground-water monitoring data.

  - If hazardous constituents have been measured in the ground water which exceed the concentration limits established under § 264.94 Table 1, or if ground water monitoring conducted at the time of permit application under

§§ 265.90 through 265.94 at the waste boundary indicates the presence of haz- ardous constituents from the facility in ground water over background con- centrations, the owner or operator must submit sufficient information, supporting data, and analyses to estab- lish a corrective action program which meets the requirements of § 264.100. However, an owner or operator is not required to submit information to es- tablish a corrective action program if he demonstrates to the Regional Ad- ministrator that alternate concentra- tion limits will protect human health and the environment after considering

## Environmental Protection Agency	§ 270.15

the criteria listed in § 264.94(b). An owner or operator who is not required to establish a corrective action pro- gram for this reason must instead sub- mit sufficient information to establish a compliance monitoring program which  meets  the  requirements  of

§ 264.99 and paragraph (c)(6) of this sec- tion. To demonstrate compliance with

§ 264.100, the owner or operator must address, at a minimum, the following items:

    - A characterization of the contami- nated ground water, including con- centrations of hazardous constituents;

    - The concentration limit for each hazardous constituent found in the ground water as set forth in § 264.94;

    - Detailed plans and an engineer- ing report describing the corrective ac- tion to be taken; and

    - A description of how the ground- water monitoring program will dem- onstrate the adequacy of the corrective action.

    - The permit may contain a sched- ule for submittal of the information re- quired in paragraphs (c)(8) (iii) and (iv) provided the owner or operator obtains written authorization from the Re- gional Administrator prior to sub- mittal of the complete permit applica- tion.

- Information requirements for solid waste management units. (1) The fol- lowing information is required for each solid waste management unit at a fa- cility seeking a permit:

- The location of the unit on the topographic map required under para- graph (b)(19) of this section.

- Designation of type of unit.

- General dimensions and struc- tural description (supply any available drawings).

- When the unit was operated.

- Specification of all wastes that have been managed at the unit, to the extent available.

- The owner or operator of any fa- cility containing one or more solid waste management units must submit all available information pertaining to any release of hazardous wastes or haz- ardous constituents from such unit or units.

- The owner/operator must conduct and provide the results of sampling and analysis of groundwater, landsurface,

and subsurface strata, surface water, or air, which may include the installation of wells, where the Director ascertains it is necessary to complete a RCRA Fa- cility Assessment that will determine if a more complete investigation is necessary.

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983]

EDITORIAL NOTE: For FEDERAL REGISTER ci-

tations affecting § 270.14, see the List of CFR Sections Affected, which appears in the Finding Aids section of the printed volume and at www.govinfo.gov.

### § 270.15 Specific part B information re- quirements for containers.

Except  as  otherwise  provided  in

§ 264.170, owners or operators of facili- ties that store containers of hazardous waste must provide the following addi- tional information:

- A description of the containment system to demonstrate compliance with § 264.175. Show at least the fol- lowing:

  - Basic design parameters, dimen- sions, and materials of construction.

  - How the design promotes drainage or how containers are kept from con- tact with standing liquids in the con- tainment system.

  - Capacity of the containment sys- tem relative to the number and volume of containers to be stored.

  - Provisions for preventing or man- aging run-on.

  - How accumulated liquids can be analyzed and removed to prevent over- flow.

- For storage areas that store con- tainers holding wastes that do not con- tain free liquids, a demonstration of compliance with § 264.175(c), including:

  - Test procedures and results or other documentation or information to show that the wastes do not contain free liquids; and

  - A description of how the storage area is designed or operated to drain and remove liquids or how containers are kept from contact with standing liquids.

- Sketches, drawings, or data dem- onstrating compliance with § 264.176 (location of buffer zone and containers holding ignitable or reactive wastes) and § 264.177(c) (location of incompat- ible wastes), where applicable.

## § 270.16

- Where incompatible wastes are stored or otherwise managed in con- tainers, a description of the procedures used  to  ensure  compliance  with

§§ 264.177 (a) and (b), and 264.17 (b) and (c).

- Information on air emission con- trol equipment as required in § 270.27.

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983; 59 FR 62952, Dec. 6, 1994]

### § 270.16 Specific part B information re- quirements for tank systems.

Except  as  otherwise  provided  in

§ 264.190, owners and operators of facili- ties that use tanks to store or treat hazardous waste must provide the fol- lowing additional information:

- A written assessment that is re- viewed and certified by a qualified Pro- fessional Engineer as to the structural integrity and suitability for handling hazardous waste of each tank system, as required under §§ 264.191 and 264.192 of this chapter;

- Dimensions and capacity of each tank;

- Description of feed systems, safe- ty cutoff, bypass systems, and pressure controls (e.g., vents);

- A diagram of piping, instrumenta- tion, and process flow for each tank system;

- A description of materials and equipment used to provide external corrosion protection, as required under

§ 264.192(a)(3)(ii);

- For new tank systems, a detailed description of how the tank system(s) will be installed in compliance with

§ 264.192 (b), (c), (d), and (e);

- Detailed plans and description of how the secondary containment system for each tank system is or will be de- signed, constructed, and operated to meet the requirements of § 264.193 (a), (b), (c), (d), (e), and (f);

- For tank systems for which a variance  from  the  requirements  of

§ 264.193  is  sought  (as  provided  by

§§ 264.193(g)):

- Detailed plans and engineering and hydrogeologic reports, as appro- priate, describing alternate design and operating practices that will, in con- junction with location aspects, prevent the migration of any hazardous waste or  hazardous  constituents  into  the

## 40 CFR Ch. I (7–1–24 Edition)

ground water or surface water during the life of the facility, or

- A detailed assessment of the sub- stantial present or potential hazards posed to human health or the environ- ment should a release enter the envi- ronment.

  - Description of controls and prac- tices to prevent spills and overflows, as required under § 264.194(b); and

  - For tank systems in which ignit- able, reactive, or incompatible wastes are to be stored or treated, a descrip- tion of how operating procedures and tank system and facility design will achieve compliance with the require- ments of §§ 264.198 and 264.199.

  - Information on air emission con- trol equipment as required in § 270.27.

[51 FR 25486, July 14, 1986; 51 FR 29431, Aug.

15, 1986; 59 FR 62952, Dec. 6, 1994; 71 FR 16914,

Apr. 4, 2006]

### § 270.17 Specific part B information re- quirements for surface impound- ments.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that store, treat or dispose of haz- ardous waste in surface impoundments must provide the following additional information:

- A list of the hazardous wastes placed or to be placed in each surface impoundment;

- Detailed plans and an engineering report describing how the surface im- poundment is designed and is or will be constructed, operated, and maintained to meet the requirements of §§ 264.19, 264.221, 264.222, and 264.223 of this chap- ter, addressing the following items:

  - The liner system (except for an existing portion of a surface impound- ment). If an exemption from the re- quirement for a liner is sought as pro- vided by § 264.221(b), submit detailed plans and engineering and hydrogeologic reports, as appropriate, describing alternate design and oper- ating practices that will, in conjunc- tion with location aspects, prevent the migration of any hazardous constitu- ents into the ground water or surface water at any future time;

  - The double liner and leak (leach-

ate) detection, collection, and removal system, if the surface impoundment must  meet  the  requirements  of

## Environmental Protection Agency	§ 270.18

§ 264.221(c) of this chapter. If an exemp- tion from the requirements for double liners and a leak detection, collection, and removal system or alternative de- sign is sought as provided by § 264.221 (d), (e), or (f) of this chapter, submit appropriate information;

  - If the leak detection system is lo- cated in a saturated zone, submit de- tailed plans and an engineering report explaining the leak detection system design and operation, and the location of the saturated zone in relation to the leak detection system;

  - The construction quality assur- ance (CQA) plan if required under

§ 264.19 of this chapter;

  - Proposed action leakage rate, with  rationale,  if  required  under

§ 264.222 of this chapter, and response action plan, if required under § 264.223 of this chapter;

  - Prevention of overtopping; and

  - Structural integrity of dikes;

- A description of how each surface impoundment, including the double liner system, leak detection system, cover system, and appurtenances for control of overtopping, will be in- spected in order to meet the require- ments of § 264.226(a), (b), and (d) of this chapter. This information must be in- cluded in the inspection plan submitted under § 270.14(b)(5);

- A certification by a qualified en- gineer which attests to the structural integrity of each dike, as required under § 264.226(c). For new units, the owner or operator must submit a state- ment by a qualified engineer that he will provide such a certification upon completion of construction in accord- ance with the plans and specifications;

- A description of the procedure to be used for removing a surface im- poundment from service, as required under § 264.227(b) and (c). This informa- tion should be included in the contin- gency   plan   submitted   under

§ 270.14(b)(7);

- A description of how hazardous waste residues and contaminated mate- rials will be removed from the unit at closure, as required under § 264.228(a)(1). For any wastes not to be removed from the unit upon closure, the owner or op- erator must submit detailed plans and an engineering report describing how

§ 264.228(a)(2) and (b) will be complied

with. This information should be in- cluded in the closure plan and, where applicable, the post-closure plan sub- mitted under § 270.14(b)(13);

- If ignitable or reactive wastes are to be placed in a surface impoundment, an explanation of how § 264.229 will be complied with;

- If incompatible wastes, or incom- patible wastes and materials will be placed in a surface impoundment, an explanation of how § 264.230 will be complied with.

- A waste management plan for EPA Hazardous Waste Nos. FO20, FO21, FO22, FO23, FO26, and FO27 describing how the surface impoundment is or will be designed, constructed, operated, and maintained to meet the requirements of § 264.231. This submission must ad- dress the following items as specified in § 264.231:

  - The volume, physical, and chem- ical characteristics of the wastes, in- cluding their potential to migrate through soil or to volatilize or escape into the atmosphere;

  - The attenuative properties of un- derlying and surrounding soils or other materials;

  - The mobilizing properties of other materials co-disposed with these wastes; and

  - The effectiveness of additional treatment, design, or monitoring tech- niques.

- Information on air emission con- trol equipment as required in § 270.27.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 2006, Jan. 14, 1985; 50 FR 28752, July 15,

1985; 57 FR 3495, Jan. 29, 1992; 59 FR 62952,

Dec. 6, 1994; 71 FR 40279, July 14, 2006]

### § 270.18 Specific part B information re- quirements for waste piles.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that store or treat hazardous waste in waste piles must provide the fol- lowing additional information:

- A list of hazardous wastes placed or to be placed in each waste pile;

- If an exemption is sought to

§ 264.251 and subpart F of part 264 as provided by § 264.250(c) or § 264.90(b)(2), an explanation of how the standards of

§ 264.250(c) will be complied with or de- tailed plans and an engineering report

## § 270.18

describing how the requirements of

§ 264.90(b)(2) will be met.

- Detailed plans and an engineering report describing how the waste pile is designed and is or will be constructed, operated, and maintained to meet the requirements of §§ 264.19, 264.251, 264.252, and 264.253 of this chapter, addressing the following items:

(1)(i) The liner system (except for an existing portion of a waste pile), if the waste pile must meet the requirements of § 264.251(a) of this chapter. If an ex- emption from the requirement for a liner  is  sought  as  provided  by

§ 264.251(b) of this chapter, submit de- tailed plans, and engineering and hydrogeological reports, as appro- priate, describing alternate designs and operating practices that will, in con- junction with location aspects, prevent the migration of any hazardous con- stituents into the ground water or sur- face water at any future time;

- The double liner and leak (leach- ate) detection, collection, and removal system, if the waste pile must meet the requirements of § 264.251(c) of this chap- ter. If an exemption from the require- ments for double liners and a leak de- tection, collection, and removal sys- tem or alternative design is sought as provided by § 264.251(d), (e), or (f) of this chapter, submit appropriate informa- tion;

- If the leak detection system is located in a saturated zone, submit de- tailed plans and an engineering report explaining the leak detection system design and operation, and the location of the saturated zone in relation to the leak detection system;

- The construction quality assur- ance (CQA) plan if required under

§ 264.19 of this chapter;

- Proposed action leakage rate, with  rationale,  if  required  under

§ 264.252 of this chapter, and response action plan, if required under § 264.253 of this chapter;

- Control of run-on;

- Control of run-off;

- Management of collection and holding units associated with run-on and run-off control systems; and

- Control of wind dispersal of par- ticulate matter, where applicable;

- A description of how each waste pile, including the double liner system,

## 40 CFR Ch. I (7–1–24 Edition)

leachate collection and removal sys- tem, leak detection system, cover sys- tem, and appurtenances for control of run-on and run-off, will be inspected in order  to  meet  the  requirements  of

§ 264.254(a), (b), and (c) of this chapter. This information must be included in the inspection plan submitted under

§ 270.14(b)(5);

- If treatment is carried out on or in the pile, details of the process and equipment used, and the nature and quality of the residuals;

- If ignitable or reactive wastes are to be placed in a waste pile, an expla- nation of how the requirements of

§ 264.256 will be complied with;

- If incompatible wastes, or incom- patible wastes and materials will be placed in a waste pile, an explanation of how § 264.257 will be complied with;

- A description of how hazardous waste residues and contaminated mate- rials will be removed from the waste pile at closure, as required under

§ 264.258(a). For any waste not to be re- moved from the waste pile upon clo- sure, the owner or operator must sub- mit detailed plans and an engineering report describing how § 264.310 (a) and

(b) will be complied with. This infor- mation should be included in the clo- sure plan and, where applicable, the post-closure  plan  submitted  under

§ 270.14(b)(13).

- A waste management plan for EPA Hazardous Waste Nos. FO20, FO21, FO22, FO23, FO26, and FO27 describing how a waste pile that is not enclosed (as defined in § 264.250(c)) is or will be designed, constructed, operated, and maintained to meet the requirements of § 264.259. This submission must ad- dress the following items as specified in § 264.259:

  - The volume, physical, and chem- ical characteristics of the wastes to be disposed in the waste pile, including their potential to migrate through soil or to volatilize or escape into the at- mosphere;

  - The attenuative properties of un- derlying and surrounding soils or other materials;

  - The mobilizing properties of other materials co-disposed with these wastes; and

## Environmental Protection Agency	§ 270.19

  - The effectiveness of additional treatment, design, or monitoring tech- niques.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 2006, Jan. 14, 1985; 50 FR 28752, July 15,

1985; 57 FR 3496, Jan. 29, 1992; 71 FR 40279,

July 14, 2006]

### § 270.19 Specific part B information re- quirements for incinerators.

Except as § 264.340 of this Chapter and

§ 270.19(e) provide otherwise, owners and operators of facilities that incin- erate hazardous waste must fulfill the requirements of paragraphs (a), (b), or

(c) of this section.

- When seeking an exemption under

§ 264.340 (b) or (c) of this chapter (Ignit- able, corrosive, or reactive wastes only):

- Documentation that the waste is listed as a hazardous waste in part 261, subpart D of this chapter, solely be- cause it is ignitable (Hazard Code I) or corrosive (Hazard Code C) or both; or

- Documentation that the waste is listed as a hazardous waste in part 261, subpart D of this chapter, solely be- cause it is reactive (Hazard Code R) for characteristics other than those listed in § 261.23(a) (4) and (5) of this chapter, and will not be burned when other haz- ardous wastes are present in the com- bustion zone; or

- Documentation that the waste is a hazardous waste solely because it possesses the characteristic of ignit- ability, corrosivity, or both, as deter- mined by the tests for characteristics of hazardous waste under part 261, sub- part C of this chapter; or

- Documentation that the waste is a hazardous waste solely because it possesses the reactivity characteristics listed in § 261.23(a) (1), (2), (3), (6), (7), or

(8) of this chapter, and that it will not be burned when other hazardous wastes are present in the combustion zone; or

- Submit a trial burn plan or the results of a trial burn, including all re- quired determinations, in accordance with § 270.62; or

- In lieu of a trial burn, the appli- cant may submit the following infor- mation:

  - An analysis of each waste or mix- ture of wastes to be burned including:

- Heat value of the waste in the form and composition in which it will be burned.

- Viscosity (if applicable), or de- scription of physical form of the waste.

- An identification of any haz- ardous organic constituents listed in part 261, appendix VIII, of this chapter, which are present in the waste to be burned, except that the applicant need not analyze for constituents listed in part 261, appendix VIII, of this chapter which would reasonably not be ex- pected to be found in the waste. The constituents excluded from analysis must be identified and the basis for their exclusion stated. The waste anal- ysis must rely on appropriate analyt- ical techniques.

- An approximate quantification of

the hazardous constituents identified in the waste, within the precision pro- duced by appropriate analytical meth- ods.

- A quantification of those haz- ardous constituents in the waste which may be designated as POHC’s based on data submitted from other trial or operational burns which demonstrate compliance with the performance standards in § 264.343 of this chapter.

  - A detailed engineering description of the incinerator, including:

    - Manufacturer’s name and model number of incinerator.

    - Type of incinerator.

    - Linear dimension of incinerator unit including cross sectional area of combustion chamber.

    - Description of auxiliary fuel sys- tem (type/feed).

    - Capacity of prime mover.

    - Description of automatic waste feed cutoff system(s).

    - Stack gas monitoring and pollu- tion control monitoring system.

    - Nozzle and burner design.

    - Construction materials.

    - Location and description of tem- perature, pressure, and flow indicating devices and control devices.

  - A description and analysis of the waste to be burned compared with the waste for which data from operational or trial burns are provided to support the contention that a trial burn is not needed. The data should include those items listed in paragraph (c)(1) of this section. This analysis should specify

## § 270.20

the POHC’s which the applicant has identified in the waste for which a per- mit is sought, and any differences from the POHC’s in the waste for which burn data are provided.

  - The design and operating condi- tions of the incinerator unit to be used, compared with that for which compara- tive burn data are available.

  - A description of the results sub- mitted from any previously conducted trial burn(s) including:

    - Sampling and analysis techniques used to calculate performance stand- ards in § 264.343 of this chapter,

    - Methods and results of moni- toring temperatures, waste feed rates, carbon monoxide, and an appropriate indicator of combustion gas velocity (including a statement concerning the precision and accuracy of this measure- ment),

  - The expected incinerator oper- ation information to demonstrate com- pliance with §§ 264.343 and 264.345 of this chapter including:

    - Expected carbon monoxide (CO) level in the stack exhaust gas.

    - Waste feed rate.

    - Combustion zone temperature.

    - Indication of combustion gas ve- locity.

    - Expected stack gas volume, flow rate, and temperature.

    - Computed residence time for waste in the combustion zone.

    - Expected hydrochloric acid re- moval efficiency.

    - Expected fugitive emissions and their control procedures.

    - Proposed waste feed cut-off lim- its based on the identified significant operating parameters.

  - Such supplemental information as the Director finds necessary to achieve the purposes of this paragraph.

  - Waste analysis data, including that submitted in paragraph (c)(1) of this section, sufficient to allow the Di- rector to specify as permit Principal Organic Hazardous Constituents (per- mit POHC’s) those constituents for which destruction and removal effi- ciencies will be required.

- The Director shall approve a per- mit application without a trial burn if he finds that:

  - The wastes are sufficiently simi- lar; and

## 40 CFR Ch. I (7–1–24 Edition)

  - The incinerator units are suffi- ciently similar, and the data from other trial burns are adequate to speci- fy (under § 264.345 of this chapter) oper- ating conditions that will ensure that the performance standards in § 264.343 of this chapter will be met by the in- cinerator.

- When an owner or operator of a hazardous waste incineration unit be- comes subject to RCRA permit require- ments after October 12, 2005, or when an owner or operator of an existing hazardous waste incineration unit dem- onstrates compliance with the air emission standards and limitations in part 63, subpart EEE, of this chapter (i.e., by conducting a comprehensive performance test and submitting a No- tification   of   Compliance   under

§§ 63.1207(j) and 63.1210(d) of this chapter documenting compliance with all ap- plicable requirements of part 63, sub- part EEE, of this chapter), the require- ments of this section do not apply, ex- cept those provisions the Director de- termines are necessary to ensure com- pliance with §§ 264.345(a) and 264.345(c) of this chapter if you elect to comply with § 270.235(a)(1)(i) to minimize emis- sions of toxic compounds from startup, shutdown, and malfunction events. Nevertheless, the Director may apply the provisions of this section, on a case-by-case basis, for purposes of in- formation collection in accordance with §§ 270.10(k), 270.10(l), 270.32(b)(2), and 270.32(b)(3).

[48 FR 14228, Apr. 1, 1983, as amended at 58

FR 46051, Aug. 31, 1993; 64 FR 53076, Sept. 30,

1999; 67 FR 6816, Feb. 13, 2002; 67 FR 77692,

Dec. 19, 2002; 70 FR 34590, June 14, 2005; 70 FR

59577, Oct. 12, 2005]

### § 270.20 Specific part B information re- quirements for land treatment fa- cilities.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that use land treatment to dispose of hazardous waste must provide the following additional information:

- A description of plans to conduct a treatment demonstration as required under § 264.272. The description must include the following information;

## Environmental Protection Agency	§ 270.20

  - The wastes for which the dem- onstration will be made and the poten- tial hazardous constituents in the waste;

  - The data sources to be used to make the demonstration (e.g., lit- erature, laboratory data, field data, or operating data);

  - Any specific laboratory or field test that will be conducted, including:

    - The type of test (e.g., column leaching, degradation);

    - Materials and methods, including analytical procedures;

    - Expected time for completion;

    - Characteristics of the unit that will be simulated in the demonstration, including treatment zone characteris- tics, climatic conditions, and operating practices.

- A description of a land treatment program, as required under § 264.271. This information must be submitted with the plans for the treatment dem- onstration, and updated following the treatment demonstration. The land treatment program must address the following items:

  - The wastes to be land treated;

  - Design measures and operating practices necessary to maximize treat- ment in accordance with § 264.273(a) in- cluding:

    - Waste application method and rate;

    - Measures to control soil pH;

    - Enhancement of microbial or chemical reactions;

    - Control of moisture content;

  - Provisions for unsaturated zone monitoring, including:

    - Sampling equipment, procedures, and frequency;

    - Procedures for selecting sampling locations;

    - Analytical procedures;

    - Chain of custody control;

    - Procedures for establishing back- ground values;

    - Statistical methods for inter- preting results;

    - The justification for any haz- ardous constituents recommended for selection as principal hazardous con- stituents, in accordance with the cri- teria for such selection in § 264.278(a);

  - A list of hazardous constituents reasonably expected to be in, or derived from, the wastes to be land treated

based on waste analysis performed pur- suant to § 264.13;

  - The proposed dimensions of the treatment zone;

- A description of how the unit is or will be designed, constructed, operated, and maintained in order to meet the requirements of § 264.273. This submis- sion must address the following items:

  - Control of run-on;

  - Collection and control of run-off;

  - Minimization of run-off of haz- ardous constituents from the treat- ment zone;

  - Management of collection and holding facilities associated with run- on and run-off control systems;

  - Periodic inspection of the unit. This information should be included in the inspection plan submitted under

§ 270.14(b)(5);

  - Control of wind dispersal of par- ticulate matter, if applicable;

- If food-chain crops are to be grown in or on the treatment zone of the land treatment unit, a description of how the demonstration required under § 264.276(a) will be conducted in- cluding:

  - Characteristics of the food-chain crop for which the demonstration will be made.

  - Characteristics of the waste, treatment zone, and waste application method and rate to be used in the dem- onstration;

  - Procedures for crop growth, sam- ple collection, sample analysis, and data evaluation;

  - Characteristics of the comparison crop including the location and condi- tions under which it was or will be grown;

- If food-chain crops are to be grown, and cadmium is present in the land-treated waste, a description of how the requirements of § 264.276(b) will be complied with;

- A description of the vegetative cover to be applied to closed portions of the facility, and a plan for maintain- ing such cover during the post-closure care  period,  as  required  under

§§ 264.280(a)(8) and 264.280(c)(2). This in- formation should be included in the closure plan and, where applicable, the post-closure care plan submitted under

§ 270.14(b)(13);

## § 270.21

- If ignitable or reactive wastes will be placed in or on the treatment zone, an explanation of how the require- ments of § 264.281 will be complied with;

- If incompatible wastes, or incom- patible wastes and materials, will be placed in or on the same treatment zone, an explanation of how § 264.282 will be complied with.

- A waste management plan for EPA Hazardous Waste Nos. FO20, FO21, FO22, FO23, FO26, and FO27 describing how a land treatment facility is or will be designed, constructed, operated, and maintained to meet the requirements of § 264.283. This submission must ad- dress the following items as specified in § 264.283:

  - The volume, physical, and chem-

ical characteristics of the wastes, in- cluding their potential to migrate through soil or to volatilize or escape into the atmosphere;

  - The attenuative properties of un- derlying and surrounding soils or other materials;

  - The mobilizing properties of other materials co-disposed with these wastes; and

  - The effectiveness of additional treatment, design, or monitoring tech- niques.

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983, as amended at 50 FR 2006, Jan. 14, 1985;

71 FR 40279, July 14, 2006]

### § 270.21 Specific part B information re- quirements for landfills.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that dispose of hazardous waste in landfills must provide the following ad- ditional information:

- A list of the hazardous wastes placed or to be placed in each landfill or landfill cell;

- Detailed plans and an engineering report describing how the landfill is de- signed and is or will be constructed, op- erated, and maintained to meet the re- quirements of §§ 264.19, 264.301, 264.302, and 264.303 of this chapter, addressing the following items:

(1)(i) The liner system (except for an existing portion of a landfill), if the landfill must meet the requirements of

§ 264.301(a) of this chapter. If an exemp- tion from the requirement for a liner is sought as provided by § 264.301(b) of this

## 40 CFR Ch. I (7–1–24 Edition)

chapter, submit detailed plans, and en- gineering and hydrogeological reports, as appropriate, describing alternate de- signs and operating practices that will, in conjunction with location aspects, prevent the migration of any hazardous constituents into the ground water or surface water at any future time;

- The double liner and leak (leach- ate) detection, collection, and removal system, if the landfill must meet the requirements of § 264.301(c) of this chap- ter. If an exemption from the require- ments for double liners and a leak de- tection, collection, and removal sys- tem or alternative design is sought as provided by § 264.301(d), (e), or (f) of this chapter, submit appropriate informa- tion;

- If the leak detection system is located in a saturated zone, submit de- tailed plans and an engineering report explaining the leak detection system design and operation, and the location of the saturated zone in relation to the leak detection system;

- The construction quality assur- ance (CQA) plan if required under

§ 264.19 of this chapter;

- Proposed action leakage rate, with  rationale,  if  required  under

§ 264.302 of this chapter, and response action plan, if required under § 264.303 of this chapter;

- Control of run-on;

- Control of run-off;

- Management of collection and holding facilities associated with run- on and run-off control systems; and

- Control of wind dispersal of par- ticulate matter, where applicable;

- A description of how each landfill, including the double liner system, leachate collection and removal sys- tem, leak detection system, cover sys- tem, and appurtenances for control of run-on and run-off, will be inspected in order to meet the requirements of

§ 264.303(a), (b), and (c) of this chapter. This information must be included in the inspection plan submitted under

§ 270.14(b)(5);

- A description of how each landfill, including the liner and cover systems, will be inspected in order to meet the requirements of § 264.303 (a) and (b). This information should be included in the inspection plan submitted under

§ 270.14(b)(5).

## Environmental Protection Agency	§ 270.22

- Detailed plans and an engineering report describing the final cover which will be applied to each landfill or land- fill cell at closure in accordance with

§ 264.310(a), and a description of how each landfill will be maintained and monitored after closure in accordance with § 264.310(b). This information should be included in the closure and post-closure  plans  submitted  under

§ 270.14(b)(13).

- If ignitable or reactive wastes will be landfilled, an explanation of how the standards of § 264.312 will be complied with;

- If incompatible wastes, or incom- patible wastes and materials will be landfilled,  an  explanation  of  how

§ 264.313 will be complied with;

- If bulk or non-containerized liq- uid waste or wastes containing free liq- uids is to be landfilled prior to May 8, 1985, an explanation of how the require- ments of § 264.314(a) will be complied with;

- If containers of hazardous waste are to be landfilled, an explanation of how the requirements of § 264.315 or

§ 264.316, as applicable, will be complied with.

- A waste management plan for EPA Hazardous Waste Nos. FO20, FO21, FO22, FO23, FO26, and FO27 describing how a landfill is or will be designed, constructed, operated, and maintained to meet the requirements of § 264.317. This submission must address the fol- lowing items as specified in § 264.317:

  - The volume, physical, and chem- ical characteristics of the wastes, in- cluding their potential to migrate through soil or to volatilize or escape into the atmosphere;

  - The attenuative properties of un- derlying and surrounding soils or other materials;

  - The mobilizing properties of other materials co-disposed with these wastes; and

  - The effectiveness of additional treatment, design, or monitoring tech- niques.

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983, as amended at 50 FR 2006, Jan. 14, 1985;

50 FR 28752, July 15, 1985; 57 FR 3496, Jan. 29,

1992]

### § 270.22 Specific part B information re- quirements for boilers and indus- trial furnaces burning hazardous waste.

When an owner or operator of a ce- ment kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid production furnace becomes subject to RCRA permit re- quirements after October 12, 2005, or when an owner or operator of an exist- ing cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid production fur- nace demonstrates compliance with the air emission standards and limita- tions in part 63, subpart EEE, of this chapter (i.e., by conducting a com- prehensive performance test and sub- mitting a Notification of Compliance under §§ 63.1207(j) and 63.1210(d) of this chapter documenting compliance with all applicable requirements of part 63, subpart EEE, of this chapter), the re- quirements of this section do not apply. The requirements of this section do apply, however, if the Director de- termines certain provisions are nec- essary  to  ensure  compliance  with

§§ 266.102(e)(1) and 266.102(e)(2)(iii) of this chapter if you elect to comply with § 270.235(a)(1)(i) to minimize emis- sions of toxic compounds from startup, shutdown, and malfunction events; or if you are an area source and elect to comply with the §§ 266.105, 266.106, and

266.107 standards and associated re- quirements for particulate matter, hy- drogen chloride and chlorine gas, and non-mercury metals; or the Director determines certain provisions apply, on a case-by-case basis, for purposes of in- formation collection in accordance with §§ 270.10(k), 270.10(l), 270.32(b)(2), and 270.32(b)(3).

  - Trial burns—(1) General. Except as provided below, owners and operators that are subject to the standards to control organic emissions provided by

§ 266.104 of this chapter, standards to control particulate matter provided by

§ 266.105 of this chapter, standards to control metals emissions provided by

§ 266.106 of this chapter, or standards to control hydrogen chloride or chlorine gas emissions provided by § 266.107 of this chapter must conduct a trial burn to demonstrate conformance with those standards and must submit a

## § 270.22

trial burn plan or the results of a trial burn, including all required determina- tions, in accordance with § 270.66.

    - A trial burn to demonstrate con- formance with a particular emission standard may be waived under provi- sions of §§ 266.104 through 266.107 of this chapter and paragraphs (a)(2) through (a)(5) of this section; and

    - The owner or operator may sub- mit data in lieu of a trial burn, as pre- scribed in paragraph (a)(6) of this sec- tion.

- Waiver of trial burn for DRE—(i) Boilers operated under special operating requirements. When seeking to be per- mitted under §§ 266.104(a)(4) and 266.110 of this chapter that automatically waive the DRE trial burn, the owner or operator of a boiler must submit docu- mentation that the boiler operates under the special operating require- ments provided by § 266.110 of this chap- ter.

- Boilers and industrial furnaces burning low risk waste. When seeking to be permitted under the provisions for low   risk   waste   provided   by

§§ 266.104(a)(5) and 266.109(a) of this chapter that waive the DRE trial burn, the owner or operator must submit:

  - Documentation that the device is operated in conformance with the re- quirements of § 266.109(a)(1) of this chapter.

  - Results of analyses of each waste to be burned, documenting the con- centrations of nonmetal compounds listed in appendix VIII of part 261 of this chapter, except for those constitu- ents that would reasonably not be ex- pected to be in the waste. The constitu- ents excluded from analysis must be identified and the basis for their exclu- sion explained. The analysis must rely on appropriate analytical techniques.

  - Documentation of hazardous waste firing rates and calculations of reasonable, worst-case emission rates of each constituent identified in para- graph (a)(2)(ii)(B) of this section using procedures provided by § 266.109(a)(2)(ii) of this chapter.

  - Results of emissions dispersion modeling for emissions identified in paragraphs (a)(2)(ii)(C) of this section using modeling procedures prescribed by § 266.106(h) of this chapter. The Di- rector will review the emission mod-

## 40 CFR Ch. I (7–1–24 Edition)

eling conducted by the applicant to de- termine conformance with these proce- dures. The Director will either approve the modeling or determine that alter- nate or supplementary modeling is ap- propriate.

  - Documentation that the max- imum annual average ground level con- centration of each constituent identi- fied in paragraph (a)(2)(ii)(B) of this section quantified in conformance with paragraph (a)(2)(ii)(D) of this section does not exceed the allowable ambient level established in appendices IV or V of part 266. The acceptable ambient concentration for emitted constituents for which a specific Reference Air Con- centration has not been established in appendix IV or Risk-Specific Dose has not been established in appendix V is

0.1 micrograms per cubic meter, as

noted in the footnote to appendix IV.

- Waiver of trial burn for metals. When seeking to be permitted under the Tier I (or adjusted Tier I) metals feed rate screening limits provided by

§ 266.106 (b) and (e) of this chapter that control metals emissions without re- quiring a trial burn, the owner or oper- ator must submit:

  - Documentation of the feed rate of hazardous waste, other fuels, and in- dustrial furnace feed stocks;

  - Documentation of the concentra- tion  of  each  metal  controlled  by

§ 266.106 (b) or (e) of this chapter in the hazardous waste, other fuels, and in- dustrial furnace feedstocks, and cal- culations of the total feed rate of each metal;

  - Documentation of how the appli- cant will ensure that the Tier I feed rate screening limits provided by

§ 266.106 (b) or (e) of this chapter will not be exceeded during the averaging period provided by that paragraph;

  - Documentation to support the determination of the terrain-adjusted effective stack height, good engineer- ing practice stack height, terrain type, and land use as provided by § 266.106 (b)(3) through (b)(5) of this chapter;

  - Documentation of compliance with the provisions of § 266.106(b)(6), if applicable, for facilities with multiple stacks;

  - Documentation that the facility does not fail the criteria provided by

## Environmental Protection Agency	§ 270.22

§ 266.106(b)(7) for eligibility to comply with the screening limits; and

  - Proposed sampling and metals analysis plan for the hazardous waste, other fuels, and industrial furnace feed stocks.

- Waiver of trial burn for particulate matter. When seeking to be permitted under the low risk waste provisions of

§ 266.109(b) which waives the particulate standard (and trial burn to dem- onstrate conformance with the particu- late standard), applicants must submit documentation supporting conform- ance with paragraphs (a)(2)(ii) and (a)(3) of this section.

- Waiver of trial burn for HCl and Cl2. When seeking to be permitted under the Tier I (or adjusted Tier I) feed rate screening limits for total chloride and chlorine provided by § 266.107 (b)(1) and

(e) of this chapter that control emis- sions of hydrogen chloride (HCl) and chlorine gas (Cl2) without requiring a trial burn, the owner or operator must submit:

  - Documentation of the feed rate of hazardous waste, other fuels, and in- dustrial furnace feed stocks;

  - Documentation of the levels of total chloride and chlorine in the haz- ardous waste, other fuels, and indus- trial furnace feedstocks, and calcula- tions of the total feed rate of total chloride and chlorine;

  - Documentation of how the appli- cant will ensure that the Tier I (or ad- justed Tier I) feed rate screening limits provided by § 266.107 (b)(1) or (e) of this chapter will not be exceeded during the averaging period provided by that para- graph;

  - Documentation to support the determination of the terrain-adjusted effective stack height, good engineer- ing practice stack height, terrain type, and  land  use  as  provided  by

§ 266.107(b)(3) of this chapter;

  - Documentation of compliance with the provisions of § 266.107(b)(4), if applicable, for facilities with multiple stacks;

  - Documentation that the facility does not fail the criteria provided by

§ 266.107(b)(3) for eligibility to comply with the screening limits; and

  - Proposed sampling and analysis plan for total chloride and chlorine for

the hazardous waste, other fuels, and industrial furnace feedstocks.

- Data in lieu of trial burn. The owner or operator may seek an exemp- tion from the trial burn requirements to  demonstrate  conformance  with

§§ 266.104 through 266.107 of this chapter and § 270.66 by providing the informa- tion required by § 270.66 from previous compliance testing of the device in conformance with § 266.103 of this chap- ter, or from compliance testing or trial or operational burns of similar boilers or industrial furnaces burning similar hazardous wastes under similar condi- tions. If data from a similar device is used to support a trial burn waiver, the design and operating information re- quired by § 270.66 must be provided for both the similar device and the device to which the data is to be applied, and a comparison of the design and oper- ating information must be provided. The Director shall approve a permit ap- plication without a trial burn if he finds that the hazardous wastes are suf- ficiently similar, the devices are suffi- ciently similar, the operating condi- tions are sufficiently similar, and the data from other compliance tests, trial burns, or operational burns are ade- quate to specify (under § 266.102 of this chapter) operating conditions that will ensure conformance with § 266.102(c) of this chapter. In addition, the following information shall be submitted:

- For a waiver from any trial burn:

  - A description and analysis of the hazardous waste to be burned compared with the hazardous waste for which data from compliance testing, or oper- ational or trial burns are provided to support the contention that a trial burn is not needed;

  - The design and operating condi- tions of the boiler or industrial furnace to be used, compared with that for which comparative burn data are avail- able; and

  - Such supplemental information as the Director finds necessary to achieve the purposes of this paragraph.

- For a waiver of the DRE trial burn, the basis for selection of POHCs used in the other trial or operational burns which demonstrate compliance with the DRE performance standard in

§ 266.104(a) of this chapter. This anal- ysis should specify the constituents in

## § 270.23

appendix VIII, part 261 of this chapter, that the applicant has identified in the hazardous waste for which a permit is sought, and any differences from the POHCs in the hazardous waste for which burn data are provided.

- Alternative HC limit for industrial

furnaces with organic matter in raw mate- rials. Owners and operators of indus- trial furnaces requesting an alternative HC limit under § 266.104(f) of this chap- ter shall submit the following informa- tion at a minimum:

- Documentation that the furnace is designed and operated to minimize HC emissions from fuels and raw mate- rials;

- Documentation of the proposed baseline flue gas HC (and CO) con- centration, including data on HC (and CO) levels during tests when the facil- ity produced normal products under normal operating conditions from nor- mal raw materials while burning nor- mal fuels and when not burning haz- ardous waste;

- Test burn protocol to confirm the baseline HC (and CO) level including information on the type and flow rate of all feedstreams, point of introduc- tion of all feedstreams, total organic carbon content (or other appropriate measure of organic content) of all nonfuel feedstreams, and operating conditions that affect combustion of fuel(s) and destruction of hydrocarbon emissions from nonfuel sources;

- Trial burn plan to:

  - Demonstrate that flue gas HC (and CO) concentrations when burning haz- ardous waste do not exceed the base- line HC (and CO) level; and

  - Identify the types and concentra- tions of organic compounds listed in appendix VIII, part 261 of this chapter, that are emitted when burning haz- ardous waste in conformance with pro- cedures prescribed by the Director;

- Implementation plan to monitor over time changes in the operation of the facility that could reduce the base- line HC level and procedures to periodi- cally confirm the baseline HC level; and

- Such other information as the Di- rector finds necessary to achieve the purposes of this paragraph.

- Alternative metals implementation approach.  When seeking to be per-

## 40 CFR Ch. I (7–1–24 Edition)

mitted under an alternative metals im- plementation	approach	under

§ 266.106(f) of this chapter, the owner or operator must submit documentation specifying how the approach ensures compliance with the metals emissions standards of § 266.106(c) or (d) and how the approach can be effectively imple- mented and monitored. Further, the owner or operator shall provide such other information that the Director finds necessary to achieve the purposes of this paragraph.

- Automatic waste feed cutoff system. Owners and operators shall submit in- formation describing the automatic waste feed cutoff system, including any pre-alarm systems that may be used.

- Direct transfer. Owners and opera- tors that use direct transfer operations to feed hazardous waste from transport vehicles (containers, as defined in

§ 266.111 of this chapter) directly to the boiler or industrial furnace shall sub- mit information supporting conform- ance with the standards for direct transfer provided by § 266.111 of this chapter.

- Residues. Owners and operators that claim that their residues are ex- cluded from regulation under the provi- sions of § 266.112 of this chapter must submit information adequate to dem- onstrate conformance with those provi- sions.

[56 FR 7235, Feb. 21, 1991; 56 FR 32691, July 17,

1991, as amended at 64 FR 53077, Sept. 30,

1999; 67 FR 6816, Feb. 13, 2002; 67 FR 77692,

Dec. 19, 2002; 70 FR 34590, June 14, 2005; 70 FR

59577, Oct. 12, 2005]

### § 270.23 Specific part B information re- quirements for miscellaneous units.

Except  as  otherwise  provided  in

§ 264.600, owners and operators of facili- ties that treat, store, or dispose of haz- ardous waste in miscellaneous units must provide the following additional information:

- A detailed description of the unit being used or proposed for use, includ- ing the following:

  - Physical characteristics, mate- rials of construction, and dimensions of the unit;

  - Detailed plans and engineering re- ports describing how the unit will be

## Environmental Protection Agency	§ 270.24

located, designed, constructed, oper- ated, maintained, monitored, in- spected, and closed to comply with the requirements of §§ 264.601 and 264.602; and

  - For disposal units, a detailed de- scription of the plans to comply with the  post-closure  requirements  of

§ 264.603.

- Detailed hydrologic, geologic, and meteorologic assessments and land-use maps for the region surrounding the site that address and ensure compli- ance of the unit with each factor in the environmental performance standards of § 264.601. If the applicant can dem- onstrate that he does not violate the environmental performance standards of § 264.601 and the Director agrees with such demonstration, preliminary hy- drologic, geologic, and meteorologic assessments will suffice.

- Information  on  the  potential

pathways of exposure of humans or en- vironmental receptors to hazardous waste or hazardous constituents and on the potential magnitude and nature of such exposures.

- For any treatment unit, a report on a demonstration of the effectiveness of the treatment based on laboratory or field data.

- Any additional information deter- mined by the Director to be necessary for evaluation of compliance of the unit with the environmental perform- ance standards of § 264.601.

### § 270.24 Specific part B information re- quirements for process vents.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that have process vents to which subpart AA of part 264 applies must provide the following additional infor- mation:

- For facilities that cannot install a closed-vent system and control device to comply with the provisions of 40 CFR 264 subpart AA on the effective date that the facility becomes subject to the provisions of 40 CFR 264 or 265 subpart AA, an implementation sched- ule as specified in § 264.1033(a)(2).

- Documentation of compliance with the process vent standards in

§ 264.1032, including:

- Information and data identifying all  affected  process  vents,  annual

throughput and operating hours of each affected unit, estimated emission rates for each affected vent and for the overall facility (i.e., the total emis- sions for all affected vents at the facil- ity), and the approximate location within the facility of each affected unit (e.g., identify the hazardous waste management units on a facility plot plan).

- Information and data supporting estimates of vent emissions and emis- sion reduction achieved by add-on con- trol devices based on engineering cal- culations or source tests. For the pur- pose of determining compliance, esti- mates of vent emissions and emission reductions must be made using oper- ating parameter values (e.g., tempera- tures, flow rates, or concentrations) that represent the conditions that exist when the waste management unit is operating at the highest load or ca- pacity level reasonably expected to occur.

- Information and data used to de- termine whether or not a process vent is  subject  to  the  requirements  of

§ 264.1032.

- Where an owner or operator ap- plies for permission to use a control de- vice other than a thermal vapor incin- erator, catalytic vapor incinerator, flare, boiler, process heater, condenser, or carbon adsorption system to comply with the requirements of § 264.1032, and chooses to use test data to determine the organic removal efficiency or the total organic compound concentration achieved by the control device, a per- formance  test  plan  as  specified  in

§ 264.1035(b)(3).

- Documentation of compliance with § 264.1033, including:

  - A list of all information ref- erences and sources used in preparing the documentation.

  - Records, including the dates, of each compliance test required by

§ 264.1033(k).

  - A design analysis, specifications, drawings, schematics, and piping and instrumentation diagrams based on the appropriate sections of ‘‘APTI Course 415: Control of Gaseous Emissions’’ (in- corporated by reference as specified in

§ 270.6) or other engineering texts ac- ceptable to the Regional Administrator

## § 270.25

that present basic control device infor- mation. The design analysis shall ad- dress the vent stream characteristics and control device operation param- eters as specified in § 264.1035(b)(4)(iii).

  - A statement signed and dated by the owner or operator certifying that the operating parameters used in the design analysis reasonably represent the conditions that exist when the haz- ardous waste management unit is or would be operating at the highest load or capacity level reasonably expected to occur.

  - A statement signed and dated by

the owner or operator certifying that the control device is designed to oper- ate at an efficiency of 95 weight per- cent or greater unless the total organic emission limits of § 264.1032(a) for af- fected process vents at the facility can be attained by a control device involv- ing vapor recovery at an efficiency less than 95 weight percent.

[55 FR 25518, June 21, 1990, as amended at 56

FR 19290, Apr. 26, 1991; 70 FR 59577, Oct. 12,

2005]

### § 270.25 Specific part B information re- quirements for equipment.

Except  as  otherwise  provided  in

§ 264.1, owners and operators of facili- ties that have equipment to which sub- part BB of part 264 applies must pro- vide the following additional informa- tion:

    - For each piece of equipment to which subpart BB of part 264 applies:

      - Equipment identification number and hazardous waste management unit identification.

      - Approximate locations within the facility (e.g., identify the hazardous waste management unit on a facility plot plan).

      - Type of equipment (e.g., a pump or pipeline valve).

      - Percent by weight total organics in the hazardous waste stream at the equipment.

      - Hazardous waste state at the equipment (e.g., gas/vapor or liquid).

      - Method of compliance with the standard (e.g., ‘‘monthly leak detection and repair’’ or ‘‘equipped with dual me- chanical seals’’).

    - For facilities that cannot install a closed-vent system and control device to comply with the provisions of 40

## 40 CFR Ch. I (7–1–24 Edition)

CFR 264 subpart BB on the effective date that the facility becomes subject to the provisions of 40 CFR 264 or 265 subpart BB, an implementation sched- ule as specified in § 264.1033(a)(2).

    - Where an owner or operator ap- plies for permission to use a control de- vice other than a thermal vapor incin- erator, catalytic vapor incinerator, flare, boiler, process heater, condenser, or carbon adsorption system and choos- es to use test data to determine the or- ganic removal efficiency or the total organic compound concentration achieved by the control device, a per- formance  test  plan  as  specified  in

§ 264.1035(b)(3).

    - Documentation that dem- onstrates compliance with the equip- ment standards in §§ 264.1052 to 264.1059. This documentation shall contain the records required under § 264.1064. The Regional Administrator may request further documentation before deciding if compliance has been demonstrated.

    - Documentation to demonstrate compliance with § 264.1060 shall include the following information:

      - A list of all information ref- erences and sources used in preparing the documentation.

      - Records, including the dates, of each  compliance  test  required  by

§ 264.1033(j).

      - A design analysis, specifications, drawings, schematics, and piping and instrumentation diagrams based on the appropriate sections of ‘‘APTI Course 415: Control of Gaseous Emissions’’ (in- corporated by reference as specified in

§ 270.6) or other engineering texts ac- ceptable to the Regional Administrator that present basic control device infor- mation. The design analysis shall ad- dress the vent stream characteristics and control device operation param- eters as specified in § 264.1035(b)(4)(iii).

      - A statement signed and dated by the owner or operator certifying that the operating parameters used in the design analysis reasonably represent the conditions that exist when the haz- ardous waste management unit is oper- ating at the highest load or capacity level reasonably expected to occur.

      - A statement signed and dated by the owner or operator certifying that

## Environmental Protection Agency	§ 270.26

the control device is designed to oper- ate at an efficiency of 95 weight per- cent or greater.

[55 FR 25518, June 21, 1990, as amended at 56

FR 19290, Apr. 26, 1991; 70 FR 59577, Oct. 12,

2005]

### § 270.26 Special part B information re- quirements for drip pads.

Except  as  otherwise  provided  by

§ 264.1 of this chapter, owners and oper- ators of hazardous waste treatment, storage, or disposal facilities that col- lect, store, or treat hazardous waste on drip pads must provide the following additional information:

        - A list of hazardous wastes placed or to be placed on each drip pad.

        - If an exemption is sought to sub- part F of part 264 of this chapter, as provided by § 264.90 of this chapter, de- tailed plans and an engineering report describing  how  the  requirements  of

§ 264.90(b)(2) of this chapter will be met.

        - Detailed plans and an engineering report describing how the drip pad is or will be designed, constructed, operated and maintained to meet the require- ments of § 264.573 of this chapter, in- cluding the as-built drawings and spec- ifications. This submission must ad- dress the following items as specified in § 264.571 of this chapter:

          - The design characteristics of the drip pad;

          - The liner system;

          - The leakage detection system, in- cluding the leak detection system and how it is designed to detect the failure of the drip pad or the presence of any releases of hazardous waste or accumu- lated liquid at the earliest practicable time;

          - Practices designed to maintain drip pads;

          - The associated collection system;

          - Control of run-on to the drip pad;

          - Control of run-off from the drip pad;

          - The interval at which drippage and other materials will be removed from the associated collection system and a statement demonstrating that the interval will be sufficient to pre- vent overflow onto the drip pad;

          - Procedures for cleaning the drip pad at least once every seven days to ensure the removal of any accumulated residues of waste or other materials,

including but not limited to rinsing, washing with detergents or other ap- propriate solvents, or steam cleaning and provisions for documenting the date, time, and cleaning procedure used each time the pad is cleaned.

          - Operating practices and proce- dures that will be followed to ensure that tracking of hazardous waste or waste constituents off the drip pad due to activities by personnel or equipment is minimized;

          - Procedures for ensuring that, after removal from the treatment ves- sel, treated wood from pressure and non-pressure processes is held on the drip pad until drippage has ceased, in- cluding recordkeeping practices;

          - Provisions for ensuring that col- lection and holding units associated with the run-on and run-off control systems are emptied or otherwise man- aged as soon as possible after storms to maintain design capacity of the sys- tem;

          - If treatment is carried out on the drip pad, details of the process equip- ment used, and the nature and quality of the residuals.

          - A description of how each drip pad, including appurtenances for con- trol of run-on and run-off, will be in- spected in order to meet the require- ments of § 264.573 of this chapter. This information should be included in the inspection  plan  submitted  under

§ 270.14(b)(5) of this part.

          - A certification signed by a quali- fied Professional Engineer, stating that the drip pad design meets the require- ments of paragraphs (a) through (f) of

§ 264.573 of this chapter.

          - A description of how hazardous waste residues and contaminated mate- rials will be removed from the drip pad at closure, as required under § 264.575(a) of this chapter. For any waste not to be removed from the drip pad upon clo- sure, the owner or operator must sub- mit detailed plans and an engineering report describing how § 264.310 (a) and

(b) of this chapter will be complied with. This information should be in- cluded in the closure plan and, where applicable, the post-closure plan sub- mitted under § 270.14(b)(13).

[55 FR 50489, Dec. 6, 1990. Redesignated and

amended at 56 FR 30198, July 1, 1991; 71 FR

16914, Apr. 4, 2006; 71 FR 40279, July 14, 2006]

## § 270.27

### § 270.27 Specific Part B information requirements for air emission con- trols for tanks, surface impound- ments, and containers.

            - Except as otherwise provided in 40 CFR 264.1, owners and operators of tanks, surface impoundments, or con- tainers that use air emission controls in accordance with the requirements of 40 CFR part 264, subpart CC shall pro- vide the following additional informa- tion:

              - Documentation for each floating

roof cover installed on a tank subject to 40 CFR 264.1084(d)(1) or 40 CFR

264.1084(d)(2) that includes information prepared by the owner or operator or provided by the cover manufacturer or vendor describing the cover design, and certification by the owner or operator that the cover meets the applicable de- sign specifications as listed in 40 CFR 264.1084(e)(1) or 40 CFR 264.1084(f)(1).

              - Identification of each container

area subject to the requirements of 40 CFR part 264, subpart CC and certifi- cation by the owner or operator that the requirements of this subpart are met.

              - Documentation for each enclosure used to control air pollutant emissions from tanks or containers in accordance with the requirements of 40 CFR 264.1084(d)(5) or 40 CFR 264.1086(e)(1)(ii)

that includes records for the most re- cent set of calculations and measure- ments performed by the owner or oper- ator to verify that the enclosure meets the criteria of a permanent total enclo- sure as specified in ‘‘Procedure T—Cri- teria for and Verification of a Perma- nent or Temporary Total Enclosure’’ under 40 CFR 52.741, appendix B.

              - Documentation for each floating membrane cover installed on a surface impoundment in accordance with the requirements of 40 CFR 264.1085(c) that includes information prepared by the owner or operator or provided by the cover manufacturer or vendor describ- ing the cover design, and certification by the owner or operator that the cover meets the specifications listed in 40 CFR 264.1085(c)(1).

              - Documentation for each closed-

vent system and control device in- stalled in accordance with the require- ments of 40 CFR 264.1087 that includes design and performance information as

## 40 CFR Ch. I (7–1–24 Edition)

specified in § 270.24 (c) and (d) of this part.

              - An emission monitoring plan for both Method 21 in 40 CFR part 60, ap- pendix A and control device monitoring methods. This plan shall include the following information: monitoring point(s), monitoring methods for con- trol devices, monitoring frequency, procedures	for	documenting exceedances, and procedures for miti- gating noncompliances.

              - When an owner or operator of a fa-

cility subject to 40 CFR part 265, sub- part CC cannot comply with 40 CFR part 264, subpart CC by the date of per- mit issuance, the schedule of imple- mentation required under 40 CFR 265.1082.

[61 FR 59996, Nov. 25, 1996]

### § 270.28 Part B information require- ments for post-closure permits.

For post-closure permits, the owner or operator is required to submit only the   information   specified   in

§§ 270.14(b)(1), (4), (5), (6), (11), (13), (14),

(16), (18) and (19), (c), and (d), unless the Regional Administrator determines that  additional  information from

§§ 270.14, 270.16, 270.17, 270.18, 270.20, or

270.21 is necessary. The owner or oper- ator is required to submit the same in- formation when an alternative author- ity is used in lieu of a post-closure per- mit as provided in § 270.1(c)(7).

[63 FR 56735, Oct. 22, 1998]

### § 270.29  Permit denial.

The Director may, pursuant to the procedures in part 124, deny the permit application either in its entirety or as to the active life of a hazardous waste management facility or unit only.

[54 FR 9607, Mar. 7, 1989]

# Subpart C—Permit Conditions

### § 270.30 Conditions applicable to all permits.

The following conditions apply to all RCRA permits, and shall be incor- porated into the permits either ex- pressly or by reference. If incorporated by reference, a specific citation to these regulations (or the corresponding approved State regulations) must be given in the permit.

## Environmental Protection Agency	§ 270.30

- Duty to comply. The permittee must comply with all conditions of this permit, except that the permittee need not comply with the conditions of this permit to the extent and for the dura- tion such noncompliance is authorized in an emergency permit. (See § 270.61). Any permit noncompliance, except under the terms of an emergency per- mit, constitutes a violation of the ap- propriate Act and is grounds for en- forcement action; for permit termi- nation, revocation and reissuance, or modification; or for denial of a permit renewal application.

- Duty to reapply. If the permittee

wishes to continue an activity regu- lated by this permit after the expira- tion date of this permit, the permittee must apply for and obtain a new per- mit.

- Need to halt or reduce activity not a defense. It shall not be a defense for a permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this permit.

- In the event of noncompliance with the permit, the permittee shall take all reasonable steps to minimize releases to the environment, and shall carry out such measures as are reason- able to prevent significant adverse im- pacts on human health or the environ- ment.

- Proper operation and maintenance. The permittee shall at all times prop- erly operate and maintain all facilities and systems of treatment and control (and related appurtenances) which are installed or used by the permittee to achieve compliance with the conditions of this permit. Proper operation and maintenance includes effective per- formance, adequate funding, adequate operator staffing and training, and ade- quate laboratory and process controls, including appropriate quality assur- ance procedures. This provision re- quires the operation of back-up or aux- iliary facilities or similar systems only when necessary to achieve compliance with the conditions of the permit.

- Permit actions. This permit may be

modified, revoked and reissued, or ter- minated for cause. The filing of a re- quest by the permittee for a permit modification,	revocation	and

reissuance, or termination, or a notifi- cation of planned changes or antici- pated noncompliance, does not stay any permit condition.

- Property rights. The permit does not convey any property rights of any sort, or any exclusive privilege.

- Duty to provide information. The permittee shall furnish to the Director, within a reasonable time, any relevant information which the Director may request to determine whether cause ex- ists for modifying, revoking and reissu- ing, or terminating this permit, or to determine compliance with this per- mit. The permittee shall also furnish to the Director, upon request, copies of records required to be kept by this per- mit.

- Inspection and entry. The per- mittee shall allow the Director, or an authorized representative, upon the presentation of credentials and other

documents as may be required by law to:

- Enter at reasonable times upon the permittee’s premises where a regu- lated facility or activity is located or conducted, or where records must be kept under the conditions of this per- mit;

- Have access to and copy, at rea- sonable times, any records that must be kept under the conditions of this permit;

- Inspect at reasonable times any facilities, equipment (including moni- toring and control equipment), prac- tices, or operations regulated or re- quired under this permit; and

- Sample or monitor at reasonable times, for the purposes of assuring per- mit compliance or as otherwise author- ized by RCRA, any substances or pa- rameters at any location.

- Monitoring and records. (1) Samples and measurements taken for the pur- pose of monitoring shall be representa- tive of the monitored activity.

- The permittee shall retain records of all monitoring information, includ- ing all calibration and maintenance records and all original strip chart re- cordings for continuous monitoring in- strumentation, copies of all reports re- quired by this permit, the certification required by § 264.73(b)(9) of this chapter, and records of all data used to com- plete the application for this permit,

## § 270.30

for a period of at least 3 years from the date of the sample, measurement, re- port, certification, or application. This period may be extended by request of the Director at any time. The per- mittee shall maintain records from all ground-water monitoring wells and as- sociated ground-water surface ele- vations, for the active life of the facil- ity, and for disposal facilities for the post-closure care period as well.

- Records for monitoring informa- tion shall include:

  - The date, exact place, and time of sampling or measurements;

  - The individual(s) who performed the sampling or measurements;

  - The date(s) analyses were per- formed;

  - The individual(s) who performed the analyses;

  - The analytical techniques or methods used; and

  - The results of such analyses.

- Signatory requirements. All appli- cations, reports, or information sub- mitted to the Director shall be signed and certified (See § 270.11.)

- Reporting requirements—(1) Planned changes. The permittee shall give no- tice to the Director as soon as possible of any planned physical alterations or additions to the permitted facility.

- Anticipated noncompliance. The permittee shall give advance notice to the Director of any planned changes in the permitted facility or activity which may result in noncompliance with permit requirements. For a new facility, the permittee may not treat, store, or dispose of hazardous waste; and for a facility being modified, the permittee may not treat, store, or dis- pose of hazardous waste in the modified portion of the facility except as pro- vided in § 270.42, until:

(i) The permittee has submitted to

the Director by certified mail or hand delivery a letter signed by the per- mittee and a registered professional engineer stating that the facility has been constructed or modified in com- pliance with the permit; and

(ii)(A) The Director has inspected the modified or newly constructed facility and finds it is in compliance with the conditions of the permit; or

(B) Within 15 days of the date of sub- mission  of  the  letter  in paragraph

## 40 CFR Ch. I (7–1–24 Edition)

(l)(2)(i) of this section, the permittee has not received notice from the Direc- tor of his or her intent to inspect, prior inspection is waived and the permittee may commence treatment, storage, or disposal of hazardous waste.

- Transfers. This permit is not transferable to any person except after notice to the Director. The Director may require modification or revocation and reissuance of the permit to change the name of the permittee and incor- porate such other requirements as may be necessary under RCRA. (See § 270.40)

- Monitoring reports. Monitoring re- sults shall be reported at the intervals specified elsewhere in this permit.

- Compliance schedules. Reports of compliance or noncompliance with, or any progress reports on, interim and final requirements contained in any compliance schedule of this permit shall be submitted no later than 14 days following each schedule date.

- Twenty-four hour reporting. (i) The permittee shall report any noncompli- ance which may endanger health or the environment orally within 24 hours from the time the permittee becomes aware of the circumstances, including:

  - Information concerning release of any hazardous waste that may cause an endangerment to public drinking water supplies.

  - Any information of a release or discharge of hazardous waste or of a fire or explosion from the HWM facil- ity, which could threaten the environ- ment or human health outside the fa- cility.

- The description of the occurrence and its cause shall include:

  - Name, address, and telephone number of the owner or operator;

  - Name, address, and telephone number of the facility;

  - Date, time, and type of incident;

  - Name and quantity of material(s) involved;

  - The extent of injuries, if any;

  - An assessment of actual or poten- tial hazards to the environment and human health outside the facility, where this is applicable; and

  - Estimated quantity and disposi- tion of recovered material that re- sulted from the incident.

- A written submission shall also be provided within 5 days of the time

## Environmental Protection Agency	§ 270.32

the permittee becomes aware of the circumstances. The written submission shall contain a description of the non- compliance and its cause; the period of noncompliance including exact dates and times, and if the noncompliance has not been corrected, the anticipated time it is expected to continue; and steps taken or planned to reduce, eliminate, and prevent reoccurrence of the noncompliance. The Director may waive the five day written notice re- quirement in favor of a written report within fifteen days.

- Manifest discrepancy report: If a significant discrepancy in a manifest is discovered, the permittee must at- tempt to reconcile the discrepancy. If not resolved within fifteen days, the permittee must submit a letter report, including a copy of the manifest, to the Director. (See 40 CFR 264.72.)

- Unmanifested waste report: This re- port must be submitted to the Director within 15 days of receipt of unmanifested waste. (See 40 CFR 264.76)

- Biennial report: A biennial report must be submitted covering facility ac- tivities during odd numbered calendar years. (See 40 CFR 264.75.)

- Other noncompliance. The per- mittee shall report all instances of noncompliance not reported under paragraphs (l)(4), (5), and (6) of this sec- tion, at the time monitoring reports are submitted. The reports shall con- tain the information listed in para- graph (l)(6) of this section.

- Other information. Where the per- mittee becomes aware that it failed to submit any relevant facts in a permit application, or submitted incorrect in- formation in a permit application or in any report to the Director, it shall promptly submit such facts or informa- tion.

- Information repository. The Direc- tor may require the permittee to estab- lish and maintain an information re- pository at any time, based on the fac- tors set forth in 40 CFR 124.33(b). The information  repository  will  be  gov-

erned by the provisions in 40 CFR 124.33(c) through (f).

(Clean Water Act (33 U.S.C. 1251 et seq.), Safe Drinking Water Act (42 U.S.C. 300f et seq.), Clean Air Act (42 U.S.C. 7401 et seq.), Re- source Conservation and Recovery Act (42 U.S.C. 6901 et seq.))

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30114, June 30, 1983; 48 FR 39622, Sept. 1,

1983; 50 FR 28752, July 15, 1985; 53 FR 37935,

Sept. 28, 1988; 60 FR 63433, Dec. 11, 1995]

### § 270.31 Requirements for recording and reporting of monitoring results.

All permits shall specify:

- Requirements concerning the proper use, maintenance, and installa- tion, when appropriate, of monitoring equipment or methods (including bio- logical monitoring methods when ap- propriate);

- Required monitoring including type, intervals, and frequency suffi- cient to yield data which are represent- ative of the monitored activity includ- ing, when appropriate, continuous monitoring;

- Applicable reporting requirements based upon the impact of the regulated activity and as specified in parts 264, 266 and 267. Reporting shall be no less frequent than specified in the above regulations.

### § 270.32 Establishing	permit	condi- tions.

- In addition to conditions required in all permits (§ 270.30), the Director shall establish conditions, as required on a case-by-case basis, in permits under §§ 270.50 (duration of permits), 270.33(a)  (schedules  of  compliance),

270.31 (monitoring), and for EPA issued permits only, 270.33(b) (alternate sched- ules of compliance) and 270.3 (consider- ations under Federal law).

(b)(1) Each RCRA permit shall in- clude permit conditions necessary to achieve compliance with the Act and regulations, including each of the ap- plicable requirements specified in parts 264 and 266 through 268 of this chapter. In satisfying this provision, the Admin- istrator may incorporate applicable re- quirements of parts 264 and 266 through

268 of this chapter directly into the permit or establish other permit condi- tions that are based on these parts.

## § 270.33

  - Each permit issued under section 3005 of this act shall contain terms and conditions as the Administrator or State Director determines necessary to protect human health and the environ- ment.

  - If, as the result of an assess- ment(s) or other information, the Ad- ministrator or Director determines that conditions are necessary in addi- tion to those required under 40 CFR parts 63, subpart EEE, 264 or 266 to en- sure protection of human health and the environment, he shall include those terms and conditions in a RCRA permit for a hazardous waste combustion unit.

- For a State issued permit, an ap- plicable requirement is a State statu- tory or regulatory requirement which takes effect prior to final administra- tive disposition of a permit. For a per- mit issued by EPA, an applicable re- quirement is a statutory or regulatory requirement (including any interim final regulation) which takes effect prior to the issuance of the permit. Section 124.14 (reopening of comment period) provides a means for reopening EPA permit proceedings at the discre- tion of the Director where new require- ments become effective during the per- mitting process and are of sufficient magnitude to make additional pro- ceedings desirable. For State and EPA administered programs, an applicable requirement is also any requirement which takes effect prior to the modi- fication or revocation and reissuance of a permit, to the extent allowed in

§ 270.41.

- New or reissued permits, and to the extent allowed under § 270.41, modi- fied or revoked and reissued permits, shall incorporate each of the applicable requirements referenced in this section and in 40 CFR 270.31.

- Incorporation. All permit condi- tions shall be incorporated either ex- pressly or by reference. If incorporated by reference, a specific citation to the applicable regulations or requirements must be given in the permit.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 28752, July 15, 1985; 51 FR 40653, Nov. 7,

1986; 65 FR 30913, May 15, 2000; 70 FR 59577,

Oct. 12, 2005]

## 40 CFR Ch. I (7–1–24 Edition)

### § 270.33  Schedules of compliance.

- The permit may, when appro- priate, specify a schedule of compli- ance leading to compliance with the Act and regulations.

  - Time for compliance. Any schedules of compliance under this section shall require compliance as soon as possible.

  - Interim dates. Except as provided in paragraph (b)(1)(ii) of this section, if a permit establishes a schedule of com- pliance which exceeds 1 year from the date of permit issuance, the schedule shall set forth interim requirements and the dates for their achievement.

    - The time between interim dates shall not exceed 1 year.

    - If the time necessary for comple- tion of any interim requirement is more than 1 year and is not readily di- visible into stages for completion, the permit shall specify interim dates for the submission of reports of progress toward completion of the interim re- quirements and indicate a projected completion date.

  - Reporting.  The permit shall be

written to require that no later than 14 days following each interim date and the final date of compliance, the per- mittee shall notify the Director in writing, of its compliance or non- compliance with the interim or final requirements.

- Alternative schedules of compliance. A RCRA permit applicant or permittee may cease conducting regulated activi- ties (by receiving a terminal volume of hazardous waste and, for treatment and storage HWM facilities, closing pursuant to applicable requirements; and, for disposal HWM facilities, clos- ing and conducting post-closure care pursuant to applicable requirements) rather than continue to operate and meet permit requirements as follows:

  - If the permittee decides to cease

conducting regulated activities at a given time within the term of a permit which has already been issued:

    - The permit may be modified to contain a new or additional schedule leading to timely cessation of activi- ties; or

    - The permittee shall cease con- ducting permitted activities before noncompliance with any interim or final compliance schedule requirement already specified in the permit.

## Environmental Protection Agency	§ 270.41

  - If the decision to cease conducting regulated activities is made before issuance of a permit whose term will include the termination date, the per- mit shall contain a schedule leading to termination which will ensure timely compliance with applicable require- ments.

  - If the permittee is undecided whether to cease conducting regulated activities, the Director may issue or modify a permit to contain two sched- ules as follows:

    - Both schedules shall contain an identical interim deadline requiring a final decision on whether to cease con- ducting regulated activities no later than a date which ensures sufficient time to comply with applicable re- quirements in a timely manner if the decision is to continue conducting reg- ulated activities;

    - One schedule shall lead to timely compliance with applicable require- ments;

    - The second schedule shall lead to cessation of regulated activities by a date which will ensure timely compli- ance with applicable requirements;

    - Each permit containing two schedules shall include a requirement that after the permittee has made a final decision under paragraph (b)(3)(i) of this section it shall follow the sched- ule leading to compliance if the deci- sion is to continue conducting regu- lated activities, and follow the sched- ule leading to termination if the deci- sion is to cease conducting regulated activities.

  - The applicant’s or permittee’s de- cision to cease conducting regulated activities shall be evidenced by a firm public commitment satisfactory to the Director, such as resolution of the board of directors of a corporation.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30114, June 30, 1983; 71 FR 40279, July 14,

2006]

# Subpart D—Changes to Permit

### § 270.40  Transfer of permits.

- A permit may be transferred by the permittee to a new owner or oper- ator only if the permit has been modi- fied or revoked and reissued (under

§ 270.40(b) or § 270.41(b)(2)) to identify the  new  permittee  and  incorporate

such other requirements as may be necessary under the appropriate Act.

- Changes in the ownership or oper- ational control of a facility may be made as a Class 1 modification with prior written approval of the Director in accordance with § 270.42 or as a rou- tine change with prior approval under 40 CFR 124.213. The new owner or oper- ator must submit a revised permit ap- plication no later than 90 days prior to the scheduled change. A written agree- ment containing a specific date for transfer of permit responsibility be- tween the current and new permittees must also be submitted to the Director. When a transfer of ownership or oper- ational control occurs, the old owner or operator shall comply with the re- quirements of 40 CFR part 264, subpart H (Financial Requirements) until the new owner or operator has dem- onstrated that he or she is complying with the requirements of that subpart. The new owner or operator must dem- onstrate compliance with subpart H re- quirements within six months of the date of the change of ownership or operational control of the facility. Upon demonstration to the Director by the new owner or operator of compli- ance with subpart H, the Director shall notify the old owner or operator that he or she no longer needs to comply with subpart H as of the date of dem- onstration.

[53 FR 37935, Sept. 28, 1988, as amended at 70

FR 53475, Sept. 8, 2005]

### § 270.41 Modification	or	revocation and reissuance of permits.

When the Director receives any infor- mation (for example, inspects the facil- ity, receives information submitted by the permittee as required in the permit (see § 270.30), receives a request for rev- ocation and reissuance under § 124.5 or conducts a review of the permit file), he or she may determine whether one or more of the causes listed in para- graphs (a) and (b) of this section for modification, or revocation and reissuance or both exist. If cause ex- ists, the Director may modify or re- voke and reissue the permit accord- ingly, subject to the limitations of paragraph (c) of this section, and may request an updated application if nec- essary. When a permit is modified, only

## § 270.42

the conditions subject to modification are reopened. If a permit is revoked and reissued, the entire permit is re- opened and subject to revision and the permit is reissued for a new term. (See

40 CFR 124.5(c)(2).) If cause does not exist under this section, the Director shall not modify or revoke and reissue the permit, except on request of the permittee. If a permit modification is requested by the permittee, the Direc- tor shall approve or deny the request according to the procedures of § 270.42, or § 270.320 and 40 CFR part 124, subpart

G. Otherwise, a draft permit must be prepared and other procedures in part

124 (or procedures of an authorized State program) followed.

- Causes for modification. The fol- lowing are causes for modification, but not revocation and reissuance, of per- mits; the following may be causes for revocation and reissuance, as well as modification, when the permittee re- quests or agrees.

  - Alterations. There are material and substantial alterations or additions to the permitted facility or activity which occurred after permit issuance which justify the application of permit conditions that are different or absent in the existing permit.

  - Information. The Director has re- ceived information. Permits may be modified during their terms for this cause only if the information was not available at the time of permit issuance (other than revised regula- tions, guidance, or test methods) and would have justified the application of different permit conditions at the time of issuance.

  - New statutory requirements or regu- lations. The standards or regulations on which the permit was based have been changed by statute, through promulga- tion of new or amended standards or regulations, or by judicial decision after the permit was issued.

  - Compliance schedules. The Director determines good cause exists for modi- fication of a compliance schedule, such as an act of God, strike, flood, or mate- rials shortage or other events over which the permittee has little or no control and for which there is no rea- sonably available remedy.

  - Notwithstanding any other provi- sion in this section, when a permit for

## 40 CFR Ch. I (7–1–24 Edition)

a land disposal facility is reviewed by the Director under § 270.50(d), the Di- rector shall modify the permit as nec- essary to assure that the facility con- tinues to comply with the currently applicable requirements in parts 124, 260 through 266, and 270.

- Causes for modification or revoca- tion and reissuance. The following are causes to modify or, alternatively, re- voke and reissue a permit:

- Cause exists for termination under § 270.43, and the Director deter- mines that modification or revocation and reissuance is appropriate.

- The Director has received notifi- cation (as required in the permit, see

§ 270.30(l)(3)) of a proposed transfer of the permit.

- The Director has received notifi- cation under 40 CFR 124.202(b) of a fa- cility owner or operator’s intent to be covered by a standardized permit.

- Facility siting. Suitability of the facility location will not be considered at the time of permit modification or revocation and reissuance unless new information or standards indicate that a threat to human health or the envi- ronment exists which was unknown at the time of permit issuance.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30114, June 30, 1983; 50 FR 28752, July 15,

1985; 52 FR 45799, Dec. 1, 1987; 53 FR 37936,

Sept. 28, 1988; 70 FR 53475, Sept. 8, 2005; 71 FR

40279, July 14, 2006]

### § 270.42 Permit modification at the re- quest of the permittee.

- Class 1 modifications. (1) Except as provided in paragraph (a)(2) of this sec- tion, the permittee may put into effect Class 1 modifications listed in appendix I of this section under the following conditions:

  - The permittee must notify the Di- rector concerning the modification by certified mail or other means that es- tablish proof of delivery within 7 cal- endar days after the change is put into effect. This notice must specify the changes being made to permit condi- tions or supporting documents ref- erenced by the permit and must ex- plain why they are necessary. Along with the notice, the permittee must provide the applicable information re- quired by §§ 270.13 through 270.21, 270.62,

and 270.63.

## Environmental Protection Agency	§ 270.42

  - The permittee must send a notice of the modification to all persons on the facility mailing list, maintained by the Director in accordance with 40 CFR 124.10(c)(viii), and the appropriate units of State and local government, as spec- ified in 40 CFR 124.10(c)(ix). This notifi- cation must be made within 90 calendar days after the change is put into effect. For the Class I modifications that re- quire prior Director approval, the noti- fication must be made within 90 cal- endar days after the Director approves the request.

  - Any person may request the Di-

rector to review, and the Director may for cause reject, any Class 1 modifica- tion. The Director must inform the permittee by certified mail that a Class 1 modification has been rejected, explaining the reasons for the rejec- tion. If a Class 1 modification has been rejected, the permittee must comply with the original permit conditions.

- Class 1 permit modifications iden- tified in appendix I by an asterisk may be made only with the prior written ap- proval of the Director.

- For a Class 1 permit modification, the permittee may elect to follow the procedures in § 270.42(b) for Class 2 modifications instead of the Class 1 procedures. The permittee must inform the Director of this decision in the no- tice required in § 270.42(b)(1).

- Class 2 modifications. (1) For Class 2 modifications, listed in appendix I of this section, the permittee must sub- mit a modification request to the Di- rector that:

- Describes the exact change to be made to the permit conditions and sup- porting documents referenced by the permit;

- Identifies that the modification is a Class 2 modification;

- Explains why the modification is needed; and

- Provides the applicable informa- tion required by §§ 270.13 through 270.21, 270.62, and 270.63.

- The permittee must send a notice of the modification request to all per- sons on the facility mailing list main- tained by the Director and to the ap- propriate units of State and local gov- ernment as specified in 40 CFR 124.10(c)(ix) and must publish this no- tice in a major local newspaper of gen-

eral circulation. This notice must be mailed and published within 7 days be- fore or after the date of submission of the modification request, and the per- mittee must provide to the Director evidence of the mailing and publica- tion. The notice must include:

- Announcement of a 60-day com- ment  period,  in  accordance  with

§ 270.42(b)(5), and the name and address of an Agency contact to whom com- ments must be sent;

- Announcement of the date, time, and place for a public meeting held in accordance with § 270.42(b)(4);

- Name and telephone number of the permittee’s contact person;

- Name and telephone number of an Agency contact person;

- Location where copies of the modification request and any sup- porting documents can be viewed and copied; and

- The following statement: ‘‘The permittee’s compliance history during the life of the permit being modified is available from the Agency contact per- son.’’

- The permittee must place a copy of the permit modification request and supporting documents in a location ac- cessible to the public in the vicinity of the permitted facility.

- The permittee must hold a public meeting no earlier than 15 days after the publication of the notice required in paragraph (b)(2) of this section and no later than 15 days before the close of the 60-day comment period. The meet- ing must be held to the extent prac- ticable in the vicinity of the permitted facility.

- The public shall be provided 60 days to comment on the modification request. The comment period will begin on the date the permittee pub- lishes the notice in the local news- paper. Comments should be submitted to the Agency contact identified in the public notice.

(6)(i) No later than 90 days after re- ceipt of the notification request, the Director must:

  - Approve the modification re- quest, with or without changes, and modify the permit accordingly;

  - Deny the request;

  - Determine that the modification request must follow the procedures in

## § 270.42

§ 270.42(c) for Class 3 modifications for the following reasons:

    - There is significant public con- cern about the proposed modification; or

    - The complex nature of the change requires the more extensive procedures of Class 3.

  - Approve the request, with or without changes, as a temporary au- thorization having a term of up to 180 days, or

  - Notify the permittee that he or she will decide on the request within the next 30 days.

- If the Director notifies the per- mittee of a 30-day extension for a deci- sion, the Director must, no later than

120 days after receipt of the modifica- tion request:

  - Approve the modification re- quest, with or without changes, and modify the permit accordingly;

  - Deny the request; or

  - Determine that the modification request must follow the procedures in

§ 270.42(c) for Class 3 modifications for the following reasons:

    - There is significant public con- cern about the proposed modification; or

    - The complex nature of the change requires the more extensive procedures of Class 3.

  - Approve the request, with or without changes, as a temporary au- thorization having a term of up to 180 days.

- If the Director fails to make one of the decisions specified in paragraph (b)(6)(ii) of this section by the 120th day after receipt of the modification request, the permittee is automatically authorized to conduct the activities de- scribed in the modification request for up to 180 days, without formal Agency action. The authorized activities must be conducted as described in the permit modification request and must be in compliance with all appropriate stand- ards of 40 CFR part 265. If the Director approves, with or without changes, or denies the modification request during the term of the temporary or auto- matic authorization provided for in paragraphs (b)(6) (i), (ii), or (iii) of this section, such action cancels the tem- porary or automatic authorization.

## 40 CFR Ch. I (7–1–24 Edition)

(iv)(A) In the case of an automatic authorization under paragraph (b)(6)(iii) of this section, or a tem- porary authorization under paragraph (b)(6) (i)(D) or (ii)(D) of this section, if the Director has not made a final ap- proval or denial of the modification re- quest by the date 50 days prior to the end of the temporary or automatic au- thorization, the permittee must within seven days of that time send a notifica- tion to persons on the facility mailing list, and make a reasonable effort to notify other persons who submitted written comments on the modification request, that:

- The permittee has been authorized temporarily to conduct the activities described in the permit modification request, and

- Unless the Director acts to give final approval or denial of the request by the end of the authorization period, the permittee will receive authoriza- tion to conduct such activities for the life of the permit.

- If the owner/operator fails to no- tify the public by the date specified in paragraph (b)(6)(iv)(A) of this section, the effective date of the permanent au- thorization will be deferred until 50 days after the owner/operator notifies the public.

- Except as provided in paragraph (b)(6)(vii) of this section, if the Direc- tor does not finally approve or deny a modification request before the end of the automatic or temporary authoriza- tion period or reclassify the modifica- tion as a Class 3, the permittee is au- thorized to conduct the activities de- scribed in the permit modification re- quest for the life of the permit unless modified later under § 270.41 or § 270.42. The activities authorized under this paragraph must be conducted as de- scribed in the permit modification re- quest and must be in compliance with all appropriate standards of 40 CFR part 265.

- In making a decision to approve or deny a modification request, includ- ing a decision to issue a temporary au- thorization or to reclassify a modifica- tion as a Class 3, the Director must consider all written comments sub- mitted to the Agency during the public comment period and must respond in

## Environmental Protection Agency	§ 270.42

writing to all significant comments in his or her decision.

- With the written consent of the permittee, the Director may extend in- definitely or for a specified period the time periods for final approval or de- nial of a modification request or for re- classifying a modification as a Class 3.

- The Director may deny or change the terms of a Class 2 permit modifica- tion request under paragraphs (b)(6) (i) through (iii) of this section for the fol- lowing reasons:

  - The modification request is in- complete;

  - The requested modification does not comply with the appropriate re- quirements of 40 CFR part 264 or other applicable requirements; or

  - The conditions of the modifica- tion fail to protect human health and the environment.

- The permittee may perform any construction associated with a Class 2 permit modification request beginning 60 days after the submission of the re- quest unless the Director establishes a later date for commencing construc- tion and informs the permittee in writ- ing before day 60.

- Class 3 modifications. (1) For Class

3 modifications listed in appendix I of this section, the permittee must sub- mit a modification request to the Di- rector that:

- Describes the exact change to be made to the permit conditions and sup- porting documents referenced by the permit;

- Identifies that the modification is a Class 3 modification;

- Explains why the modification is needed; and

- Provides the applicable informa- tion required by 40 CFR 270.13 through 270.22, 270.62, 270.63, and 270.66.

(2) The permittee must send a notice of the modification request to all per- sons on the facility mailing list main- tained by the Director and to the ap- propriate units of State and local gov- ernment as specified in 40 CFR 124.10(c)(ix) and must publish this no- tice in a major local newspaper of gen- eral circulation. This notice must be mailed and published within seven days before or after the date of submission of the modification request, and the permittee must provide to the Director

evidence of the mailing and publica- tion. The notice must include:

  - Announcement of a 60-day com- ment period, and a name and address of an Agency contact to whom comments must be sent;

  - Announcement of the date, time, and place for a public meeting on the modification request, in accordance with § 270.42(c)(4);

  - Name and telephone number of the permittee’s contact person;

  - Name and telephone number of an Agency contact person;

  - Location where copies of the modification request and any sup- porting documents can be viewed and copied; and

  - The following statement: ‘‘The permittee’s compliance history during the life of the permit being modified is available from the Agency contact per- son.’’

- The permittee must place a copy of the permit modification request and supporting documents in a location ac- cessible to the public in the vicinity of the permitted facility.

- The permittee must hold a public meeting no earlier than 15 days after the publication of the notice required in paragraph (c)(2) of this section and no later than 15 days before the close of the 60-day comment period. The meet- ing must be held to the extent prac- ticable in the vicinity of the permitted facility.

- The public shall be provided at least 60 days to comment on the modi- fication request. The comment period will begin on the date the permittee publishes the notice in the local news- paper. Comments should be submitted to the Agency contact identified in the notice.

- After the conclusion of the 60-day comment period, the Director must grant or deny the permit modification request according to the permit modi- fication procedures of 40 CFR part 124. In addition, the Director must consider and respond to all significant written comments received during the 60-day comment period.

- Other modifications. (1) In the case of modifications not explicitly listed in appendix I of this section, the per- mittee may submit a Class 3 modifica- tion request to the Agency, or he or she

## § 270.42

may request a determination by the Director that the modification should be reviewed and approved as a Class 1 or Class 2 modification. If the per- mittee requests that the modification be classified as a Class 1 or 2 modifica- tion, he or she must provide the Agen- cy with the necessary information to support the requested classification.

- The Director shall make the de- termination described in paragraph (d)(1) of this section as promptly as practicable. In determining the appro- priate class for a specific modification, the Director shall consider the simi- larity of the modification to other modifications codified in appendix I and the following criteria:

  - Class 1 modifications apply to minor changes that keep the permit current with routine changes to the fa- cility or its operation. These changes do not substantially alter the permit conditions or reduce the capacity of the facility to protect human health or the environment. In the case of Class 1 modifications, the Director may re- quire prior approval.

  - Class 2 modifications apply to changes that are necessary to enable a permittee to respond, in a timely man- ner, to,

    - Common variations in the types and quantities of the wastes managed under the facility permit,

    - Technological advancements, and

    - Changes necessary to comply with new regulations, where these changes can be implemented without substantially changing design speci- fications or management practices in the permit.

  - Class 3 modifications substan- tially alter the facility or its oper- ation.

- Temporary authorizations. (1) Upon request of the permittee, the Director may, without prior public notice and comment, grant the permittee a tem- porary authorization in accordance with this subsection. Temporary au- thorizations must have a term of not more than 180 days.

(2)(i) The permittee may request a temporary authorization for:

- Any Class 2 modification meeting the criteria in paragraph (e)(3)(ii) of this section, and

## 40 CFR Ch. I (7–1–24 Edition)

- Any Class 3 modification that meets the criteria in paragraph (3)(ii)

(A) or (B) of this section; or that meets the criteria in paragraphs (3)(ii) (C) through (E) of this section and provides improved management or treatment of a hazardous waste already listed in the facility permit.

- The temporary authorization re- quest must include:

  - A description of the activities to be conducted under the temporary au- thorization;

  - An explanation of why the tem- porary authorization is necessary; and

  - Sufficient information to ensure compliance with 40 CFR part 264 stand- ards.

- The permittee must send a no- tice about the temporary authorization request to all persons on the facility mailing list maintained by the Direc- tor and to appropriate units of State and local governments as specified in

40 CFR 124.10(c)(ix). This notification must be made within seven days of sub- mission of the authorization request.

- The Director shall approve or deny the temporary authorization as quickly as practical. To issue a tem- porary authorization, the Director must find:

  - The authorized activities are in compliance with the standards of 40 CFR part 264.

  - The temporary authorization is necessary to achieve one of the fol- lowing objectives before action is like- ly to be taken on a modification re- quest:

    - To facilitate timely implementa- tion of closure or corrective action ac- tivities;

    - To allow treatment or storage in tanks or containers, or in containment buildings in accordance with 40 CFR part 268;

    - To prevent disruption of ongoing waste management activities;

    - To enable the permittee to re- spond to sudden changes in the types or quantities of the wastes managed under the facility permit; or

    - To facilitate other changes to protect human health and the environ- ment.

- A temporary authorization may be reissued for one additional term of

## Environmental Protection Agency	§ 270.42

up to 180 days provided that the per- mittee has requested a Class 2 or 3 per- mit modification for the activity cov- ered in the temporary authorization, and:

  - The reissued temporary authoriza- tion constitutes the Director’s decision on a Class 2 permit modification in ac- cordance with paragraph (b)(6)(i)(D) or (ii)(D) of this section, or

  - The Director determines that the reissued temporary authorization in- volving a Class 3 permit modification request is warranted to allow the au- thorized activities to continue while the modification procedures of para- graph (c) of this section are conducted.

- Public notice and appeals of permit modification decisions. (1) The Director shall notify persons on the facility mailing list and appropriate units of State and local government within 10 days of any decision under this section to grant or deny a Class 2 or 3 permit modification request. The Director shall also notify such persons within 10 days after an automatic authorization for a Class 2 modification goes into ef- fect under § 270.42(b)(6) (iii) or (v).

- The Director’s decision to grant or deny a Class 2 or 3 permit modifica- tion request under this section may be appealed under the permit appeal pro- cedures of 40 CFR 124.19.

- An automatic authorization that goes into effect under paragraph (b)(6)(iii) or (v) of this section may be appealed under the permit appeal pro- cedures of 40 CFR 124.19; however, the permittee may continue to conduct the activities pursuant to the automatic authorization unless and until a final determination is made by the Environ- mental Appeals Board to grant review and remand the permit decision.

- Newly regulated wastes and units.

- The permittee is authorized to con- tinue to manage wastes listed or iden- tified as hazardous under part 261 of this chapter, or to continue to manage hazardous waste in units newly regu- lated as hazardous waste management units, if:

  - The unit was in existence as a haz- ardous waste facility with respect to the newly listed or characterized waste or newly regulated waste management unit on the effective date of the final

rule listing or identifying the waste, or regulating the unit;

  - The permittee submits a Class 1 modification request on or before the date on which the waste or unit be- comes subject to the new requirements;

  - The permittee is in compliance with the applicable standards of 40 CFR parts 265 and 266 of this chapter;

  - The permittee also submits a complete Class 2 or 3 modification re- quest within 180 days of the effective date of the rule listing or identifying the waste, or subjecting the unit to RCRA Subtitle C management stand- ards;

  - In the case of land disposal units, the permittee certifies that each such unit is in compliance with all applica- ble requirements of part 265 of this chapter for groundwater monitoring and financial responsibility on the date

12 months after the effective date of the rule identifying or listing the waste as hazardous, or regulating the unit as a hazardous waste management unit. If the owner or operator fails to certify compliance with all these re- quirements, he or she will lose author- ity to operate under this section.

- New wastes or units added to a fa- cility’s permit under this subsection do not constitute expansions for the pur- pose of the 25 percent capacity expan- sion limit for Class 2 modifications.

- Military hazardous waste munitions treatment and disposal. The permittee is authorized to continue to accept waste military munitions notwithstanding any permit conditions barring the per- mittee from accepting off-site wastes, if:

  - The facility was in existence as a hazardous waste facility, and the facil- ity was already permitted to handle the waste military munitions, on the date when the waste military muni- tions became subject to hazardous waste regulatory requirements;

  - On or before the date when the waste military munitions become sub- ject to hazardous waste regulatory re- quirements, the permittee submits a Class 1 modification request to remove or amend the permit provision restrict- ing the receipt of off-site waste muni- tions; and

  - The permittee submits a complete Class 2 modification request within 180

## § 270.42

days of the date when the waste mili- tary munitions became subject to haz- ardous waste regulatory requirements.

    - Permit modification list. The Direc- tor must maintain a list of all ap- proved permit modifications and must publish a notice once a year in a State- wide newspaper that an updated list is available for review.

    - Combustion facility changes to meet part 63 MACT standards. The following procedures apply to hazardous waste combustion facility permit modifica- tions requested under appendix I of this section, section L(9).

      - Facility owners or operators must have complied with the Notification of Intent to Comply (NIC) requirements of 40 CFR 63.1210 that were in effect prior to October 11, 2000, (See 40 CFR part 63

§§ 63.1200–63.1499 revised as of July 1, 2000) in order to request a permit modi- fication under this section for the pur- pose of technology changes needed to meet the standards under 40 CFR 63.1203, 63.1204, and 63.1205.

      - Facility owners or operators must comply with the Notification of Intent to Comply (NIC) requirements of 40 CFR 63.1210(b) and 63.1212(a) before a permit modification can be requested under this section for the purpose of technology changes needed to meet the

40 CFR 63.1215, 63.1216, 63.1217, 63.1218,

63.1219, 63.1220, and 63.1221 standards

promulgated on October 12, 2005.

      - If the Director does not approve or deny the request within 90 days of re- ceiving it, the request shall be deemed approved. The Director may, at his or her discretion, extend this 90 day dead- line one time for up to 30 days by noti- fying the facility owner or operator.

## 40 CFR Ch. I (7–1–24 Edition)

    - Waiver of RCRA permit conditions in support of transition to the part 63 MACT standards. (1) You may request to have specific RCRA operating and emissions limits waived by submitting a Class 1 permit modification request under appendix I of this section, sec- tion L(10). You must:

- Identify the specific RCRA permit operating and emissions limits which you are requesting to waive;

- Provide an explanation of why the changes are necessary in order to minimize or eliminate conflicts be- tween the RCRA permit and MACT compliance; and

- Discuss how the revised provi- sions will be sufficiently protective.

- The Director shall approve or deny the request within 30 days of re- ceipt of the request. The Director may, as his or her discretion, extend this 30 day deadline one time for up to 30 days by notifying the facility owner or oper- ator.

- To request this modification in conjunction with MACT performance testing where permit limits may only be waived during actual test events and pretesting, as defined under 40 CFR 63.1207(h)(2)(i) and (ii), for an aggregate time not to exceed 720 hours of oper- ation (renewable at the discretion of the Administrator) you must:

  - Submit your modification request to the Director at the same time you submit your test plans to the Adminis- trator; and

  - The Director may elect to ap- prove or deny the request continent upon approval of the test plans.

    - [Reserved]

Appendix I to § 270.42—Classification of Permit Modification

| Modifications | Class |
| --- | --- |
| A. General Permit Provisions |  |
| 1. Administrative and informational changes ................................................................................................................. | 1 |
| 2. Correction of typographical errors ............................................................................................................................. | 1 |
| 3. Equipment replacement or upgrading with functionally equivalent components (e.g., pipes, valves, pumps, con- |  |
| veyors, controls) ......................................................................................................................................................... | 1 |
| 4. Changes in the frequency of or procedures for monitoring, reporting, sampling, or maintenance activities by the |  |
| permittee: |  |
| a. To provide for more frequent monitoring, reporting, sampling, or maintenance ............................................... | 1 |
| b. Other changes .................................................................................................................................................... | 2 |
| 5. Schedule of compliance: |  |
| a. Changes in interim compliance dates, with prior approval of the Director ........................................................ | 1 1 |
| b. Extension of final compliance date ..................................................................................................................... | 3 |
| 6. Changes in expiration date of permit to allow earlier permit termination, with prior approval of the Director ......... | 1 1 |
| 7. Changes in ownership or operational control of a facility, provided the procedures of § 270.40(b) are followed .... | 1 1 |
| 8. Changes to remove permit conditions that are no longer applicable (i.e., because the standards upon which |  |
| they are based are no longer applicable to the facility). ............................................................................................ | 1 1 |

## Environmental Protection Agency	§ 270.42

APPENDIX I TO § 270.42—CLASSIFICATION OF PERMIT MODIFICATION—Continued

| Modifications | Class |
| --- | --- |
| - Changes to remove permit conditions applicable to a unit excluded under the provisions of § 261.4. ................... - Changes in the expiration date of a permit issued to a facility at which all units are excluded under the provi- sions of § 261.4. ......................................................................................................................................................... B. General Facility Standards - Changes to waste sampling or analysis methods: - To conform with agency guidance or regulations .............................................................................................. - To incorporate changes associated with F039 (multi-source leachate) sampling or analysis methods ........... - To incorporate changes associated with underlying hazardous constituents in ignitable or corrosive wastes - Other changes .................................................................................................................................................... - Changes to analytical quality assurance/control plan: - To conform with agency guidance or regulations .............................................................................................. - Other changes .................................................................................................................................................... - Changes in procedures for maintaining the operating record ................................................................................... - Changes in frequency or content of inspection schedules ....................................................................................... - Changes in the training plan: - That affect the type or decrease the amount of training given to employees ................................................... - Other changes .................................................................................................................................................... - Contingency plan: - Changes in emergency procedures (i.e., spill or release response procedures) .............................................. - Replacement with functionally equivalent equipment, upgrade, or relocate emergency equipment listed ....... - Removal of equipment from emergency equipment list ..................................................................................... - Changes in name, address, or phone number of coordinators or other persons or agencies identified in the plan ...................................................................................................................................................................... - Construction quality assurance plan: - Changes that the CQA officer certifies in the operating record will provide equivalent or better certainty that the unit components meet the design specifications .......................................................................................... - Other changes .................................................................................................................................................... NOTE: When a permit modification (such as introduction of a new unit) requires a change in facility plans or other general facility standards, that change shall be reviewed under the same procedures as the permit modification. C. Ground-Water Protection - Changes to wells: - Changes in the number, location, depth, or design of upgradient or downgradient wells of permitted ground- water monitoring system ..................................................................................................................................... - Replacement of an existing well that has been damaged or rendered inoperable, without change to loca- tion, design, or depth of the well ......................................................................................................................... - Changes in ground-water sampling or analysis procedures or monitoring schedule, with prior approval of the Di- rector ........................................................................................................................................................................... - Changes in statistical procedure for determining whether a statistically significant change in ground-water qual- ity between upgradient and downgradient wells has occurred, with prior approval of the Director .......................... - Changes in point of compliance ................................................................................................................................ - Changes in indicator parameters, hazardous constituents, or concentration limits (including ACLs): - As specified in the groundwater protection standard ......................................................................................... - As specified in the detection monitoring program .............................................................................................. - Changes to a detection monitoring program as required by § 264.98(h), unless otherwise specified in this ap- pendix ......................................................................................................................................................................... - Compliance monitoring program: - Addition of compliance monitoring program as required by §§ 264.98(g)(4) and 264.99 ................................. - Changes to a compliance monitoring program as required by § 264.99(j), unless otherwise specified in this appendix .............................................................................................................................................................. - Corrective action program: - Addition of a corrective action program as required by §§ 264.99(h)(2) and 264.100 ...................................... - Changes to a corrective action program as required by § 264.100(h), unless otherwise specified in this ap- pendix .................................................................................................................................................................. D. Closure - Changes to the closure plan: - Changes in estimate of maximum extent of operations or maximum inventory of waste on-site at any time during the active life of the facility, with prior approval of the Director .............................................................. - Changes in the closure schedule for any unit, changes in the final closure schedule for the facility, or ex- tension of the closure period, with prior approval of the Director ...................................................................... - Changes in the expected year of final closure, where other permit conditions are not changed, with prior approval of the Director ....................................................................................................................................... - Changes in procedures for decontamination of facility equipment or structures, with prior approval of the Di- rector ................................................................................................................................................................... - Changes in approved closure plan resulting from unexpected events occurring during partial or final clo- sure, unless otherwise specified in this appendix .............................................................................................. - Extension of the closure period to allow a landfill, surface impoundment or land treatment unit to receive non-hazardous wastes after final receipt of hazardous wastes under § 264.113 (d) and (e) ............................ - Creation of a new landfill unit as part of closure ....................................................................................................... - Addition of the following new units to be used temporarily for closure activities: - Surface impoundments ....................................................................................................................................... - Incinerators ......................................................................................................................................................... - Waste piles that do not comply with § 264.250(c) .............................................................................................. | 1 1 1 1 1 1 1 1 2 1 2 1 2 2 1 2 1 2 1 1 2 2 1 1 1 1 1 2 3 2 2 3 2 3 2 1 1 1 1 1 1 1 1 2 2 3 3 3 3 |

## § 270.42

40 CFR Ch. I (7–1–24 Edition)

APPENDIX I TO § 270.42—CLASSIFICATION OF PERMIT MODIFICATION—Continued

| Modifications | Class |
| --- | --- |
| - Waste piles that comply with § 264.250(c) ......................................................................................................... - Tanks or containers (other than specified below) .............................................................................................. - Tanks used for neutralization, dewatering, phase separation, or component separation, with prior approval of the Director ...................................................................................................................................................... - Staging piles ....................................................................................................................................................... E. Post-Closure - Changes in name, address, or phone number of contact in post-closure plan ........................................................ - Extension of post-closure care period ....................................................................................................................... - Reduction in the post-closure care period ................................................................................................................. - Changes to the expected year of final closure, where other permit conditions are not changed ............................ - Changes in post-closure plan necessitated by events occurring during the active life of the facility, including par- tial and final closure ................................................................................................................................................... F. Containers - Modification or addition of container units: - Resulting in greater than 25% increase in the facility’s container storage capacity, except as provided in F(1)(c) and F(4)(a) below .................................................................................................................................... - Resulting in up to 25% increase in the facility’s container storage capacity, except as provided in F(1)(c) and F(4)(a) below ................................................................................................................................................ - Or treatment processes necessary to treat wastes that are restricted from land disposal to meet some or all of the applicable treatment standards or to treat wastes to satisfy (in whole or in part) the standard of ‘‘use of practically available technology that yields the greatest environmental benefit’’ contained in § 268.8(a)(2)(ii), with prior approval of the Director. This modification may also involve addition of new waste codes or narrative descriptions of wastes. It is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ............................................................................................................................. 2. - Modification of a container unit without increasing the capacity of the unit ...................................................... - Addition of a roof to a container unit without alteration of the containment system ......................................... - Storage of different wastes in containers, except as provided in (F)(4) below: - That require additional or different management practices from those authorized in the permit ...................... - That do not require additional or different management practices from those authorized in the permit .......... Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes. - Storage or treatment of different wastes in containers: - That require addition of units or change in treatment process or management standards, provided that the wastes are restricted from land disposal and are to be treated to meet some or all of the applicable treat- ment standards, or that are to be treated to satisfy (in whole or in part) the standard of ‘‘use of practically available technology that yields the greatest environmental benefit’’ contained in § 268.8(a)(2)(ii). This modi- fication is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..................... - That do not require the addition of units or a change in the treatment process or management standards, and provided that the units have previously received wastes of the same type (e.g., incinerator scrubber water). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..................................................................................................................................................................... G. Tanks - Modification or addition of tank units resulting in greater than 25% increase in the facility’s tank capacity, except as provided in G(1)(c), G(1)(d), and G(1)(e) below ................................................................................ - Modification or addition of tank units resulting in up to 25% increase in the facility’s tank capacity, except as provided in G(1)(d) and G(1)(e) below .......................................................................................................... - Addition of a new tank that will operate for more than 90 days using any of the following physical or chem- ical treatment technologies: neutralization, dewatering, phase separation, or component separation .............. - After prior approval of the Director, addition of a new tank that will operate for up to 90 days using any of the following physical or chemical treatment technologies: neutralization, dewatering, phase separation, or component separation ......................................................................................................................................... - Modification or addition of tank units or treatment processes necessary to treat wastes that are restricted from land disposal to meet some or all of the applicable treatment standards or to treat wastes to satisfy (in whole or in part) the standard of ‘‘use of practically available technology that yields the greatest environ- mental benefit’’ contained in § 268.8(a)(2)(ii), with prior approval of the Director. This modification may also involve addition of new waste codes. It is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) .............................................................................................................................................. - Modification of a tank unit or secondary containment system without increasing the capacity of the unit .............. - Replacement of a tank with a tank that meets the same design standards and has a capacity within 10% of the replaced tank provided ......................................................................................................................................... —The capacity difference is no more than 1500 gallons, —The facility’s permitted tank capacity is not increased, and —The replacement tank meets the same conditions in the permit. - Modification of a tank management practice ............................................................................................................. - Management of different wastes in tanks: - That require additional or different management practices, tank design, different fire protection specifica- tions, or significantly different tank treatment process from that authorized in the permit, except as provided in (G)(5)(c) below ................................................................................................................................................ - That do not require additional or different management practices, tank design, different fire protection spec- ifications, or significantly different tank treatment process than authorized in the permit, except as provided in (G)(5)(d) ........................................................................................................................................................... | 2 2 1 1 2 1 2 3 1 2 3 2 1 1 2 1 3 2 1 1 1 1 3 2 2 1 1 1 1 2 1 2 3 2 |

## Environmental Protection Agency	§ 270.42

APPENDIX I TO § 270.42—CLASSIFICATION OF PERMIT MODIFICATION—Continued

| Modifications | Class |
| --- | --- |
| c. That require addition of units or change in treatment processes or management standards, provided that the wastes are restricted from land disposal and are to be treated to meet some or all of the applicable treatment standards or that are to be treated to satisfy (in whole or in part) the standard of ‘‘use of prac- tically available technology that yields the greatest environmental benefit’’ contained in § 268.8(a)(2)(ii). The modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ............. d. That do not require the addition of units or a change in the treatment process or management standards, and provided that the units have previously received wastes of the same type (e.g., incinerator scrubber water). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..................................................................................................................................................................... Note: See § 270.42(g) for modification procedures to be used for the management of newly lilsted or identified wastes. H. Surface Impoundments - Modification or addition of surface impoundment units that result in increasing the facility’s surface impound- ment storage or treatment capacity ........................................................................................................................... - Replacement of a surface impoundment unit ............................................................................................................ - Modification of a surface impoundment unit without increasing the facility’s surface impoundment storage or treatment capacity and without modifying the unit’s liner, leak detection system, or leachate collection system .... - Modification of a surface impoundment management practice ................................................................................. - Treatment, storage, or disposal of different wastes in surface impoundments: - That require additional or different management practices or different design of the liner or leak detection system than authorized in the permit .................................................................................................................. - That do not require additional or different management practices or different design of the liner or leak de- tection system than authorized in the permit ...................................................................................................... - That are wastes restricted from land disposal that meet the applicable treatment standards or that are treat- ed to satisfy the standard of ‘‘use of practically available technology that yields the greatest environmental benefit’’ contained in § 269.8(a)(2)(ii), and provided that the unit meets the minimum technological require- ments stated in § 268.5(h)(2). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ............................................................................................................................. - That are residues from wastewater treatment or incineration, provided that disposal occurs in a unit that meets the minimum technological requirements stated in § 268.5(h)(2), and provided further that the surface impoundment has previously received wastes of the same type (for example, incinerator scrubber water). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..... - Modifications of unconstructed units to comply with §§ 264.221(c), 264.222, 264.223, and 264.226(d) ................. - Changes in response action plan: - Increase in action leakage rate .......................................................................................................................... - Change in a specific response reducing its frequency or effectiveness ............................................................ - Other changes .................................................................................................................................................... Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes I. Enclosed Waste Piles. For all waste piles except those complying with § 264.250(c), modifications are treated the same as for a landfill. The following modifications are applicable only to waste piles complying with § 264.250(c). - Modification or addition of waste pile units: - Resulting in greater than 25% increase in the facility’s waste pile storage or treatment capacity ................... - Resulting in up to 25% increase in the facility’s waste pile storage or treatment capacity ............................... - Modification of waste pile unit without increasing the capacity of the unit ............................................................... - Replacement of a waste pile unit with another waste pile unit of the same design and capacity and meeting all waste pile conditions in the permit ............................................................................................................................. - Modification of a waste pile management practice ................................................................................................... - Storage or treatment of different wastes in waste piles: - That require additional or different management practices or different design of the unit ................................ - That do not require additional or different management practices or different design of the unit .................... - Conversion of an enclosed waste pile to a containment building unit ...................................................................... Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes. J. Landfills and Unenclosed Waste Piles - Modification or addition of landfill units that result in increasing the facility’s disposal capacity .............................. - Replacement of a landfill ........................................................................................................................................... - Addition or modification of a liner, leachate collection system, leachate detection system, run-off control, or final cover system .............................................................................................................................................................. - Modification of a landfill unit without changing a liner, leachate collection system, leachate detection system, run-off control, or final cover system .......................................................................................................................... - Modification of a landfill management practice ......................................................................................................... - Landfill different wastes: - That require additional or different management practices, different design of the liner, leachate collection system, or leachate detection system ................................................................................................................. - That do not require additional or different management practices, different design of the liner, leachate col- lection system, or leachate detection system ..................................................................................................... - That are wastes restricted from land disposal that meet the applicable treatment standards or that are treat- ed to satisfy the standard of ‘‘use of practically available technology that yields the greatest environmental benefit’’ contained in § 268.8(a)(2)(ii), and provided that the landfill unit meets the minimum technological requirements stated in § 268.5(h)(2). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..................................................................................................................... | 1 1 1 3 3 2 2 3 2 1 1 1 1 3 3 2 3 2 2 1 2 3 2 2 3 3 3 2 2 3 2 1 |

## § 270.42

40 CFR Ch. I (7–1–24 Edition)

APPENDIX I TO § 270.42—CLASSIFICATION OF PERMIT MODIFICATION—Continued

| Modifications | Class |
| --- | --- |
| d. That are residues from wastewater treatment or incineration, provided that disposal occurs in a landfill unit that meets the minimum technological requirements stated in § 268.5(h)(2), and provided further that the landfill has previously received wastes of the same type (for example, incinerator ash). This modification is not applicable to dioxin-containing wastes (F020, 021, 022, 023, 026, 027, and 028) ..................................... - Modifications of unconstructed units to comply with §§ 264.251(c), 264.252, 264.253, 264.254(c), 264.301(c), 264.302, 264.303(c), and 264.304 ............................................................................................................................. - Changes in response action plan: - Increase in action leakage rate .......................................................................................................................... - Change in a specific response reducing its frequency or effectiveness ............................................................ - Other changes .................................................................................................................................................... Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes. K. Land Treatment - Lateral expansion of or other modification of a land treatment unit to increase areal extent .................................. - Modification of run-on control system ........................................................................................................................ - Modify run-off control system .................................................................................................................................... - Other modifications of land treatment unit component specifications or standards required in permit .................... - Management of different wastes in land treatment units: - That require a change in permit operating conditions or unit design specifications ......................................... - That do not require a change in permit operating conditions or unit design specifications .............................. Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes - Modification of a land treatment unit management practice to: - Increase rate or change method of waste application ....................................................................................... - Decrease rate of waste application .................................................................................................................... - Modification of a land treatment unit management practice to change measures of pH or moisture content, or to enhance microbial or chemical reactions ................................................................................................................... - Modification of a land treatment unit management practice to grow food chain crops, to add to or replace exist- ing permitted crops with different food chain crops, or to modify operating plans for distribution of animal feeds resulting from such crops ........................................................................................................................................... - Modification of operating practice due to detection of releases from the land treatment unit pursuant to § 264.278(g)(2) ........................................................................................................................................................... - Changes in the unsaturated zone monitoring system, resulting in a change to the location, depth, number of sampling points, or replace unsaturated zone monitoring devices or components of devices with devices or com- ponents that have specifications different from permit requirements ........................................................................ - Changes in the unsaturated zone monitoring system that do not result in a change to the location, depth, num- ber of sampling points, or that replace unsaturated zone monitoring devices or components of devices with de- vices or components having specifications different from permit requirements ........................................................ - Changes in background values for hazardous constituents in soil and soil-pore liquid ......................................... - Changes in sampling, analysis, or statistical procedure ......................................................................................... - Changes in land treatment demonstration program prior to or during the demonstration ..................................... - Changes in any condition specified in the permit for a land treatment unit to reflect results of the land treat- ment demonstration, provided performance standards are met, and the Director’s prior approval has been re- ceived ......................................................................................................................................................................... - Changes to allow a second land treatment demonstration to be conducted when the results of the first dem- onstration have not shown the conditions under which the wastes can be treated completely, provided the con- ditions for the second demonstration are substantially the same as the conditions for the first demonstration and have received the prior approval of the Director ....................................................................................................... - Changes to allow a second land treatment demonstration to be conducted when the results of the first dem- onstration have not shown the conditions under which the wastes can be treated completely, where the condi- tions for the second demonstration are not substantially the same as the conditions for the first demonstration ... - Changes in vegetative cover requirements for closure ........................................................................................... L. Incinerators, Boilers, and Industrial Furnaces: - Changes to increase by more than 25% any of the following limits authorized in the permit: A thermal feed rate limit, a feedstream feed rate limit, a chlorine/chloride feed rate limit, a metal feed rate limit, or an ash feed rate limit. The Director will require a new trial burn to substantiate compliance with the regulatory performance stand- ards unless this demonstration can be made through other means ......................................................................... - Changes to increase by up to 25% any of the following limits authorized in the permit: A thermal feed rate limit, a feedstream feed rate limit, a chlorine/chloride feed rate limit, a metal feed rate limit, or an ash feed rate limit. The Director will require a new trial burn to substantiate compliance with the regulatory performance standards unless this demonstration can be made through other means ................................................................................. - Modification of an incinerator, boiler, or industrial furnace unit by changing the internal size or geometry of the primary or secondary combustion units, by adding a primary or secondary combustion unit, by substantially changing the design of any component used to remove HCl/Cl2, metals, or particulate from the combustion gases, or by changing other features of the incinerator, boiler, or industrial furnace that could affect its capability to meet the regulatory performance standards. The Director will require a new trial burn to substantiate compli- ance with the regulatory performance standards unless this demonstration can be made through other means ... - Modification of an incinerator, boiler, or industrial furnace unit in a manner that would not likely affect the capa- bility of the unit to meet the regulatory performance standards but which would change the operating conditions or monitoring requirements specified in the permit. The Director may require a new trial burn to demonstrate compliance with the regulatory performance standards ............................................................................................ - Operating requirements:. | 1 1 1 3 3 2 3 2 3 2 3 2 3 1 2 3 3 3 2 2 2 2 1 1 1 1 3 2 3 2 3 2 |

## Environmental Protection Agency	§ 270.42

APPENDIX I TO § 270.42—CLASSIFICATION OF PERMIT MODIFICATION—Continued

| Modifications | Class |
| --- | --- |
| a. Modification of the limits specified in the permit for minimum or maximum combustion gas temperature, |  |
| minimum combustion gas residence time, oxygen concentration in the secondary combustion chamber, flue |  |
| gas carbon monoxide and hydrocarbon concentration, maximum temperature at the inlet to the particulate |  |
| matter emission control system, or operating parameters for the air pollution control system. The Director |  |
| will require a new trial burn to substantiate compliance with the regulatory performance standards unless |  |
| this demonstration can be made through other means ...................................................................................... | 3 |
| b. Modification of any stack gas emission limits specified in the permit, or modification of any conditions in the |  |
| permit concerning emergency shutdown or automatic waste feed cutoff procedures or controls ..................... | 3 |
| c. Modification of any other operating condition or any inspection or recordkeeping requirement specified in |  |
| the permit ............................................................................................................................................................ | 2 |
| 6. Burning different wastes:. |  |
| a. If the waste contains a POHC that is more difficult to burn than authorized by the permit or if burning of the |  |
| waste requires compliance with different regulatory performance standards than specified in the permit. The |  |
| Director will require a new trial burn to substantiate compliance with the regulatory performance standards |  |
| unless this demonstration can be made through other means .......................................................................... | 3 |
| b. If the waste does not contain a POHC that is more difficult to burn than authorized by the permit and if |  |
| burning of the waste does not require compliance with different regulatory performance standards than |  |
| specified in the permit ......................................................................................................................................... | 2 |
| Note: See § 270.42(g) for modification procedures to be used for the management of newly listed or identified wastes |  |
| 7. Shakedown and trial burn: |  |
| a. Modification of the trial burn plan or any of the permit conditions applicable during the shakedown period |  |
| for determining operational readiness after construction, the trial burn period, or the period immediately fol- |  |
| lowing the trial burn ............................................................................................................................................. | 2 |
| b. Authorization of up to an additional 720 hours of waste burning during the shakedown period for deter- |  |
| mining operational readiness after construction, with the prior approval of the Director ................................... | 11 |
| c. Changes in the operating requirements set in the permit for conducting a trial burn, provided the change is |  |
| minor and has received the prior approval of the Director ................................................................................. | 11 |
| d. Changes in the ranges of the operating requirements set in the permit to reflect the results of the trial burn, |  |
| provided the change is minor and has received the prior approval of the Director ........................................... | 11 |
| 8. Substitution of an alternative type of nonhazardous waste fuel that is not specified in the permit ......................... | 1 |
| 9. Technology changes needed to meet standards under 40 CFR part 63 (Subpart EEE—National Emission |  |
| Standards for Hazardous Air Pollutants From Hazardous Waste Combustors), provided the procedures of |  |
| § 270.42(j) are followed. | 1 1 |
| 10. Changes to RCRA permit provisions needed to support transition to 40 CFR part 63 (Subpart EEE—National |  |
| Emission Standards for Hazardous Air Pollutants From Hazardous Waste Combustors), provided the procedures |  |
| of § 270.42(k) are followed. |  |
| M. Containment Buildings. |  |
| 1. Modification or addition of containment building units: |  |
| a. Resulting in greater than 25% increase in the facility’s containment building storage or treatment capacity ......... | 3 |
| b. Resulting in up to 25% increase in the facility’s containment building storage or treatment capacity ..................... | 2 |
| 2. Modification of a containment building unit or secondary containment system without increasing the capacity of the |  |
| unit ..................................................................................................................................................................................... | 2 |
| 3. Replacement of a containment building with a containment building that meets the same design standards provided: |  |
| a. The unit capacity is not increased ............................................................................................................................. | 1 |
| b. The replacement containment building meets the same conditions in the permit .................................................... | 1 |
| 4. Modification of a containment building management practice ......................................................................................... | 2 |
| 5. Storage or treatment of different wastes in containment buildings: |  |
| a. That require additional or different management practices ....................................................................................... | 3 |
| b. That do not require additional or different management practices ........................................................................... | 2 |
| N. Corrective Action: |  |
| 1. Approval of a corrective action management unit pursuant to § 264.552 ................................................................. | 3 |
| 2. Approval of a temporary unit or time extension for a temporary unit pursuant to § 264.553 ................................... | 2 |
| 3. Approval of a staging pile or staging pile operating term extension pursuant to § 264.554 ..................................... | 2 |
| O. Burden Reduction |  |
| 1. [Reserved] |  |
| 2. Development of one contingency plan based on Integrated Contingency Plan Guidance pursuant to § 264.52(b) ....... | 1 |
| 3. Changes to recordkeeping and reporting requirements pursuant to: §§ 264.56(i), 264.343(a)(2), 264.1061(b)(1),(d), |  |
| 264.1062(a)(2), 264.196(f), 264.100(g), and 264.113(e)(5) | 1 |
| 4. Changes to inspection frequency for tank systems pursuant to § 264.195(b) | 1 |
| 5. Changes to detection and compliance monitoring program pursuant to §§ 264.98(d), (g)(2), and (g)(3), 264.99(f), and |  |
| (g) | 1 |

1 Class 1 modifications requiring prior Agency approval.

[53 FR 37936, Sept. 28, 1988]

EDITORIAL NOTE: For FEDERAL REGISTER citations affecting § 270.42, see the List of CFR Sec- tions Affected, which appears in the Finding Aids section of the printed volume and at www.govinfo.gov.

## § 270.43

### § 270.43  Termination of permits.

- The following are causes for ter- minating a permit during its term, or for denying a permit renewal applica- tion:

  - Noncompliance by the permittee with any condition of the permit;

  - The permittee’s failure in the ap- plication or during the permit issuance process to disclose fully all relevant facts, or the permittee’s misrepresenta- tion of any relevant facts at any time; or

  - A determination that the per- mitted activity endangers human health or the environment and can only be regulated to acceptable levels by permit modification or termination.

- The Director shall follow the ap- plicable procedures in part 124 or part 22, as appropriate or State procedures in terminating any permit under this section.

[48 FR 14228, Apr. 1, 1983, as amended at 65

FR 30913, May 15, 2000]

# Subpart E—Expiration and Continuation of Permits

### § 270.50  Duration of permits.

- RCRA permits shall be effective for a fixed term not to exceed 10 years.

- Except as provided in § 270.51, the term of a permit shall not be extended by modification beyond the maximum duration specified in this section.

- The Director may issue any per- mit for a duration that is less than the full allowable term under this section.

- Each permit for a land disposal fa- cility shall be reviewed by the Director five years after the date of permit issuance or reissuance and shall be modified as necessary, as provided in

§ 270.41.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 28752, July 15, 1985]

### § 270.51 Continuation of expiring per- mits.

- EPA permits. When EPA is the per- mit-issuing authority, the conditions of an expired permit continue in force under 5 U.S.C. 558(c) until the effective date of a new permit (see § 124.15) if:

  - The permittee has submitted a timely application under § 270.14 and the  applicable  sections  in  §§ 270.15

## 40 CFR Ch. I (7–1–24 Edition)

through 270.29 which is a complete (under § 270.10(c)) application for a new permit; and

  - The Regional Administrator through no fault of the permittee, does not issue a new permit with an effec- tive date under § 124.15 on or before the expiration date of the previous permit (for example, when issuance is imprac- ticable due to time or resource con- straints).

- Effect. Permits continued under this section remain fully effective and enforceable.

- Enforcement. When the permittee is not in compliance with the condi- tions of the expiring or expired permit, the Regional Administrator may choose to do any or all of the following:

  - Initiate enforcement action based upon the permit which has been contin- ued;

  - Issue a notice of intent to deny the new permit under § 124.6. If the per- mit is denied, the owner or operator would then be required to cease the ac- tivities authorized by the continued permit or be subject to enforcement ac- tion for operating without a permit;

  - Issue a new permit under part 124 with appropriate conditions; or

  - Take other actions authorized by these regulations.

- State continuation. In a State with a hazardous waste program authorized under 40 CFR part 271, if a permittee has submitted a timely and complete application under applicable State law and regulations, the terms and condi- tions of an EPA-issued RCRA permit continue in force beyond the expiration date of the permit, but only until the effective date of the State’s issuance or denial of a State RCRA permit.

- Standardized permits. (1) The condi- tions of your expired standardized per- mit continue until the effective date of your new permit (see 40 CFR 124.15) if all of the following are true:

- If EPA is the permit-issuing au- thority.

- If you submit a timely and com- plete Notice of Intent under 40 CFR 124.202(b) requesting coverage under a RCRA standardized permit; and

## Environmental Protection Agency	§ 270.61

- If the Director, through no fault on your part, does not issue your per- mit before your previous permit ex- pires (for example, where it is imprac- tical to make the permit effective by that date because of time or resource constraints).

(2) In some cases, the Director may notify you that you are not eligible for a standardized permit (see 40 CFR 124.206). In those cases, the conditions of your expired permit will continue if you submit the information specified in paragraph (a)(1) of this section (that is, a complete application for a new permit) within 60 days after you re- ceive our notification that you are not eligible for a standardized permit.

(Clean Water Act (33 U.S.C. 1251 et seq.), Safe Drinking Water Act (42 U.S.C. 300f et seq.), Clean Air Act (42 U.S.C. 7401 et seq.), Re- source Conservation and Recovery Act (42 U.S.C. 6901 et seq.))

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 39622, Sept. 1, 1983; 70 FR 53475, Sept. 8,

2005]

# Subpart F—Special Forms of Permits

### § 270.60  Permits by rule.

Notwithstanding any other provision of this part or part 124, the following shall be deemed to have a RCRA permit if the conditions listed are met:

- Ocean disposal barges or vessels. The owner or operator of a barge or other vessel which accepts hazardous waste for ocean disposal, if the owner or operator:

  - Has a permit for ocean dumping issued under 40 CFR part 220 (Ocean Dumping, authorized by the Marine Protection, Research, and Sanctuaries Act, as amended, 33 U.S.C. 1420 et seq.);

  - Complies with the conditions of that permit; and

  - Complies with the following haz- ardous waste regulations:

    - 40 CFR 264.11, Identification num- ber;

    - 40 CFR 264.71, Use of manifest system;

    - 40 CFR 264.72, Manifest discrep- ancies;

    - 40 CFR 264.73(a) and (b)(1), Oper- ating record;

    - 40 CFR 264.75, Biennial report; and

    - 40 CFR 264.76, Unmanifested waste report.

- Injection wells. The owner or oper- ator of an injection well disposing of hazardous waste, if the owner or oper- ator:

  - Has a permit for underground in- jection issued under part 144 or 145; and

  - Complies with the conditions of that permit and the requirements of

§ 144.14 (requirements for wells man- aging hazardous waste).

  - For UIC permits issued after No- vember 8, 1984:

    - Complies with 40 CFR 264.101; and

    - Where the UIC well is the only unit at a facility which requires a RCRA permit, complies with 40 CFR 270.14(d).

- Publicly owned treatment works. The owner or operator of a POTW which accepts for treatment hazardous waste, if the owner or operator:

  - Has an NPDES permit;

  - Complies with the conditions of that permit; and

  - Complies with the following regu- lations:

    - 40 CFR 264.11, Identification num- ber;

    - 40 CFR 264.71, Use of manifest system;

    - 40 CFR 264.72, Manifest discrep- ancies;

    - 40 CFR 264.73(a) and (b)(1), Oper- ating record;

    - 40 CFR 264.75, Biennial report;

    - 40 CFR 264.76, Unmanifested waste report; and

    - For NPDES permits issued after November 8, 1984, 40 CFR 264.101.

  - If the waste meets all Federal, State, and local pretreatment require- ments which would be applicable to the waste if it were being discharged into the POTW through a sewer, pipe, or similar conveyance.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 28752, July 15, 1985; 52 FR 45799, Dec. 1,

1987]

### § 270.61  Emergency permits.

- Notwithstanding any other provi- sion of this part or part 124, in the event the Director finds an imminent and substantial endangerment to human health or the environment the Director may issue a temporary emer- gency permit: (1) To a non-permitted

## § 270.62

facility to allow treatment, storage, or disposal of hazardous waste or (2) to a permitted facility to allow treatment, storage, or disposal of a hazardous waste not covered by an effective per- mit.

- This emergency permit:

  - May be oral or written. If oral, it shall be followed in five days by a writ- ten emergency permit;

  - Shall not exceed 90 days in dura- tion;

  - Shall clearly specify the haz- ardous wastes to be received, and the manner and location of their treat- ment, storage, or disposal;

  - May be terminated by the Direc- tor at any time without process if he or she determines that termination is ap- propriate to protect human health and the environment;

  - Shall be accompanied by a public notice published under § 124.10(b) in- cluding:

    - Name and address of the office granting the emergency authorization;

    - Name and location of the per- mitted HWM facility;

    - A brief description of the wastes involved;

    - A brief description of the action authorized and reasons for authorizing it; and

    - Duration of the emergency per- mit; and

  - Shall incorporate, to the extent possible and not inconsistent with the emergency situation, all applicable re- quirements of this part and 40 CFR parts 264 and 266.

[48 FR 14228, Apr. 1, 1983, as amended at 48

FR 30114, June 30, 1983; 60 FR 63433, Dec. 11,

1996]

### § 270.62 Hazardous waste incinerator permits.

When an owner or operator of a haz- ardous waste incineration unit be- comes subject to RCRA permit require- ments after October 12, 2005, or when an owner or operator of an existing hazardous waste incineration unit dem- onstrates compliance with the air emission standards and limitations in part 63, subpart EEE, of this chapter (i.e., by conducting a comprehensive performance test and submitting a No- tification   of   Compliance   under

§§ 63.1207(j) and 63.1210(d) of this chapter

## 40 CFR Ch. I (7–1–24 Edition)

documenting compliance with all ap- plicable requirements of part 63, sub- part EEE, of this chapter), the require- ments of this section do not apply, ex- cept those provisions the Director de- termines are necessary to ensure com- pliance with §§ 264.345(a) and 264.345(c) of this chapter if you elect to comply with § 270.235(a)(1)(i) to minimize emis- sions of toxic compounds from startup, shutdown, and malfunction events. Nevertheless, the Director may apply the provisions of this section, on a case-by-case basis, for purposes of in- formation collection in accordance with §§ 270.10(k), 270.10(l), 270.32(b)(2), and 270.32(b)(3).

- For the purposes of determining operational readiness following com- pletion of physical construction, the Director must establish permit condi- tions, including but not limited to al- lowable waste feeds and operating con- ditions, in the permit to a new haz- ardous waste incinerator. These permit conditions will be effective for the min- imum time required to bring the incin- erator to a point of operational readi- ness to conduct a trial burn, not to ex- ceed 720 hours operating time for treat- ment of hazardous waste. The Director may extend the duration of this oper- ational period once, for up to 720 addi- tional hours, at the request of the ap- plicant when good cause is shown. The permit may be modified to reflect the extension according to § 270.42 of this chapter.

  - Applicants must submit a state- ment, with part B of the permit appli- cation, which suggests the conditions necessary to operate in compliance with  the  performance  standards  of

§ 264.343 of this chapter during this pe- riod. This statement should include, at a minimum, restrictions on waste con- stituents, waste feed rates and the op- erating  parameters  identified  in

§ 264.345 of this chapter.

  - The Director will review this statement and any other relevant in- formation submitted with part B of the permit application and specify require- ments for this period sufficient to meet the performance standards of § 264.343 of this chapter based on his engineer- ing judgment.

## Environmental Protection Agency	§ 270.62

- For the purposes of determining feasibility of compliance with the per- formance standards of § 264.343 of this chapter and of determining adequate operating conditions under § 264.345 of this chapter, the Director must estab- lish conditions in the permit for a new hazardous waste incinerator to be ef- fective during the trial burn.

  - Applicants must propose a trial burn plan, prepared under paragraph (b)(2) of this section with a part B of the permit application.

  - The trial burn plan must include the following information:

    - An analysis of each waste or mix- ture of wastes to be burned which in- cludes:

      - Heat value of the waste in the form and composition in which it will be burned.

      - Viscosity (if applicable), or de- scription of the physical form of the waste.

      - An identification of any haz- ardous organic constituents listed in part 261, appendix VIII of this chapter, which are present in the waste to be burned, except that the applicant need not analyze for constituents listed in part 261, appendix VIII, of this chapter which would reasonably not be ex- pected to be found in the waste. The constituents excluded from analysis must be identified, and the basis for the exclusion stated. The waste anal- ysis must rely on appropriate analyt- ical techniques.

      - An approximate quantification of

the hazardous constituents identified in the waste, within the precision pro- duced by appropriate analytical meth- ods.

    - A detailed engineering descrip- tion of the incinerator for which the permit is sought including:

      - Manufacturer’s name and model number of incinerator (if available).

      - Type of incinerator.

      - Linear dimensions of the inciner- ator unit including the cross sectional area of combustion chamber.

      - Description of the auxiliary fuel system (type/feed).

      - Capacity of prime mover.

      - Description of automatic waste feed cut-off system(s).

      - Stack gas monitoring and pollu- tion control equipment.

      - Nozzle and burner design.

      - Construction materials.

      - Location and description of tem- perature, pressure, and flow indicating and control devices.

- A detailed description of sam- pling and monitoring procedures, in- cluding sampling and monitoring loca- tions in the system, the equipment to be used, sampling and monitoring fre- quency, and planned analytical proce- dures for sample analysis.

- A detailed test schedule for each waste for which the trial burn is planned including date(s), duration, quantity of waste to be burned, and other factors relevant to the Director’s decision under paragraph (b)(5) of this section.

- A detailed test protocol, includ- ing, for each waste identified, the ranges of temperature, waste feed rate, combustion gas velocity, use of auxil- iary fuel, and any other relevant pa- rameters that will be varied to affect the destruction and removal efficiency of the incinerator.

- A description of, and planned op- erating conditions for, any emission control equipment which will be used.

- Procedures for rapidly stopping waste feed, shutting down the inciner- ator, and controlling emissions in the event of an equipment malfunction.

- Such other information as the Director reasonably finds necessary to determine whether to approve the trial burn plan in light of the purposes of this paragraph and the criteria in para- graph (b)(5) of this section.

  - The Director, in reviewing the trial burn plan, shall evaluate the suf- ficiency of the information provided and may require the applicant to sup- plement this information, if necessary, to achieve the purposes of this para- graph.

  - Based on the waste analysis data in the trial burn plan, the Director will specify as trial Principal Organic Haz- ardous Constituents (POHCs), those constituents for which destruction and removal efficiencies must be calculated during the trial burn. These trial POHCs will be specified by the Director based on his estimate of the difficulty of incineration of the constituents identified in the waste analysis, their concentration or mass in the waste

## § 270.62

feed, and, for wastes listed in part 261, subpart D, of this chapter, the haz- ardous waste organic constituent or constituents identified in appendix VII of that part as the basis for listing.

  - The Director shall approve a trial burn plan if he finds that:

    - The trial burn is likely to deter- mine whether the incinerator perform- ance standard required by § 264.343 of this chapter can be met;

    - The trial burn itself will not present an imminent hazard to human health or the environment;

    - The trial burn will help the Di- rector to determine operating require- ments to be specified under § 264.345 of this chapter; and

    - The information sought in para- graphs (b)(5) (i) and (ii) of this section cannot reasonably be developed through other means.

  - The Director must send a notice to all persons on the facility mailing list as set forth in 40 CFR 124.10(c)(1)(ix) and to the appropriate units of State and local government as set forth in 40 CFR 124.10(c)(1)(x) an- nouncing the scheduled commence- ment and completion dates for the trial burn. The applicant may not com- mence the trial burn until after the Di- rector has issued such notice.

    - This notice must be mailed within a reasonable time period before the scheduled trial burn. An additional no- tice is not required if the trial burn is delayed due to circumstances beyond the control of the facility or the per- mitting agency.

    - This notice must contain:

      - The name and telephone number of the applicant’s contact person;

      - The name and telephone number of the permitting agency’s contact of- fice;

      - The location where the approved trial burn plan and any supporting doc- uments can be reviewed and copied; and

      - An expected time period for com- mencement and completion of the trial burn.

  - During each approved trial burn (or as soon after the burn as is prac- ticable), the applicant must make the following determinations:

## 40 CFR Ch. I (7–1–24 Edition)

- A quantitative analysis of the trial POHCs in the waste feed to the in- cinerator.

- A quantitative analysis of the ex- haust gas for the concentration and mass emissions of the trial POHCs, ox- ygen (O2) and hydrogen chloride (HCl).

- A quantitative analysis of the scrubber water (if any), ash residues, and other residues, for the purpose of estimating the fate of the trial POHCs.

- A computation of destruction and removal efficiency (DRE), in ac- cordance with the DRE formula speci- fied in § 264.343(a) of this chapter.

- If the HCl emission rate exceeds

1.8 kilograms of HCl per hour (4 pounds per hour), a computation of HCl re- moval efficiency in accordance with

§ 264.343(b) of this chapter.

- A computation of particulate emissions,   in   accordance   with

§ 264.343(c) of this chapter.

- An identification of sources of fugitive emissions and their means of control.

- A measurement of average, maximum, and minimum temperatures and combustion gas velocity.

- A continuous measurement of carbon monoxide (CO) in the exhaust gas.

- Such other information as the Di- rector may specify as necessary to en- sure that the trial burn will determine compliance with the performance standards in § 264.343 of this chapter and to establish the operating condi- tions required by § 264.345 of this chap- ter as necessary to meet that perform- ance standard.

  - The applicant must submit to the Director a certification that the trial burn has been carried out in accord- ance with the approved trial burn plan, and must submit the results of all the determinations required in paragraph (b)(6) of this section. This submission shall be made within 90 days of comple- tion of the trial burn, or later if ap- proved by the Director.

  - All data collected during any trial burn must be submitted to the Director following the completion of the trial burn.

  - All submissions required by this paragraph must be certified on behalf of the applicant by the signature of a

## Environmental Protection Agency	§ 270.63

person authorized to sign a permit ap- plication or a report under § 270.11.

  - Based on the results of the trial burn, the Director shall set the oper- ating requirements in the final permit according to § 264.345 of this chapter. The permit modification shall proceed according to § 270.42.

- For the purposes of allowing oper- ation of a new hazardous waste inciner- ator following completion of the trial burn and prior to final modification of the permit conditions to reflect the trial burn results, the Director may es- tablish permit conditions, including but not limited to allowable waste feeds and operating conditions suffi- cient  to  meet  the  requirements  of

§ 264.345 of this chapter, in the permit to a new hazardous waste incinerator. These permit conditions will be effec- tive for the minimum time required to complete sample analysis, data com- putation and submission of the trial burn results by the applicant, and modification of the facility permit by the Director.

- Applicants must submit a state- ment, with part B of the permit appli- cation, which identifies the conditions necessary to operate in compliance with  the  performance  standards  of

§ 264.343 of this chapter, during this pe- riod. This statement should include, at a minimum, restrictions on waste con- stituents, waste feed rates, and the op- erating parameters in § 264.345 of this chapter.

- The Director will review this statement and any other relevant in- formation submitted with part B of the permit application and specify those requirements for this period most like- ly to meet the performance standards of § 264.343 of this chapter based on his engineering judgment.

- For the purpose of determining feasibility of compliance with the per- formance standards of § 264.343 of this chapter and of determining adequate operating conditions under § 264.345 of this chapter, the applicant for a permit for an existing hazardous waste incin- erator must prepare and submit a trial burn plan and perform a trial burn in accordance with § 270.19(b) and para- graphs (b)(2) through (b)(5) and (b)(7) through (b)(10) of this section or, in- stead,  submit  other  information  as

specified in § 270.19(c). The Director must announce his or her intention to approve the trial burn plan in accord- ance with the timing and distribution requirements of paragraph (b)(6) of this section. The contents of the notice must include: the name and telephone number of a contact person at the fa- cility; the name and telephone number of a contact office at the permitting agency; the location where the trial burn plan and any supporting docu- ments can be reviewed and copied; and a schedule of the activities that are re- quired prior to permit issuance, includ- ing the anticipated time schedule for agency approval of the plan and the time period during which the trial burn would be conducted. Applicants sub- mitting information under § 270.19(a) are exempt from compliance with 40 CFR 264.343 and 264.345 and, therefore, are exempt from the requirement to conduct a trial burn. Applicants who submit trial burn plans and receive ap- proval before submission of a permit application must complete the trial burn and submit the results, specified in paragraph (b)(7) of this section, with part B of the permit application. If completion of this process conflicts with the date set for submission of the part B application, the applicant must contact the Director to establish a later date for submission of the part B application or the trial burn results. Trial burn results must be submitted prior to issuance of the permit. When the applicant submits a trial burn plan with part B of the permit application, the Director will specify a time period prior to permit issuance in which the trial burn must be conducted and the results submitted.

[48 FR 14228, Apr. 1, 1983, as amended at 53

FR 37939, Sept. 28, 1988; 58 FR 46051, Aug. 31,

1993; 60 FR 63433, Dec. 11, 1995; 64 FR 53077,

Sept. 30, 1999; 67 FR 6816, Feb. 13, 2002; 67 FR

77692, Dec. 19, 2002; 70 FR 34590, June 14, 2005;

70 FR 59578, Oct. 12, 2005]

### § 270.63 Permits for land treatment demonstrations using field test or laboratory analyses.

- For the purpose of allowing an owner or operator to meet the treat- ment demonstration requirements of

§ 264.272 of this chapter, the Director may issue a treatment demonstration

## § 270.64

permit. The permit must contain only those requirements necessary to meet the standards in § 264.272(c). The permit may be issued either as a treatment or disposal permit covering only the field test or laboratory analyses, or as a two-phase facility permit covering the field tests, or laboratory analyses, and design, construction operation and maintenance of the land treatment unit.

  - The Director may issue a two- phase facility permit if he finds that, based on information submitted in part B of the application, substantial, al- though incomplete or inconclusive, in- formation already exists upon which to base the issuance of a facility permit.

  - If the Director finds that not enough information exists upon which he can establish permit conditions to attempt to provide for compliance with all of the requirements of subpart M, he must issue a treatment demonstra- tion permit covering only the field test or laboratory analyses.

- If the Director finds that a phased permit may be issued, he will establish, as requirements in the first phase of the facility permit, conditions for con- ducting the field tests or laboratory analyses. These permit conditions will include design and operating param- eters (including the duration of the tests or analyses and, in the case of field tests, the horizontal and vertical dimensions of the treatment zone), monitoring procedures, post-dem- onstration clean-up activities, and any other conditions which the Director finds  may  be  necessary  under

§ 264.272(c). The Director will include

conditions in the second phase of the facility permit to attempt to meet all subpart M requirements pertaining to unit design, construction, operation, and maintenance. The Director will es- tablish these conditions in the second phase of the permit based upon the sub- stantial but incomplete or inconclusive information contained in the part B ap- plication.

- The first phase of the permit will be effective as provided in § 124.15(b) of this chapter.

- The second phase of the permit will be effective as provided in para- graph (d) of this section.

## 40 CFR Ch. I (7–1–24 Edition)

- When the owner or operator who has been issued a two-phase permit has completed the treatment demonstra- tion, he must submit to the Director a certification, signed by a person au- thorized to sign a permit application or report under § 270.11, that the field tests or laboratory analyses have been car- ried out in accordance with the condi- tions specified in phase one of the per- mit for conducting such tests or anal- yses. The owner or operator must also submit all data collected during the field tests or laboratory analyses with- in 90 days of completion of those tests or analyses unless the Director ap- proves a later date.

- If the Director determines that

the results of the field tests or labora- tory analyses meet the requirements of

§ 264.272 of this chapter, he will modify the second phase of the permit to in- corporate any requirements necessary for operation of the facility in compli- ance with part 264, subpart M, of this chapter, based upon the results of the field tests or laboratory analyses.

- This permit modification may proceed under § 270.42, or otherwise will proceed  as  a  modification  under

§ 270.41(a)(2). If such modifications are necessary, the second phase of the per- mit will become effective only after those modifications have been made.

- If no modifications of the second phase of the permit are necessary, the Director will give notice of his final de- cision to the permit applicant and to each person who submitted written comments on the phased permit or who requested notice of the final decision on the second phase of the permit. The second phase of the permit then will become  effective  as  specified  in

§ 124.15(b).

[48 FR 14228, Apr. 1, 1983, as amended at 53

FR 37939, Sept. 28, 1988]

### § 270.64  Interim permits for UIC wells.

The Director may issue a permit under this part to any Class I UIC well (see § 144.6) injecting hazardous wastes within a State in which no UIC pro- gram has been approved or promul- gated. Any such permit shall apply and insure compliance with all applicable requirements of 40 CFR part 264, sub- part R (RCRA standards for wells), and shall be for a term not to exceed two

## Environmental Protection Agency	§ 270.66

years. No such permit shall be issued after approval or promulgation of a UIC program in the State. Any permit under this section shall contain a con- dition providing that it will terminate upon final action by the Director under a UIC program to issue or deny a UIC permit for the facility.

[48 FR 14228, Apr. 1, 1983; 48 FR 30114, June 30,

1983]

### § 270.65 Research, development, and demonstration permits.

  - The Administrator may issue a research, development, and demonstra- tion permit for any hazardous waste treatment facility which proposes to utilize an innovative and experimental hazardous waste treatment technology or process for which permit standards for such experimental activity have not been promulgated under part 264 or

266. Any such permit shall include such terms and conditions as will assure protection of human health and the en- vironment. Such permits:

    - Shall provide for the construction of such facilities as necessary, and for operation of the facility for not longer than one year unless renewed as pro- vided in paragraph (d) of this section, and

    - Shall provide for the receipt and treatment by the facility of only those types and quantities of hazardous waste which the Administrator deems necessary for purposes of determining the efficacy and performance capabili- ties of the technology or process and the effects of such technology or proc- ess on human health and the environ- ment, and

    - Shall include such requirements

as the Administrator deems necessary to protect human health and the envi- ronment (including, but not limited to, requirements regarding monitoring, operation, financial responsibility, clo- sure, and remedial action), and such re- quirements as the Administrator deems necessary regarding testing and providing of information to the Admin- istrator with respect to the operation of the facility.

  - For the purpose of expediting re-

view and issuance of permits under this section, the Administrator may, con- sistent with the protection of human health and the environment, modify or

waive permit application and permit issuance requirements in parts 124 and 270 except that there may be no modi- fication or waiver of regulations re- garding financial responsibility (in- cluding insurance) or of procedures re- garding public participation.

  - The Administrator may order an immediate termination of all oper- ations at the facility at any time he determines that termination is nec- essary to protect human health and the environment.

  - Any permit issued under this sec- tion may be renewed not more than three times. Each such renewal shall be for a period of not more than 1 year.

[50 FR 28752, July 15, 1985]

### § 270.66 Permits for boilers and indus- trial furnaces burning hazardous waste.

When an owner or operator of a ce- ment kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid production furnace becomes subject to RCRA permit re- quirements after October 12, 2005 or when an owner or operator of an exist- ing cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid production fur- nace demonstrates compliance with the air emission standards and limita- tions in part 63, subpart EEE, of this chapter (i.e., by conducting a com- prehensive performance test and sub- mitting a Notification of Compliance under §§ 63.1207(j) and 63.1210(d) of this chapter documenting compliance with all applicable requirements of part 63, subpart EEE, of this chapter), the re- quirements of this section do not apply. The requirements of this section do apply, however, if the Director de- termines certain provisions are nec- essary  to  ensure  compliance  with

§§ 266.102(e)(1) and 266.102(e)(2)(iii) of this chapter if you elect to comply with § 270.235(a)(1)(i) to minimize emis- sions of toxic compounds from startup, shutdown, and malfunction events; or if you are an area source and elect to comply with the §§ 266.105, 266.106, and

266.107 standards and associated re- quirements for particulate matter, hy- drogen chloride and chlorine gas, and non-mercury metals; or the Director determines certain provisions apply, on

## § 270.66

a case-by-case basis, for purposes of in- formation collection in accordance with §§ 270.10(k), 270.10(l), 270.32(b)(2), and 270.32(b)(3).

- General. Owners and operators of new boilers and industrial furnaces (those not operating under the interim status standards of § 266.103 of this chapter) are subject to paragraphs (b) through (f) of this section. Boilers and industrial furnaces operating under the interim status standards of § 266.103 of this chapter are subject to paragraph

(g) of this section.

- Permit operating periods for new boilers and industrial furnaces. A permit for a new boiler or industrial furnace shall specify appropriate conditions for the following operating periods:

  - Pretrial burn period. For the period beginning with initial introduction of hazardous waste and ending with initi- ation of the trial burn, and only for the minimum time required to bring the boiler or industrial furnace to a point of operational readiness to conduct a trial burn, not to exceed 720 hours op- erating time when burning hazardous waste, the Director must establish in the Pretrial Burn Period of the permit conditions, including but not limited to, allowable hazardous waste feed rates and operating conditions. The Di- rector may extend the duration of this operational period once, for up to 720 additional hours, at the request of the applicant when good cause is shown. The permit may be modified to reflect the extension according to § 270.42.

    - Applicants must submit a state-

ment, with part B of the permit appli- cation, that suggests the conditions necessary to operate in compliance with the standards of §§ 266.104 through

266.107 of this chapter during this pe- riod. This statement should include, at a minimum, restrictions on the appli- cable operating requirements identified in § 266.102(e) of this chapter.

    - The Director will review this statement and any other relevant in- formation submitted with part B of the permit application and specify require- ments for this period sufficient to meet the performance standards of §§ 266.104 through 266.107 of this chapter based on his/her engineering judgment.

  - Trial burn period. For the duration of the trial burn, the Director must es-

## 40 CFR Ch. I (7–1–24 Edition)

tablish conditions in the permit for the purposes of determining feasibility of compliance with the performance standards of §§ 266.104 through 266.107 of this chapter and determining adequate operating conditions under § 266.102(e) of this chapter. Applicants must pro- pose a trial burn plan, prepared under paragraph (c) of this section, to be sub- mitted with part B of the permit appli- cation.

  - Post-trial burn period. (i) For the period immediately following comple- tion of the trial burn, and only for the minimum period sufficient to allow sample analysis, data computation, and submission of the trial burn results by the applicant, and review of the trial burn results and modification of the facility permit by the Director to reflect the trial burn results, the Direc- tor will establish the operating re- quirements most likely to ensure com- pliance with the performance standards of §§ 266.104 through 266.107 of this chap- ter based on his engineering judgment.

- Applicants must submit a state-

ment, with part B of the application, that identifies the conditions necessary to operate during this period in compli- ance with the performance standards of

§§ 266.104 through 266.107 of this chap- ter. This statement should include, at a minimum, restrictions on the oper- ating  requirements  provided  by

§ 266.102(e) of this chapter.

- The Director will review this statement and any other relevant in- formation submitted with part B of the permit application and specify require- ments for this period sufficient to meet the performance standards of §§ 266.104 through 266.107 of this chapter based on his/her engineering judgment.

  - Final permit period. For the final period of operation, the Director will develop operating requirements in con- formance with § 266.102(e) of this chap- ter that reflect conditions in the trial burn plan and are likely to ensure com- pliance with the performance standards of §§ 266.104 through 266.107 of this chap- ter. Based on the trial burn results, the Director shall make any necessary modifications to the operating require- ments to ensure compliance with the performance standards. The permit modification shall proceed according to

§ 270.42.

## Environmental Protection Agency	§ 270.66

- Requirements for trial burn plans. The trial burn plan must include the following information. The Director, in reviewing the trial burn plan, shall evaluate the sufficiency of the infor- mation provided and may require the applicant to supplement this informa- tion, if necessary, to achieve the pur- poses of this paragraph:

  - An analysis of each feed stream, including hazardous waste, other fuels, and industrial furnace feed stocks, as fired, that includes:

    - Heating value, levels of antimony, arsenic, barium, beryllium, cadmium, chromium, lead, mercury, silver, thal- lium, total chlorine/chloride, and ash;

    - Viscosity or description of the physical form of the feed stream;

  - An analysis of each hazardous waste, as fired, including:

    - An identification of any hazardous organic constituents listed in appendix VIII, part 261, of this chapter that are present in the feed stream, except that the applicant need not analyze for con- stituents listed in appendix VIII that would reasonably not be expected to be found in the hazardous waste. The con- stituents excluded from analysis must be identified and the basis for this ex- clusion explained. The waste analysis must be conducted in accordance with appropriate analytical techniques.

    - An approximate quantification of the hazardous constituents identified in the hazardous waste, within the pre- cision produced by appropriate analyt- ical methods.

    - A description of blending proce- dures, if applicable, prior to firing the hazardous waste, including a detailed analysis of the hazardous waste prior to blending, an analysis of the material with which the hazardous waste is blended, and blending ratios.

  - A detailed engineering description of the boiler or industrial furnace, in- cluding:

    - Manufacturer’s name and model number of the boiler or industrial fur- nace;

    - Type of boiler or industrial fur- nace;

    - Maximum design capacity in ap- propriate units;

    - Description of the feed system for the hazardous waste, and, as appro-

priate, other fuels and industrial fur- nace feedstocks;

    - Capacity of hazardous waste feed system;

    - Description of automatic haz- ardous waste feed cutoff system(s);

    - Description of any air pollution control system; and

    - Description of stack gas moni- toring and any pollution control moni- toring systems.

  - A detailed description of sampling and monitoring procedures including sampling and monitoring locations in the system, the equipment to be used, sampling and monitoring frequency, and planned analytical procedures for sample analysis.

  - A detailed test schedule for each hazardous waste for which the trial burn is planned, including date(s), du- ration, quantity of hazardous waste to be burned, and other factors relevant to the Director’s decision under para- graph (b)(2) of this section.

  - A detailed test protocol, includ- ing, for each hazardous waste identi- fied, the ranges of hazardous waste feed rate, and, as appropriate, the feed rates of other fuels and industrial furnace feedstocks, and any other relevant pa- rameters that may affect the ability of the boiler or industrial furnace to meet the performance standards in §§ 266.104 through 266.107 of this chapter.

  - A description of, and planned op- erating conditions for, any emission control equipment that will be used.

  - Procedures for rapidly stopping the hazardous waste feed and control- ling emissions in the event of an equip- ment malfunction.

  - Such other information as the Di- rector reasonably finds necessary to determine whether to approve the trial burn plan in light of the purposes of this paragraph and the criteria in para- graph (b)(2) of this section.

- Trial burn procedures. (1) A trial burn must be conducted to dem- onstrate conformance with the stand- ards of §§ 266.104 through 266.107 of this chapter under an approved trial burn plan.

- The Director shall approve a trial burn plan if he/she finds that:

  - The trial burn is likely to deter- mine whether the boiler or industrial furnace  can  meet  the  performance

## § 270.66

standards of §§ 266.104 through 266.107 of this chapter;

  - The trial burn itself will not present an imminent hazard to human health and the environment;

  - The trial burn will help the Di- rector to determine operating require- ments to be specified under § 266.102(e) of this chapter; and

  - The information sought in the trial burn cannot reasonably be devel- oped through other means.

- The Director must send a notice to all persons on the facility mailing list as set forth in 40 CFR 124.10(c)(1)(ix) and to the appropriate units of State and local government as set forth in 40 CFR 124.10(c)(1)(x) an- nouncing the scheduled commence- ment and completion dates for the trial burn. The applicant may not com- mence the trial burn until after the Di- rector has issued such notice.

  - This notice must be mailed within a reasonable time period before the trial burn. An additional notice is not required if the trial burn is delayed due to circumstances beyond the control of the facility or the permitting agency.

  - This notice must contain:

    - The name and telephone number of applicant’s contact person;

    - The name and telephone number of the permitting agency contact of- fice;

    - The location where the approved trial burn plan and any supporting doc- uments can be reviewed and copied; and

    - An expected time period for com- mencement and completion of the trial burn.

- The applicant must submit to the Director a certification that the trial burn has been carried out in accord- ance with the approved trial burn plan, and must submit the results of all the determinations required in paragraph

(c) of this section. This submission shall be made within 90 days of comple- tion of the trial burn, or later if ap- proved by the Director.

- All data collected during any trial burn must be submitted to the Director following completion of the trial burn.

- All submissions required by this paragraph must be certified on behalf of the applicant by the signature of a

## 40 CFR Ch. I (7–1–24 Edition)

person authorized to sign a permit ap- plication or a report under § 270.11.

- Special procedures for DRE trial burns. When a DRE trial burn is re- quired under § 266.104(a) of this chapter, the Director will specify (based on the hazardous waste analysis data and other information in the trial burn plan) as trial Principal Organic Haz- ardous Constituents (POHCs) those compounds for which destruction and removal efficiencies must be calculated during the trial burn. These trial POHCs will be specified by the Director based on information including his/her estimate of the difficulty of destroying the constituents identified in the haz- ardous waste analysis, their concentra- tions or mass in the hazardous waste feed, and, for hazardous waste con- taining or derived from wastes listed in part 261, subpart D of this chapter, the hazardous waste organic constituent(s) identified in Appendix VII of that part as the basis for listing.

- Determinations based on trial burn.

During each approved trial burn (or as soon after the burn as is practicable), the applicant must make the following determinations:

- A quantitative analysis of the lev- els of antimony, arsenic, barium, beryl- lium, cadmium, chromium, lead, mer- cury, thallium, silver, and chlorine/ chloride, in the feed streams (haz- ardous waste, other fuels, and indus- trial furnace feedstocks);

- When a DRE trial burn is required under § 266.104(a) of this chapter:

  - A quantitative analysis of the trial POHCs in the hazardous waste feed;

  - A quantitative analysis of the stack gas for the concentration and mass emissions of the trial POHCs; and

  - A computation of destruction and removal efficiency (DRE), in ac- cordance with the DRE formula speci- fied in § 266.104(a) of this chapter;

- When a trial burn for chlorinated dioxins and furans is required under

§ 266.104(e) of this chapter, a quan- titative analysis of the stack gas for the concentration and mass emission rate of the 2,3,7,8-chlorinated tetra- octa congeners of chlorinated dibenzo- p-dioxins and furans, and a computa- tion showing conformance with the emission standard;

## Environmental Protection Agency	§ 270.67

- When a trial burn for particulate matter, metals, or HCl/Cl2 is required under §§ 266.105, 266.106 (c) or (d), or

266.107 (b)(2) or (c) of this chapter, a quantitative analysis of the stack gas for the concentrations and mass emis- sions of particulate matter, metals, or hydrogen chloride (HCl) and chlorine (Cl2), and computations showing con- formance with the applicable emission performance standards;

- When a trial burn for DRE, met- als,  or  HCl/Cl2  is  required  under

§§ 266.104(a), 266.106 (c) or (d), or 266.107

(b)(2) or (c) of this chapter, a quan- titative analysis of the scrubber water (if any), ash residues, other residues, and products for the purpose of esti- mating the fate of the trial POHCs, metals, and chlorine/chloride;

- An identification of sources of fu- gitive emissions and their means of control;

- A continuous measurement of car- bon monoxide (CO), oxygen, and where required, hydrocarbons (HC), in the stack gas; and

- Such other information as the Di- rector may specify as necessary to en- sure that the trial burn will determine compliance with the performance standards in §§ 266.104 through 266.107 of this chapter and to establish the oper- ating conditions required by § 266.102(e) of this chapter as necessary to meet those performance standards.

- Interim status boilers and industrial furnaces. For the purpose of deter- mining feasibility of compliance with the performance standards of § 266.104 through 266.107 of this chapter and of determining adequate operating condi- tions under § 266.103 of this chapter, ap- plicants owning or operating existing boilers or industrial furnaces operated under the interim status standards of

§ 266.103 of this chapter must either pre- pare and submit a trial burn plan and perform a trial burn in accordance with the requirements of this section or sub- mit other information as specified in

§ 270.22(a)(6). The Director must an- nounce his or her intention to approve of the trial burn plan in accordance with the timing and distribution re- quirements of paragraph (d)(3) of this section. The contents of the notice must include: the name and telephone number of a contact person at the fa-

cility; the name and telephone number of a contact office at the permitting agency; the location where the trial burn plan and any supporting docu- ments can be reviewed and copied; and a schedule of the activities that are re- quired prior to permit issuance, includ- ing the anticipated time schedule for agency approval of the plan and the time periods during which the trial burn would be conducted. Applicants who submit a trial burn plan and re- ceive approval before submission of the part B permit application must com- plete the trial burn and submit the re- sults specified in paragraph (f) of this section with the part B permit applica- tion. If completion of this process con- flicts with the date set for submission of the part B application, the applicant must contact the Director to establish a later date for submission of the part B application or the trial burn results. If the applicant submits a trial burn plan with part B of the permit applica- tion, the trial burn must be conducted and the results submitted within a time period prior to permit issuance to be specified by the Director.

[56 FR 7239, Feb. 21, 1991; 56 FR 32692, July 17,

1991, as amended at 58 FR 46051, Aug. 31, 1993;

60 FR 63433, Dec. 11, 1995; 64 FR 53077, Sept.

30, 1999; 67 FR 77692, Dec. 19, 2002; 70 FR 34590,

June 14, 2005; 70 FR 59578, Oct. 12, 2005]

### § 270.67 RCRA standardized permits for storage and treatment units.

RCRA standardized permits are spe- cial forms of permits for TSD owners or operators that:

- Generate hazardous waste and then non-thermally treat or store the hazardous waste on-site in tanks, con- tainers, or containment buildings; or

- Receive hazardous waste gen- erated off-site by a generator under the same ownership as the receiving facil- ity, and then store or non-thermally treat the hazardous waste in con- tainers, tanks, or containment build- ings. Standardized permit facility own- ers or operators are regulated under subpart J of this part, part 124 subpart G of this chapter, and part 267 of this chapter.

[70 FR 53475, Sept. 8, 2005]

## § 270.68

### § 270.68  Remedial Action Plans (RAPs).

Remedial Action Plans (RAPs) are special forms of permits that are regu- lated under subpart H of this part.

[63 FR 65941, Nov. 30, 1998]

# Subpart G—Interim Status

### § 270.70  Qualifying for interim status.

  - Any person who owns or operates an ‘‘existing HWM facility’’ or a facil- ity in existence on the effective date of statutory or regulatory amendments under the Act that render the facility subject to the requirement to have a RCRA permit shall have interim status and shall be treated as having been issued a permit to the extent he or she has:

    - Complied with the requirements of section 3010(a) of RCRA pertaining to notification of hazardous waste ac- tivity.

[Comment: Some existing facilities may not be required to file a notification under sec- tion 3010(a) of RCRA. These facilities may qualify for interim status by meeting para- graph (a)(2) of this section.]

    - Complied with the requirements of § 270.10 governing submission of part A applications;

  - Failure to qualify for interim sta- tus. If EPA has reason to believe upon examination of a part A application that it fails to meet the requirements of § 270.13, it shall notify the owner or operator in writing of the apparent de- ficiency. Such notice shall specify the grounds for EPA’s belief that the appli- cation is deficient. The owner or oper- ator shall have 30 days from receipt to respond to such a notification and to explain or cure the alleged deficiency in his part A application. If, after such notification and opportunity for re- sponse, EPA determines that the appli- cation is deficient it may take appro- priate enforcement action.

  - Paragraph (a) of this section shall

not apply to any facility which has been previously denied a RCRA permit or if authority to operate the facility under RCRA has been previously termi- nated.

[48 FR 14228, Apr. 1, 1983, as amended at 49

FR 17718, Apr. 24, 1984; 50 FR 28753, July 15,

1985; 71 FR 40279, July 14, 2006]

## 40 CFR Ch. I (7–1–24 Edition)

### § 270.71 Operation during interim sta- tus.

- During the interim status period the facility shall not:

  - Treat, store, or dispose of haz- ardous waste not specified in part A of the permit application;

  - Employ processes not specified in part A of the permit application; or

  - Exceed the design capacities spec- ified in part A of the permit applica- tion.

- Interim status standards. During interim status, owners or operators shall comply with the interim status standards at 40 CFR part 265.

### § 270.72 Changes during interim sta- tus.

- Except as provided in paragraph (b), the owner or operator of an interim status facility may make the following changes at the facility:

  - Treatment, storage, or disposal of new hazardous wastes not previously identified in part A of the permit appli- cation (and, in the case of newly listed or identified wastes, addition of the units being used to treat, store, or dis- pose of the hazardous wastes on the ef- fective date of the listing or identifica- tion) if the owner or operator submits a revised part A permit application prior to such treatment, storage, or disposal;

  - Increases in the design capacity of

processes used at the facility if the owner or operator submits a revised part A permit application prior to such a change (along with a justification ex- plaining the need for the change) and the Director approves the changes be- cause:

    - There is a lack of available treat- ment, storage, or disposal capacity at other hazardous waste management fa- cilities, or

    - The change is necessary to com- ply with a Federal, State, or local re- quirement.

  - Changes in the processes for the treatment, storage, or disposal of haz- ardous waste or addition of processes if the owner or operator submits a re- vised part A permit application prior to such change (along with a justifica- tion explaining the need for the change) and the Director approves the change because:

## Environmental Protection Agency	§ 270.72

    - The change is necessary to pre- vent a threat to human health and the environment because of an emergency situation, or

    - The change is necessary to com- ply with a Federal, State, or local re- quirement.

  - Changes in the ownership or oper- ational control of a facility if the new owner or operator submits a revised part A permit application no later than 90 days prior to the scheduled change. When a transfer of operational control of a facility occurs, the old owner or operator shall comply with the require- ments of 40 CFR part 265, subpart H (Financial Requirements), until the new owner or operator has dem- onstrated to the Director that he is complying with the requirements of that subpart. The new owner or oper- ator must demonstrate compliance with subpart H requirements within six months of the date of the change in ownership or operational control of the facility. Upon demonstration to the Di- rector by the new owner or operator of compliance with subpart H, the Direc- tor shall notify the old owner or oper- ator in writing that he no longer needs to comply with subpart H as of the date of demonstration. All other in- terim status duties are transferred ef- fective immediately upon the date of the change in ownership or operational control of the facility.

  - Changes made in accordance with

an interim status corrective action order issued by EPA under section 3008(h) or other Federal authority, by an authorized State under comparable State authority, or by a court in a ju- dicial action brought by EPA or by an authorized State. Changes under this paragraph are limited to the treat- ment, storage, or disposal of solid waste from releases that originate within the boundary of the facility.

  - Addition of newly regulated units for the treatment, storage, or disposal of hazardous waste if the owner or op- erator submits a revised part A permit application on or before the date on which the unit becomes subject to the new requirements.

- Except as specifically allowed under this paragraph, changes listed under paragraph (a) of this section may not be made if they amount to recon-

struction of the hazardous waste man- agement facility. Reconstruction oc- curs when the capital investment in the changes to the facility exceeds 50 percent of the capital cost of a com- parable entirely new hazardous waste management facility. If all other re- quirements are met, the following changes may be made even if they amount to a reconstruction:

- Changes made solely for the pur- poses of complying with the require- ments of 40 CFR 265.193 for tanks and ancillary equipment.

- If necessary to comply with Fed- eral, State, or local requirements, changes to an existing unit, changes solely involving tanks or containers, or addition of replacement surface im- poundments that satisfy the standards of section 3004(o).

- Changes that are necessary to allow owners or operators to continue handling newly listed or identified haz- ardous wastes that have been treated, stored, or disposed of at the facility prior to the effective date of the rule establishing the new listing or identi- fication.

- Changes during closure of a facil- ity or of a unit within a facility made in accordance with an approved closure plan.

- Changes necessary to comply with an interim status corrective action order issued by EPA under section 3008(h) or other Federal authority, by an authorized State under comparable State authority, or by a court in a ju- dicial proceeding brought by EPA or an authorized State, provided that such changes are limited to the treatment, storage, or disposal of solid waste from releases that originate within the boundary of the facility.

- Changes to treat or store, in tanks, containers, or containment buildings, hazardous wastes subject to land disposal restrictions imposed by part 268 of this chapter or RCRA sec- tion 3004, provided that such changes are made solely for the purpose of com- plying with part 268 of this chapter or RCRA section 3004.

- Addition of newly regulated units under paragraph (a)(6) of this section.

## § 270.73

- Changes necessary to comply with standards under 40 CFR part 63, Sub- part EEE—National Emission Stand- ards for Hazardous Air Pollutants From Hazardous Waste Combustors.

[54 FR 9608, Mar. 7, 1989, as amended at 56 FR

7239, Feb. 21, 1991; 57 FR 37282, Aug. 18, 1992;

63 FR 33829, June 19, 1998; 71 FR 40279, July

14, 2006]

### § 270.73  Termination of interim status.

Interim status terminates when:

  - Final administrative disposition of a permit application, except an ap- plication for a remedial action plan (RAP) under subpart H of this part, is made.

  - Interim status is terminated as provided in § 270.10(e)(5).

  - For owners or operators of each land disposal facility which has been granted interim status prior to Novem- ber 8, 1984, on November 8, 1985, unless:

    - The owner or operator submits a part B application for a permit for such facility prior to that date; and

    - The owner or operator certifies that such facility is in compliance with all applicable ground-water monitoring and financial responsibility require- ments.

  - For owners or operators of each land disposal facility which is in exist- ence on the effective date of statutory or regulatory amendments under the Act that render the facility subject to the requirement to have a RCRA per- mit and which is granted interim sta- tus, twelve months after the date on which the facility first becomes subject to such permit requirement unless the owner or operator of such facility:

    - Submits a part B application for a

RCRA permit for such facility before the date 12 months after the date on which the facility first becomes subject to such permit requirement; and

    - Certifies that such facility is in compliance with all applicable ground water monitoring and financial respon- sibility requirements.

  - For owners or operators of any land disposal unit that is granted au- thority to operate under § 270.72(a) (1),

(2) or (3), on the date 12 months after the effective date of such requirement, unless the owner or operator certifies that such unit is in compliance with all applicable  ground-water  monitoring

## 40 CFR Ch. I (7–1–24 Edition)

and financial responsibility require- ments.

  - For owners and operators of each incinerator facility which has achieved interim status prior to November 8, 1984, interim status terminates on No- vember 8, 1989, unless the owner or op- erator of the facility submits a part B application for a RCRA permit for an incinerator facility by November 8, 1986.

  - For owners or operators of any fa- cility (other than a land disposal or an incinerator facility) which has achieved interim status prior to No- vember 8, 1984, interim status termi- nates on November 8, 1992, unless the owner or operator of the facility sub- mits a part B application for a RCRA permit for the facility by November 8, 1988.

[48 FR 14228, Apr. 1, 1983, as amended at 50

FR 28753, July 15, 1985; 54 FR 9609, Mar. 7,

1989; 56 FR 7239, Feb. 21, 1991; 56 FR 32692,

July 17, 1991; 63 FR 65941, Nov. 30, 1998]

# Subpart H—Remedial Action Plans (RAPs)

SOURCE: 63 FR 65941, Nov. 30, 1998, unless

otherwise noted.

### § 270.79 Why is this subpart written in a special format?

This subpart is written in a special format to make it easier to understand the regulatory requirements. Like other Environmental Protection Agen- cy (EPA) regulations, this establishes enforceable legal requirements. For this subpart, ‘‘I’’ and ‘‘you’’ refer to the owner/operator.

General Information

### § 270.80  What is a RAP?

- A RAP is a special form of RCRA permit that you, as an owner or oper- ator, may obtain, instead of a permit issued under §§ 270.3 through 270.66, to authorize you to treat, store, or dis- pose of hazardous remediation waste (as defined in § 260.10 of this chapter) at a remediation waste management site. A RAP may only be issued for the area of contamination where the remedi- ation wastes to be managed under the RAP originated, or areas in close prox- imity to the contaminated area, except

## Environmental Protection Agency	§ 270.105

as allowed in limited circumstances under § 270.230.

- The requirements in §§ 270.3 through 270.66 do not apply to RAPs unless those requirements for tradi- tional RCRA permits are specifically required under §§ 270.80 through 270.230. The definitions in § 270.2 apply to RAPs.

- Notwithstanding any other provi- sion of this part or part 124 of this chapter, any document that meets the requirements in this section con- stitutes a RCRA permit under RCRA section 3005(c).

- A RAP may be:

  - A stand-alone document that in- cludes only the information and condi- tions required by this subpart; or

  - Part (or parts) of another docu- ment that includes information and/or conditions for other activities at the remediation waste management site, in addition to the information and condi- tions required by this subpart.

- If you are treating, storing, or dis- posing of hazardous remediation wastes as part of a cleanup compelled by Fed- eral or State cleanup authorities, your RAP does not affect your obligations under those authorities in any way.

- If you receive a RAP at a facility operating under interim status, the RAP does not terminate your interim status.

### § 270.85  When do I need a RAP?

- Whenever you treat, store, or dis- pose of hazardous remediation wastes in a manner that requires a RCRA per- mit under § 270.1, you must either ob- tain:

  - A  RCRA  permit  according  to

§§ 270.3 through 270.66; or

  - A RAP according to this subpart.

- Treatment units that use combus- tion of hazardous remediation wastes at a remediation waste management site are not eligible for RAPs under this subpart.

- You may obtain a RAP for man- aging hazardous remediation waste at an already permitted RCRA facility. You must have these RAPs approved as a modification to your existing permit according  to  the  requirements  of

§ 270.41 or § 270.42 instead of the require- ments in this subpart. When you sub- mit an application for such a modifica-

tion, however, the information require- ments in § 270.42(a)(1)(i), (b)(1)(iv), and (c)(1)(iv) do not apply; instead, you must submit the information required under § 270.110. When your permit is modified the RAP becomes part of the RCRA permit. Therefore when your permit (including the RAP portion) is modified, revoked and reissued, termi- nated or when it expires, it will be modified according to the applicable requirements in §§ 270.40 through 270.42, revoked and reissued according to the applicable requirements in §§ 270.41 and 270.43, terminated according to the ap- plicable requirements in § 270.43, and expire according to the applicable re- quirements in §§ 270.50 and 270.51.

### § 270.90 Does my RAP grant me any rights or relieve me of any obliga- tions?

The provisions of § 270.4 apply to RAPs.  (NOTE:  The  provisions  of

§ 270.4(a) provide you assurance that, as long as you comply with your RAP, EPA will consider you in compliance with Subtitle C of RCRA, and will not take enforcement actions against you. However, you should be aware of four exceptions to this provision that are listed in § 270.4.)

Applying for a RAP

### § 270.95  How do I apply for a RAP?

To apply for a RAP, you must com- plete an application, sign it, and sub- mit it to the Director according to the requirements in this subpart.

### § 270.100  Who must obtain a RAP?

When a facility or remediation waste management site is owned by one per- son, but the treatment, storage or dis- posal activities are operated by an- other person, it is the operator’s duty to obtain a RAP, except that the owner must also sign the RAP application.

### § 270.105 Who must sign the applica- tion and any required reports for a RAP?

Both the owner and the operator must sign the RAP application and any required reports according to § 270.11(a), (b), and (c). In the application, both the owner and the operator must also make the certification required under

## § 270.110

§ 270.11(d)(1). However, the owner may choose the alternative certification under § 270.11(d)(2) if the operator cer- tifies under § 270.11(d)(1).

### § 270.110 What must I include in my application for a RAP?

You must include the following infor- mation in your application for a RAP:

- The name, address, and EPA iden- tification number of the remediation waste management site;

- The name, address, and telephone number of the owner and operator;

- The latitude and longitude of the site;

- The United States Geological Sur- vey (USGS) or county map showing the location of the remediation waste man- agement site;

- A scaled drawing of the remedi- ation waste management site showing:

  - The remediation waste manage- ment site boundaries;

  - Any significant physical struc- tures; and

  - The boundary of all areas on-site where remediation waste is to be treat- ed, stored or disposed;

- A specification of the hazardous remediation waste to be treated, stored or disposed of at the facility or remedi- ation waste management site. This must include information on:

  - Constituent concentrations and other properties of the hazardous reme- diation wastes that may affect how such materials should be treated and/or otherwise managed;

  - An estimate of the quantity of these wastes; and

  - A description of the processes you will use to treat, store, or dispose of this waste including technologies, han- dling systems, design and operating pa- rameters you will use to treat haz- ardous remediation wastes before dis- posing of them according to the LDR standards of part 268 of this chapter, as applicable;

- Enough information to dem- onstrate that operations that follow the provisions in your RAP application will ensure compliance with applicable requirements of parts 264, 266, and 268 of this chapter;

- Such information as may be nec- essary to enable the Regional Adminis- trator to carry out his duties under

## 40 CFR Ch. I (7–1–24 Edition)

other Federal laws as is required for traditional  RCRA  permits  under

§ 270.14(b)(20);

- Any other information the Direc- tor decides is necessary for dem- onstrating compliance with this sub- part or for determining any additional RAP conditions that are necessary to protect human health and the environ- ment.

### § 270.115 What if I want to keep this information confidential?

Part 2 (Public Information) of this chapter allows you to claim as con- fidential any or all of the information you submit to EPA under this subpart. You must assert any such claim at the time that you submit your RAP appli- cation or other submissions by stamp- ing the words ‘‘confidential business information’’ on each page containing such information. If you do assert a claim at the time you submit the infor- mation, EPA will treat the information according to the procedures in part 2 of this chapter. If you do not assert a claim at the time you submit the infor- mation, EPA may make the informa- tion available to the public without further notice to you. EPA will deny any requests for confidentiality of your name and/or address.

### § 270.120 To whom must I submit my RAP application?

You must submit your application for a RAP to the Director for approval.

### § 270.125 If I submit my RAP applica- tion as part of another document, what must I do?

If you submit your application for a RAP as a part of another document, you must clearly identify the compo- nents of that document that constitute your RAP application.

Getting a RAP Approved

### § 270.130 What is the process for ap- proving or denying my application for a RAP?

- If the Director tentatively finds that your RAP application includes all of the information required by § 270.110 and that your proposed remediation waste management activities meet the regulatory standards, the Director will make a tentative decision to approve

## Environmental Protection Agency	§ 270.145

your RAP application. The Director will then prepare a draft RAP and pro- vide an opportunity for public com- ment before making a final decision on your RAP application, according to this subpart.

- If the Director tentatively finds that your RAP application does not in- clude all of the information required by

§ 270.110 or that your proposed remedi- ation waste management activities do not meet the regulatory standards, the Director may request additional infor- mation from you or ask you to correct deficiencies in your application. If you fail or refuse to provide any additional information the Director requests, or to correct any deficiencies in your RAP application, the Director may make a tentative decision to deny your RAP application. After making this ten- tative decision, the Director will pre- pare a notice of intent to deny your RAP application (‘‘notice of intent to deny’’) and provide an opportunity for public comment before making a final decision on your RAP application, ac- cording to the requirements in this Subpart. The Director may deny the RAP application either in its entirety or in part.

### § 270.135 What must the Director in- clude in a draft RAP?

If the Director prepares a draft RAP, it must include the:

- Information  required  under

§ 270.110(a) through (f);

- The following terms and condi- tions:

  - Terms and conditions necessary to ensure that the operating requirements specified in your RAP comply with ap- plicable requirements of parts 264, 266, and 268 of this chapter (including any recordkeeping and reporting require- ments). In satisfying this provision, the Director may incorporate, ex- pressly or by reference, applicable re- quirements of parts 264, 266, and 268 of this chapter into the RAP or establish site-specific conditions as required or allowed by parts 264, 266, and 268 of this chapter;

  - Terms and conditions in § 270.30;

  - Terms and conditions for modi- fying, revoking and reissuing, and ter- minating your RAP, as provided in

§ 270.170; and

  - Any additional terms or condi- tions that the Director determines are necessary to protect human health and the environment, including any terms and conditions necessary to respond to spills and leaks during use of any units permitted under the RAP; and

- If the draft RAP is part of another document, as described in § 270.80(d)(2), the Director must clearly identify the components of that document that con- stitute the draft RAP.

### § 270.140 What else must the Director prepare in addition to the draft RAP or notice of intent to deny?

Once the Director has prepared the draft RAP or notice of intent to deny, he must then:

- Prepare a statement of basis that briefly describes the derivation of the conditions of the draft RAP and the reasons for them, or the rationale for the notice of intent to deny;

- Compile an administrative record, including:

  - The RAP application, and any supporting data furnished by the appli- cant;

  - The draft RAP or notice of intent to deny;

  - The statement of basis and all documents cited therein (material readily available at the issuing Re- gional office or published material that is generally available need not be phys- ically included with the rest of the record, as long as it is specifically re- ferred to in the statement of basis); and

  - Any other documents that support the decision to approve or deny the RAP; and

- Make information contained in the administrative record available for review by the public upon request.

### § 270.145 What are the procedures for public comment on the draft RAP or notice of intent to deny?

- The Director must:

  - Send notice to you of his inten- tion to approve or deny your RAP ap- plication, and send you a copy of the statement of basis;

  - Publish a notice of his intention to approve or deny your RAP applica- tion in a major local newspaper of gen- eral circulation;

## § 270.150

  - Broadcast his intention to ap- prove or deny your RAP application over a local radio station; and

  - Send a notice of his intention to approve or deny your RAP application to each unit of local government hav- ing jurisdiction over the area in which your site is located, and to each State agency having any authority under State law with respect to any construc- tion or operations at the site.

- The notice required by paragraph

(a) of this section must provide an op- portunity for the public to submit written comments on the draft RAP or notice of intent to deny within at least 45 days.

- The notice required by paragraph

(a) of this section must include:

- The name and address of the office processing the RAP application;

- The name and address of the RAP applicant, and if different, the remedi- ation waste management site or activ- ity the RAP will regulate;

- A brief description of the activity the RAP will regulate;

- The name, address and telephone number of a person from whom inter- ested persons may obtain further infor- mation, including copies of the draft RAP or notice of intent to deny, state- ment of basis, and the RAP applica- tion;

- A brief description of the com- ment procedures in this section, and any other procedures by which the pub- lic may participate in the RAP deci- sion;

- If a hearing is scheduled, the date, time, location and purpose of the hear- ing;

- If a hearing is not scheduled, a statement of procedures to request a hearing;

- The location of the administrative record, and times when it will be open for public inspection; and

- Any additional information the Director considers necessary or proper.

- If, within the comment period, the Director receives written notice of opposition to his intention to approve or deny your RAP application and a re- quest for a hearing, the Director must hold an informal public hearing to dis- cuss issues relating to the approval or denial of your RAP application. The Director may also determine on his

## 40 CFR Ch. I (7–1–24 Edition)

own initiative that an informal hearing is appropriate. The hearing must in- clude an opportunity for any person to present written or oral comments. Whenever possible, the Director must schedule this hearing at a location con- venient to the nearest population cen- ter to the remediation waste manage- ment site and give notice according to the requirements in paragraph (a) of this section. This notice must, at a minimum, include the information re- quired by paragraph (c) of this section and:

- Reference to the date of any pre- vious public notices relating to the RAP application;

- The date, time and place of the hearing; and

- A brief description of the nature and purpose of the hearing, including the applicable rules and procedures.

### § 270.150 How will the Director make a final decision on my RAP applica- tion?

  - The Director must consider and respond to any significant comments raised during the public comment pe- riod, or during any hearing on the draft RAP or notice of intent to deny, and revise your draft RAP based on those comments, as appropriate.

  - If the Director determines that your RAP includes the information and terms  and  conditions  required  in

§ 270.135, then he will issue a final deci- sion approving your RAP and, in writ- ing, notify you and all commenters on your draft RAP that your RAP applica- tion has been approved.

  - If the Director determines that your RAP does not include the infor- mation required in § 270.135, then he will issue a final decision denying your RAP and, in writing, notify you and all commenters on your draft RAP that your RAP application has been denied.

  - If the Director’s final decision is that the tentative decision to deny the RAP application was incorrect, he will withdraw the notice of intent to deny and proceed to prepare a draft RAP, ac- cording to the requirements in this subpart.

  - When the Director issues his final RAP decision, he must refer to the pro- cedures for appealing the decision under § 270.155.

## Environmental Protection Agency	§ 270.170

  - Before issuing the final RAP deci- sion, the Director must compile an ad- ministrative record. Material readily available at the issuing Regional office or published materials which are gen- erally available and which are included in the administrative record need not be physically included with the rest of the record as long as it is specifically referred to in the statement of basis or the response to comments. The admin- istrative record for the final RAP must include information in the administra- tive record for the draft RAP (see

§ 270.140(b)) and:

- All comments received during the public comment period;

- Tapes or transcripts of any hear- ings;

- Any written materials submitted at these hearings;

- The responses to comments;

- Any new material placed in the record since the draft RAP was issued;

(6) Any other documents supporting the RAP; and (7) A copy of the final RAP.

  - The Director must make informa- tion contained in the administrative record available for review by the pub- lic upon request.

### § 270.155 May the decision to approve or deny my RAP application be ad- ministratively appealed?

- Any commenter on the draft RAP or notice of intent to deny, or any par- ticipant in any public hearing(s) on the draft RAP, may appeal the Director’s decision to approve or deny your RAP application to EPA’s Environmental Appeals Board under § 124.19 of this chapter. Any person who did not file comments, or did not participate in any public hearing(s) on the draft RAP, may petition for administrative review only to the extent of the changes from the draft to the final RAP decision. Ap- peals of RAPs may be made to the same extent as for final permit deci- sions under § 124.15 of this chapter (or a decision under § 270.29 to deny a permit for the active life of a RCRA hazardous waste management facility or unit).

- This appeal is a prerequisite to

seeking judicial review of these EPA actions.

[63 FR 65941, Nov. 30, 1998, as amended at 78

FR 5288, Jan. 25, 2013]

### § 270.160 When does my RAP become effective?

Your RAP becomes effective 30 days after the Director notifies you and all commenters that your RAP is approved unless:

- The Director specifies a later ef- fective date in his decision;

- You or another person has ap- pealed your RAP under § 270.155 (if your RAP is appealed, and the request for review is granted under § 270.155, condi- tions of your RAP are stayed according to § 124.16 of this chapter); or

- No commenters requested a change in the draft RAP, in which case the RAP becomes effective imme- diately when it is issued.

### § 270.165 When may I begin physical construction of new units permitted under the RAP?

You must not begin physical con- struction of new units permitted under the RAP for treating, storing or dis- posing of hazardous remediation waste before receiving a finally effective RAP.

How May My RAP Be Modified, Re- voked and Reissued, or Termi- nated?

### § 270.170 After my RAP is issued, how may it be modified, revoked and re- issued, or terminated?

In your RAP, the Director must specify, either directly or by reference, procedures for future modifications, revocations and reissuance, or termi- nations of your RAP. These procedures must provide adequate opportunities for public review and comment on any modification,	revocation	and reissuance, or termination that would significantly change your management of your remediation waste, or that oth- erwise merits public review and com- ment. If your RAP has been incor- porated into a traditional RCRA per- mit, as allowed under § 270.85(c), then the RAP will be modified according to the applicable requirements in §§ 270.40 through 270.42, revoked and reissued according to the applicable require- ments in §§ 270.41 and 270.43, or termi- nated according to the applicable re- quirements of § 270.43.

## § 270.175

### § 270.175 For what reasons may the Di- rector choose to modify my final RAP?

  - The Director may modify your final RAP on his own initiative only if one or more of the following reasons listed in this section exist(s). If one or more of these reasons do not exist, then the Director will not modify your final RAP, except at your request. Rea- sons for modification are:

    - You made material and substan- tial alterations or additions to the ac- tivity that justify applying different conditions;

    - The Director finds new informa- tion that was not available at the time of RAP issuance and would have justi- fied applying different RAP conditions at the time of issuance;

    - The standards or regulations on which the RAP was based have changed because of new or amended statutes, standards or regulations, or by judicial decision after the RAP was issued;

    - If your RAP includes any sched- ules of compliance, the Director may find reasons to modify your compliance schedule, such as an act of God, strike, flood, or materials shortage or other events over which you as the owner/op- erator have little or no control and for which there is no reasonably available remedy;

    - You are not in compliance with conditions of your RAP;

    - You failed in the application or during the RAP issuance process to dis- close fully all relevant facts, or you misrepresented any relevant facts at the time;

    - The Director has determined that the activity authorized by your RAP endangers human health or the envi- ronment and can only be remedied by modifying; or

    - You have notified the Director (as required in the RAP under § 270.30(l)(3)) of a proposed transfer of a RAP.

  - Notwithstanding any other provi- sion in this section, when the Director reviews a RAP for a land disposal facil- ity under § 270.195, he may modify the permit as necessary to assure that the facility continues to comply with the currently applicable requirements in parts 124, 260 through 266 and 270 of this chapter.

## 40 CFR Ch. I (7–1–24 Edition)

  - The Director will not reevaluate the suitability of the facility location at the time of RAP modification unless new information or standards indicate that a threat to human health or the environment exists that was unknown when the RAP was issued.

### § 270.180 For what reasons may the Di- rector choose to revoke and reissue my final RAP?

- The Director may revoke and re- issue your final RAP on his own initia- tive only if one or more reasons for revocation and reissuance exist(s). If one or more reasons do not exist, then the Director will not modify or revoke and reissue your final RAP, except at your request. Reasons for modification or revocation and reissuance are the same as the reasons listed for RAP modifications in § 270.175(a)(5) through

(8) if the Director determines that rev- ocation and reissuance of your RAP is appropriate.

- The Director will not reevaluate the suitability of the facility location at the time of RAP revocation and reissuance, unless new information or standards indicate that a threat to human health or the environment ex- ists that was unknown when the RAP was issued.

### § 270.185 For what reasons may the Di- rector choose to terminate my final RAP, or deny my renewal applica- tion?

The Director may terminate your final RAP on his own initiative, or deny your renewal application for the same reasons as those listed for RAP modifications in § 270.175(a)(5) through

(7) if the Director determines that ter- mination of your RAP or denial of your RAP renewal application is appro- priate.

### § 270.190 May the decision to approve or deny a modification, revocation and reissuance, or termination of my RAP be administratively ap- pealed?

- Any commenter on the modifica- tion, revocation and reissuance or ter- mination, or any person who partici- pated in any hearing(s) on these ac- tions, may appeal the Director’s deci- sion to approve a modification, revoca- tion and reissuance, or termination of

## Environmental Protection Agency	§ 270.215

your RAP, according to § 270.155. Any person who did not file comments or did not participate in any public hear- ing(s) on the modification, revocation and reissuance or termination, may pe- tition for administrative review only of the changes from the draft to the final RAP decision.

- Any commenter on the modifica- tion, revocation and reissuance or ter- mination, or any person who partici- pated in any hearing(s) on these ac- tions, may informally appeal the Di- rector’s decision to deny a request for modification,	revocation	and reissuance, or termination to EPA’s Environmental Appeals Board. Any person who did not file comments, or did not participate in any public hear- ing(s) on the modification, revocation and reissuance or termination may pe- tition for administrative review only of the changes from the draft to the final RAP decision.

- The process for informal appeals of RAPs is as follows:

  - The person appealing the decision must send a letter to the Environ- mental Appeals Board. The letter must briefly set forth the relevant facts.

  - The Environmental Appeals Board has 60 days after receiving the letter to act on it.

  - If the Environmental Appeals Board does not take action on the let- ter within 60 days after receiving it, the appeal shall be considered denied.

- This informal appeal is a pre- requisite to seeking judicial review of these EPA actions.

### § 270.195  When will my RAP expire?

RAPs must be issued for a fixed term, not to exceed 10 years, although they may be renewed upon approval by the Director in fixed increments of no more than ten years. In addition, the Director must review any RAP for haz- ardous waste land disposal five years after the date of issuance or reissuance and you or the Director must follow the requirements for modifying your RAP as necessary to assure that you continue to comply with currently ap- plicable requirements in RCRA sec- tions 3004 and 3005.

### § 270.200 How may I renew my RAP if it is expiring?

If you wish to renew your expiring RAP, you must follow the process for application for and issuance of RAPs in this subpart.

### § 270.205 What happens if I have ap- plied correctly for a RAP renewal but have not received approval by the time my old RAP expires?

If you have submitted a timely and complete application for a RAP re- newal, but the Director, through no fault of yours, has not issued a new RAP with an effective date on or before the expiration date of your previous RAP, your previous RAP conditions continue in force until the effective date of your new RAP or RAP denial.

Operating Under Your RAP

### § 270.210 What records must I main- tain concerning my RAP?

You are required to keep records of:

- All data used to complete RAP applications and any supplemental in- formation that you submit for a period of at least 3 years from the date the ap- plication is signed; and

- Any operating and/or other records the Director requires you to maintain as a condition of your RAP.

### § 270.215 How are time periods in the requirements in this subpart and my RAP computed?

- Any time period scheduled to begin on the occurrence of an act or event must begin on the day after the act or event. (For example, if your RAP specifies that you must close a staging pile within 180 days after the operating term for that staging pile ex- pires, and the operating term expires on June 1, then June 2 counts as day one of your 180 days, and you would have to complete closure by November 28.)

- Any time period scheduled to begin before the occurrence of an act or event must be computed so that the pe- riod ends on the day before the act or event. (For example, if you are trans- ferring ownership or operational con- trol of your site, and wish to transfer your RAP, the new owner or operator must submit a revised RAP application

## § 270.220

no later than 90 days before the sched- uled change. Therefore, if you plan to change ownership on January 1, the new owner/operator must submit the revised RAP application no later than October 3, so that the 90th day would be December 31.)

- If the final day of any time period falls on a weekend or legal holiday, the time period must be extended to the next working day. (For example, if you wish to appeal the Director’s decision to modify your RAP, then you must pe- tition the Environmental Appeals Board within 30 days after the Director has issued the final RAP decision. If the 30th day falls on Sunday, then you may submit your appeal by the Mon- day after. If the 30th day falls on July 4th, then you may submit your appeal by July 5th.)

- Whenever a party or interested

person has the right to or is required to act within a prescribed period after the service of notice or other paper upon him by mail, 3 days must be added to the prescribed term. (For example, if you wish to appeal the Director’s deci- sion to modify your RAP, then you must petition the Environmental Ap- peals Board within 30 days after the Di- rector has issued the final RAP deci- sion. However, if the Director notifies you of his decision by mail, then you may have 33 days to petition the Envi- ronmental Appeals Board.)

### § 270.220 How may I transfer my RAP to a new owner or operator?

- If you wish to transfer your RAP to a new owner or operator, you must follow the requirements specified in your RAP for RAP modification to identify the new owner or operator, and incorporate any other necessary requirements. These modifications do not constitute ‘‘significant’’ modifica- tions for purposes of § 270.170. The new owner/operator must submit a revised RAP application no later than 90 days before the scheduled change along with a written agreement containing a spe- cific date for transfer of RAP responsi- bility between you and the new permit- tees.

- When a transfer of ownership or

operational control occurs, you as the old owner or operator must comply with the applicable requirements in

## 40 CFR Ch. I (7–1–24 Edition)

part 264, subpart H (Financial Require- ments), of this chapter until the new owner or operator has demonstrated that he is complying with the require- ments in that subpart. The new owner or operator must demonstrate compli- ance with part 264, subpart H, of this chapter within six months of the date of the change in ownership or oper- ational control of the facility or reme- diation waste management site. When the new owner/operator demonstrates compliance with part 264, subpart H, of this chapter to the Director, the Direc- tor will notify you that you no longer need to comply with part 264, subpart H, of this chapter as of the date of dem- onstration.

### § 270.225 What must the State or EPA Region report about noncompliance with RAPs?

The State or EPA Region must re- port noncompliance with RAPs accord- ing to the provisions of § 270.5.

Obtaining a RAP for an Off-Site Location

### § 270.230 May I perform remediation waste management activities under a RAP at a location removed from the area where the remediation wastes originated?

- You may request a RAP for reme- diation waste management activities at a location removed from the area where the remediation wastes origi- nated if you believe such a location would be more protective than the con- taminated area or areas in close prox- imity.

- If the Director determines that an alternative location, removed from the area where the remediation waste originated, is more protective than managing remediation waste at the area of contamination or areas in close proximity, then the Director may ap- prove a RAP for this alternative loca- tion.

- You must request the RAP, and the Director will approve or deny the RAP, according to the procedures and requirements in this subpart.

- A RAP for an alternative location must also meet the following require- ments, which the Director must in- clude in the RAP for such locations:

## Environmental Protection Agency	§ 270.235

  - The RAP for the alternative loca- tion must be issued to the person re- sponsible for the cleanup from which the remediation wastes originated;

  - The RAP is subject to the ex- panded public participation require- ments in §§ 124.31, 124.32, and 124.33 of this chapter;

  - The RAP is subject to the public notice requirements in § 124.10(c) of this chapter;

  - The site permitted in the RAP may not be located within 61 meters or

200 feet of a fault which has had dis- placement in the Holocene time (you must demonstrate compliance with this standard through the require- ments in § 270.14(b)(11)) (See definitions of terms in § 264.18(a) of this chapter);

NOTE TO PARAGRAPH (d)(4): Sites located in political jurisdictions other than those listed in Appendix VI of part 264 of this chapter, are assumed to be in compliance with this requirement.

- These alternative locations are remediation waste management sites, and retain the following benefits of re- mediation waste management sites:

  - Exclusion from facility-wide cor- rective action under § 264.101 of this chapter; and

  - Application of § 264.1(j) of this chapter in lieu of part 264, subparts B, C, and D, of this chapter.

# Subpart I—Integration with Max- imum Achievable Control Technology (MACT) Stand- ards

### § 270.235 Options for incinerators, ce- ment kilns, lightweight aggregate kilns, solid fuel boilers, liquid fuel boilers and hydrochloric acid pro- duction furnaces to minimize emis- sions from startup, shutdown, and malfunction events.

(a) Facilities with existing permits—(1) Revisions to permit conditions after docu- menting compliance with MACT. The owner or operator of a RCRA-permitted incinerator, cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid pro- duction furnace may request that the Director address permit conditions that minimize emissions from startup, shutdown, and malfunction events under any of the following options

when requesting removal of permit conditions that are no longer applica- ble according to §§ 264.340(b) and 266.100(b) of this chapter:

- Retain relevant permit conditions.

Under this option, the Director will:

  - Retain permit conditions that ad- dress releases during startup, shut- down, and malfunction events, includ- ing releases from emergency safety vents, as these events are defined in the facility’s startup, shutdown, and malfunction  plan  required  under

§ 63.1206(c)(2) of this chapter; and

  - Limit applicability of those per- mit conditions only to when the facil- ity is operating under its startup, shut- down, and malfunction plan.

- Revise relevant permit conditions.

- Under this option, the Director will:

  - Identify a subset of relevant exist- ing permit requirements, or develop al- ternative permit requirements, that ensure emissions of toxic compounds are minimized from startup, shutdown, and malfunction events, including re- leases from emergency safety vents, based on review of information includ- ing the source’s startup, shutdown, and malfunction plan, design, and oper- ating history.

  - Retain or add these permit re- quirements to the permit to apply only when the facility is operating under its startup, shutdown, and malfunction plan.

- Changes that may significantly in- crease emissions. (1) You must notify the Director in writing of changes to the startup, shutdown, and malfunction plan or changes to the design of the source that may significantly increase emissions of toxic compounds from startup, shutdown, or malfunction events, including releases from emer- gency safety vents. You must notify the Director of such changes within five days of making such changes. You must identify in the notification rec- ommended revisions to permit condi- tions necessary as a result of the changes to ensure that emissions of toxic compounds are minimized during these events.

- The Director may revise permit

conditions as a result of these changes to ensure that emissions of toxic com- pounds are minimized during startup,

## § 270.235

shutdown, or malfunction events, in- cluding releases from emergency safety vents either:

  - Upon permit renewal, or, if war- ranted;

  - By modifying the permit under

§§ 270.41(a) or 270.42.

  - Remove permit conditions. Under this option:

    - The owner or operator must docu- ment that the startup, shutdown, and malfunction  plan  required  under

§ 63.1206(c)(2) of this chapter has been approved by the Administrator under

§ 63.1206(c)(2)(ii)(B) of this chapter; and

    - The Director will remove permit conditions that are no longer applica- ble according to §§ 264.340(b) and 266.100(b) of this chapter.

- Addressing permit conditions upon permit reissuance. The owner or oper- ator of an incinerator, cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydro- chloric acid production furnace that has conducted a comprehensive per- formance test and submitted to the Ad- ministrator a Notification of Compli- ance documenting compliance with the standards of part 63, subpart EEE, of this chapter may request in the appli- cation to reissue the permit for the combustion unit that the Director con- trol emissions from startup, shutdown, and malfunction events under any of the following options:

  - RCRA option A. (A) Under this op-

tion, the Director will:

    - Include, in the permit, conditions that   ensure   compliance   with

§§ 264.345(a)   and   264.345(c)   or

§§ 266.102(e)(1) and 266.102(e)(2)(iii) of this chapter to minimize emissions of toxic compounds from startup, shut- down, and malfunction events, includ- ing releases from emergency safety vents; and

    - Specify that these permit require- ments apply only when the facility is operating under its startup, shutdown, and malfunction plan.; or

  - RCRA option B. (A) Under this op- tion, the Director will:

    - Include, in the permit conditions, that ensure emissions of toxic com- pounds are minimized from startup, shutdown, and malfunction events, in- cluding releases from emergency safety vents, based on review of information

## 40 CFR Ch. I (7–1–24 Edition)

including the source’s startup, shut- down, and malfunction plan, design, and operating history; and

    - Specify that these permit require- ments apply only when the facility is operating under its startup, shutdown, and malfunction plan.

(B) Changes that may significantly in- crease emissions. (1) You must notify the Director in writing of changes to the startup, shutdown, and malfunction plan or changes to the design of the source that may significantly increase emissions of toxic compounds from startup, shutdown, or malfunction events, including releases from emer- gency safety vents. You must notify the Director of such changes within five days of making such changes. You must identify in the notification rec- ommended revisions to permit condi- tions necessary as a result of the changes to ensure that emissions of toxic compounds are minimized during these events.

- The Director may revise permit

conditions as a result of these changes to ensure that emissions of toxic com- pounds are minimized during startup, shutdown, or malfunction events, in- cluding releases from emergency safety vents either:

  - Upon permit renewal, or, if war- ranted;

  - By modifying the permit under

§§ 270.41(a) or 270.42; or

  - CAA option. Under this option:

    - The owner or operator must docu- ment that the startup, shutdown, and malfunction  plan  required  under

§ 63.1206(c)(2) of this chapter has been approved by the Administrator under

§ 63.1206(c)(2)(ii)(B) of this chapter; and

    - The Director will omit from the permit conditions that are not applica- ble under §§ 264.340(b) and 266.100(b) of this chapter.

- Interim status facilities—(1) Interim status operations. In compliance with

§§ 265.340 and 266.100(b), the owner or op- erator of an incinerator, cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydro- chloric acid production furnace that is operating under the interim status standards of part 265 or 266 of this chapter may control emissions of toxic compounds during startup, shutdown, and malfunction events under either of

## Environmental Protection Agency	§ 270.260

the following options after conducting a comprehensive performance test and submitting to the Administrator a No- tification of Compliance documenting compliance with the standards of part 63, subpart EEE, of this chapter.

  - RCRA option. Under this option, the owner or operator continues to comply with the interim status emis- sion standards and operating require- ments of part 265 or 266 of this chapter relevant to control of emissions from startup, shutdown, and malfunction events. Those standards and require- ments apply only during startup, shut- down, and malfunction events; or

  - CAA option. Under this option, the owner or operator is exempt from the interim status standards of part 265 or 266 of this chapter relevant to con- trol of emissions of toxic compounds during startup, shutdown, and mal- function events upon submission of written notification and documenta- tion to the Director that the startup, shutdown, and malfunction plan re- quired under § 63.1206(c)(2) of this chap- ter has been approved by the Adminis- trator under § 63.1206(c)(2)(ii)(B) of this chapter.

(2) Operations under a subsequent RCRA permit. When an owner or oper- ator of an incinerator, cement kiln, lightweight aggregate kiln, solid fuel boiler, liquid fuel boiler, or hydro- chloric acid production furnace that is operating under the interim status standards of parts 265 or 266 of this chapter submits a RCRA permit appli- cation, the owner or operator may re- quest that the Director control emis- sions from startup, shutdown, and mal- function events under any of the op- tions provided by paragraphs (a)(2)(i), (a)(2)(ii), or (a)(2)(iii) of this section.

- New units. Hazardous waste incin- erator, cement kiln, lightweight aggre- gate kiln, solid fuel boiler, liquid fuel boiler, or hydrochloric acid production furnace units that become subject to RCRA permit requirements after Octo- ber 12, 2005 must control emissions of toxic compounds during startup, shut- down, and malfunction events under ei- ther of the following options:

- Comply with the requirements specified in § 63.1206(c)(2) of this chap- ter; or

- Request to include in the RCRA permit, conditions that ensure emis- sions of toxic compounds are mini- mized from startup, shutdown, and malfunction events, including releases from emergency safety vents, based on review of information including the source’s startup, shutdown, and mal- function plan and design. The director will specify that these permit condi- tions apply only when the facility is operating under its startup, shutdown, and malfunction plan.

[67 FR 6817, Feb. 13, 2002, as amended at 70

FR 59578, Oct. 12, 2005]

# Subpart J—RCRA Standardized Permits for Storage and Treat- ment Units

SOURCE: 70 FR 53475, Sept. 8, 2005, unless

otherwise noted.

General Information About Standardized Permits

### § 270.250 What is a RCRA standardized permit?

A RCRA standardized permit (RCRA) is a special type of permit that author- izes you to manage hazardous waste. It is issued under 40 CFR part 124, subpart G and subpart J of this part.

### § 270.255 Who is eligible for a stand- ardized permit?

- You may be eligible for a stand- ardized permit if:

  - You generate hazardous waste and then store or non-thermally treat the hazardous waste on-site in containers, tanks, or containment buildings; or

  - You receive hazardous waste gen- erated off-site by a generator under the same ownership as the receiving facil- ity, and then store or non-thermally treat the hazardous waste in con- tainers, tanks, or containment build- ings.

  - We will inform you of your eligi- bility when we make a decision on your permit application.

- [Reserved]

### § 270.260 What requirements of part 270 apply to a standardized permit?

The following subparts and sections of this part 270 apply to a standardized permit:

## § 270.270

- Subpart A—General Information: All sections.

- Subpart B—Permit Application:

§§ 270.10, 270.11, 270.12, 270.13 and 270.29.

- Subpart C—Permit Conditions: All sections.

- Subpart D—Changes to Permit:

§§ 270.40, 270.41, and 270.43.

- Subpart E—Expiration and Con- tinuation of Permits: All sections.

- Subpart F—Special Forms of Per- mits: § 270.67.

- Subpart G—Interim Status: All sections.

- Subpart	H—Remedial	Action Plans: Does not apply.

- Subpart J—Standardized Permits: All sections.

Applying for a Standardized Permit

### § 270.270 How do I apply for a stand- ardized permit?

You apply for a standardized permit by following the procedures in 40 CFR part 124, subpart G and this subpart.

### § 270.275 What information must I sub- mit to the permitting agency to sup- port my standardized permit appli- cation?

The information in paragraphs (a) through (j) of this section will be the basis of your standardized permit ap- plication. You must submit it to the Director when you submit your Notice of Intent under 40 CFR 124.202(b) re- questing coverage under a RCRA stand- ardized permit:

- The Part A information described

in § 270.13.

- A meeting summary and other materials required by 40 CFR 124.31.

- Documentation of compliance with the location standards of 40 CFR

267.18 and § 270.14(b)(11).

- Information that allows the Di- rector to carry out our obligations under other Federal laws required in

§ 270.3.

- Solid waste management unit in- formation required by § 270.14(d).

- A certification meeting the re- quirements of § 270.280, and an audit of the facility’s compliance status with 40 CFR part 267 as required by § 270.280.

- A closure plan prepared in accord- ance with part 267, subpart G.

- The most recent closure cost esti- mate for your facility prepared under

## 40 CFR Ch. I (7–1–24 Edition)

§ 267.142 and a copy of the documenta- tion required to demonstrate financial assurance under § 267.143. For a new fa- cility, you may gather the required documentation 60 days before the ini- tial receipt of hazardous wastes.

- If you manage wastes generated off-site, the waste analysis plan.

- If you manage waste generated from off-site, documentation showing that the waste generator and the off- site facility are under the same owner- ship.

### § 270.280 What are the certification re- quirements?

You must submit a signed certifi- cation based on your audit of your fa- cility’s compliance with 40 CFR part 267.

- Your certification must read: I certify under penalty of law that:

  - I have personally examined and am familiar with the report containing the results of an audit conducted of my facility’s compliance status with 40 CFR part 267, which supports this cer- tification. Based on my inquiry of those individuals immediately respon- sible for conducting the audit and pre- paring the report, I believe that my (include paragraph (a)(1)(i) and (ii) this section, whichever applies):

    - My existing facility complies with all applicable requirements of 40 CFR part 267 and will continue to comply until the expiration of the permit; or

    - My facility has been designed, and will be constructed and operated to comply with all applicable require- ments of 40 CFR part 267, and will con- tinue to comply until expiration of the permit.

  - I will make all information that I am required to maintain at my facility by §§ 270.290 through 277.315 readily available for review by the permitting agency and the public; and,

  - I will continue to make all infor- mation required by §§ 270.290 through

277.315 available until the permit ex- pires. I am aware that there are signifi- cant penalties for submitting false in- formation, including the possibility of fine and imprisonment for knowing violation.

- You must sign this certification following   the   requirements   of

§ 270.11(a)(1) through (3).

## Environmental Protection Agency	§ 270.290

- This certification must be based upon an audit that you conduct of your facility’s compliance status with 40 CFR part 267. A written audit report, signed and certified as accurate by the auditor, must be submitted to the Di- rector with the 40 CFR 124.202(b) Notice of Intent.

Information That Must Be Kept at

Your Facility

### § 270.290 What general types of infor- mation must I keep at my facility?

You must keep the following infor- mation at your facility:

- A general description of the facil- ity.

- Chemical and physical analyses of the hazardous waste and hazardous de- bris handled at the facility. At a min- imum, these analyses must contain all the information you must know to treat or store the wastes properly under the requirements of 40 CFR part 267.

- A copy of the waste analysis plan required by 40 CFR 267.13(b).

- A description of the security pro- cedures and equipment required by 40 CFR 267.14.

- A copy of the general inspection schedule required by 40 CFR 267.15(b). You must include in the inspection schedule applicable requirements of 40 CFR 267.174, 267.193, 267.195, 264.1033,

264.1052, 264.1053, 264.1058, and 264.1088.

- A justification of any modifica- tion of the preparedness and prevention requirements of 40 CFR part 267, sub- part C (§§ 267.30 to 267.35).

- A copy of the contingency plan re- quired by 40 CFR part 267, subpart D.

- A description of procedures, structures, or equipment used at the facility to:

  - Prevent hazards in unloading op- erations (for example, use ramps, spe- cial forklifts),

  - Prevent runoff from hazardous waste handling areas to other areas of the facility or environment, or to pre- vent flooding (for example, with berms, dikes, trenches),

  - Prevent contamination of water supplies,

  - Mitigate effects of equipment fail- ure and power outages,

  - Prevent undue exposure of per- sonnel to hazardous waste (for exam- ple, requiring protective clothing), and

  - Prevent releases to atmosphere,

- A description of precautions to prevent accidental ignition or reaction of ignitable, reactive, or incompatible wastes as required by 40 CFR 267.17.

- Traffic pattern, estimated volume (number, types of vehicles) and control (for example, show turns across traffic lanes, and stacking lanes; describe ac- cess road surfacing and load bearing capacity; show traffic control signals).

- [Reserved]

- An outline of both the introduc- tory and continuing training programs you will use to prepare employees to operate or maintain your facility safe- ly as required by 40 CFR 267.16. A brief description of how training will be de- signed to meet actual job tasks under 40 CFR 267.16(a)(3) requirements.

- A copy of the closure plan re- quired by 40 CFR 267.112. Include, where applicable, as part of the plans, specific requirements in 40 CFR 267.176, 267.201,

and 267.1108.

- [Reserved]

- The most recent closure cost esti- mate for your facility prepared under 40 CFR 267.142 and a copy of the docu- mentation required to demonstrate fi- nancial assurance under 40 CFR 267.143. For a new facility, you may gather the required documentation 60 days before the initial receipt of hazardous wastes.

- [Reserved]

- Where applicable, a copy of the insurance policy or other documenta- tion that complies with the liability requirements of 40 CFR 267.147. For a new facility, documentation showing the amount of insurance meeting the specification of 40 CFR 267.147(a) that you plan to have in effect before initial receipt of hazardous waste for treat- ment or storage.

- Where appropriate, proof of cov- erage by a State financial mechanism, as required by 40 CFR 267.149 or 267.150.

- A topographic map showing a dis- tance of 1,000 feet around your facility at a scale of 2.5 centimeters (1 inch) equal to not more than 61.0 meters (200 feet). The map must show elevation contours. The contour interval must show the pattern of surface water flow

## § 270.300

in the vicinity of and from each oper- ational unit of the facility. For exam- ple, contours with an interval of 1.5 meters (5 feet), if relief is greater than

6.1 meters (20 feet), or an interval of 0.6 meters (2 feet), if relief is less than 6.1 meters (20 feet). If your facility is in a mountainous area, you should use large contour intervals to adequately show topographic profiles of facilities. The map must clearly show the fol- lowing:

  - Map scale and date.

  - 100-year flood plain area.

  - Surface waters including inter- mittent streams.

  - Surrounding land uses (residen- tial, commercial, agricultural, rec- reational).

  - A wind rose (i.e., prevailing wind- speed and direction).

  - Orientation of the map (north arrow).

  - Legal boundaries of your facility site.

  - Access control (fences, gates).

  - Injection and withdrawal wells both on-site and off-site.

  - Buildings; treatment, storage, or disposal operations; or other structure (recreation areas, runoff control sys- tems, access and internal roads, storm, sanitary, and process sewerage sys- tems, loading and unloading areas, fire control facilities, etc.)

  - Barriers for drainage or flood control.

  - Location of operational units within your facility, where hazardous waste is (or will be) treated or stored. (Include equipment cleanup areas.)

### § 270.300 What container information must I keep at my facility?

If you store or treat hazardous waste in containers, you must keep the fol- lowing information at your facility:

    - A description of the containment system to demonstrate compliance with the container storage area provi- sions of 40 CFR 267.173. This description must show the following:

      - Basic design parameters, dimen- sions, and materials of construction.

      - How the design promotes drainage or how containers are kept from con- tact with standing liquids in the con- tainment system.

## 40 CFR Ch. I (7–1–24 Edition)

      - Capacity of the containment sys- tem relative to the number and volume of containers to be stored.

      - Provisions for preventing or man- aging run-on.

      - How accumulated liquids can be analyzed and removed to prevent over- flow.

    - For storage areas that store con- tainers holding wastes that do not con- tain free liquids, a demonstration of compliance with 40 CFR 267.173(c), in- cluding:

      - Test procedures and results or other documentation or information to show that the wastes do not contain free liquids.

      - A description of how the storage area is designed or operated to drain and remove liquids or how containers are kept from contact with standing liquids.

    - Sketches, drawings, or data dem- onstrating compliance with 40 CFR

267.174 (location of buffer zone (15m or 50ft) and containers holding ignitable or reactive wastes) and 40 CFR 267.175(c) (location of incompatible wastes in relation to each other), where applicable.

    - Where incompatible wastes are stored or otherwise managed in con- tainers, a description of the procedures used to ensure compliance with 40 CFR 267.175(a) and (b), and 267.17(b) and (c).

    - Information on air emission con- trol equipment as required by § 270.315.

### § 270.305 What tank information must I keep at my facility?

If you use tanks to store or treat haz- ardous waste, you must keep the fol- lowing information at your facility:

- A written assessment that is re- viewed and certified by an independent, qualified, registered professional engi- neer on the structural integrity and suitability for handling hazardous waste of each tank system, as required under 40 CFR 267.191 and 267.192.

- Dimensions and capacity of each tank.

- Description of feed systems, safe- ty cutoff, bypass systems, and pressure controls (e.g., vents).

- A diagram of piping, instrumenta- tion, and process flow for each tank system.

## Environmental Protection Agency	§ 270.315

- A description of materials and equipment used to provide external corrosion protection, as required under 40 CFR 267.191.

- For new tank systems, a detailed description of how the tank system(s) will be installed in compliance with 40 CFR 267.192 and 267.194.

- Detailed plans and description of how the secondary containment system for each tank system is or will be de- signed, constructed, and operated to meet  the  requirements  of  40  CFR

267.195 and 267.196.

- [Reserved]

- Description of controls and prac- tices to prevent spills and overflows, as required under 40 CFR 267.198.

- For tank systems in which ignit- able, reactive, or incompatible wastes are to be stored or treated, a descrip- tion of how operating procedures and tank system and facility design will achieve compliance with the require- ments of 40 CFR 267.202 and 267.203.

- Information on air emission con- trol equipment as required by § 270.315.

### § 270.310 What equipment information must I keep at my facility?

If your facility has equipment to which 40 CFR part 264, subpart BB ap- plies, you must keep the following in- formation at your facility:

- For each piece of equipment to which 40 CFR part 264 subpart BB ap- plies:

  - Equipment identification number and hazardous waste management unit identification.

  - Approximate locations within the facility (e.g., identify the hazardous waste management unit on a facility plot plan).

  - Type of equipment (e.g., a pump or a pipeline valve).

  - Percent by weight of total organics in the hazardous waste stream at the equipment.

  - Hazardous waste state at the equipment (e.g., gas/vapor or liquid).

  - Method of compliance with the standard (e.g., monthly leak detection and repair, or equipped with dual me- chanical seals).

- For facilities that cannot install a closed-vent system and control device to comply with 40 CFR part 264, sub- part BB on the effective date that the

facility becomes subject to the subpart BB provisions, an implementation schedule as specified in 40 CFR 264.1033(a)(2).

- Documentation that demonstrates compliance with the equipment stand- ards in 40 CFR 264.1052 and 264.1059.

This documentation must contain the records required under 40 CFR 264.1064.

- Documentation to demonstrate compliance with 40 CFR 264.1060 must include the following information:

  - A list of all information ref- erences and sources used in preparing the documentation.

  - Records, including the dates, of each compliance test required by 40 CFR 264.1033(j).

  - A design analysis, specifications, drawings, schematics, and piping and instrumentation diagrams based on the appropriate sections of ‘‘Course 415: Control of Gaseous Emissions’’ (incor- porated by reference as specified in 40 CFR 260.11) or other engineering texts acceptable to the Director that present basic control device design informa- tion. The design analysis must address the vent stream characteristics and control device operation parameters as specified in 40 CFR 264.1035(b)(4)(iii).

  - A statement you signed and dated certifying that the operating param- eters used in the design analysis rea- sonably represent the conditions that exist when the hazardous waste man- agement unit is operating at the high- est load or capacity level reasonable expected to occur.

  - A statement you signed and dated certifying that the control device is de- signed to operate at an efficiency of 95 weight percent or greater.

### § 270.315 What air emissions control information must I keep at my facil- ity?

If you have air emission control equipment subject to 40 CFR part 264, subpart CC, you must keep the fol- lowing information at your facility:

- Documentation for each floating roof cover installed on a tank subject to 40 CFR 264.1084(d)(1) or (d)(2) that in- cludes information you prepared or the cover manufacturer/vendor provided describing the cover design, and your

## § 270.320

certification that the cover meets ap- plicable design specifications listed in 40 CFR 264.1084(e)(1) or (f)(1).

- Identification of each container area subject to the requirements of 40 CFR part 264, subpart CC and your cer- tification that the requirements of this subpart are met.

- Documentation for each enclosure used to control air pollutant emissions from tanks or containers under re- quirements of 40 CFR 264.1084(d)(5) or 264.1086(e)(1)(ii). You must include records for the most recent set of cal- culations and measurements you per- formed to verify that the enclosure meets the criteria of a permanent total enclosure as specified in ‘‘Procedure T—Criteria for and Verification of a Permanent or Temporary Total Enclo- sure’’ under 40 CFR 52.741, appendix B.

- [Reserved]

- Documentation for each closed- vent system and control device in- stalled under requirements of 40 CFR 264.1087 that includes design and per- formance information as specified in

§ 270.24 (c) and (d).

- An emission monitoring plan for both Method 21 in 40 CFR part 60, ap- pendix A and control device monitoring methods. This plan must include the following information: monitoring point(s), Monitoring methods for con- trol devices, monitoring frequency, procedures	for	documenting exceedences, and procedures for miti- gating noncompliances.

Modifying a Standardized Permit

### § 270.320 How do I modify my RCRA standardized permit?

You can modify your RCRA stand- ardized permit by following the proce- dures found in 40 CFR 124.211 through 124.214.

# PART 271—REQUIREMENTS FOR AU- THORIZATION OF STATE HAZ- ARDOUS WASTE PROGRAMS

### Subpart A—Requirements for Final Authorization

Sec.

  - Purpose and scope.

  - Definitions.

  - Availability of final authorization.

  - Consistency.

## 40 CFR Ch. I (7–1–24 Edition)

  - Elements of a program submission.

  - Program description.

  - Attorney General’s statement.

  - Memorandum of Agreement with the Regional Administrator.

  - Requirements for identification and listing of hazardous wastes.

  - Requirements of generators of haz- ardous wastes.

  - Requirements for transporters of haz- ardous wastes.

  - Requirements for hazardous waste management facilities.

  - Requirements with respect to permits and permit applications.

  - Requirements for permitting.

  - Requirements for compliance evalua- tion programs.

  - Requirements for enforcement au- thority.

  - Sharing of information.

  - Coordination with other programs.

  - EPA review of State permits.

  - Approval process.

  - Procedures for revision of State pro- grams.

  - Criteria for withdrawing approval of State programs.

  - Procedures for withdrawing approval of State programs.

  - Interim authorization under section 3006(g) of RCRA.

  - HSWA requirements.

  - Requirements for used oil manage- ment.

  - Interim authorization-by-rule for the revised Corrective Action Management Unit rule.

### Subpart B [Reserved]

AUTHORITY: 42 U.S.C. 6905, 6912(a), 6926, and 6939g.

SOURCE: 48 FR 14248, Apr. 1, 1983, unless

otherwise noted.

# Subpart A—Requirements for Final Authorization

### § 271.1  Purpose and scope.

    - This subpart specifies the proce- dures EPA will follow in approving, re- vising, and withdrawing approval of State programs and the requirements State programs must meet to be ap- proved by the Administrator under sec- tions 3006(b), (f) and (h) of RCRA.

    - State submissions for program ap- proval must be made in accordance with the procedures set out in this sub- part.

    - The substantive provisions which must be included in State programs for