- CHAPTER 62-340

DELINEATION OF THE LANDWARD EXTENT OF WETLANDS AND SURFACE WATERS

62-340.100 	Intent

62-340.200 	Definitions

62-340.300 	Delineation of Wetlands

62-340.400 	Selection of Appropriate Vegetative Stratum

62-340.450 	Vegetative Index

62-340.500 	Hydrologic Indicators

62-340.550 	Wetland Hydrology

62-340.600 	Surface Waters

62-340.700 	Exemptions for Treatment or Disposal Systems

62-340.750 	Exemption for Surface Waters or Wetlands Created by Mosquito Control Activities

- 62-340.100 Intent.

(1) This rule’s intent is to provide a unified statewide methodology for the delineation of the extent of wetlands and surface waters to satisfy the mandate of Section 373.421, F.S. This delineation methodology is intended to approximate the combined landward extent of wetlands as determined by a water management district and the Department immediately before the effective date of this rule. Before implementing the specific provisions of this methodology, the regulating agency shall attempt to identify wetlands according to the definition for wetlands in subsection 373.019(25), F.S., and subsection 62-340.200(19), F.A.C., below. The landward extent of wetlands shall be determined by the dominance of plant species, soils and other hydrologic evidence indicative of regular and periodic inundation or saturation. In all cases, attempts shall be made to locate the landward extent of wetlands visually by on site inspection, or aerial photointerpretation in combination with ground truthing, without quantitative sampling. If this cannot be accomplished, the quantitative methods in paragraph 62-301.400(1)(c), F.A.C., shall be used unless the applicant or petitioner and regulating agency agree, in writing, on an alternative method for quantitatively analyzing the vegetation on site. The methodology shall not be used to delineate areas which are not wetlands as defined in subsection 62-340.200(19), F.A.C., nor to delineate as wetlands or surface waters areas exempted from delineation by statute or agency rule.

(2) The Department shall be responsible for ensuring statewide coordination and consistency in the delineation of surface waters and wetlands pursuant to this rule, by providing training and guidance to the Department, Districts, and local governments in implementing the methodology.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.100.

- 62-340.200 Definitions.

When used in this chapter, the following terms shall mean:

(1) “Aquatic plant” means a plant, including the roots, which typically floats on water or requires water for its entire structural support, or which will desiccate outside of water.

(2) “Canopy” means the plant stratum composed of all woody plants and palms with a trunk four inches or greater in diameter at breast height, except vines.

(3) “Diameter at Breast Height (DBH)” means the diameter of a plant’s trunk or main stem at a height of 4.5 feet above the ground.

(4) “Facultative plants” means those plant species listed in subsection 62-340.450(3), F.A.C., of this chapter. For the purposes of this rule, facultative plants are not indicators of either wetland or upland conditions.

(5) “Facultative Wet plants” means those plant species listed in subsection 62-340.450(2), F.A.C., of this chapter.

(6) “Ground Cover” means the plant stratum composed of all plants not found in the canopy or subcanopy, except vines and aquatic plants.

(7) “Ground truthing” means verification on the ground of conditions on a site.

(8) “Hydric Soils” means soils that are saturated, flooded, or ponded long enough during the growing season to develop anaerobic conditions in the upper part of the soil profile.

(9) “Hydric Soil Indicators” means those indicators of hydric soil conditions as identified in Soil and Water Relationships of Florida's Ecological Communities (Florida Soil Conservation ed. Staff 1992).

(10) “Inundation” means a condition in which water from any source regularly and periodically covers a land surface.

(11) “Obligate plants” means those plant species listed in subsection 62-340.450(1), F.A.C., of this chapter.

(12) “Regulating agency” means the Department of Environmental Protection, the water management districts, state or regional agencies, local governments, and any other governmental entities.

(13) “Riverwash” means areas of unstabilized sandy, silty, clayey, or gravelly sediments. These areas are flooded, washed, and reworked by rivers or streams so frequently that they may support little or no vegetation.

(14) “Saturation” means a water table six inches or less from the soil surface for soils with a permeability equal to or greater than six inches per hour in all layers within the upper 12 inches, or a water table 12 inches or less from the soil surface for soils with a permeability less than six inches per hour in any layer within the upper 12 inches.

(15) “Seasonal High Water” means the elevation to which the ground and surface water can be expected to rise due to a normal wet season.

(16) “Subcanopy” means the plant stratum composed of all woody plants and palms, exclusive of the canopy, with a trunk or main stem with a DBH between one and four inches, except vines.

(17) “Upland plants” means those plant species, not listed as Obligate, Facultative Wet, or Facultative by this rule, excluding vines, aquatic plants, and any plant species not introduced into the State of Florida as of the effective date of this rule.

(18) “U.S.D.A.-S.C.S.” means the United States Department of Agriculture, Soil Conservation Service.

(19) “Wetlands,” as defined in subsection 373.019(25), F.S., means those areas that are inundated or saturated by surface water or ground water at a frequency and a duration sufficient to support, and under normal circumstances do support, a prevalence of vegetation typically adapted for life in saturated soils. Soils present in wetlands generally are classified as hydric or alluvial, or possess characteristics that are associated with reducing soil conditions. The prevalent vegetation in wetlands generally consists of facultative or obligate hydrophytic macrophytes that are typically adapted to areas having soil conditions described above. These species, due to morphological, physiological, or reproductive adaptations, have the ability to grow, reproduce or persist in aquatic environments or anaerobic soil conditions. Florida wetlands generally include swamps, marshes, bayheads, bogs, cypress domes and strands, sloughs, wet prairies, riverine swamps and marshes, hydric seepage slopes, tidal marshes, mangrove swamps and other similar areas. Florida wetlands generally do not include longleaf or slash pine flatwoods with an understory dominated by saw palmetto.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.200.

- 62-340.300 Delineation of Wetlands.

The landward extent (i.e., the boundary) of wetlands as defined in subsection 62-340.200(19), F.A.C., shall be determined by applying reasonable scientific judgment to evaluate the dominance of plant species, soils, and other hydrologic evidence of regular and periodic inundation and saturation as set forth below. In applying reasonable scientific judgment, all reliable information shall be evaluated in determining whether the area is a wetland as defined in subsection 62-340.200(19), F.A.C.

(1) Before using the wetland delineation methodology described below, the regulating agency shall attempt to identify and delineate the landward extent of wetlands by direct application of the definition of wetlands in subsection 62-340.200(19), F.A.C., with particular attention to the vegetative communities which the definition lists as wetlands and non-wetlands. If the boundary cannot be located easily by use of the definition in subsection 62-340.200(19), F.A.C., the provisions of this rule shall be used to locate the landward extent of a wetland. In applying the provisions of this rule, the regulating agency shall attempt to locate the landward extent of wetlands visually by on site inspection, or aerial photointerpretation in combination with ground truthing.

(2) The landward extent of a wetland as defined in subsection 62-340.200(19), F.A.C., shall include any of the following areas:

(a) Those areas where the aereal extent of obligate plants in the appropriate vegetative stratum is greater than the areal extent of all upland plants in that stratum, as identified using the method in Rule 62-340.400, F.A.C., and either:

1. The substrate is composed of hydric soils or riverwash, as identified using standard U.S.D.A.-S.C.S. practices for Florida, including the approved hydric soil indicators, except where the hydric soil is disturbed by a nonhydrological mechanical mixing of the upper soil profile and the regulating agency establishes through data or evidence that hydric soil indicators would be present but for the disturbance;

2. The substrate is nonsoil, rock outcrop-soil complex, or the substrate is located within an artificially created wetland area; or

3. One or more of the hydrologic indicators listed in Rule 62-340.500, F.A.C., are present and reasonable scientific judgment indicates that inundation or saturation is present sufficient to meet the wetland definition of subsection 62-340.200(19), F.A.C.

(b) Those areas where the areal extent of obligate or facultative wet plants, or combinations thereof, in the appropriate stratum is equal to or greater than 80% of all the plants in that stratum, excluding facultative plants, and either:

1. The substrate is composed of hydric soils or riverwash, as identified using standard U.S.D.A.-S.C.S. practices for Florida, including the approved hydric soil indicators, except where the hydric soil is disturbed by a nonhydrologic mechanical mixing of the upper soil profile and the regulating agency establishes through data or evidence that hydric soil indicators would be present but for the disturbance;

2. The substrate is nonsoil, rock outcrop-soil complex, or the substrate is located within an artificially created wetland area; or

3. One or more of the hydrologic indicators listed in Rule 62-340.500, F.A.C., are present and reasonable scientific judgment indicates that inundation or saturation is present sufficient to meet the wetland definition of subsection 62-340.200(19), F.A.C.

(c) Those areas, other than pine flatwoods and improved pastures, with undrained hydric soils which meet, in situ, at least one of the criteria listed below. A hydric soil is considered undrained unless reasonable scientific judgment indicates permanent artificial alterations to the on site hydrology have resulted in conditions which would not support the formation of hydric soils.

1. Soils classified according to United States Department of Agriculture’s Keys to Soil Taxonomy (4th ed. 1990) as Umbraqualfs, Sulfaquents, Hydraquents, Humaquepts, Histosols (except Folists), Argiaquolls, or Umbraquults.

2. Saline sands (salt flats-tidal flats).

3. Soil within a hydric mapping unit designated by the U.S.D.A.-S.C.S. as frequently flooded or depressional, when the hydric nature of the soil has been field verified using the U.S.D.A.-S.C.S. approved hydric soil indicators for Florida. If a permit applicant, or a person petitioning for a formal determination pursuant to subsection 373.421(2), F.S., disputes the boundary of a frequently flooded or depressional mapping unit, the applicant or petitioner may request that the regulating agency, in cooperation with the U.S.D.A.-S.C.S., confirm the boundary. For the purposes of subsection 120.60(2), F.S., a request for a boundary confirmation pursuant to this subparagraph shall have the same effect as a timely request for additional information by the regulating agency. The regulating agency’s receipt of the final response provided by the U.S.D.A.-S.C.S. to the request for boundary confirmation shall have the same effect as a receipt of timely requested additional information.

4. For the purposes of this paragraph only, “pine flatwoods” means a plant community type in Florida occurring on flat terrain with soils which may experience a seasonal high water table near the surface. The canopy species consist of a monotypic or mixed forest of long leaf pine or slash pine. The subcanopy is typically sparse or absent. The ground cover is dominated by saw palmetto with areas of wire grass, gallberry, and other shrubs, grasses, and forbs, which are not obligate or facultative wet species. Pine flatwoods do not include those wetland communities as listed in the wetland definition contained in subsection 62-340.200(19), F.A.C., which may occur in the broader landscape setting of pine flatwoods and which may contain slash pine. Also for the purposes of this paragraph only, “improved pasture” means areas where the dominant native plant community has been replaced with planted or natural recruitment of herbaceous species which are not obligate or facultative wet species and which have been actively maintained for livestock through mechanical means or grazing.

(d) Those areas where one or more of the hydrologic indicators listed in Rule 62-340.500, F.A.C., are present, and which have hydric soils, as identified using the U.S.D.A.-S.C.S. approved hydric soil indicators for Florida, and reasonable scientific judgment indicates that inundation or saturation is present sufficient to meet the wetland definition of subsection 62-340.200(19), F.A.C. These areas shall not extent beyond the seasonal high water elevation.

(3)(a) If the vegetation or soils of an upland or wetland area have been altered by natural or man-induced factors such that the boundary between wetlands and uplands cannot be delineated reliably by use of the methodology in subsection 62-340.300(2), F.A.C., as determined by the regulating agency, and the area has hydric soils or riverwash, as identified using standard U.S.D.A.-S.C.S. practices for Florida, including the approved hydric soil indicators, except where the hydric soil is disturbed by a non hydrologic mechanical mixing of the upper soil profile and the regulating agency establishes through data or evidence that hydric soil indicators would be present but for the disturbance, then the most reliable available information shall be used with reasonable scientific judgment to determine where the methodology in subsection 62-340.300(2), F.A.C., would have delineated the boundary between wetlands and uplands. Reliable available information may include, but is not limited to, aerial photographs, remaining vegetation, authoritative site-specific documents, or topographical consistencies.

(b) This subsection shall not apply to any area where regional or site-specific permitted activity, or activities which did not require a permit, under Sections 253.123 and 253.124, F.S. (1957), as subsequently amended, the provisions of Chapter 403, F.S. (1983), relating to dredging and filling activities, Chapter 84-79, Laws of Florida, and Part IV of Chapter 373, F.S., have altered the hydrology of the area to the extent that reasonable scientific judgment, or application of the provisions of Section 62-340.550, F.A.C., indicate that under normal circumstances the area no longer inundates or saturates at a frequency and duration sufficient to meet the wetland definition in subsection 62-340.200(19), F.A.C.

(c) This subsection shall not be construed to limit the type of evidence which may be used to delineate the landward extent of a wetland under this chapter when an activity violating the regulatory requirements of Sections 253.123 and 253.124, F.S. (1957), as subsequently amended, the provisions of Chapter 403, F.S. (1983), relating to dredging and filling activities, Chapter 84-79, Laws of Florida, and Part IV of Chapter 373, F.S., has disturbed the vegetation or soils of an area.

(4) The regulating agency shall maintain sufficient soil scientists on staff to provide evaluation or consultation regarding soil determinations in applying the methodologies set forth in subsection 62-340.300(2) or (3), F.A.C. Services provided by the U.S.D.A.-S.C.S., or other competent soil scientists, under contract or agreement with the regulating agency, may be used in lieu of, or to augment, agency staff.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.300.

- 62-340.400 Selection of Appropriate Vegetative Stratum.

Dominance of plant species, as described in paragraphs 62-340.300(2)(a) and 62-340.300(2)(b), F.A.C., shall be determined in a plant stratum (canopy, subcanopy, or ground cover). The top stratum shall be used to determine dominance unless the top stratum, exclusive of facultative plants, constitutes less than 10 percent areal extent, or unless reasonable scientific judgment establishes that the indicator status of the top stratum is not indicative of the hydrologic conditions on site. In such cases, the stratum most indicative of on site hydrologic conditions, considering the seasonal variability in the amount and distribution of rainfall, shall be used. The evidence concerning the presence or absence of regular and periodic inundation or saturation shall be based on in situ data. All facts and factors relating to the presence or absence of regular and periodic inundation or saturation shall be weighed in deciding whether the evidence supports shifting to a lower stratum. The presence of obligate, facultative wet, or upland plants in a lower stratum does not by itself constitute sufficient evidence to shift strata, but can be considered along with other physical data in establishing the weight of evidence necessary to shift to a lower stratum. The burden of proof shall be with the party asserting that a stratum other than the top stratum should be used to determine dominance. Facultative plants shall not be considered for purposes of determining appropriate strata or dominance.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.400.

- 62-340.450 Vegetative Index.

(1) Obligate Species

Acer	maple, silver

saccharinum

Acoelorraphe		palm, paurotis

wrightii

Acrostichum	leather fern

spp.

Aeschynomene	joint-vetch,

pratensis	meadow

Agalinis linifolia	false-foxglove,

flax-leaf

Agalinis	false-foxglove,

maritima	saltmarsh

Alisma	water-plantain,

subcordatum	subcordate

Alnus serrulata	alder, hazel

Alternanthera	alligator-weed

philoxeroides

Alternanthera	alligator weed,

sessilis	sessile

Amaranthus	amaranth,

australis	southern

Amaranthus	amaranth,

cannabinus	tidemarsh

Amaranthus	amaranth,

floridanus	Florida

Ammannia spp.	toothcup

Annona glabra	pond apple

Aristida affinis	three-awn

grass, long-leaf

Armoracia	lakecress

aquatica

Arnoglossum	indian-plantain,

sulcatum	Georgia

Asclepias	milkweed,

incarnata	swamp

Asclepias	milkweed,

lanceolata	fen-flower

Asclepias	milkweed,

perennis	aquatic

Asclepias rubra	milkweed, red

Aster	aster, climbing

carolinianus

Aster elliottii	aster, Elliott’s

Aster subulatus	aster, saltmarsh

Aster tenuifolius	aster, saltmarsh

Avicennia	mangrove, black

germinans

Baccharis	false-willow

angustifolia

Bacopa spp.	water-hyssop

Batis maritima	saltwort

Betula nigra	birch, river

Bidens spp.	beggar-ticks

except

Bidens	beggar-ticks,

pilosa	white (FAC)

Bidens	Spanish needles

bipinnata	(U)

Boehmeria	false-nettle,

cylindrica	small-spike

Borrichia spp.	sea oxeye

Burmannia spp.	burmannia

Callitriche spp.	water-starwort

Campanula	bellflower

floridana

Canna spp.	canna

except

Canna x	canna, common

generalis	(FAC)

Cardamine	bitter-cress

bulbosa

Cardamine	spring-cress

pensylvanica

Carex atlantica	sedge, prickly

bog

Carex comosa	sedge, bearded

Carex crinita	sedge, fringed

Carex	sedge,

crus-corvi	raven-foot

Carex	sedge,

decomposita	cypress-knee

Carex elliottii	sedge, Elliott’s

Carex folliculata	sedge, long

Carex gigantea	sedge, large

Carex howei	sedge, Howe’s

Carex	sedge, shoreline

hyalinolepis

Carex leptalea	sedge,

bristly-stalk

Carex	sedge, Louisiana

louisianica

Carex lupulina	sedge, hop

Carex lurida	sedge, shallow

Carex stipata	sedge,

stalk-grain

Carex	sedge, Walter’s

walteriana

Carya aquatica	hickory, water

Cephalanthus	buttonbush

occidentalis

Chamaecyparis	cedar, Atlantic

thyoides	white

Cicuta spp.	water-hemlock

Cirsium muticum		thistle, swamp

Cladium spp.	sawgrass

Cleistes	rosebud

divaricata

Colocasia	elephant’s ear

esculenta

Coreopsis	tickseed,

nudata	Georgia

Cornus amomum		dogwood, silky

Crataegus	mayhaw

aestivalis

Crinum	swamp-lily,

americanum	southern

Cyperus	flatsedge,

alternifolius	alternate-leaf

Cyperus	flatsedge, jointed

articulatus

Cyperus	flatsedge,

difformis	variable

Cyperus	flatsedge,

distinctus	marshland

Cyperus	flatsedge

drummondii

Cyperus	flatsedge

entrerianus

Cyperus	flatsedge,

erythrorhizos	red-root

Cyperus haspan	flatsedge,

sheathed

Cyperus	flatsedge,

lanceolatus	epiphytic

Cyperus papyrus		flatsedge,

papyrus

Decodon	swamp-loosestr

verticillatus	ife

Dichromena	white-top

latifolia	sedge, giant

Distichlis spicata		saltgrass,

seashore

Drosera	sundew,

filiformis	thread-leaf

Drosera	sundew,

intermedia	spoon-leaf

Drosera tracyi	sundew, Gulf

coast

Dulichium	sedge, three-way

arundinaceum

Echinodorus	burhead

spp.

Eleocharis spp.	spikerush

Erianthus	plumegrass,

giganteus	sugarcane

Erianthus	plumegrass,

strictus	narrow

Eriocaulon spp.	pipewort

Eryngium	corn snakeroot

aquaticum

Eupatorium	marsh

leptophyllum	thoroughwort

Fimbristylis spp.	fringe-rush

except

Fimbristylis	fringe-rush,

annua	annual (FACW)

F. puberula	fringe-rush,

Vahl’s (FACW)

F. spathacea	hurricane-grass

(FAC)

Fraxinus spp.	ash

except

Fraxinum	ash, white (U)

americana

Fuirena spp.	umbrella-sedge

Gleditsia	water-locust

aquatica

Glyceria striata	fowl mannagrass

Heteranthera	mud-plantain,

reniformis	kidney-leaf

Hibiscus	rosemallow,

coccineus	scarlet

Hibiscus	rosemallow,

grandiflorus	swamp

Hibiscus laevis	rosemallow,

halberd-leaf

Hibiscus	rosemallow,

moscheutos	swamp

Hydrochloa	watergrass

caroliniensis

Hydrocleis	water-poppy

nymphoides

Hydrocotyle	penny-wort,

ranunculoides	floating

Hydrolea spp.	false-fiddle-leaf

Hygrophila spp.	hygrophila

Hymenachne	trompetilla

amplexicaulis

Hymenocallis	spider-lily

spp.

Hypericum	St. John’s-wort,

chapmanii	Chapman’s

Hypericum	St. John’s-wort,

edisonianum	Edison’s

Hypericum	St. John’s-wort,

fasciculatum	marsh

Hypericum	St. John’s-wort,

lissophloeus	smooth-bark

Hypericum	St. John’s-wort,

nitidum	Carolina

Ilex amelanchier	holly, sarvis

Ilex cassine	holly, dahoon

Ilex myrtifolia	holly, myrtle

Ilex verticillata	winterberry

Illicium	anise, Florida

floridanum

Impatiens	touch-me-not,

capensis	spotted

Iris spp.	iris

except I. verna	dwarf iris (U)

Isoetes spp.	quillwort

Itea virginica	virginia willow

Iva frutescens	marsh elder

Juncus spp.	rush

except J. tenuis	rush (FAC)

J. marginatus	rush (FACW)

Justicia spp.	water-willow

except J. 	shrimp plant (U)

brandegeana

Kosteletzkya	mallow, seashore

virginica

Lachnocaulon	bogbutton,

digynum	pineland

Lachnocaulon	bogbutton,

engleri	Engler’s

Lachnocaulon	bogbutton,

minus	Small’s

Laguncularia	mangrove, white

racemosa

Leersia spp.	cutgrass

Leitneria	corkwood

floridana

Lilaeopsis spp.	lilaeopsis

Lilium iridollae	lily, panhandle

Limnobium	frogbit

spongia

Limnophila spp.	marshweed

Limonium	sea-lavender

carolinianum

Lindera	spicebush,

melissaefolia	southern

Linum westii	flax, West’s

Liparis elata =	liparis, tall

(L. nervosa)

Litsea aestivalis	pondspice

Lobelia	cardinal flower

cardinalis

Lobelia	lobelia, Florida

floridana

Ludwigia spp.	ludwigia;

water-primrose

except	seedbox, hairy

Ludwigia hirtella	(FACW)

Ludwigia	seedbox,

maritima	seaside (FACW)

L. suffruticosa	seedbox,

headed (FACW)

Ludwigia	seedbox,

virgata	savanna

(FACW)

Lycium	Christmas berry

carolinianum

Lycopus spp.	bugleweed

Lysimachia spp.	loosestrife

Lythrum spp.	marsh loosestrife

Macranthera	flameflower

flammea

Magnolia	magnolia,

virginiana	sweetbay

var. australis

Malaxis spicata	adder’s-mouth,

Florida

Maxillaria	orchid, hidden

crassifolia

Melanthium	bunchflower,

virginicum	Virginia

Micranthemum	baby tears

spp.

Micromeria	savory, Brown’s

brownei

Mimulus alatus	monkey-flower

Monanthochloe	keygrass

littoralis

Muhlenbergia	muhly grass

capillaris

Nasturtium spp.	water-cress

Nelumbo spp.	water-lotus

Nuphar luteum	cow-lily, yellow

Nymphaea spp.	water-lily

Nymphoides	floating hearts

spp.

Nyssa aquatica	tupelo, water

Nyssa ogeche	tupelo, ogeechee

Nyssa sylvatica	tupelo, swamp

var. biflora

Orontium	golden club

aquaticum

Osmunda regalis	fern, royal

Oxypolis spp.	water drop-wort

Panicum	panic grass

ensifolium

Panicum	witchgrass,

erectifolium	erect-leaf

Panicum	panicum,

gymnocarpon	savannah

Panicum	maidencane

hemitomon

Panicum	panicum, tall

longifolium	thin

Panicum	panicum, woolly

scabriusculum

Panicum tenerum		panicum,

bluejoint

Parnassia spp.	grass-of-parnas

sus

Paspalidium	water panicum

geminatum

Paspalum	paspalum,

dissectum	mudbank

Paspalum	paspalum, joint

distichum

Paspalum	paspalum, gulf

monostachyum

Paspalum	paspalum, early

praecox

Paspalum	paspalum, water

repens

Peltandra spp.	arum; spoon

flower

Penthorum	ditch stonecrop

sedoides

Pentodon	pentodon, Hall’s

pentandrus

Persea palustris	bay, swamp

Phragmites	reed, common

australis

Physostegia	dragon-head,

godfreyi	Godfrey’s

Physostegia	dragon-head,

leptophylla	slender-leaf

Pinckneya	fever-tree

bracteata

Pinguicula spp.	butterwort

Planera aquatica	planer tree

Platanthera spp.	orchid, fringed

Pleea tenuifolia	rush-featherling

Pogonia	pogonia, rose

ophioglossoides

Polygala cymosa		milkwort, tall

Polygonum spp.	smartweed

except

P. argyrocoleon	smartweed,

silversheath (U)

P. virginianum	jumpseed

(FACW)

Pontederia	pickerelweed

cordata

Populus	cottonwood,

heterophylla	swamp

Proserpinaca	mermaid-weed

spp.

Psilocarya spp.	baldrush

Quercus lyrata	oak, overcup

Rhexia	meadow-beauty

parviflora	white

Rhexia	meadow-beauty

salicifolia	panhandle

Rhizophora	mangrove, red

mangle

Rhynchospora	beakrush,

cephalantha	clustered

Rhynchospora	beakrush,

chapmanii	Chapman’s

Rhynchospora	beakrush,

corniculata	short-bristle

Rhynchospora

decurrens	beakrush,

swamp-forest

Rhynchospora	beakrush,

divergens	spreading

Rhynchospora	beakrush,

harperi	Harper’s

Rhynchospora	beakrush, horned

inundata

Rhynchospora	beakrush, large

macra

Rhynchospora	beakrush,

microcarpa	southern

Rhynchospora	beakrush, millet

miliacea

Rhynchospora	beakrush,

mixta	mingled

Rhynchospora	beakrush,

oligantha	few-flower

Rhynchospora	beakrush,

stenophylla	Chapman’s

Rhynchospora	beakrush,

tracyi	Tracy’s

Rorippa spp.	yellow-cress

Rosa palustris	rose, swamp

Rotala ramosior	toothcup

Rudbeckia	coneflower,

mohrii	Mohr’s

Sabatia	rose-gentian,

bartramii	Bartram’s

Sabatia calycina	rose-gentian,

coast

Sabatia	rose-gentian,

dodecandra	large

Sacciolepis	cupscale,

striata	American

Sagittaria spp.	arrowhead

Salicornia spp.	glasswort

Salix spp.	willow

Samolus spp.	pimpernel, water

Sarracenia spp.	pitcher-plant

except

Sarracenia	pitcher-plant,

minor	hooded (FACW)

Saururus	lizard’s tail

cernuus

Scirpus spp.	bulrush

Scutellaria	skullcap, blue

lateriflora

Scutellaria	skullcap

racemosa

Senecio aureus	ragwort, golden

Senecio	butterweed

glabellus

Setaria magna	foxtail

Sium suave	water-parsnip

Solidago elliottii	golden-rod,

Elliott’s

Solidago patula	golden-rod,

rough-leaf

Sparganium	burreed

americanum

Spartina	cordgrass,

alterniflora	saltmarsh

Spartina	cordgrass, big

cynosuroides

Spartina	cordgrass, gulf

spartinae

Spergularia	sandspurry,

marina	saltmarsh

Sphagnum spp.	sphagnum moss

Sphenopholis	wedgescale,

pensylvanica	swamp

Sporobolus	dropseed,

virginicus	seashore

Stachys	hedgenettle

lythroides

Stillingia	corkwood

aquatica

Styrax	snowbell; storax

americana

Suaeda spp.	sea-blite

Taxodium	cypress, pond

ascendens

Taxodium	cypress, bald

distichum

Thalia	thalia; fire flag

geniculata

Tofieldia	false-asphodel,

racemosa	coastal

Triadenum spp.	St. John’s-wort,

marsh

Triglochin	arrow-grass

striatam

Typha spp.	cattail

Utricularia spp.	bladderwort

Veronica	speedwell, water

anagallis-aquat

ica

Vicia ocalensis	vetch, Ocala

Viola lanceolata	violet, lance-leaf

Websteria	water-meal

confervoides

Woodwardia	chainfern

aereolata

Xyris spp.	yellow-eyed

grass

except

Xyris	yellow-eyed

caroliniana	grass, Carolina

(FACW)

Xyris jupicai	yellow-eyed

grass, tropical

(FACW)

Zizania aquatica	wildrice

Zizaniopsis	wildrice,

miliacea	southern

(2) Facultative Wet Species

Abildgaardia	rush, flat-spike

ovata

Acer negundo	box-elder

Acer rubrum	maple, red

Aeschynomene	joint-vetch, India

indica

Agalinis aphylla	false-foxglove,

scale-leaf

Agalinis	false-foxglove

pinetorum

(=A. pulchella)

Agalinis	false-foxglove,

purpurea	large purple

Agarista	hobble-bush

populifolia

Agrostis	redtop

stolonifera

Amorpha	indigo-bush

fruticosa

Amphicarpum	blue maidencane

muhlenbergian

um

Amsonia rigida	slimpod, stiff

Amsonia	slimpod, eastern

tabernaemonta

na

Andropogon	bluestem, bushy

glomeratus

(Campbell)

Andropogon	bluestem,

liebmanii	Mohr’s

var. pungensis

(Campbell) (A.

mohrii)

Anthaenantia	silky-scale,

rufa	purple

Apteria aphylla	nodding nixie

Arenaria	stitchwort,

godfreyi	Godfrey’s

Arisaema spp.	jack-in-the-pulp

it; green-dragon

Aristida	three-awn

purpurascens	grass, wand-like

(s.l.)

Arnoglossum	indian-plantain,

diversifolium	variable-leaf

Arnoglossum	indian-plantain,

ovatum	egg-leaf

Aronia	red chokeberry

arbutifolia

Arundinaria	giant cane

gigantea

Asclepias	milkweed,

connivens	large-flower

Asclepias	milkweed,

longifolia	long-leaf

Asclepias	milkweed,

pedicellata	savannah

Asclepias	milkweed,

viridula	southern

Aster chapmanii	aster, savannah

Aster	aster,

eryngiifolius	coyote-thistle

Aster lateriflorus	aster, calico

Aster spinulosus	aster, bog

Aster vimineus	aster, small

white

Athyrium	fern, subarctic

filix-femina	lady

Atriplex patula	saltbush,

halberd-leaf

Balduina	honeycomb-hea

atropurpurea	d, purple

Balduina	honeycomb-hea

uniflora	d, one-flower

Bartonia spp.	screwstem

Bigelowia	golden-rod,

nudata	rayless

Blechnum	swamp fern

serrulatum

Boltonia spp.	boltonia

Brachiaria	paragrass

purpurascens

Cacalia	indian-plantain,

suaveolens	sweet-scent

Calamovilfa	Curtiss’ reed

curtissii	grass

Calopogon spp.	grass-pinks

Calycocarpum	cupseed

lyonii

Caperonia spp.	caperonia

Capparis	caper-tree

flexuosa

Carex spp.	sedges

except

Carex atlantica	sedge, prickly

bog (OBL)

Carex comosa	sedge, bearded

(OBL)

Carex crinita	sedge, fringed

(OBL)

Carex	sedge,

crus-corvi	raven-foot

(OBL)

Carex	sedge,

decomposita	cypress-knee

(OBL)

Carex elliottii	sedge, Elliott’s

(OBL)

Carex	sedge, long

folliculata	(OBL)

Carex gigantea	sedge, large

(OBL)

Carex howei	sedge, Howe’s

(OBL)

Carex	sedge, shoreline

hyalinolepis	(OBL)

Carex leptalea	sedge,

bristly-stalk

(OBL)

Carex	sedge,

louisianica	Louisiana

(OBL)

Carex lupulina	sedge, hop

(OBL)

Carex lurida	sedge, shallow

(OBL)

Carex stipata	sedge,

stalk-grain

(OBL)

Carex	sedge, Walter’s

walteriana	(OBL)

Carphephorus	chaffhead,

carnosus	pineland

Carphephorus	chaffhead,

pseudoliatris	bristle-leaf

Carpinus	hornbeam,

caroliniana	American

Celtis laevigata	sugar-berry;

hackberry

Centella asiatica	coinwort

Chaptalia	sunbonnet;

tomentosa	pineland daisy

Chasmanthium	spanglegrass

spp.

except

C. latifolum

C. sessiliflorum	longleaf

Chasmanthium

Chrysobalanus	cocoplum

icaco

Cirsium lecontei	thistle, Leconte’s

Cirsium nuttallii	thistle, Nuttall’s

Clethra alnifolia	sweet pepper

bush

Cliftonia	buckwheat-tree

monophylla

Commelina spp.	dayflower

except

Commelina	dayflower,

erecta	sandhill (U)

Conocarpus	buttonwood

erectus

Coreopsis	tickseed, sickle

falcata

Coreopsis	tickseed, Florida

floridana

Coreopsis	tickseed,

gladiata	southeastern

Coreopsis	tickseed,

integrifolia	ciliate-leaf

Coreopsis	tickseed,

leavenworthii	Leavenworth’s

Coreopsis	tickseed, Texas

linifolia

Cornus foemina	swamp dogwood

Crataegus	haw, parsley

marshallii

Crataegus	haw, green

viridis

Croton elliottii	croton, Elliott’s

Ctenitis	fern,

submarginalis	brown-hair comb

Ctenium spp.	toothache grass

Cuphea aspera	common

waxweed

Cyperus spp.	flatsedge

except

C. alternifolius	flatsedge,

alternate-leaf

(OBL)

Cyperus	flatsedge,

articulatus	jointed (OBL)

Cyperus	flatsedge,

difformis	variable (OBL)

Cyperus	flatsedge,

distinctus	marshland

(OBL)

Cyperus	flatsedge (OBL)

drummondii

Cyperus	flatsedge (OBL)

entrerianus

C. erythrorhizos		flatsedge,

red-root (OBL)

Cyperus haspan	flatsedge,

sheathed (OBL)

Cyperus	flatsedge,

lanceolatus	epiphytic (OBL)

Cyperus	flatsedge,

papyrus	papyrus (OBL)

Cyperus	flatsedge,

cuspidatus	coastal-plain

(FAC)

Cyperus	flatsedge (FAC)

esculentus

Cyperus	flatsedge (FAC)

giganteus

Cyperus	flatsedge,

globulosus	baldwin (FAC)

Cyperus	flatsedge, black

huarmensis	knotty-root

(FAC)

Cyperus metzii	flatsedge (FAC)

Cyperus	flatsedge (FAC)

retrorsus

Cyperus	flatsedge,

rotundus	purple (FAC)

Cyperus	flatsedge,

filiculmis	sandhill (U)

Cyperus	flatsedge (U)

ovularis

Cyperus	flatsedge (U)

reflexus

Cyperus	flatsedge (U)

refractus

C. retrofractus	flatsedge (U)

Cyperus	flatsedge (U)

tetragonus

Dichromena	white-top

colorata	sedge, starbrush

Dichromena	white-top

floridensis	sedge,

Everglades

Dicliptera	mudwort, wild

brachiata

Digitaria	everglades grass

pauciflora

Diodia	button-weed

virginiana

Dionaea	Venu’ flytrap

muscipula

Drosera	sundew, dwarf

brevifolia

Drosera	sundew, pink

capillaris

Dryopteris	shield-fern,

ludoviciana	southern

Dyschoriste	dyschoriste,

humistrata	swamp

Echinochloa	jungle-rice;

spp.	cockspur grass

Eclipta alba	yerba de Tajo

Elyonurus	balsam-scale,

tripsacoides	Pan-American

Equisetum	horsetail

hyemale

Erianthus	plume grass,

brevibarbus	short-beard

Erigeron vernus	fleabane, early

whitetop

Eriochloa spp.	cupgrass

Eryngium	coyote-thistle,

integrifolium	blue-flower

Eryngium	coyote-thistle,

prostratum	creeping

Eryngium	rattlesnake

yuccifolium	master

Erythrodes	erythrodes, low

querceticola

Eulophia alta	coco, wild

Eupatoriadelph	joe-pye-weed

us fistulosus

Eupatorium	thoroughwort,

leucolepis	white-bract

Eupatorium	thoroughwort,

mikanioides	semaphore

Eupatorium	boneset

perfoliatum

Euphorbia	broomspurge,

humistrata	spreading

(=Chamaesyce

humistrata)

Euphorbia	spurge, Florida

inundata

Euphorbia	spurge,

polyphylla	many-leaved

Eustachys	fingergrass,

glauca	saltmarsh

(=Chloris

glauca)

Eustoma	prairie-gentian

exaltatum

Evolvulus	evolvulus

convolvuloides

Evolvulus	silky bindweed

sericeus

Fimbristylis	fimbry, annual

annua

Fimbristylis	fimbry, Vahl’s

puberula	hairy

Flaveria	yellowtop

floridana

Flaveria linearis	yellowtop

Forestiera	privet, swamp

acuminata

Fothergilla	witch-alder,

gardenii	dwarf

Galium	bedstraw, stiff

tinctorium	marsh

Gaylussacia	woolly-berry

mosieri

Gentiana spp.	gentian

Gleditsia	honey-locust

triacanthos

Gordonia	bay, loblolly

lasianthus

Gratiola spp.	hedgehyssop

except

Gratiola hispida	hedgehyssop

(FAC)

Habenaria spp.	rein orchid

Halesia diptera	silver-bell

Harperocallis	Harper’s beauty

flava

Hartwrightia	hartwrightia,

floridana	Florida

Hedychium	ginger

coronarium

Helenium spp.	sneezeweed

except

Helenium	sneezeweed,

amarum	pasture (FAC)

Helianthus	sunflower,

agrestis	southeastern

Helianthus	sunflower,

angustifolius	swamp

Helianthus	sunflower,

carnosus	lakeside

Helianthus	sunflower,

heterophyllus	wetland

Helianthus	sunflower, muck

simulans

Heliotropium	heliotrope,

procumbens	four-spike

Hemicarpha	dwarf-bullrush

spp.

Hibiscus	rosemallow

aculeatus

Hydrocotyle	pennywort

spp.

except

H.	pennywort,

ranunculoides	floating (OBL)

Hypericum spp.	St. John’s-wort

except

Hypericum	St. John’s-wort,

chapmanii	Chapman’s

(OBL)

H. edisonianum	St. John’s-wort,

Edison’s (OBL)

H. fasciculatum	St. John’s-wort,

marsh (OBL)

H. lissophloeus	St. John’s-wort,

smooth-bark

(OBL)

Hypericum	St. John’s-wort,

nitidum	Carolina (OBL)

H. hypericoides	St. Andrew’s

cross (FAC)

H. tetrapetalum	St. John’s-wort,

four-petal (FAC)

H. cumulicola	St. John’s-wort,

scrub (U)

H. drummondii	St. John’s-wort,

Drummond’s (U)

H. gentianoides	pineweed (U)

H. microsepalum		St. John’s-wort,

small-sepal (U)

H. prolificum	St. John’s-wort,

shrubby (U)

Hypericum	St. John’s-wort,

punctatum	dotted (U)

Hypericum	St. John’s-wort,

reductum	Atlantic (U)

Hypolepis	fern, bead

repens

Hypoxis spp.	stargrasses,

yellow

Hyptis alata	musky mint

Ilex coriacea	holly, bay-gall

Ilex decidua	holly, deciduous

Illicium	star anise

parviflorum

Iva	little marsh elder

microcephala

Juncus	shore rush

marginatus

Kalmia latifolia	laurel, mountain

Lachnocaulon	bogbutton,

anceps	white-head

Lachnocaulon	bogbutton,

beyrichianum	southern

Laportea	wood-nettle,

canadensis	Canada

Leptochloa spp.	sprangle-top

except

Leptochloa	sprangle-top,

virgata	tropic (FAC)

Leucothoe spp.	dog-hobble

Liatris garberi	gayfeather,

garber’s

Lindera benzoin	spicebush,

northern

Lindernia spp.	false-pimpernel

except

Lindernia	false-pimpernel,

crustacea	Malayan

(FAC)

Linum carteri	flax, Carter’s

Linum striatum	flax, ridged

yellow

Lipocarpha spp.	lipocarpha

Liquidambar	sweetgum

styraciflua

Liriodendron	tulip tree

tulipifera

Listera spp.	twayblade

Lobelia spp.	lobelia

except

Lobelia	flower, cardinal

cardinalis	(OBL)

Lobelia	lobelia, Florida

floridana	(OBL)

Lophiola	golden-crest

americana

Ludwigia hirtella	seedbox, hairy

Ludwigia	seedbox, seaside

maritima

Ludwigia	seedbox, headed

suffruticosa

Ludwigia virgata	seedbox,

savanna

Lycopodium	clubmoss

spp.

Lyonia lucida	fetter-bush

Lyonia mariana	fetter-bush

Macbridea spp.	birds-in-a-nest

Manisuris spp.	jointgrass

except

M. cylindrica	jointgrass,

pitted (FAC)

Marshallia	barbara’s-button

graminifolia	s, grass-leaf

Marshallia	barbara’s-button

tenuifolia	s, slim-leaf

Mecardonia	mecardonia

spp.

Melanthera	squarestem

nivea

Mitreola spp.	hornpod

Muhlenbergia	nimblewill

schreberi

Myrica	bayberry,

heterophylla	evergreen

Myrica inodora	bayberry,

odorless

Nemastylis	pleatleaf,

floridana	fall-flowering

Nemophila	baby-blue-eyes,

aphylla	small-flower

Oldenlandia	bluets, water

spp.

Onoclea	fern, sensitive

sensibilis

Osmunda	fern, cinnamon

cinnamomea

Panicum	cut-throat grass

abscissum (Hall)

Panicum	panicum, fall

dichotomiflorum

Panicum	panicum

dichotomum

Panicum	panicum

pinetorum

Panicum repens	grass, torpedo

Panicum	panicum, red-top

rigidulum

Panicum	panicum

scoparium

Panicum spretum		panicum

Panicum	panicum, warty

verrucosum

Panicum	switchgrass

virgatum

Paspalum	paspalum, brook

acuminatum

Paspalum	paspalum, bull

boscianum

Paspalum	paspalum,

floridanum	Florida

Paspalum laeve	paspalum, field

Paspalum	paspalum,

pubiflorum	hairy-seed

Pavonia spicata	mangrove

mallow

Philoxerus	silverhead

vermicularis

Phyllanthus	leaf-flower,

caroliniensis	Carolina

Phyllanthus	leaf-flower,

liebmannianus	Florida

Physostegia	dragon-head,

purpurea	purple

Physostegia	dragon-head,

virginiana	false

Pieris	fetter-bush,

phillyreifolia	climbing

Pilea spp.	clearweed

Pinus glabra	pine, spruce

Pinus serotina	pine, pond

Platanus	sycamore

occidentalis

Pluchea spp.	camphor-weed

Polygala spp.	milkwort

except

Polygala	milkwort, tall

cymosa	yellow (OBL)

P. leptostachys	milkwort,

sandhill (U)

Polygala	milkwort, scrub

lewtonii	(U)

Polygala	milkwort,

polygama	racemed (U)

P. verticillata	milkwort,

whorled (U)

Polygonum	jumpseed

virginianum

Ponthieva	shadow-witch

racemosa

Populus	cotton-wood,

deltoides	eastern

Pteris tripartita	brake, giant

Ptilimnium	mock

capillaceum	bishop-weed

Pycnanthemum	mountain-mint,

nudum	coastal-plain

Quercus	oak, laurel

laurifolia

Quercus	oak, swamp

michauxii	chestnut

Quercus nigra	oak, water

Quercus pagoda	oak, cherry-bark

Quercus phellos	oak, willow

Ranunculus spp.	butter-cup

Reimarochloa	grass, Florida

oligostachya	reimar

Rhapidophyllu	palm, needle

m hystrix

Rhexia spp.	meadow-beauty

except

Rhexia	meadow-beauty

parviflora	white (OBL)

Rhexia	meadow-beauty

salicifolia	panhandle

(OBL)

Rhododendron	azalea, swamp

viscosum

Rhynchospora	beakrush

spp.

except

R. cephalantha	beakrush,

clustered (OBL)

R. chapmanii	beakrush,

Chapman's

(OBL)

R. corniculata	beakrush,

short-bristle

(OBL)

R. decurrens	beakrush,

swamp-forest

(OBL)

R. divergens	beakrush,

spreading (OBL)

R. harperi	beakrush,

Harper’s (OBL)

R. inundata	beakrush,

horned (OBL)

Rhynchospora	beakrush, large

macra	(OBL)

R. microcarpa	beakrush,

southern (OBL)

R. miliacea	beakrush, millet

(OBL)

Rhynchospora	beakrush,

mixta	mingled (OBL)

R. oligantha	beakrush,

few-flower

(OBL)

R. stenophylla	beakrush,

Chapman’s

(OBL)

Rhynchospora	beakrush,

tracyi	Tracy’s (OBL)

Rhynchospora	beakrush,

grayi	Gray’s (U)

R. intermedia	beakrush,

pinebarren (U)

R. megalocarpa	beakrush,

giant-fruited (U)

Roystonea spp.	palm, royal

Rudbeckia	coneflower,

fulgida	orange

Rudbeckia	coneflower,

graminifolia	grass-leaf

Rudbeckia	coneflower,

laciniata	cut-leaf

Rudbeckia nitida	coneflower,

shiny

Ruellia	wild-petunia,

noctiflora	night-flowering

Rumex spp.	dock

Sabal minor	palmetto, dwarf

Sabatia spp.	rose-gentian

except

Sabatia	rose-gentian,

bartramii	Bartram’s (OBL)

Sabatia calycina	rose-gentian,

coast (OBL)

Sabatia	rose-gentian,

dodecandra	large (OBL)

Sachsia	sachsia

polycephala

Sarracenia	pitcher-plant,

minor	hooded

Schoenolirion

croceum	sunny bells

Schoenolirion	sunny bells

elliottii

Schoenus	black-sedge

nigricans

Scleria spp.	nutrush

Sclerolepis	hardscale, one

uniflora	flower

Selaginella	spike-moss,

apoda	meadow

Sesuvium spp.	sea-purslane

Sisyrinchium	blue-eye-grass,

atlanticum	eastern

Sisyrinchium	blue-eye-grass

capillare

Sisyrinchium	blue-eye-grass,

mucronatum	Michaux’s

Solanum	canker-berry

bahamense

Solanum	night-shade,

erianthum	shrub

Solidago	golden-rod,

fistulosa	marsh

Solidago	golden-rod,

leavenworthii	leavenworth’s

Solidago	golden-rod,

sempervirens	seaside

Solidago stricta	golden-rod,

willow-leaf

Sophora	coast sophora

tomentosa

Spartina bakeri	cordgrass, sand

Spartina patens	cordgrass,

saltmeadow

Spermacoce	button-plant,

glabra	smooth

Sphenoclea	chicken-spike

zeylandica

Sphenostigma	ixia, Bartram’s

coelestinum

Spigelia	pink-root

loganioides

Spilanthes	spotflower,

americana	creeping

Spiranthes spp.	ladies’-tresses

Sporobolus	dropseed,

floridanus	Florida

Staphylea trifolia	bladdernut,

American

Stenandrium	stenandrium

floridanum

Stenanthium	feather-bells,

gramineum	eastern

Stipa	grass, Florida

avenacioides	needle

Stokesia laevis	stokesia

Syngonanthus	bantam-buttons

flavidulus

Teucrium	germander,

canadense	American

Thalictrum spp.	meadow-rue

Thelypteris spp.	shield fern

Tilia americana	American

basswood

Toxicodendron	poison sumac

vernix

Trachelosperm	climbing-dogba

um difforme	ne

Trepocarpus	trepocarpus,

aethusae	aethusa-like

Trianthema	horse-purslane

portulacastrum

Tridens	tridens, savannah

ambiguus

Tridens strictus	tridens,

long-spike

Triphora spp.	pogonias,

nodding

Ulmus spp.	elm

except

Ulmus rubra	elm, slippery

(U)

Urechites lutea	allamanda, wild

Uvularia	bellwort, Florida

floridana

Vaccinium	blueberry,

corymbosum	highbush

Verbena scabra	vervain,

sandpaper

Verbesina	crownbeard,

chapmanii	Chapman’s

Verbesina	crownbeard,

heterophylla	diverse-leaf

Vernonia spp.	ironweed

except

V. angustifolia	ironweed,

narrow-leaf (U)

Veronicastrum	culver’s root

virginicum

Viburnum	arrow-wood

dentatum

Viburnum nudum		viburnum,

possum-haw

Viburnum	viburnum,

obovatum	walter

Vicia acutifolia	vetch, four-leaf

Vicia floridana	vetch, Florida

Viola affinis	violet, Leconte’s

Viola esculenta	violet, edible

Viola	violet,

primulifolia	primrose-leaf

Woodwardia	chainfern

virginica

Xanthorhiza	yellow-root,

simplicissima	shrubby

Xanthosoma	elephant ear

sagittifolium

Xyris	yellow-eyed-gr

caroliniana	ass, Carolina

Xyris jupicai	yellow-eyed-gr

ass, Richard’s

Yeatesia	yeatesia,

viridiflora	green-flower

Zephyranthes	lily, atamasco

atamasco

Zigadenus	crow poison

densus

Zigadenus	deathcamas,

glaberrimus	Atlantic

Within Monroe County and the Key Largo portion of Miami-Dade County only, the following species shall be listed as Facultative Wet:

Alternanthera	beach

maritima	alternanthera

Morinda royoc	Keys rhubarb

Strumpfia	strumpia

maritima

(3) Facultative Species

Acacia	ear-leaved acacia

auriculiformis

Aletris spp.	colic-root

Alopecurus	foxtail, tufted

carolinianus

Anagallis pumila	pimpernel,

Florida

Andropogon	bluestem,

arctatus	savannah

(Campbell)

Andropogon	bluestem,

brachystachys	short-spike

(Campbell)

Andropogon	bluestem, big

gerardii

(Campbell)

Andropogon	bluestem, slim

perangustatus

(Campbell)

Andropogon	broom-sedge

virginicus

(Campbell)

Ardisia spp.	marlberry

Aristida	grass,

rhizomophora	rhizomatous

three-awn

Aristida	bottlebrush,

spiciformis	three-awn

Aristida stricta	grass, pineland

three-awn

Arundo donax	reed, giant

Aster dumosus	aster, bushy

Aster umbellatus	aster, flat-top

white

Axonopus spp.	carpet grass

Baccharis dioica	false-willow,

broom-bush

Baccharis	groundsel tree

glomeruliflora

Baccharis	false-willow,

halimifolia	eastern

Bidens pilosa	beggar-ticks,

hairy

Bucida buceras	gregory wood

Bumelia	bumelia, coastal

celastrina

Bumelia	bumelia,

lycioides	buckthorn

Bumelia	bumelia

reclinata

Campanula	bellflower,

americana	American

Canna x	garden canna

generalis

Carphephorus	vanilla plant

odoratissimus

Carphephorus	deer-tongue

paniculatus

Casuarina spp.	casuarina

Cayaponia	cyaponia,

guingueloba	five-lobe

Cestrum diurnum		day jessamine

Chasmanthium	spangle grass

latifolium

Chasmanthium	longleaf

sessiliflorum	Chasmanthium

Chiococca spp.	snowberry

Colubrina	snakewood,

asiatica	Asian

Conoclinium	mistflower

coelestinum

Coreopsis	tickseed, tall

tripteris

Cupaniopsis	carrotwood

anacardioides

Cuphea	waxweed,

carthagenensis	Columbia

Cyperus	flatsedge,

cuspidatus	coastal-plain

Cyperus	flatsedge

giganteus

Cyperus	flatsedge,

globulosus	baldwin

Cyperus	flatsedge, black

huarmensis	knotty-root

Cyperus metzii	flatsedge

Cyperus	flatsedge

retrorsus

Cyperus	flatsedge, purple

rotundus

Cypselea	panal

humifusa

Cyrilla	cyrilla, swamp

racemiflora

Dichondra	pony-foot

caroliniensis

Digitaria	crabgrass, dwarf

serotina

Diospyros	persimmon,

virginiana	common

Drymaria	West Indian

cordata	chickweed

Elytraria	scaly-stem,

caroliniensis	Carolina

Eragrostis spp.	lovegrass

Erechites	fireweed

hieraciifolia

Erigeron	fleabane

guercifolius

Erithralis	black torchwood

fruticosa

Eryngium	coyote-thistle,

bladwini	Baldwin’s

Eupatorium spp.	thoroughworts

except

E. leptophyllum	thoroughwort,

secund (OBL)

E. leucolepis	thoroughwort,

white-bract

(FACW)

E. mikanioides	thoroughwort,

semaphore

(FACW)

E. perfoliatum	boneset,

common

(FACW)

Eustachys	finger grass

petracea

Euthamia spp.	bushy goldenrod

Ficus aurea	fig, Florida

strangler

Fimbristylis	hurricane-grass

spathacea

Flaveria bidentis	yellowtop

Flaveria	yellowtop

trinervia

Forestiera	privet, Florida

segregata

Gaylussacia	dwarf

dumosa	huckleberry

Gaylussacia	dangleberry

frondosa

Gratiola hispida	hyssop, hispid

Helenium	sneezeweed,

amarum	pasture

Helianthus	sunflower,

floridanus	Florida

Heliotropium	heliotrope,

curassavicum	seaside

Heliotropium	heliotrope

polyphyllum

Hibiscus	rosemallow, sea

tiliaceus

Hypericum	St. Andrew’s

hypericoides	cross

Ilex opaca var.	American holly

opaca

Ilex vomitoria	yaupon holly

Jacquinia	Joewood

keyensis

Juncus tenuis	rush, path

Kosteletzkya	mallow, coastal

pentasperma

Lachnanthes	redroot

caroliniana

Leptochloa	sprangle-top,

virgata	tropic

Liatris gracilis	blazing star

Liatris spicata	gayfeather,

spiked

Lilium catesbaei	lily, southern red

Lindernia	false-pimpernel

crustacea	Malayan

Linum	flax, Florida

floridanum	yellow

Linum medium	flax, stiff yellow

Lyonia ligustrina	maleberry

Manisuris	joint grass, pitted

cylindrica

Maytenus	Florida mayten

phyllanthoides

Melaleuca	punk tree

guinguenervia

Melochia	chocolate-weed

corchorifolia

Metopium	poison wood

toxiferum

Mimosa pigra	mimosa, black

Morus rubra	mulberry, red

Muhlenbergia	cutover muhly

expansa

Murdannia spp.	dewflower

Myosurus	mouse-tail, tiny

minimus

Myrica cerifera	bayberry,

southern

Myrsine	myrsine, guiana

guianensis

Nephrolepis	sword ferns

spp.

Neyraudia	reed, silk

reynaudiana

Oplismenus	grass, woods

setarius

Oryza sativa	rice, cultivated

Panicum anceps	panicum, beaked

Panicum	panicum

commutatum

(Hall)

Panicum hians	panicum, gaping

Panicum	panicum

strigosum

Panicum tenue	panicum

Parietaria spp.	pellitory

Paspalum	paspalum, sour

conjugatum

Paspalum	dallisgrass

dilatatum

Paspalum	paspalum,

fimbriatum	Panama

Paspalum	paspalum,

plicatulum	brown-seed

Paspalum	paspalum, thin

setaceum

Paspalum	grass, vasey

urvillei

Pennisetum	elephant ear

purpureum	grass

Phalaris spp.	grass, canary

Phyla spp.	frog-fruit

Phyllanthus	leaf-flower,

urinaria	water

Piriqueta	piriqueta

caroliniana

Polypogon spp.	grass,

rabbit-foot

Polypremium	rustweed

procumbens

Psidium	guava,

cattleianun	strawberry

Psychotria spp.	wild coffee

Rhodomyrtus	downy rose

tomentosus	myrtle

Rubus spp.	blackberries

Ruellia	wild-petunia,

brittoniana	Britton’s

Ruellia	wild petunia

caroliniensis

Sabal palmetto	palm, cabbage

Sacciolepis	grass, glenwood

indica

Sambucus	elderberry

canadensis

Sapium sebiferum		tallow-tree,

Chinese

Schinus	pepper-tree,

terebinthifolius	Brazilian

Schizachyrium	bluestem

spp.

Scoparia dulcis	sweet broom

Scutellaria	skullcap

floridana

Scutellaria	rough skullcap

integrifolia

Sebastiana	sebastian-bush,

fruticosa	gulf

Sesbania spp.	rattle-bush

Setaria	grass, bristle

geniculata

Seymeria	black senna

cassiodes

Solidago rugosa	golden-rod,

wrinkled

Stillingia	queen’s-delight,

sylvatica	marsh

var. tenuis

Suriana	bay-cedar

maritima

Syzygium spp.	Java plum

Thespesia	seaside mahoe

populnea

Tradescantia	trailing

fluminensis	spiderwort

Trema spp.	trema

Tripsacum	grass, eastern

dactyloides	gama

Vaccinium	blueberry,

elliottii	Elliott

Verbesina	crownbeard,

virginica	white

Wedelia	creeping ox-eye

trilobata

Within Monroe County and the Key Largo portion of Miami-Dade County only, the following species shall be listed as Facultative:

Alternanthera	smooth

paronychioides	chaff-flower

Byrsonima	locust-berry

lucida

Ernodea	golden creeper

littoralis

Guapira	blolly

discolor

Manilkara	wild dilly

bahamensis

Pisonia	pisonia

rotundata

Pithecellobium	blackbead

keyensis

Pithecellobium	catsclaw

unguis-cati

Randia aculeata	box briar

Reynosia	darling plum

septentrionalis

Thrinax radiata	Florida thatch

palm

(4) Nomenclature. Use of plants in this rule is based solely on the scientific names. Common names are included in the above lists for information purposes only. The following references shall be used by the regulating agency to resolve any uncertainty about the nomenclature or taxonomy of any plant listed by a given scientific name in this section: R. Godfrey, Trees, Shrubs and Woody Vines of Northern Florida and Adjacent Georgia & Alabama (Univ. Ga. Press, Athens 1988) and D. Lellinger, Ferns & Fern-Allies of the United States & Canada (Smithsonian Institution Press, Washington D.C. 1985) for all species covered by these references. For all other listed scientific names the following references will be followed unless the species list in this section designates a different authority next to an individual species name: R. Godfrey & J. Wooten, Aquatic and Wetland Plants of Southeastern United States: Monocotyledons (Univ. Ga. Press, Athens 1979); R. Godfrey & J. Wooten, Aquatic and Wetland Plants of Southeastern United States: Dicotyledons (Univ. Ga. Press, Athens 1979); D. & H. Correll, Flora of the Bahama Archipelago (A.R. Gantner, Germany 1982). When the species list in this section designates a different authority next to an individual species name, the regulating agency shall resolve any ambiguity in nomenclature by using the name identified in D. Hall, The Grasses of Florida (Doctoral Dissertation, Univ. of Fla., Gainesville 1978); or C. Campbell, Systematics of the Andropogon Virginicus Complex (GRAMINEAE), 64 Journal of the Arnold Arboretum 171-254 (1983).

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.450.

- 62-340.500 Hydrologic Indicators.

The indicators below may be used as evidence of inundation or saturation when used as provided in Rule 62-340.300, F.A.C. Several of the indicators reflect a specific water elevation. These specific water elevation indicators are intended to be evaluated with meteorological information, surrounding topography and reliable hydrologic data or analyses when provided, to ensure that such indicators reflect inundation or saturation of a frequency and duration sufficient to meet the wetland definition in subsection 62-340.200(19), F.A.C., and not rare or aberrant events. These specific water elevation indicators are not intended to be extended from the site of the indicator into surrounding areas when reasonable scientific judgment indicates that the surrounding areas are not wetlands as defined in subsection 62-340.200(19), F.A.C.

(1) Algal mats. The presence or remains of nonvascular plant material which develops during periods of inundation and persists after the surface water has receded.

(2) Aquatic mosses or liverworts on trees or substrates. The presence of those species of mosses or liverworts tolerant of or dependent on surface water inundation.

(3) Aquatic plants. Defined in subsection 62-340.200(1), F.A.C.

(4) Aufwuchs. The presence or remains of the assemblage of sessile, attached or free-living, nonvascular plants and invertebrate animals (including protozoans) which develop a community on inundated surfaces.

(5) Drift lines and rafted debris. Vegetation, litter, and other natural or manmade material deposited in discrete lines or locations on the ground or against fixed objects, or entangled above the ground within or on fixed objects in a form and manner which indicates that the material was waterborne. This indicator should be used with caution to ensure that the drift lines or rafted debris represent usual and recurring events typical of inundation or saturation at a frequency and duration sufficient to meet the wetland definition of subsection 62-340.200(19), F.A.C.

(6) Elevated lichen lines. A distinct line, typically on trees, formed by the water-induced limitation on the growth of lichens.

(7) Evidence of aquatic fauna. The presence or indications of the presence of animals which spend all or portions of their life cycle in water. Only those life stages which depend on being in or on water for daily survival are included in this indicator.

(8) Hydrologic data. Reports, measurements, or direct observation of inundation or saturation which support the presence of water to an extent consistent with the provisions of the definition of wetlands and the criteria within this rule, including evidence of a seasonal high water table at or above the surface according to methodologies set forth in Soil and Water Relationships of Florida's Ecological Communities (Florida Soil Conservation Staff 1992).

(9) Morphological plant adaptations. Specialized structures or tissues produced by certain plants in response to inundation or saturation which normally are not observed when the plant has not been subject to conditions of inundation or saturation.

(10) Secondary flow channels. Discrete and obvious natural pathways of water flow landward of the primary bank of a stream watercourse and typically parallel to the main channel.

(11) Sediment deposition. Mineral or organic matter deposited in or shifted to positions indicating water transport.

(12) Vegetated tussocks or hummocks. Areas where vegetation is elevated above the natural grade on a mound built up of plant debris, roots, and soils so that the growing vegetation is not subject to the prolonged effects of soil anoxia.

(13) Water marks. A distinct line created on fixed objects, including vegetation, by a sustained water elevation.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.500.

- 62-340.550 Wetland Hydrology.

A wetland delineation using the methodology described above, can be refuted by either reliable hydrologic records or site specific hydrologic data which indicate that neither inundation for at least seven consecutive days, nor saturation for at least twenty consecutive days, occurs during conditions which represent long-term hydrologic conditions. Hydrologic records or site specific hydrologic data must be of such a duration, frequency, and accuracy to demonstrate that the records or data are representative of the long-term hydrologic conditions, including the variability in quantity and seasonality of rainfall. When sufficient amounts of either reliable hydrologic records or site specific hydrologic data are not available to prove that the wetland area of concern does not inundate or saturate as described above, a site-specific field-verified analytic or numerical model may be used to demonstrate that the wetland area no longer inundates or saturates regularly or periodically under typical long-term hydrologic conditions. Before initiating the use of a model to evaluate if a wetland delineation should be refuted based on hydrologic conditions, the applicant or petitioner shall first meet with the appropriate regulating agency and reach an agreement on the terms of study, including data collection, the specific model, model development and calibration, and model verification. If the data, analyses, or models are deemed inadequate based on the hydrologic conditions being addressed, the regulating agency shall provide a case-by-case review of the applicability of any data, analyses, or models and shall provide specific reasons, based on generally accepted scientific and engineering practices, why they are inadequate.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211 FS. History–New 7-1-94, Formerly 17-340.550.

- 62-340.600 Surface Waters.

(1) For the purposes of Section 373.421, F.S., surface waters are waters on the surface of the earth, contained in bounds created naturally or artificially, including, the Atlantic Ocean, the Gulf of Mexico, bays, bayous, sounds, estuaries, lagoons, lakes, ponds, impoundments, rivers, streams, springs, creeks, branches, sloughs, tributaries, and other watercourses. However, state water quality standards apply only to those waters defined in subsection 403.031(13), F.S.

(2) The landward extent of a surface water in the State for the purposes of implementing Section 373.414, F.S., shall be the more landward of the following:

(a) Wetlands as located by Rule 62-340.300, F.A.C., of this chapter;

(b) The mean high water line elevation for tidal water bodies;

(c) The ordinary high water line for non-tidal natural water bodies;

(d) The top of the bank for artificial lakes, borrow pits, canals, ditches and other artificial water bodies with side slopes of 1 foot vertical to 4 feet horizontal or steeper, excluding spoil banks when the canals and ditches have resulted from excavation into the ground; or

(e) The seasonal high water line for artificial lakes, borrow pits, canals, ditches, and other artificial water bodies with side slopes flatter than 1 foot vertical to 4 feet horizontal along with any artificial water body created by diking or impoundment above the ground.

(3) Determinations made pursuant to paragraphs (2)(b) and (2)(c) shall be for regulatory purposes and are not intended to be a delineation of the boundaries of lands for the purposes of title.

Specific Authority 373.421 FS. Law Implemented 373.421, 373.4211, 403.031(13) FS. History–New 7-1-94, Formerly 17-340.600.

- 62-340.700 Exemptions for Treatment or Disposal Systems.

(1) Alteration and maintenance of the following shall be exempt from the rules adopted by the department and the water management districts to implement subsections 373.414(1) through 373.414(6), 373.414(8) and 373.414(10), F.S.; and subsection 373.414(7), F.S., regarding any authority to apply state water quality standards within any works, impoundments, reservoirs, and other watercourses described in this subsection and any authority granted pursuant to Section 373.414, F.S. (1991):

(a) Works, impoundments, reservoirs, and other watercourses constructed and operated solely for wastewater treatment or disposal in accordance with a valid permit reviewed or issued under Rules 62-28.700, 62-302.520, F.A.C., Chapters 62-17, 62-600, 62-610, 62-640, 62-650, 62-660, 62-670, 62-671, 62-673, or 62-701, F.A.C., or Section 403.0885, F.S., or rules implementing Section 403.0885, F.S., except for treatment wetlands or receiving wetlands permitted to receive wastewater pursuant to Chapter 62-611, F.A.C., or Section 403.0885, F.S., or its implementing rules;

(b) Works, impoundments, reservoirs, and other watercourses constructed solely for wastewater treatment or disposal before a construction permit was required under Chapter 403, F.S., and operated solely for wastewater treatment or disposal in accordance with a valid permit reviewed or issued under Rules 62-28.700, 62-302.520, F.A.C., Chapters 62-17, 62-600, 62-610, 62-640, 62-650, 62-660, 62-670, 62-671, 62-673, or 62-701, F.A.C., or Section 403.0885, F.S., or rules implementing Section 403.0885, F.S., except for treatment wetlands or receiving wetlands permitted to receive wastewater pursuant to Chapter 62-611, F.A.C., or Section 403.0885, F.S., or its implementing rules;

(c) Works, impoundments, reservoirs, and other watercourses of less than 0.5 acres in combined area on a project-wide basis, constructed and operated solely for stormwater treatment in accordance with a noticed exemption under Chapter 62-25, F.A.C., or a valid permit issued under Chapters 62-25 (excluding Rule 62-25.042), 62-330, 40B-4, 40C-4, 40C-42 (excluding Rule 40C-42.0265), 40C-44, 40D-4, 40D-40, 40D-45, or 40E-4, F.A.C., except those permitted as wetland stormwater treatment systems; or

(d) Works, impoundments, reservoirs, and other watercourses of less than 0.5 acres in combined area on a project-wide basis, constructed and operated solely for stormwater treatment before a permit was required under Chapters 62-25, 40B-4, 40C-4, 40C-42, 40C-44, 40D-4, 40D-40, 40D-45, or 40E-4, F.A.C.

(2) Alteration and maintenance of the following shall be exempt from the rules adopted by the department and the water management districts to implement subsections 373.414(1), 373.414(2)(a), 373.414(8), and 373.414(10), F.S.; and subsections 373.414(3) through 373.414(6), F.S.; and subsection 373.414(7), F.S., regarding any authority to apply state water quality standards within any works, impoundments, reservoirs, and other watercourses described in this subsection and any authority granted pursuant to Section 373.414, F.S. (1991), except for authority to protect threatened and endangered species in isolated wetlands:

(a) Works, impoundments, reservoirs, and other watercourses of 0.5 acre or greater in combined area on a project-wide basis, constructed and operated solely for stormwater treatment in accordance with a noticed exemption under Chapter 62-25, F.A.C., or a valid permit issued under Chapters 62-25 (excluding Rule 62-25.042), 62-330, 40B-4, 40C-4, 40C-42 (excluding Rule 40C-42.0265), 40C-44, 40D-4, 40D-40, 40D-45, 40E-4, except those permitted as wetland stormwater treatment systems; or

(b) Works, impoundments, reservoirs, and other watercourses of 0.5 acres or greater in combined area on a project-wide basis, constructed and operated solely for stormwater treatment before a permit was required under Chapters 62-25, 40B-4, 40C-4, 40C-42, 40C-44, 40D-4, 40D-40, 40D-45, or 40E-4, F.A.C.

(3) The exemptions in subsections 62-340.700(1) and (2) shall not apply to works, impoundments, reservoirs or other watercourses that

(a) Are currently wetlands which existed before construction of the stormwater treatment system and were incorporated in it;

(b) Are proposed to be altered through expansion into wetlands or other surface waters; or

(c) Are wetlands created, enhanced, or restored as mitigation for wetland or surface water impacts under a permit issued by the Department or a water management district.

(4) Alterations and maintenance of works, impoundments, reservoirs, and other watercourses exempt under this subsection shall not be considered in determining whether any wetland permitting threshold is met or exceeded under part IV of Chapter 373, F.S.

(5) Works, impoundments, reservoirs, and other watercourses exempt under this subsection, other than isolated wetlands in systems described in subsection 62-340.700(2), F.A.C., above, shall not be delineated under Section 373.421, F.S.

(6) This exemption shall not affect the application of state water quality standards, including those applicable to Outstanding Florida Waters, at the point of discharge to waters as defined in subsection 403.031(13), F.S.

(7) As used in this subsection, “solely for” means the reason for which a work, impoundment, reservoir, or other watercourse is constructed and operated; and such construction and operation would not have occurred but for the purposes identified in subsection 62-340.700(1) or 62-340.700(2), F.A.C. Furthermore, the phrase does not refer to a work, impoundment, reservoir, or other watercourse constructed or operated for multiple purposes. Incidental uses, such as occasional recreational uses, will not render the exemption inapplicable, so long as the incidental uses are not part of the original planned purpose of the work, impoundment, reservoir, or other watercourse. However, for those works, impoundments, reservoirs, or other watercourses described in paragraphs 62-340.700(1)(c) and 62-340.700(2)(a), F.A.C., use of the system for flood attenuation, whether originally planned or unplanned, shall be considered an incidental use, so long as the works, impoundments, reservoirs, and other watercourses are no more than 2 acres larger than the minimum area required to comply with the stormwater treatment requirements of the district or department. For the purposes of this subsection, reuse from a work, impoundment, reservoir, or other watercourse is part of treatment or disposal.

Specific Authority 373.414(9) FS. Law Implemented 373.414(9) FS. History–New 7-1-94, Formerly 17-340.700.

- 62-340.750 Exemption for Surface Waters or Wetlands Created by Mosquito Control Activities.

Construction, alteration, operation, maintenance, removal, and abandonment of stormwater management systems, dams, impoundments, reservoirs, appurtenant works, or works, in, on or over lands that have become surface waters or wetlands solely because of mosquito control activities undertaken as part of a governmental mosquito control program, and which lands were neither surface waters nor wetlands before such activities, shall be exempt from the rules adopted by the department and water management districts to implement subsections 373.414(1) through 373.414(6), 373.414(8), and 373.414(10), F.S.; and subsection 373.414(7), F.S., regarding any authority granted pursuant to Section 373.414, F.S. (1991). Activities exempted under this section shall not be considered in determining whether any wetland permitting threshold is met or exceeded under part IV of Chapter 373, F.S. This exemption shall not affect the regulation of impacts on other surface waters or wetlands, or the application of state water quality standards to waters as defined in subsection 403.031(13), F.S., including standards applicable to Outstanding Florida Waters.

Specific Authority 373.414(9) FS. Law Implemented 373.414(9) FS. History–New 7-1-94, Formerly 17-340.750.