WASTEWATER APPLICATION FORM 2CG, PERMIT TO DISCHARGE PROCESS WASTEWATER FROM NEW OR EXISTING INDUSTRIAL WASTEWATER FACILITIES TO GROUND WATER

Incorporated by reference in subsection 62-620.910(4), F.A.C.

INSTRUCTIONS - FORM 2CG

This form must be completed by all applicants who check "yes" to Item II-D in DEP Form 62-620.910(1) (Form 1).

Submit this completed form and supporting documentation and the application fee to the wastewater program of the appropriate DEP district office or local delegated program. Electronic submittal is preferred and may be available at the DEP Business Portal http://www.fldepportal.com/go/ To locate the appropriate DEP district office, go to: https://floridadep.gov/comm/comm/content/districts. All applications submitted after December 21, 2028, shall be submitted electronically.

Public Availability of Submitted Information.

You may not claim as confidential any information required by this form or Form 1, whether the information is reported on the forms or in an attachment. This information will be made available to the public upon request. Any information you submit to the Department which goes beyond that required by this form or Form 1 you may claim as confidential but claims for information which is effluent data will be denied. If you do not assert a claim of confidentiality at the time of submitting the information, the Department may make the information public without further notice to you. Claims of confidentiality must be in accordance with Rule 62-620.302, Florida Administrative Code.

Completeness

Your application will not be considered complete unless you answer every question on this form and on Form 1. If an item does not apply to you, enter "NA" (for "not applicable") to show that you considered the question. Also, you may need a Ground Water Monitoring Plan (GWMP) required by Rule 62-522.600, Florida Administrative Code. Please contact the Department for more information.

Follow-up Requirements (for New or Substantially Modified Facilities)

Although you are now required to submit estimated data on this form, please note that no later than six months after you begin discharging from the proposed or substantially modified facility, you must complete and submit items V and VI of this Form 2CG. However, you need not complete those portions of Item V requiring test which you have already performed under the discharge monitoring requirements of your permit.

Definitions

All significant terms used in these instructions and in the form are defined in the glossary found in the General Instructions which accompany Form 1.

DEP ID Number

If you are applying for a renewal of an existing permit or for a substantial revision to an existing permit, fill in your DEP Identification Number at the top of each page of Form 2CG. You may copy this number directly from Item 1 of Form 1. If you are applying for a permit for a proposed facility, leave the DEP Identification Number blank. The Department will assign a number.

Item I

You may use the map you provided for Item XI of Form 1 to determine the latitude and longitude of each of your discharge locations.

Item II

Describe any land application of effluent.

Item III

A. The line drawing should show generally the route taken by water in your facility from intake to discharge. Show all operations contributing wastewater, including process and production areas, sanitary flows, cooling water, and stormwater runoff. You may group similar operations into a single unit, labeled to correspond to the more detailed listing in Item III B. The water balance should show average flows. Show all significant losses of water to products, atmosphere, and discharge. You should use actual measurements whenever available; otherwise, use your best estimate.

B. List all sources of wastewater to each discharge point. Operations may be described in general terms (for example, "dye-making reactor" or "distillation tower"). You may estimate the flow contributed by each source if no data are available. For stormwater discharges you may estimate the average flow, but you must indicate the rainfall event upon which the estimate is based and the method of estimation. For each treatment unit, indicate its size, flow rate, and retention time, and describe the ultimate disposal of any solid or liquid wastes not discharged. Treatment units should be listed in order, and you should select the proper code from Table 2CG-1 to fill in column 3-b for each treatment unit. Insert "XX" into column 3-b if no code corresponds to a treatment unit you list.

C. A discharge is intermittent unless it occurs without interruption during the operating hours of the facility, except for infrequent shut-downs for maintenance, process changes, or other similar activities. A discharge is seasonal if it occurs only during certain parts of the year. Fill in every applicable column in this item for each source of intermittent or seasonal discharges. Base your answers on actual data whenever available; otherwise, provide your best estimate. Report the highest daily value for flow rate and total volume in the "Max. Daily" columns (columns 4-a and 4-b). Report the average of all daily values measured during days when the discharge occurred within the last year in the "Long Term Avg." columns (columns 4-a and 4-b).

Item IV

A. If you check “yes” to this question, complete all parts of the following chart.

B. If you check “yes” to this question, complete all parts of the following chart.

C. Answer “yes” or “no”.

Item V

A. If you check "yes" to this question, complete all parts of the chart, or attach a copy of any previous submission you have made to the Department containing the same information.

B. You are not required to submit a description of future pollution control projects if you do not wish to or if none is planned.

Item VI (A, B, C, and D, including Tables VI-A, VI-B, and VI-C)

This item requires you to collect and report data on the pollutants discharged from each of your discharge points. Each part of this item addresses a different set of pollutants and must be completed in accordance with the specific instructions for that part. The following general instructions apply to the entire item.

General Instructions

Part A requires you to report at least one analysis for each pollutant listed. Parts B and C require you to report analytical data in two ways. For some pollutants, you may be required to mark "X" in the "Testing Required" column (column 2-a, Part C), and test (sample and analyze) and report the levels of the pollutants in your discharge whether or not you expect them to be present in your discharge. For all other, you must mark "X" in either the "Believe Present" column or the "Believe Absent" column (columns 2-a or 2-b, Part B, and Columns 2-b or 2-c, Part C) based on your best estimate, and test for those which you believe to be present. (See specific instructions on the form and below for Parts A through D.). Base your determination that a pollutant is present in or absent from your discharge on your knowledge of your raw materials, maintenance chemicals, intermediate and final products and by-products, and any previous analyses known to you of your effluent or similar effluent. (For example, if you manufacture pesticides, you should expect those pesticides to be present in contaminated stormwater runoff.). If you would expect a pollutant to be present solely as a result of its presence in your intake water, you must mark "Believe Present" but you are not required to analyze for that pollutant. Instead, mark an "X" in the "Intake" column.

A. Reporting

All levels must be reported as concentration and as total mass. You may report some or all of the required data by attaching separate sheets of paper instead of filling out pages VI-1 to VI-8 if the separate sheets contain all the required information in a format which is consistent with pages VI-1 to VI-8 in spacing and in identification of pollutants and columns. (For example, the data systems used in your GC/MS analysis may be able to print data in the proper format.) Use the following abbreviations in the columns headed "Units" (column 3, Part A, and Column 4, Parts B and C).

All reporting of values for metals must be in terms of "total recoverable metal," unless (1) an applicable, promulgated effluent limitation or standard specifies the limitation for the metal in dissolved, valent, or total form; or (2) all approved analytical methods for the metal inherently measure only its dissolved form (e.g., hexavalent chromium). If you measure only one daily value, complete only "Max. Daily Values" columns and insert "1" into the "Number of Analyses" column (columns 2-a and 2-d, Part A, and column 3-a, 3-d, Parts B and C). The Department may require you to conduct additional analyses to further characterize your discharges. For composite sample, the daily value is the total mass or average concentration found in a composite sample taken over the operating hours of the facility during a 24-hour period; for grab samples, the daily value is the arithmetic or flow-weighted total mass or average concentration found in a series of at least four grab samples taken over the operating hours of the facility during a 24-hour period. If you measure more than one daily value for a pollutant and those values are representative of your waste stream, you must report them. You must describe your method of testing and data analysis. You also must determine the average of all values within the last year and report the concentration and mass under the "Long Term Avg. Values" columns (column 2-c, Part A, and column 3-c, Parts B and C), and the total number of daily values under the "Number of Analyses" columns (column 2-d, Part A, and columns 3-d, Parts B and C). Also determine the average of all daily values taken during each calendar month, and report the highest average under the "Max. 30-day Values" columns (column 2-c, Part A, and column 3-b, Parts B and C).

Concentration

ppm - parts per million

mg/l - milligrams per liter

ppb - parts per billion

ug/l - micrograms per liter

Mass

lbs - pounds

ton - tons (English tons)

mg - milligrams

g - grams

kg - kilograms

T - tonnes (metric tons)

B. Sampling

The collection of the samples for the reported analyses should be supervised by a person experienced in performing sampling of industrial wastewater. Any specific requirements contained in the applicable analytical methods should be followed for sample containers, sample preservation, holding times, the collection of duplicate samples, etc. The time when you sample should be representative of your normal operation, to the extent feasible, with all processes which contribute wastewater in normal operation, and with your treatment system operating properly with no system upsets. Samples should be collected from the center of the flow channel, where turbulence is at a maximum, at a site specified in your present permit, or at any site adequate for the collection of a representative sample. Sampling for metals that are hardness-dependent shall also include sampling for hardness.

For pH, temperature, cyanide, total phenols, residual chlorine, oil and grease, and fecal coliform, grab samples must be used. For all other pollutants 24-hour composite samples must be used. However, a minimum of one grab sample may be taken for effluents from holding ponds, or other impoundments with a retention period of greater than 24 hours. For stormwater discharges a minimum of one to four grab samples may be taken, depending on the duration of the discharge. One grab must be taken in the first hour (or less) of discharge, with one additional grab (up to a minimum of four) taken in each succeeding hour of discharge for discharges lasting four or more hours. The Department may waive composite sampling for any discharge point for which you demonstrate that use of an automatic sampler is infeasible and that a minimum of four grab samples will be representative of your discharge.

Grab and composite samples are defined as follows:

Grab sample: An individual sample or at least 100 milliliters collected at a randomly-selected time over a period not exceeding 15 minutes.

Composite sample: A combination of at least 8 sample aliquots of a least 100 milliliters, collected at periodic intervals during the operating hours of a facility over a 24-hour period. The composite must be flow proportional; either the time interval between each aliquot or the volume of each aliquot must be proportional to either the stream flow at the time of sampling or the total stream flow since the collection of the previous aliquot. Aliquots may be collected manually or automatically. For GC/MS Volatile Organic Analysis (VOA), aliquots must be combined in the laboratory immediately before analysis. Four (4) (rather than eight) aliquots or grab samples should be collected for VOA. These four samples should be collected during actual hours of discharge over a 24-hour period and need not be flow proportioned. Only one analysis is required.

Data from samples taken in the past may be used if all data requirement are met; sampling was done no more than three years before submission; and all data are representative of the present discharge. Among the factors which would cause the data to be unrepresentative are significant changes in production level; changes in raw materials, processes, or final products; and changes in wastewater treatment. When EPA promulgates new analytical methods in 40 CFR Part 136, EPA will provide information as to when you should use the new methods to generate data on your discharges. The Department may promulgate new methods in Chapter 160, Florida Administrative Code, with the date when the new methods are to be used. Always be sure you have current copies of these two documents before you take samples or submit sampling data to the Department. If you have submitted data from past sampling, the Department may request additional information, including current quantitative data, if it is determined to be necessary to assess your discharges.

C. Analysis

You must use test methods promulgated in 40 CFR Part 136 or Chapter 160, Florida Administrative Code; however, if none has been promulgated for a particular pollutant, you may use any suitable method for measuring the level of the pollutant in your discharge if you submit a description of the method or a reference to a published method. Your description should include the sample holding time, preservation techniques, and the quality control measures which you used. If you have two or more substantially identical discharge points, you may request permission from the Department to sample and analyze only one point and submit the results of the analysis for other substantially identical points. If your request is granted by the Department, or a separate sheet attached to the application form identify which point you did test and describe why the other points you did not test are substantially identical to the point which you did test.

D. Reporting of Intake Data

You are not required to report data under the "Intake" columns unless you wish to demonstrate your eligibility for a "net" effluent limitation for one or more pollutants, that is, an effluent limitation adjusted by subtracting the average level of the pollutant(s) present in your intake water. To demonstrate your eligibility, under the "Intake" columns report the average of the results of analyses on your intake water (If your water is treated before use, test the water after it is treated.), and discuss the requirements for a new limitation with the appropriate district office.

Part VI-A

Part VI-A must be completed by all applicants for all discharge points including discharges of non-contact cooling water or storm runoff. However, at your request, the Department may waive the requirement to test for one or more of these pollutants, upon a determination that available information is adequate to support issuance of the permit with less stringent reporting requirements for these pollutants. Use composite samples for all pollutants in this Part, except use grab samples for pH and temperature. See the discussion in General Instructions to item V for definitions of the columns in Part A. The "Long Term Avg. Values" column (column 2-c) and "Max. 30-day Values" column (column 2-b) are not compulsory but should be filled out if data are available.

Part VI-B

Part VI-B must be completed by all applicants for all discharge points, including points containing only non-contact cooling water or storm runoff. You must report quantitative data if the pollutant(s) in question is limited in an effluent limitation either directly or indirectly but expressly through a limitation on an indicator (e.g., use of TSS as an indicator to control the discharge of iron and aluminum). For other discharged pollutants you must provide quantitative data or explain their presence in your discharge. The Department will consider a request to eliminate the requirement to test for pollutants for an industrial category or subcategory. Your request must be supported by data representative of the industrial category or subcategory in question. The data must demonstrate that individual testing for each applicant is unnecessary, because the facilities in the category or subcategory discharge substantially identical levels of the pollutant or discharge the pollutant uniformly at sufficiently low levels. Use composite samples for all pollutants you analyze for in this part, except use grab samples for residual chlorine, oil and grease, and fecal coliform. The "Long Term Avg. Values" column (column 2-c) and "Max. 30-day Values" column (column 2-b) are not compulsory but should be filled out if data are available.

Part VI-C

Table 2CG-2 at the end of these instructions lists 34 primary industry categories. For each discharge point, if any of your processes which contribute wastewater falls into one of those categories, you must mark "X" in "Testing Required" column (column 2-a) and test for (1) all of the toxic metals, cyanide, and total phenols; and (2) the organic toxic pollutants contained in Table 2CG-3 as applicable to your category. The organic toxic pollutants are listed by GC/MS fractions on pages VI-3 to VI-8 in Part VI-C. The inclusion of total phenols in Part VI-C is not intended to classify total phenols as a toxic pollutant. When you determine which industry category you are in to find your testing requirements, you are not determining your category for any other purpose and you are not giving up your right to challenge your inclusion in that category before your permit is issued. For all other cases (secondary industries, non-process wastewater discharge points, and GC/MS fractions that are not required), you must mark "X" in either the "Believed Present" column or the "Believed Absent" column for each pollutant.

You must report quantitative data as follows:

For every pollutant you know or have reason to believe is present in your discharge in concentrations of 10 ppb or greater;

For acrolein; acrylonitrile; 2,4 dinitrophenol; and 2-methyl-4,6 dinitrophenol where you expect these four pollutants to be discharged in concentrations of 100 ppb or greater; and

For every pollutant expected to be discharged in concentrations less that the thresholds specified above. For pollutants in this last category, in lieu of quantitative data, you may briefly describe the reasons the pollutant is expected to be discharged.

You are required to mark "Testing Required" for dioxin if you use or manufacture one of the following compounds:

(a) 2,4,5-trichlorophenoxy acetic acid, (2,4,5-T);

(b) 2-(2,4,5-trichlorophenoxy) propanoic acid, (Silvex, 2,4,5-TP);

(c) 2-(2,4,5-trichlorophenoxy) ethyl 2,2-dichloropropionate, (Erbon);

(d) 0,0-dimethyl 0-(2,4,5-trichlorophenyl) phosphorothioate, (Ronnel);

(e) 2,4,5-trichlorophenol, (TCP); or

(f) hexachlorophene, (HCP).

If you mark "testing Required" or "Believed Present," you must perform a screening analysis for dioxins, using gas chromatography with an electron capture detector. A TCDD standard for quantitation is not required. Describe the results of this analysis in the space provided: for example, "no measurable baseline deflection at the retention time of TCDD" or "a measurable peak within the tolerances of the retention time of TCDD." The Department may require you to perform a quantitative analysis if you report a quantitative analysis if you report a positive result.

Part VI-D

List any pollutants in Table 2CG-3 that you believe to be present and explain why you believe them to be present. No analysis is required, but if you have analytical data, you must report it. Table 2CG-4 lists certain hazardous substances which may be exempt from reporting requirements contained in section 311 of the Clean Water Act. If you discharge any of these substances, please contact the Department for further information.

Item VII

This requirement applies to current use or manufacture of a toxic pollutant as an intermediate or final product or by-product. The Department may waive or modify the requirement if you demonstrate that it would be unduly burdensome to identify each toxic pollutant and the Department has adequate information to issue your permit. You may not claim this information as confidential; however, you do not have to distinguish between use or production of the pollutants or list the amounts.

Item VIII

This item is self explanatory.

Item IX

This item is self explanatory.

Item X

This item is self explanatory.

Item XI

There are severe penalties for submitting false information on this application form. Chapter 62-620, Florida Administrative Code, requires, in addition to the certification provided by a professional engineer, a certification from the owner or responsible authority of the facility as follows:

A. For a corporation: by a responsible corporate official. For purposes of this section, a responsible corporate official means (1) a president, secretary, treasurer, or vice-president of the corporation in charge of a principal business function, or any other person who performs similar policy- or decision-making functions for the corporation; or (2) the manager of one or more manufacturing, production or operating facilities employing more than 250 person or have gross annual sales or expenditures exceeding $25 million (in second-quarter 1980 dollars), if authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

B. For a partnership or sole proprietorship: by a general partner or the proprietor, respectively; or

C. For a municipality, State, Federal, or other public agency: by either a principal executive officer or ranking elected official. A principal executive officer includes the chief executive officer of the agency or a senior executive officer having the responsibility for the overall operations of a principal geographic unit of the agency, for example, a regional or district administrator.

TABLE 2CG-1

CODES FOR TREATMENT UNITS

| PHYSICAL TREATMENT PROCESSES |  |  |  |
| --- | --- | --- | --- |
| 1-A | Ammonia Stripping | 1-N | Microstraining |
| 2-B | Dialysis | 1-O | Mixing |
| 1-C | Diatomaceous Earth Filtration | 1-P | Moving Bed Filters |
| 1-D | Distillation | 1-Q | Multimedia Filtration |
| 1-E | Electrodialysis | 1-R | Rapid Sand Filtration |
| 1-F | Evaporation | 1-S | Reverse Osmosis (Hyperfiltration) |
| 1-G | Flocculation | 1-T | Screening |
| 1-H | Flotation | 1-U | Sedimentation (Settling) |
| 1-I | Foam Fractionation | 1-V | Slow Sand Filtration |
| 1-J | Freezing | 1-W | Solvent Extraction |
| 1-K | Gas-Phase Separation | 1-X | Sorption |
| 1-L | Grinding (Comminutors) | 1-Y | Percolation Pond |
| 1-M | Grit Removal |  |  |
| CHEMICAL TREATMENT PROCESSES |  |  |  |
| 2-A | Carbon Adsorption | 2-G | Disinfection (Ozone) |
| 2-B | Chemical Oxidation | 2-H | Disinfection (Other) |
| 2-C | Chemical Precipitation | 2-I | Electrochemical Treatment |
| 2-D | Coagulation | 2-J | Ion Exchange |
| 2-E | Dechlorination | 2-K | Neutralization |
| 2-F | Disinfection (Chlorine) | 2-L | Reduction |
| BIOLOGICAL TREATMENT PROCESSES |  |  |  |
| 3-A | Activated Sludge | 3-E | Pre-Aeration |
| 3-B | Aerated Lagoons | 3-F | Spray Irrigation/Land Application |
| 3-C | Anaerobic Treatment | 3-G | Stabilization Ponds |
| 3-D | Nitrification-Denitrification | 3-H | Trickling Filter |
| OTHER PROCESSES |  |  |  |
| 4-A | Discharge to Surface Water | 4-C | Reuse/Recycle of Treated Effluent |
| 4-B | Ocean Discharge Through Outfall | 4-D | Underground Injection |
| SLUDGE TREATMENT AND DISPOSAL PROCESSES |  |  |  |
| 5-A | Aerobic Digestion | 5-M | Heat Drying |
| 5-B | Anaerobic Digestion | 5-N | Heat Treatment |
| 5-C | Belt Filtration | 5-O | Incineration |
| 5-D | Centrifugation | 5-P | Land Application |
| 5-E | Chemical Conditioning | 5-Q | Landfill |
| 5-F | Chlorine Treatment | 5-R | Pressure Filtration |
| 5-G | Composting | 5-S | Pyrolysis |
| 5-H | Drying Beds | 5-T | Sludge Lagoons |
| 5-I | Elutriation | 5-U | Vacuum Filtration |
| 5-J | Flotation Thickening | 5-V | Vibration |
| 5-K | Freezing | 5-W | Wet Oxidation |
| 5-L | Gravity Thickening |  |  |

TABLE 2CG-2

TESTING REQUIREMENTS FOR ORGANIC TOXIC POLLUTANTS INDUSTRY CATEGORY

|  | GC/MS FRACTION |  |  |  |
| --- | --- | --- | --- | --- |
| INDUSTRY CATEGORY | Volatile | Acid | Base/Neutral | Pesticide |
| Adhesives and sealants | X | X | X |  |
| Aluminum forming | X | X | X |  |
| Auto and other laundries | X | X | X | X |
| Battery manufacturing | X |  | X |  |
| Coal mining | X | X | X | X |
| Coil coating | X | X | X |  |
| Copper forming | X | X | X |  |
| Electric and electronic compounds | X | X | X | X |
| Electroplating | X | X | X |  |
| Explosives manufacturing |  | X | X |  |
| Foundries | X | X | X |  |
| Gum and wood chemicals | X | X | X | X |
| Inorganic chemicals manufacturing | X | X | X |  |
| Iron and steel manufacturing | X | X | X |  |
| Leather tanning and finishing | X | X | X | X |
| Mechanical products manufacturing | X | X | X |  |
| Nonferrous metals manufacturing | X | X | X | X |
| Ore mining | X | X | X | X |
| Organic chemicals manufacturing | X | X | X | X |
| Paint and ink formulation | X | X | X | X |
| Pesticides | X | X | X | X |
| Petroleum refining | X | X | X | X |
| Pharmaceutical preparations | X | X | X |  |
| Photographic equipment and supplies | X | X | X | X |
| Plastic and synthetic materials manufacturing | X | X | X | X |
| Plastic processing | X |  |  |  |
| Porcelain enameling | X |  | X | X |
| Printing and publishing | X | X | X | X |
| Pulp and paperboard mills | X | X | X | X |
| Rubber processing | X | X | X |  |
| Soap and detergent manufacturing | X | X | X |  |
| Steam electric power plants | X | X | X |  |
| Textile mills | X | X | X | X |
| Timber products processing | X | X | X | X |

TABLE 2CG-3

TOXIC POLLUTANTS AND HAZARDOUS SUBSTANCES REQUIRED TO BE IDENTIFIED

BY APPLICANTS IF EXPECTED TO BE PRESENT

| Toxic Pollutant | Hazardous Substances | Hazardous Substances |
| --- | --- | --- |
| Asbestos | 2,2 Dichloropropionic acid | Monomethyl amine |
|  | Dichlorvos | Naled |
| Hazardous Substances | Diethyl amine | Naphthenic acid |
| Acetaldehyde | Dimethyl amine | Nitrotoluene |
| Allyl alcohol | Dintrobenzene | Parathion |
| Allylchloride | Diquat | Phenolsulfonate |
| Amyl acetate | Disulfoton | Phosgene |
| Anline | Diuron | Propargite |
| Benzonitrile | Epichlorohydrin | Propylene oxide |
| Benzyl chloride | Ethion | Pyrethrins |
| Butyl acetate | Ethylene diamine | Quinoline |
| Butylamine | Formaldehyde | Resorcinol |
| Captan | Furfural | Strontium |
| Carbaryl | Guthion | Strychinine |
| Carbofuran | Isoprene | 2,4,5-T (2,4,5-Trichlorophenoxyacetic acid) |
| Carbon disulfide | Isopropanolamine dodecylbenzenesulfonate | TDE (Terochlorodiphenyl ethane) |
| Chlopyrifos | Kelthane | 2,4,5-TP [2-(2,4,5-Trichlorophenosyl) propanic acid |
| Coumpahos | Kepone | Trichlorofon |
| Cresol | Malathion | Triethanolamine dodecylbenzenesulfonate |
| Crotonaldehyde | Mercaptodimethur | Triethylamine |
| Cyclohexane | Methoxychlor | Uranium |
| 2,4-D (2,4-Dichlorophenoxyacetic acid) | Methyl mercaptan | Vanadium |
| Diazinon | Methyl methacrylate | Vinyl acetate |
| Dicamba | Methyl parathion | Xylene |
| Dichlobenil | Mevinphos | Xylenol |
| Dichlone | Mexacarbate | Zirconium |
|  | Monoethyl amine |  |

TABLE 2CG-4

HAZARDOUS SUBSTANCES

1. Acetaldehyde

2. Acetic acid

3. Acetic anhydride

4. Acetone cyanohydrin

5. Acetyl bromide

6. Acetyl chloride

7. Acrolein

8. Acrylonitrile

9. Adipic acid

10. Aldrin

11. Allyl alcohol

12. Alyll chloride

13. Aluminum sulfate

14. Ammonia

15. Ammonium acetate

16. Ammonium benzoate

17. Ammonium bicarbonate

18. Ammonium bichromate

19. Ammonium bifluoride

20. Ammonium bisulfite

21. Ammonium carbamate

22. Ammonium carbonate

23. Ammonium chloride

24. Ammonium chromate

25. Ammonium citrate

26. Ammonium fluoroborate

27. Ammonium fluoride

28. Ammonium hydroxide

29. Ammonium oxalate

30. Ammonium silicofluoride

31. Ammonium sulfamate

32. Ammonium sulfide

33. Ammonium sulfite

34. Ammonium tartrate

35. Ammonium thiocyanate

36. Ammonium thiosulfate

37. Amyl acetate

38. Aniline

39. Antimony pentachloride

40. Antimony potassium tartrate

41. Antimony tribromide

42. Antimony trichloride

43. Antimony trifluoride

44. Antimony trioxide

45. Arsenic disulfide

46. Arsenic pentoxide

47. Arsenic trichloride

48. Arsenic trioxide

49. Arsenic trisulfide

50. Barium cyanide

51. Benzene

52. Benzoic acid

53. Benzonitrile

54. Benzoyl chloride

55. Benzyl chloride

56. Beryllium chloride

57. Beryllium fluoride

58. Beryllium nitrate

59. Butylacetate

60. n-Butylphthalate

61. Butylamine

62. Butyric acid

63. Cadmium acetate

64. Cadmium bromide

65. Cadmium chloride

66. Calcium arsenate

67. Calcium arsenite

68. Calcium carbide

69. Calcium chromate

70. Calcium cyanide

71. Calcium dodecylbenzenesulfonate

72. Calcium hypochlorite

73. Captan

74. Carbaryl

75. Carbofuran

76. Carbon disulfide

77. Carbon tetrachloride

78. Chlordane

79. Chlorine

80. Chlorobenzene

81. Chloroform

82. Chloropyrifos

83. Chlorosulfonic acid

84. Chromic acetate

85. Chromic acid

86. Chromic sulfate

87. Chromous chloride

88. Cobaltous bromide

89. Cobaltous formate

90. Cobaltous sulfamate

91. Coumaphos

92. Cresol

93. Crotonaldehyde

94. Cupric acetate

95. Cupric acetoarsenite

96. Cupric chloride

97. Cupric nitrate

98. Cupric oxalate

99. Cupric sulfate

100. Cupric sulfate ammoniated

101. Cupric tartrate

102. Cyanogen chloride

103. Cyclohexane

104. 2,4-D acid (2,4-Dichlorophenoxyacetic acid)

105. 2,4-D esters (2,4-Dichlorophenoxyacetic

acid esters)

106. DDT

107. Diazinon

108. Dicamba

109. Dichlobenil

110. Dichlone

111. Dichlorobenzene

112. Dichloropropane

113. Dichloropropene

114. Dichloropropene-Dichloropropane mix

115. 2,2-Dichloropropionic acid

116. Dichlorvos

117. Dieldrin

118. Diethylamine

119. Dimethylamine

120. Dinitrobenzene

121. Dinitrophenol

122. Dinitrotoluene

123. Diquat

124. Disulfoton

125. Diuron

126. Dodecylbenzesulfonic acid

127. Endosulfan

128. Endrin

129. Epichlorohydrin

130. Ethion

131. Ethylbenzene

132. Ethylenediamine

133. Ethylene dibromide

134. Ethylene dichloride

135. Ethylene Diaminetetracetic acid (EDTA)

136. Ferric ammonium citrate

137. Ferric ammonium oxalate

138. Ferric chloride

139. Ferric fluoride

140. Ferric nitrate

141. Ferric sulfate

142. Ferrous ammonium sulfate

143. Ferrous chloride

144. Ferrous sulfate

145. Formaldehyde

146. Formic acid

147. Fumaric acid

148. Furfural

149. Guthion

150. Heptachlor

151. Hexachlorocyclopentadiene

152. Hydrochloric acid

153. Hydrofluoric acid

154. Hydrogen cyanide

155. Hydrogen sulfide

156. Isoprene

157. Isopropanolamine dodecylbenzenesulfonate

158. Kelthane

159. Kepone

160. Lead acetate

161. Lead arsenate

162. Lead chloride

163. Lead fluoborate

164. Lead fluorite

165. Lead iodide

166. Lead nitrate

167. Lead stearate

168. Lead sulfate

169. Lead sulfide

170. Lead thiocyanate

171. Lindane

172. Lithium chromate

173. Malathion

174. Maleic acid

175. Maleic anhydride

176. Mercaptodimethur

177. Mercuric cyanide

178. Mercuric nitrate

179. Mercuric sulfate

180. Mercuric thiocyanate

181. Mercurous nitrate

182. Methoxychlor

183. Methyl mercaptan

184. Methyl methacrylate

185. Methyl parathion

186. Mevinphos

187. Mexacarbate

188. Monoethylamine

189. Monomethylamine

190. Naled

191. Naphthalene

192. Naphthenic acid

193. Nickel ammonium sulfate

194. Nickel chloride

195. Nickel hydroxide

196. Nickel nitrate

197. Nickel sulfate

198. Nitric acid

199. Nitrobenzene

200. Nitrogen dioxide

201. Nitrophenil

202. Nitrotoluene

203. Paraformaldehyde

204. Parathion

205. Pentachlorophenol

206. Phenol

207. Phosoene

208. Phosphoric acid

209. Phosphorus

210. Phosphorus oxychloride

211. Phosphorus pentasulfide

212. Phosphorus trichloride

213. Polychlorinated biphenyls (PCB)

214. Potassium arsenate

215. Potassium arsenite

216. Potassium bichromate

217. Potassium chromate

218. Potassium cyanide

219. Potassium hydroxide

220. Potassium permanganate

221. Propargite

222. Propionic acid

223. Propionic anhydride

224. Propylene oxide

225. Pyrethrins

226. Quinoline

227. Resorcinol

228. Selenium oxide

229. Silver nitrate

230. Sodium

231. Sodium arsenate

232. Sodium arsenite

233. Sodium bichromate

234. Sodium bifluoride

235. Sodium bisulfite

236. Sodium chromate

237. Sodium cyanide

238. Sodium dodecylbenzensulfonate

239. Sodium fluoride

240. Sodium hydrosulfide

241. Sodium hydroxide

242. Sodium hypochlorite

243. Sodium methylate

244. Sodium nitrate

245. Sodium phospate (dibasic)

246. Sodium phosphate (tribasic)

247. Sodium selenite

248. Strontium chromate

249. Strychnine

250. Styrene

251. Sulfuric acid

252. Sulfur monochloride

253. 2,4,5-T acid (2,4,5-Trichlorophenoxy acetic

acid)

254. 2,4,5-T amines (2,4,5-Trichlorophenoxy

acetic acid amines)

255. 2,4,5-T esters (2,4,5-Trichlorophenoxy

acetic acid esters)

256. 2,4,5-T salts (2,4,5-Trichlorophenoxy acetic

acid salts)

257. 2,4,5-TP acid (2,4,5-Trichlorophenoxy

propanoic acid)

258. 2,4,5-TP acid esters (2,4,5-Trichlorophenoxy

propanoic acid esters)

259. TDE (Tetrachlorodiphenyl ethane)

260. Tetraethyl lead

261. Tetraethyl pyrophosphate

262. Thallium sulfate

263. Toluene

264. Toxaphene

265. Trichlorofon

266. Trichloroethylene

267. Trichlorophenol

268. Triethanolamine dodecylbenzenesulfonate

269. Triethylamine

270. Trimethylamine

271. Uranyl acetate

272. Uranyl nitrate

273. Vanadium pentoxide

274. Vanadyl sulfate

275. Vinyl acetate

276. Vinylidene chloride

277. Xylene

278. Xylenol

279. Zinc acetate

280. Zinc ammonium chloride

281. Zinc borate

282. Zinc bromide

283. Zinc carbonate

284. Zinc chloride

285. Zinc cyanide

286. Zinc fluoride

287. Zinc formate

288. Zinc hydrosulfite

289. Zinc nitrate

290. Zinc phenolsulfonate

291. Zinc phosphide

292. Zinc silcofluoride

293. Zinc sulfate

294. Zirconium nitrate

295. Zirconium potassium fluoride

296. Zirconium sulfate

297. Zirconium tetrachloride

FORM 2CG, WASTEWATER APPLICATION PERMIT TO DISCHARGE PROCESS WASTEWATER FROM NEW OR EXISTING INDUSTRIAL WASTEWATER FACILITIES TO GROUND WATER

| Facility I.D. Number: |  |
| --- | --- |

Please print or type information in the appropriate areas.

I DISCHARGE LOCATION For each location, list the X,Y coordinates in decimal degrees and, where applicable, the name of the land application site.

| (1) Discharge | (2) Latitude (N) | (3) Longitude (W) | (4) Name of |
| --- | --- | --- | --- |
| Location No. (list) |  |  | Land Application |

II LAND APPLICATION DESIGN

| (1) Discharge No. | (2) Design Configuration and Construction Materials | (3) Cover Crop | (4) Application Rate | (5) Land Application Area | (6) Ground Water Classification |
| --- | --- | --- | --- | --- | --- |

| Facility I.D. Number: |  |
| --- | --- |

III FLOWS, SOURCES OF POLLUTION, AND TREATMENT TECHNOLOGIES

A. Attach a line drawing showing the water flow through the facility. Indicate sources of intake water, operations contributing wastewater to the effluent, and treatment units labeled to correspond to the more detailed descriptions in Item B. Construct a water balance on the line drawing by showing average flows between intakes, operations, treatment units, and discharge points. If a water balance cannot be determined (e.g., for certain mining activities), provide a pictorial description of nature and amount of any sources of water and any collection or treatment measures.

B. For each discharge location, provide a description of:

1. All operations contributing wastewater to the effluent; including process wastewater, sanitary wastewater, cooling water, and stormwater runoff;

2. The average flow contributed by each operation; and

3. The treatment received by the wastewater.

Use the space below. Continue on additional sheets, if necessary.

| (1) Discharge | (2) Operation(s) Contributing Flow |  | (3) Treatment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location No. | (a) Operation (list) | (b) Avg. Flow & Units | (a) Description | (b) List Code from Table 2CG-1 |  | (c) Final Disposal of Wastes Other Than by Discharge |

| Facility I.D. Number: |  |
| --- | --- |

C. Except for storm runoff, leaks, or spills, are any of the discharges described in Items II-A or B intermittent or seasonal?

Yes (complete the following table)	 No (go to D. below)

| (1) | (2) | (3) Frequency* |  | (4) Flow |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outfall # (List) | Operations(s) Contributing Flow (List) | (a) Days per Week (specify avg.) | (b) Months per Yr. (specify avg.) | (a) Flow Rate (in mgd) |  | (b) Total Volume (specify with units) |  | (c) Duration (in days) |
|  |  |  |  | Long Term Avg | Max. Daily | Long Term Avg. | Max. Daily |  |

*For land application systems, also include the Rest Period.

D. Describe practices to be followed to ensure adequate wastewater treatment during emergencies such as power loss and equipment failures causing shutdown of pollution abatement equipment of the proposed/permitted facilities.

E. List the method(s) and location(s) of flow measurement.

IV PRODUCTION

A. Do any effluent limitation guidelines (ELGs) promulgated by EPA under Section 304 of the CWA apply to your

facility?

Yes (complete the following table)	 No

| (1) ELG Category | (2) ELG Subcategory | (3) Regulatory Citation |
| --- | --- | --- |

B. Are any of the applicable ELGs expressed in terms of production (or other measure of operation)?

Yes (complete the following table)	 No

| (1) Outfall Number | (2) Operation, Production, or Material | (3) Quantity per Day | (4) Unit of Measure |
| --- | --- | --- | --- |

C. Are you requesting alternative limits based on an anticipated increase in the actual production during the next permit

term? (Consult with your NPDES permitting authority to determine what information needs to be submitted and

when.) 	 Yes	 No

| Facility I.D. Number: |  |
| --- | --- |

V IMPROVEMENTS

A. Are you now required by any Federal, State or local authority to meet any implementation schedule for the construction, upgrading or operation of wastewater treatment equipment or practices or any other environmental programs which may affect the discharges described in this application? This includes, but is not limited to, permit conditions, administrative or enforcement orders, enforcement compliance schedule letters, stipulations, court orders, and grant or loan conditions.

Yes (complete the following table)	 No

| (1) Identification of | (2) Affected Discharge Locations |  | (3) Brief Description | (4) Final Compliance Date |  |
| --- | --- | --- | --- | --- | --- |
| Condition, Agreement, Etc. | (a) No. | (b) Source of Discharge | Of Project | (a) Required | (b) Projected |

B. OPTIONAL: You may attach additional sheets describing any additional water pollution control programs (or other environmental projects which may affect your discharges) you now have underway or which you plan. Indicate whether each program is now underway or planned and indicate your actual or planned schedules for construction.

Mark "X" if description of additional control programs is attached.

VI INTAKE AND EFFLUENT CHARACTERISTICS

A, B, & C: See instructions before proceeding to Tables A, B, and C – Complete one set of tables for each discharge location – Annotate the location number in the space provided. NOTE: Tables VI-A, VI-B, and VI-C are included on separate sheets number VI-1 through VI-8.

D. Use the space below to list any of the pollutants listed in Table 2CG-3 of the instructions, which you know or have reason to believe is discharged or may be discharged from any outfall. For every pollutant you list, briefly describe the reasons you believe it to be present and report any analytical data in your possession.

| (1) Pollutant | (2) Source | (1) Pollutant | (2) Source |
| --- | --- | --- | --- |

VII POTENTIAL DISCHARGES NOT COVERED BY ANALYSIS

Is any pollutant listed in Item VI-C a substance or a component of a substance which you currently use or manufacture as an intermediate or final product or by-product?

YES (list all such pollutants below) 	 NO

| Facility I.D. Number: |  |
| --- | --- |

VIII BIOLOGICAL TOXICITY TESTING DATA

Do you have any knowledge or reason to believe that any biological test for acute or chronic toxicity has been made on any of your discharges or on a receiving water in relation to your discharge within the last 3 years?

YES (identify the test(s) and describe their purposes below)	 NO

IX CONTRACT ANALYSIS INFORMATION

Were any of the analyses reported in Item VI performed by a contract laboratory or consulting firm?

YES (list the name, address, telephone number, and certification number of, and pollutants analyzed by each such 	laboratory or firm below)

NO

| (1) Name | (2) Address | (3) Telephone (area code & no.) | (4) Pollutants Analyzed (list) |
| --- | --- | --- | --- |

X CONNECTION TO REGIONAL POTW

A. Indicate the relationship between this project and area regional planning for wastewater treatment. List steps to be taken for this industrial wastewater facility to become part of an area-wide wastewater treatment system.

| Facility I.D. Number: |  |
| --- | --- |

XI-A CERTIFICATIONS FOR NEW OR MODIFIED FACILITIES

(For NPDES permits, do not sign or submit electronically. This may change after December 21, 2028. Check with the appropriate DEP district office.)

This is to certify the engineering features of this pollution control project have been designed by me and found to be in conformity with sound engineering principles, applicable to the treatment and disposal of pollutants characterized in the permit application. There is reasonable assurance, in my professional judgment, that the pollution control facilities, when properly maintained and operated, will discharge an effluent that complies with all applicable statutes of the State of Florida and the rules of the Department. It is also agreed that the undersigned, if authorized by the owner, will furnish the applicant a set of instructions for the proper maintenance and operation of the pollution control facilities and, if applicable, pollution sources.

| Signature |  | Company Name |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Address |  |  |  |  |
| Name (please type) |  |  |  |  |  |  |
| (Affix Seal) |  | Florida Registration No.: |  |  |  |  |
|  |  | Telephone No:: |  |  | Date |  |

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gather and evaluate the information submitted. Based on my inquiry of the person or persons who manage the system or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

| Name (type or print) |  | Signature |
| --- | --- | --- |
| Title |  |  |
|  |  | Date Signed |
| Telephone No. (area code & No.) |  |  |

| Facility I.D. Number: |  |
| --- | --- |

XI-B CERTIFICATIONS FOR PERMIT RENEWALS

(For NPDES permits, do not sign or submit electronically. This may change after December 21, 2028. Check with the appropriate DEP district office.)

This is to certify the engineering features of this pollution control project have been examined by me and found to be in conformity with sound engineering principles, applicable to the treatment and disposal of pollutants characterized in the permit application. There is reasonable assurance, in my professional judgment, that the pollution control facilities, when properly maintained and operated, will discharge an effluent that complies with all applicable statutes of the State of Florida and the rules of the Department.

| Signature |  | Company Name |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Address |  |  |  |  |
| Name (please type) |  |  |  |  |  |  |
| (Affix Seal) |  | Florida Registration No.: |  |  |  |  |
|  |  | Telephone No:: |  |  | Date |  |

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gather and evaluate the information submitted. Based on my inquiry of the person or persons who manage the system or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

| Name (type or print) |  | Signature |
| --- | --- | --- |
| Title |  |  |
|  |  | Date Signed |
| Telephone No. (area code & No.) |  |  |

| Facility ID. Number: |  | Outfall No. |  |
| --- | --- | --- | --- |

PLEASE PRINT OR TYPE ONLY: You may report some or all of this information on separate sheets instead of completing these pages. Use the same format.

SEE INSTRUCTIONS.

# VI. INTAKE AND EFFLUENT CHARACTERISTICS

PART A - You must provide the results of at least one analysis for every pollutant in this table. Complete one table for each outfall. See instructions for additional details.

| 1. | 2. Effluent |  |  |  |  |  |  | 3.. Units |  | 4. Intake (optional) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pollutant | a. Max. Daily Value |  | b. Max. 30-day Value |  | c. Annual Avg. Value |  | d. No. of | a. Conc. | b. Mass | a. Long Term Avg. Value |  | b. No. of |
|  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | Analyses |  |  | (1) Conc. | (2) Mass | Analyses |
| a. CBOD5, Carbonaceous BOD, 5-day, 20 Deg. C. |  |  |  |  |  |  |  |  |  |  |  |  |
| b. Chemical Oxygen Demand (COD) |  |  |  |  |  |  |  |  |  |  |  |  |
| c. Total Organic Carbon (TOC) |  |  |  |  |  |  |  |  |  |  |  |  |
| d. Total Suspended Solids (TSS) |  |  |  |  |  |  |  |  |  |  |  |  |
| e. Total Nitrogen (as N) |  |  |  |  |  |  |  |  |  |  |  |  |
| f. Total Phosphorus (as P) |  |  |  |  |  |  |  |  |  |  |  |  |
| g. Ammonia (as N) |  |  |  |  |  |  |  |  |  |  |  |  |
| h. Flow - actual or projected | Value |  | Value |  | Value |  |  |  |  | Value |  |  |
| i. Flow - design | Value |  | Value |  | Value |  |  |  |  | Value |  |  |
| j. Specific Conductivity | Value |  | Value |  | Value |  |  |  |  | Value |  |  |
| k. Temperature (winter) | Value |  | Value |  | Value |  |  | °C |  | Value |  |  |
| l. Temperature (summer) | Value |  | Value |  | Value |  |  | °C |  | Value |  |  |
| m. pH | Min. | Max. | Min. | Max. |  |  |  | STANDARD UNITS |  |  |  |  |

*Conc. = Concentration

PART B - Mark "X" in column 2a for each pollutant you know or have reason to believe is present. Mark "X" in column 2b for each pollutant you believe to be absent. If you mark column 2a for any pollutant which is limited either directly, or indirectly but expressly, in an effluent limitations guideline, you must provide the results of at least one analysis for that pollutant. For other pollutants for which you mark column 2a, you must provide quantitative data or an explanation of their presence in your discharge. Complete one table for each outfall. See the instructions for additional details and requirements.

|  | 2. Mark "X" |  | 3. Effluent |  |  |  |  |  |  |  | 4. Units |  | 5. Intake (optional) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Pollutant and CAS No. (if available) | a. believed | b. believed | a. Maximum Daily Value |  |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  | d. No. of Analyses | a. Conc. | b. Mass | a. Long Term Avg. Value |  | b. No. of Analyses |
|  | present | absent | (1) Conc. | (2) Mass | (1) Conc. |  | (2) Mass | (1) Conc. | (2) Mass |  |  |  | (1) Conc. | (2) Mass |  |
| a. Bromide (24949-67-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b. Chlorine, Total Residual |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c. Color |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d. Fecal Coliform |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| e. Fluoride (16984-48-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f. Nitrate-Nitrite (as N) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Item VI-B Contd |  |  |  |  |  |  |  |  | Facility ID. Number: |  |  |  |  |  | Outfall No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2. Mark "X" |  | 3. Effluent |  |  |  |  |  |  |  |  | 4. Units |  |  | 5. Intake (optional) |  |  |  |
| 1. Pollutant and CAS No. (if available) | a.. believed present | b. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  | d. No. of Analyses |  | a. Conc. | b. Mass |  | a. Long Term Avg. Value |  |  | b. No. of Analyses |
|  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | (1) Conc. |  | (2) Mass |  |  |  |  |  | (1) Conc. |  | (2) Mass |  |
| g. Nitrogen, Total Organic (as N) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| h. Oil and grease |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i. Phosphorus, Total (as P) (7723-14-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| j. Radioactivity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Alpha, Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (2) Beta, Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (3) Radium, Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (4) Radium 226, Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| k. Sulfate (as SO4 ) (14808-79-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| l. Sulfide (as S) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| m. Sulfite (as SO3 ) (14265-45-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n. Surfactants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| o. Aluminum, Total (7429-90-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| p. Barium, Total (7440-39-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| q. Boron, Total (7440-42-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| r. Cobalt, Total (7440-48-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| s. Iron, Total (7439-89-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| t. Magnesium, Total (7439-95-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| u. Molybdenum, Total (7439-98-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v. Manganese, Total (7439-96-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| w. Tin, Total (7440-31-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| x. Titanium, Total (7440-32-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  | Outfall No. |  |
| --- | --- | --- | --- |

PART C - If you are a primary industry and this outfall contains process wastewater, refer to Table 2CG-2 in the instructions to determine which of the GC/MS fractions you must test for. Mark "X" in column 2a for all GC/MS fractions that apply to your industry and for ALL toxic metals, cyanides, and total phenols. If you are not required to mark column 2a (secondary industries, non-process wastewater outfalls, and non-required GC/MS fractions), mark "X" in column 2b for each pollutant you know or have reason to believe is present. Mark "X" in column 2c for each pollutant you believe is absent. If you mark column 2a for any pollutant, you must provide the results of at least one analysis for that pollutant. If you mark column 2b for any pollutant, you must provide the results of at least one analysis for that pollutant if you know or have reason to believe it will be discharged in concentrations of 10 ppb or greater. If you mark column 2b for acrolein, acrylonitrile, 2,4,dinitrophenol, or 2-methyl-4,6 dinitrophenol, you must provide the results of at least one analysis for each of these pollutants which you know or have reason to believe that you discharge in concentrations of 100 ppb or greater. Otherwise, for pollutants for which you mark column 2b, you must either submit at least one analysis or briefly describe the reasons the pollutant is expected to be discharged. Note that there are 7 pages to this part; please review each carefully. Complete one table (all 7 pages) for each outfall. See instructions for additional details and requirements.

|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  | 4. Units |  | 5. Intake (optional) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  | d. No. of Analyses | a. Conc. | b. Mass | a. Long Term Avg. Value |  | b. No. of Analyses |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass |  |  |  |  | (1) Conc. | (2) Mass |  |
| METALS, CYANIDE, AND TOTAL PHENOLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1M. Antimony, Total (7440-36-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2M. Arsenic, Total (7440-38-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3M. Beryllium, Total (7440-41-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4M. Cadmium, Total (7440-43-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5M. Chromium, Total (7440-47-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6M. Copper, Total (7440-50-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7M. Lead, Total (7439-92-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8M. Mercury, Total (7439-97-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9M. Nickel, Total (7440-02-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10M. Selenium, Total (7782-49-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11M. Silver, Total (7440-22-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12M. Thallium, Total (7440-28-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13M. Zinc, Total (7440-66-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14M. Cyanide, Total (57-12-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15M. Phenols, Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIOXIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2,3,7,8-Tetra- chlorodibenzo-P-Dioxin (1764-01-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GC/MS FRACTION -VOLATILE COMPOUNDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1V. Acrolein (107-02-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2V. Acrylonitrile (107-13-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  | Outfall No. |  |
| --- | --- | --- | --- |

|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  |  | 4. Units |  | 5. Intake (optional) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  |  | d. No. of Analyses | a. Conc. | b. Mass | a. Long Term Avg. Value |  | b. No. of Analyses |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass |  | (1) Conc. | (2) Mass |  |  |  |  | (1) Conc. | (2) Mass |  |
| GC/MS FRACTION - VOLATILE COMPOUNDS (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3V. Benzene (71-43-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4V. Bis (Chloromethyl) Ether (542-88-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5V. Bromoform (75-25-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6V. Carbon Tetrachloride (56-23-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7V Chlorobenzene (108-90-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8V. Chlorodi- bromomethane (124-48-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9V. Chloroethane (75-00-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10V. 2-Chloro-ethylvinyl Ether (110-75-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11V. Chloroform (67-66-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12V. Dichloro- bromomethane (75-27-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13V. Dichloro- difluoromethane (75-71-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14V. 1,1-Dichloroethane (75-34-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15V. 1,2-Dichloroethane (107-06-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16V. 1,1-Dichloroethylene (75-35-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17V. 1,2-Dichloropropane (78-87-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18V. 1,3-Dichloropropylene (542-75-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19V. Ethylbenzene (100-41-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20V. Methyl Bromide (74-83-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21V. Methyl Chloride (74-87-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22V. Methylene Chloride (75-09-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23V. 1,1,2,2-Tetra- chloroethane (79-34-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24V. Tetrachloroethylene (127-18-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  | Outfall No. |  |
| --- | --- | --- | --- |

|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  | 4. Units |  | 5. Intake (optional) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  | d. No. of Analyses | a. Conc. | b.Mass | a. Long Term Avg. Value |  | b. No. of Analyses |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass |  | (1) Conc. | (2) Mass |  |  |  | (1) Conc. | (2) Mass |  |
| GC/MS FRACTION - VOLATILE COMPOUNDS (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25V. Toluene (108-88-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26V. 1,2-Trans- Dichloroethylene (156-60-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27V. 1,1,1-Trichloroethane (71-55-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28V. 1,1,2-Trichloroethane (79-00-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29V. Trichloroethylene (79-01-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30V. Trichloro- fluoromethane (75-69-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31V. Vinyl Chloride (75-01-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GC/MS FRACTION - ACID COMPOUNDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1A. 2-Chlorophenol (95-57-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2A. 2,4-Dichlorophenol (120-83-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3A. 2,4-Dimethylphenol (105-67-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4A. 4,6-Dinitro-O-Cresol (534-52-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5A. 2,4-Dinitrophenol (51-28-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6A. 2-Nitrophenol (88-75-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7A. 4-Nitrophenol (100-02-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A P-Chloro-M-Cresol (59-50-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9A Pentachlorophenol (87-86-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10A Phenol (108-95-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11A 2,4,6-Trichloro- phenol (88-06-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GC/MS FRACTION - BASE/NEUTRAL COMPOUNDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1B. Acenaphthene (83-32-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2B. Acenaphtylene (208-96-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3B. Anthracene (120-12-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4B. Benzidine (92-87-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  |  |  |  |  |  | Outfall No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  |  |  | 4. Units |  |  | 5. Intake (optional) |  |  |  |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  |  | d. No. of Analyses |  | a. Conc. | b. Mass |  | a. Long Term Avg. Value |  |  | b. No. of Analyses |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | (1) Conc. |  | (2) Mass |  |  |  |  |  |  | (1) Conc. |  | (2) Mass |  |
| 5B. Benzo (a) Anthracene (56-55-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B. Benzo (a) Pyrene (50-32-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7B. 3,4-Benzo-fluoranthene (205-99-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8B. Benzo (ghi) Perylene (191-24-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9B. Benzo (k) Fluoranthene (207-08-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10B. Bis (2-Chloroethoxy) Methane (111-91-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11B. Bis (2-chloroethyl) Ether (111-44-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12B. Bis (2-Chloroisopropyl) Ether (39638-32-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13B. Bis (2-Ethylhexyl) Phthalate (117-81-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14B. 4-Bromophenyl Phenyl Ether (101-55-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15B Butyl Benzyl Phthalate (85-68-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16B. 2-Chloronaphthalene (91-58-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17B. 4-Chlorophenyl Phenyl Ether (7005-72-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18B. Chrysene (218-01-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19B. Dibenzo (a,h) Anthracene (53-70-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20B. 1,2-Dichlorobenzene (95-50-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21B. 1,3-Dichlorobenzene (541-73-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22B. 1,4-Dichlorobenzene (106-46-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23B. 3,3'-Dichlorobenzidine (91-94-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24B. Diethyl Phthalate (84-66-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25B. Dimethyl Phthalate (131-11-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26B. Di-N-Butyl Phthalate (84-74-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27B. 2,4-Dinitrotoluene (121-14-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28B. 2,6-Dinitrotoluene (606-20-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  |  |  |  |  |  | Outfall No. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  |  |  |  |  | 4. Units |  |  | 5. Intake (optional) |  |  |  |  |  |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  |  | c. Long Term Avg. Value (if available) |  |  |  |  | d. No. of Analyses |  | a. Conc. | b. Mass |  | a. Long Term Avg. Value |  |  |  | b. No. of Analyses |  |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass |  |  | (1) Conc. |  | (2) Mass |  |  |  |  |  |  | (1) Conc. | (2) Mass |  |  |  |  |
| 29B. Di-N-Octyl Phthalate (117-84-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30B. 1,2-Diphenylhydrazine (as Azobenzene) (122-66-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31B. Fluoranthene (206-44-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32B. Fluorene (86-73-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33B. Hexachlorobenzene (118-74-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34B. Hexachlorobutadiene (87-68-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35B. Hexachlorocyclopentadiene (77-47-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36B. Hexachloroethane (67-72-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37B. Indeno (1,2,3-cd) Pyrene (193-39-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38B. Isophorone (78-59-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39B. Naphthalene (91-20-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40B. Nitrobenzene (98-95-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41B N-Nitrosodimethylamine (62-75-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42B. N-Nitrosodi-N- Propylamine (621-64-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43B. N-Nitro-sodiphenylamine (86-30-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44B Phenanthrene (85-01-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 45B. Pyrene (129-00-0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46B. 1,2,4-Trichlorobenzene (120-82-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GC/MS FRACTION - PESTICIDES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1P. Aldrin (309-00-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2P. alpha-BHC (319-84-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3P beta-BHC (319-85-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4P. gamma-BHC (Lindane) (58-89-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5P. delta-BHC (319-86-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Facility ID. Number: |  |  |  |  |  | Outfall No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2. Mark "X" |  |  | 3. Effluent |  |  |  |  |  |  |  |  | 4. Units |  |  | 5. Intake (optional) |  |  |  |
| 1. Pollutant and CAS No. (if available) | a. . testing required | b. believed present | c. believed absent | a. Maximum Daily Value |  | b. Max. 30-day Value (if available) |  | c. Long Term Avg. Value (if available) |  |  | d. No. of Analyses |  | a. Conc. | b. Mass |  | a. Long Term Avg. Value |  |  | b. No. of Analyses |
|  |  |  |  | (1) Conc. | (2) Mass | (1) Conc. | (2) Mass | (1) Conc. |  | (2) Mass |  |  |  |  |  | (1) Conc. | (2) Mass |  |  |
| 6P. Chlordane (57-74-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7P. 4,4'-DDT (50-29-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8P. 4,4'-DDE (72-55-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9P. 4,4'-DDD (72-54-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10P. Dieldrin (60-57-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11P. alpha-Endosulfan (959-98-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12P. beta-Endosulfan (33213-65-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13P. Endosulfan Sulfate (1031-07-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14P. Endrin (72-20-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15P. Endrin Aldehyde (7421-93-4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16P. Heptachlor (76-44-8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17P. Heptachlor Epoxide (1024-57-3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18P. PCB-1242 (53469-21-9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19P. PCB-1254 (11097-69-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20P. PCB-1221 (11104-28-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21P. PCB-1232 (11141-16-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22P. PCB-1248 (12672-29-6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23P. PCB-1260 (11096-82-5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24P. PCB-1016 (12674-11-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25P. Toxaphene (8001-35-2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sampling requirements are periodically reviewed in light of recent research on testing methods. Upon completion of the review, changes to sampling requirements may be made. Before starting any required sampling or submitting past sampling to the Department, be sure that you have a current copy of 40 CFR Part 136 or Chapter 62-160, Florida Administrative Code.

The pollutants in each fraction are listed in Item VI-C

X - Testing required.