## FT 3100.  Stream and River Habitat Assessment

This sampling procedure requires specific training and a demonstration of competency due to the expert judgment exercised during field sampling.  Individuals conducting this procedure must train with trainers who are currently in “pass” status per FA 5720 (it is recommended to train with DEP staff via workshops and/or participating in field sampling).  It is required that samplers complete the training and testing requirements listed in FA 5720 to submit data to FDEP.

##### Equipment and Supplies

###### Completed Physical/Chemical Characterization Field Sheet (FD 9000-3) or other datasheet to capture documentation required in FD 5311; see FT 3001, section 2

###### Completed Stream/River Habitat Sketch Sheet (FD 9000-4) or other datasheet to capture documentation required in FD 5312; see FT 3001, section 2.4

###### Stream/River Habitat Assessment Field Sheet (required DEP Form FD 9000-5)

###### Pen

###### D-frame dip net with U.S. No. 30 mesh and handle marked in 0.1-m increments

##### Methods

##### Fill in the information requested at the top of the Stream/River Habitat Assessment Field Sheet (FD 9000-5), including the STORET station number, sampling date, sampling location, field identification and receiving body of water.  Record the time of sampling as described in FT 3001, section 2.1.

##### Follow the criteria given on the data sheet within each category to determine the appropriate score for that category.

##### Score the Substrate Diversity by evaluating the number of different kinds of productive substrates present.  Refer to the documentation obtained from conducting the Physical/Chemical Characterization per FT 3001.  The following substrates are considered productive:  snags (woody debris or logs larger than thumb diameter); roots (less than thumb diameter, with finer roots usually being more productive); aquatic vegetation (in contact with the water); leaf packs/mats in association with flow (leaves must be partially decomposed to be better habitat; leaf mats at the bottom may be productive if sufficient oxygen is present, but anaerobic leaf mats are not considered productive habitat); rocky substrate (usually limestone outcrops with rock diameters greater than 5 cm).  A minimum occurrence of two square meters of a particular substrate (habitat) in the reach is necessary to count that habitat as being “major;” those productive habitats with less than two square meters shall be considered “minor.”  Once the number of major habitats has been determined, assign a score for substrate diversity in the appropriate spot on the sheet.  (Higher values indicate a better condition than lower values.)  The quality of the substrates present should then be given consideration in the scoring process.  For example, partially decomposed leaf packs and “old” snags are better than fresh substrates and should be given higher scores within the same category.   See SOP SCI 1100 for more descriptions of habitat quality and selection.

##### Substrate Availability is the relative spatial abundance of productive habitats present.  Refer to documentation obtained from conducting the Physical/Chemical Characterization.  A minimum occurrence of two square meters of a particular substrate in the reach is necessary to count that substrate as being “major.”  Include only major productive habitats in the scoring process, even if your map included productive habitats that had less than two square meters coverage.  Score substrate availability on the data sheet based on the sum of the percentages of major productive habitats in the stream reach.

##### Using the ranges given on the data sheet (FD 9000-5), assign a Water Velocity score based on the maximum velocity observed at the typical cross-section of stream or river as determined during the Physical/Chemical Characterization (FT 3001 section 2.10).  Avoid areas immediately before or after snags or other material that restrict or enhance the velocity unless this is typical of the majority of the run. Note that in the majority of Florida streams, velocities over 1 m/s are considered unusually high, and should be included in the “poor” category.  An exception to this policy would be in narrow or shallow areas of streams with natural limestone bottoms, where velocities approaching 1 m/s may be normal and, thus, would be scored in the “optimal” category.  Velocities 0.33 m/s or greater but less than 1 m/s shall receive a 20.

##### The Habitat Smothering parameter is an assessment of sand and silt deposition onto what would otherwise be productive habitats.  Scoring is a two-step process.  Assign a habitat smothering score as determined by the following two steps:

Pool

Prevailing Depth

First, determine  if adequate stable pools are present.  For large, wide rivers it may be more appropriate to base the estimate on the actual amount of smothering on the habitats rather than the number of pools.  A pool is defined as an area where the depth is at least 2 times the prevailing depth and is expected to maintain that depth throughout rain events.

Shallow Pool

Prevailing Depth

A natural system should have 1 to 2 pools every 12 times the width of the stream.  For example, a 3 meter wide stream should have at least 1 pool every 36 meters or a total of 3-6 pools per 100 meter reach (100m/36m = 2.8 segments).  If there are no stable pools; i.e., the stream depth is nearly the same throughout the 100m reach, assign a score in the “poor” category.  If there are minimal (less than 1 pool every 12 times the width) or shallow pools (a shallow pool is any pool where the depth is much less than 2 times the prevailing depth), score the stream in the “marginal” category.

##### Pools should occur on the outside of curves in the stream and on the downstream side of large, woody debris.  A score in the “suboptimal” or “optimal” categories should be assigned to a stream with adequate pools based on the percent smothering as described in 2.6.2 below.

##### Second, check for deposition of sand or silt, or excessive growth of algae (> 6 mm thick), on visible habitats.  While a light dusting of sand or silt and some algal growth is normal, excessively thick coatings will reduce habitability of the substrate.  Smothering on visible habitats is indicated if sand, silt, or algae is present on a substrate in an amount greater than typically expected (3-6 mm).  Determine a percentage value for visible habitats that are not habitable due to sand and/or silt and/or algal smothering.   If less than 25% of habitats are smothered and adequate stable pools are present, score in the optimal category, and score in the suboptimal if more than 25% of habitats are smothered, for any kind of smothering (including algal).  If there is a high degree (> 50%) of algal smothering but adequate stable pools are present, score in the suboptimal, and use the rapid periphyton survey (DEP SOP FS 7230) for further investigation of algal coverage.

##### Add the scores for the primary habitat components (see sections 2.3-2.6 above) and record this primary score on the form.  The primary habitat components refer to in-stream features.

##### Observe whether or not the reach of stream or river in the sampling area is artificially channelized.  Assign a score for Artificial Channelization using the following guide:

##### Poor-  A highly physically altered system with ALL of the following; straightened stream channel (there may be human engineered “bends”, but not natural sinuousity), a trapezoidal or box-cut cross-section, and a lack of the required pools as described in 2.6 above (generally a monotypic depth).  Spoil banks or other indications of dredging may be visible.

##### Marginal-  A physically altered, channelized system with a trapezoidal cross section, but with either a small degree of sinuosity, often developed within the old dredged area. Spoil banks may be visible.

##### Suboptimal-  Good sinuosity has developed within and outside of the old channelized area.  Spoil banks may be visible, but generally have established vegetation growing on them.

##### Optimal-  A system with expected stream channel sinuosity given the width and slope of the stream; a stream should have as many bends as pools, as described in 2.6.1, unless the pools were formed solely by scouring behind trees or snags.  No evidence of dredging or artificial straightening.

##### Refer to FD 9000-4 for areas along the bank that have eroded or have the potential for bank sloughing.  Score artificially stable banks such as concrete according to bank stability, not according to natural vs. artificial stability.  Determine the extent of erosion potential for the site and assign a Bank Stability score for each bank (The “left bank” is on your left when you are looking upstream).

##### First, determine where “bankfull” is in relation to the height of each bank.  Bankfull is defined as the stage at which channel maintenance is most effective and occurs on average every 1-2 years.  For most natural Florida streams, bankfull is the height of the lowest bank, where the stream is connected to the floodplain.  For stream sites with a wetland floodplain, bankfull is usually the elevation of the flat floodplain.  For stream sites with an upland floodplain, bankfull is usually the inflection point on the bank.

Bankfull

Floodplain

##### Other indicators of bankfull (especially in larger systems) are the tops of point bars, staining and vegetation lines.  If the substrate at bankfull is limestone, pipe clay or concrete, then automatically score the bank in the “optimal” category and skip 2.9.2 and 2.9.3 below.  Ideally, bankfull should be greater than 60% of the bank height or above the woody root zone.  If this is the case, the bank gets a “plus” for this subcomponent.  Otherwise, bankfull is less than 60% of bank height and below the woody root zone and it should receive a “minus.”

##### Second, determine the slope of the portion of bank above bankfull.  The more gentle the slope the more stable the bank.  Score a bank with a slope less than 60° with a plus for this subcomponent.  A bank with a slope of greater than 60° warrants a minus.

##### Third, determine if bankfull is above or below the root zone.  If bankfull is above the root zone and there are few raw or eroded areas, score this subcomponent a plus.  Otherwise, score it a minus.  Woody vegetation/roots are more stable than herbaceous and should be scored accordingly.

##### Lastly, count up the number of pluses from each subcomponent (a total of 3 possible) and score within each category as described below:

##### Poor- 0 pluses

##### Marginal- 1 plus

##### Suboptimal- 2 pluses

##### Optimal- 3 pluses

##### Assign a score for the Riparian Buffer Zone Width that best characterizes the width of vegetation on each side of the channel.  This zone is measured from the edge of the stream bank to where clearing or other adverse human activity begins.  Take into account the intensity of the disturbance and score accordingly.  For example, a footpath that runs along one bank for 20 meters is much less intense than a paved road that runs along the same 20 meter stretch. A native vegetated buffer zone of greater than 18 m (approximately 60 feet) is currently considered optimal. A riparian zone that is vegetated but mowed regularly is considered poor.

##### Identify the plants in the riparian zone, determining the extent of coverage and whether the vegetation is native or exotic.  Look for these classes of plants:  bottomland or mesic hardwoods, understory shrubs and non-woody macrophytes.  Assign a Riparian Zone Vegetation Quality score based on the classes of plants present, the degree of bank vegetative cover, and how closely the plant community at the site approaches that expected of an undisturbed community in the region.  If there is obvious riparian substrate disruption by exotic animals or humans, these disturbances should be considered during the scoring process.

##### Add the scores for the secondary habitat components (see sections 2.8-2.11) and record this secondary score on the form.  The secondary habitat components refer to morphological and riparian zone features.

##### Add the primary score (see section 2.7) and the secondary score (see section 2.12) to get the habitat assessment total score.  Record the habitat assessment total score on the form.

##### Sign and date the form (FD 9000-5).