PART III – STORMWATER QUANTITY/FLOOD CONTROL

3.0	General Flood Control Performance Criteria

Urbanization increases total runoff volume, peak discharge rates, and the

magnitude and frequency of flood events. With an increase in the number of flood events a stream is subjected to, the potential for accelerated erosion of both the stream banks and channel bottom is enhanced. Proper design of detention systems to limit post-development peak discharge rates to predevelopment rates can minimize some of the stormwater effects of urbanization.

3.1	Projects That Must Meet Water Quantity Criteria

Projects that do not exceed the thresholds listed in paragraphs 3.1 (a)–(h) below 	shall have a rebuttable presumption that they meet the criteria for issuance in 	paragraphs 2.0 (a)–(c) if they are designed to meet the standards listed in 	subsections 3.2.1 (a) and 3.3.1. Projects that exceed any of the thresholds listed in 	paragraphs 3.1 (a)–(h) shall have a rebuttable presumption that they meet the 	criteria for issuance in paragraphs 2.0 (a)–(c) if they are designed to meet the 	standards in subsections 3.2.1, 3.3.1, 3.3.2, 3.4.1, 3.5.1 and 3.5.2.

(a) 	Is capable of impounding a volume of water of 40 or more acre-			feet;

(b) 	Serves a project with a total land area equal to, or exceeding 40 			acres;

(c) 	Provides for the placement of 12 or more acres of impervious 			surface which constitutes 40 or more percent of the total land area;

(d) 	Is wholly or partially located in, on, or over any wetland or other 			surface water.

(e)	Serves a project with a total land area equal to or exceeding ten acres, 		when any part of the project is located within the Wekiva River 			Hydrologic Basin north of State Road 436, within the 				Econlockhatchee River Hydrologic Basin, within the Tomoka River 			Hydrologic Basin, or within the Spruce Creek Hydrologic Basin;

(f)	Provides for the placement of one-half acre or more of impervious 			surface, when any of the impervious surface is located within the 			Wekiva River Hydrologic Basin north of State Road 436;

(g)	Provides for the placement of two acres or more of impervious 			surface, when any of the impervious surface is located within the 			Econlockhatchee River Hydrologic Basin, within the Tomoka River 			Hydrologic Basin, or within the Spruce Creek Hydrologic Basin; or

(h)	Is wholly or partially located within the Wekiva River Hydrologic 			Basin's 	Riparian Habitat Protection Zone as described in Paragraph 			40C-41.063(3)(e).

3.2	Design Standards for Flood Protection

3.2.1	Water Quantity

(a)	The post-development peak discharge rate must not exceed the 			pre-development peak rate of discharge for the mean annual 24-			hour storm for systems serving both of the following:

(1)	New construction area greater than 50% impervious 				(excluding waterbodies)

(2)	Projects for the construction of new developments that 				exceed the thresholds in paragraphs 62-330.020(2)(b) or 				(c), F.A.C.

Note: Both of these conditions must be met before a project is required to comply

with the peak discharge criterion. Also, projects which modify existing systems are exempt from this criterion pursuant to condition 2., above. Pervious concrete and turf blocks are not considered impervious surface for this purpose, however,

compacted soils and limerock are considered impervious for purposes of this subsection.

(b)	The post-development peak rate of discharge must not exceed the 			pre-development peak rate of discharge for the 25-year frequency, 			24-hour duration storm for all areas of the District except:

(1)	For those systems which discharge directly into the St. 			Johns River north of Lake George, the man-made portions 			of the Intracoastal Waterway, the Intracoastal Waterway 			north of the Matanzas Inlet, or the Atlantic Ocean.

(2)	Where separate basin criteria have been adopted (see 				section 13.0).  Projects located in areas for which

separate basin criteria have been developed must meet the flood 			protection design standards specified by the basin criteria.

(c)	The post-development volume of direct runoff must not exceed the 		pre-development volume of direct runoff for the 25-year 				frequency, 96-hour duration storm for systems discharging to land-			locked lakes which are adjacent to properties of more than one 			ownership.  These systems shall not cause an increase in the total 			pre-development flood stage. This can be accomplished through 			retention with percolation or, if the soil conditions are not 				sufficient for percolation, then through detention for a duration 			sufficient to mitigate adverse impacts on flood stages.

(d)	Systems which are within areas for which separate basin criteria 			have been adopted pursuant to Chapter 40C-41, F.A.C., must meet 			the applicable volume and rate requirements found in section 13.0.

3.2.2	Alternative Peak Discharge Criteria

The applicant has two options for providing reasonable assurances that the standards referenced in subsection 3.2.1 (a) and (b) are met. The applicant may make such demonstration through compliance with the criteria specified in subsection 3.2.1 (a) and (b) or applicants may propose to utilize the applicable storm event duration, or criteria specified by a local government, state agency (including FDOT), or stormwater utility with jurisdiction over the project or  by use of alternative methods as may be appropriate for the specific system. However, the District must review and approve the use of the alternative criteria.  Applicants proposing to use alternative criteria are encouraged to have a preapplication conference with District staff.

3.2.3 	Methodologies

(a)	A peak discharge analysis typically consists of generating pre-			development and post-development runoff hydrographs, routing the 			post-development hydrograph through a detention basin, and sizing 			an overflow structure to control post-development discharges at or 			below predevelopment rates.

Peak discharge computations should consider the duration, 				frequency, and intensity of rainfall, the antecedent moisture 				conditions, upper soil zone and surface storage, time of 				concentration, tailwater conditions, changes in land use or land 			cover, and any other changes in topographic and hydrologic 				characteristics. Large systems should be divided into subbasins 			according to artificial or natural drainage divides to allow for more 			accurate hydrologic simulations.

The Natural Resources Conservation Service (NRCS) Curve 				Number Method is an example of an accepted methodology (see SJ 			No. 85-5:  A Guide to SCS Runoff Procedures (1985) and

incorporated by reference in 40C-4.091(1)(e)).

(b)	The modified rational method (see Appendix C of this Volume, incorporated

by reference in 40C-4.091(1)(a)) is a popular method for estimating peak

runoff rates for small urban areas.  The rational method gives peak discharge

rates rather than a runoff hydrograph.  The modified rational formula can be

modified to generate a runoff hydrograph by 	utilizing the rainfall intensity

for various increments of a design storm.  Similar to the rational method, use

of the modified rational hydrograph method should be limited to small

drainage basins with short times of concentration.  Therefore, the modified

rational method shall only be used for systems meeting the following

criteria:

(1)	The drainage area is less than 40 acres.

(2)	The predevelopment time of concentration for the system is 			less than 60 minutes.

(3)	The post-development time of concentration for the system is 		less than 30 minutes.

The modified rational hydrograph method shall only be utilized for the mean annual return frequency, 24-hour duration storm.

3.2.4	Computer Programs Accepted by the District

Numerous computer programs have been written to solve the runoff hydrograph and

detention basin routing calculations required in a peak discharge analysis.  If the

model is sound from a theoretical standpoint and the results compare favorably with

those of a benchmark standard model (such as HEC-1), the program is accepted for

use.

3.2.5 	Aggregate Discharge

Depending on the location and design of large systems where multiple off-site

discharges are designed to occur, the District may allow the total post-development

peak discharge not to exceed the pre-development peak discharge for the combined

discharges rather than for each individual discharge. Such a consideration shall be

made only if the combined discharges meet all other requirements of chapter 62-

330, F.A.C., and discharge to the same receiving water body.

3.2.6 	Rainfall Intensity

In determining peak discharge rates, intensity of rainfall values shall be obtained

through a statistical analysis of historical long term rainfall data or from sources or

methods generally accepted as good engineering practice.

(a)	Examples of acceptable sources include:

(1)	USDA Soil Conservation Service, "Rainfall Frequency Atlas of Alabama, Florida, Georgia, and South Carolina for Durations from 30 Minutes to 24 Hours and Return Periods from 1 to 100 Years" January 1978; Gainesville, Florida.

(2)	U.S. Weather Bureau Technical Paper No. 49.

(3)	U.S. Weather Bureau Technical Paper No. 40.

(4)	U.S. Department of Interior, Bureau of Reclamation, "Design of Small Dams", 2nd Edition.

(b)	For a drainage basin greater than 10 square miles, the areal rainfall can be calculated from point rainfall using a method that has been well documented. The converting factor as described in U.S. Weather Bureau Technical Paper No. 49 may be used.

3.2.7	System Recovery

Where basins are designed for reducing post-development peak rate discharge and volume, the outlet and regulation schedule should be designed to provide necessary design detention and retention storage within 14 days following any storm event.

3.2.8 	Upper Soil Zone Storage and Surface Storage

In most instances, the upper soil zone storage and surface storage capacities will have an effect on the pre-development and post-development peak discharges and should be considered in these computations. Any generally accepted and well documented method may be used to develop the upper soil zone storage and surface storage values.

(a)	The soil zone storage at the beginning of a storm should be estimated by

using reasonable and appropriate parameters to reflect drainage practices,

average wet season water table elevation, the antecedent moisture condition

(AMC II) and any underlying soil characteristics which would limit or

prevent percolation of storm water into the entire soil column. In no case

should the soil storage 	used in the computation exceed the difference

between the maximum soil water capacity and the field capacity (i.e.,

gravitational water) for the soil columns above any impervious layer or

seasonal ground water table.

(b)	Surface storage, including that available in wetlands and low lying areas,

shall be considered as depression storage. Depression storage shall be

analyzed for its effect on peak discharge and the time of concentration.

Depression storage can also be considered in post-development storage

routing which would require development of stage-storage relationships; if

depression storage is considered, then both pre-development and post-

development storage routing must be considered.

3.3	Storage and Conveyance

3.3.1	Conveyance

Projects which alter existing conveyance systems (e.g., rerouting an existing 	ditch) must not adversely affect existing conveyance capabilities. It is presumed a 	system will meet this criterion if one of the following are met:

(a)	The existing hydraulic capacity is maintained in the new system. 			This can be accomplished by maintaining existing headwater and 			tailwater conditions.

(b)	The applicant demonstrates that changes in flood elevation and 			velocities will not adversely impact upstream or downstream off-			site property. For example, this criterion may be satisfied by 			demonstrating that there is no increase in damages to existing off-			site property (e.g., roads, buildings) resulting from changes in the 			existing flood elevations. Also, the applicant should demonstrate 			that proposed velocities are non-erosive or that erosion control 			measures (e.g., rip-rap, concrete lined channels, etc.) are sufficient 			to safely convey the flow.

(c)	The criterion in paragraph 3.3.2(b) is met.

(d)	As an alternative, the applicant may propose to utilize an 				applicable criterion established by a local government, state 			agency, or stormwater utility with jurisdiction over the project. 			However, District staff must approve the use of this criterion.

3.3.2 	Floodways and Floodplain Storage

Floodways and floodplains, and levels of flood flows or velocities of adjacent streams, impoundments or other water courses must not be altered so as to adversely impact the off-site storage and conveyance capabilities of the water resource.  It is presumed a system will meet this criterion if the following are met:

(a)	A system may not cause a net reduction in flood storage within a 10-	year floodplain except for structures elevated on pilings or traversing works. Traversing works, works or other structures shall cause no more than a one-foot increase in the 100-year flood elevation immediately upstream and no more than one tenth of a foot increase 	in the 100-year flood elevation 500 feet upstream. A system will not cause a net reduction in flood storage within a 10-year floodplain if compensating storage is provided outside the 10-year floodplain.

(b)	A system may not cause a reduction in the flood conveyance 	capabilities provided by a floodway except for structure elevated on pilings or traversing works. Such works, or other structures shall cause no more than a one-foot increase in the 100-year flood 	elevation immediately upstream and no more than one-tenth of a foot increase in the 100-year flood elevation 500 feet upstream.

(c)	An applicant may only be permitted to contravene the requirements of (a) or (b) if the applicant gives reasonable assurance that were all other persons who could impact the surface water of any impoundment, stream, or other watercourse by floodplain encroachment to exceed (a) and (b) above to the same degree as the applicant proposes, the cumulative impacts would not contravene subparagraphs 62-330.301(1)(a)-(c), F.A.C.

3.3.3	Level of Service

In evaluating the potential for flood damages to residences, public buildings, or public streets and roadways, the following criteria will be utilized:

(a)	Any proposed streets and roadways must be flood-free as required by local government criteria; and

(b)	The first floor of any proposed building used for residence or as a public facility, must be set at or above an elevation adopted by local ordinance or, where a local ordinance has not been adopted, at the 100-year flood elevation calculated by the District, or approved by the District based upon the determination of the applicant.

3.3.4 	Floodway and Floodplain Elevation Determination

(a)	The floodway and floodplain criteria contained in subsection 	3.3.2 are applicable only to locations downstream of the point on a stream or watercourse where the drainage area is five square miles.

(b)	The District has detailed information regarding floodplain and floodway

elevations for many of the streams and water courses in its jurisdiction. The

applicant is encouraged to consult with the District prior to making

calculations. Other sources of information include the most recently

published data from flood insurance rate studies or relevant engineering

reports. If data are not available from the District, the flood insurance rate

studies, or published reports, the applicant will be responsible for making

determination of floodplain elevations or floodway limits using the

procedure of "Normal Depth Analysis", extrapolation of existing data, or any

other acceptable technique.

3.3.5	Flow Velocity

Good engineering practices shall be employed to minimize the flow velocity to avoid transport of soil particles and other suspended solids from one location and deposition in another location. Many different measures, structural or non- structural, may be used to reduce erosion from the bottom and side slopes of a conveyance system or around the control structures. However, velocity reduction measures and re-vegetation with naturally occurring species of the area should be considered before using other methods of bottom and side slope consolidation.

3.4	Dams and Impoundments

3.4.1 	Dams greater than six feet in height or which have a storage capacity of greater than 15 acre-feet of water which could be released in the event of dam failure shall have a spillway system with a capacity to pass the flow resulting from the design storm indicated in Table 3.4.1-1, without overtopping the dam unless the applicant provides analyses to show that the design flood can be stored, passed through, or passed over the dam without failure occurring.

TABLE 3.4.1-1

| Hazard Classification | Storage Capacity (acre-feet) | Height (feet) | Design Storm |
| --- | --- | --- | --- |
| A | > 15 | > 6 | 25-yr. |
| B | < 1000 | < 40 | 25-yr |
|  |  | 40-100 | 1/2 PMP |
|  |  | > 100 | PMP |
|  | 1000-50,000 | < 100 | 1/2 PMP |
|  |  | > 100 | PMP |
|  | 50,000 | > 6 | PMP |
| C | < 1000 | < 40 | 1/2 PMP |
|  |  | > 40 | PMP |
|  | > 1000 | > 6 | PMP |

3.4.2	Dams greater than six feet in height or which have a storage capacity of greater than 15 acre-feet of water which could be released in the event of dam failure shall have a spillway system capable of removing from the reservoir at least 80% of the water detained in the 	reservoir above the principal spillway within ten days after passage of the design storm.

3.5   Low Flow and Base Flow Maintenance

Flows of adjacent streams, impoundments or other water courses must not be decreased so as to cause adverse impacts.

3.5.1	Low Flow

(a)	Only systems with both of the following conditions meet the low flow

performance criteria in (b) and (c).

(1)	Systems which impound water for purposes in addition to temporary

detention storage. Water impounded longer than a 14-day bleed

down period is considered conservation storage for benefits other

than detention storage (i.e., recreation, irrigation, etc.).

(2)	Systems that impound a stream or other water course which, under

pre-development conditions, discharged surface water off-site to

receiving water during 5-year, 30-day drought frequency conditions.

(b)	Any system meeting the conditions of (a) above shall be designed with an outlet structure to maintain a low flow discharge of available 	conservation storage. When the conservation storage is at the average dry season design stage, the low flow discharge should equal the average pre-development surface water discharge which occurred from the project site to receiving waters during the 5-year, 30-day drought.

(c)	The system shall be operated to provide a low flow discharge whenever

water is impounded. However, discharge may be discontinued, if desired,

during the wet season (considered as June through October) unless a water

shortage condition is declared by the District. The actual discharge will vary

according to the water stage in the impoundment. When conservation storage

is at the average dry season design stage, the discharge will be the average 5

year, 30-day low flow. When storage is below the average dry season design

stage, the discharge may be less than the average 5-year, 30-day low flow.

3.5.2	Base Flow

It is presumed that an adverse impact will result if the system causes the ground water table to be lowered:

(a)	more than an average three feet lower over the project area than the 			average dry season low water table; or

(b)	at any location, more than five feet lower than the average dry season 		low water table; or

(c)	to a level that would decrease the flows or levels of surface water bodies below any minimum level or flow established by the Governing Board pursuant to section 373.042, F.S.