# Biological Reconnaissance Field Method

The Biological Reconnaissance (BioRecon) sampling procedure requires specific training and a demonstration of competency due to the expert judgment exercised during field sampling.  Individuals conducting these procedures must train with DEP staff (via Stream Condition Index [SCI] SCI training workshops and/or participating in field sampling), complete the training requirements outlined in BRN 1200 and pass initial and continuing proficiency demonstrations per BRN 1300.  All BioRecon sampling and analysis shall be conducted according to the requirements of this BioRecon method and the SCI Primer (Sampling and Use of the Stream Condition Index [SCI] for Assessing Flowing Waters: A Primer [DEP-SAS-001/11]).  The SCI Primer provides comprehensive guidance on use of the BioRecon and other biological measures in the context of specific study objectives.  The use of this BioRecon method must adhere to the assessment principles discussed in the SCI Primer.

## Biological Reconnaissance (BioRecon) Sampling Method

See also the following sections:

###### FT 3100 Aquatic Habitat Characterization

###### FT 3001 Physical/Chemical Characterization

###### FT 1000 General Field Testing and Measurement

##### Introduction

##### This method was designed to be primarily a screening tool to determine the macroinvertebrate community health of flowing freshwater streams and rivers.

##### Equipment and Supplies

###### Completed Physical/Chemical Characterization Field Sheet (FD 9000-3) or other datasheet to capture documentation required in FD 5311

###### Completed Stream/River Habitat Sketch Sheet (FD 9000-4) or other datasheet to capture documentation required in FD 5312

###### Completed Stream/River Habitat Assessment Field Sheet (FD 9000-5) or other datasheet to capture documentation required in FD 5312

###### BioRecon Field Sheet (FD 9000-1) or other datasheet to capture documentation required in SCI 1110

###### Forceps

###### Transfer pipettes

###### White picking pans

###### 10X hand lens

###### Jars filled with alcohol (80% ethanol)

###### D-frame dip net with No. 30 mesh and handle marked in 0.1-m increments

###### Meters (DO, pH, Conductivity)

###### Wide-mouth jug

###### Cooler with ice

##### Methods

##### The SCI Primer must be read and followed prior to carrying out this SOP. Ensure that the site and conditions are appropriate for the study objectives (see SCI Primer).  Visually examine the area or reach to be sampled.  Either walk or boat throughout the aquatic system, paying close attention to its physical and habitat characteristics.  Be very careful when walking through the system not to disturb aquatic habitats.  Such disturbances could lead to inaccurate BioRecon and/or habitat assessment results.  The length of a discrete BioRecon station consists of a 100-m stretch of stream, and the width is from bank to bank.  Do not sample if site conditions (habitat, hydrology, etc.) are not consistent with study objectives, as described in the SCI Primer.

##### You must be familiar with the rainfall, stage height patterns, and stream flows in the area to be sampled.  If you do not know how the water has fluctuated in the stream, do not sample.  If information indicates that the stream has been completely dry (i.e., with no refugia for the aquatic organisms), wait a minimum of six months (180 days) after dry conditions have abated and ensure that the stream has maintained a minimum of 0.05 m/sec velocity for 28 days before performing the BioRecon (unless study objectives dictate otherwise, see SCI Primer).  If the stream has had low flow with insufficient habitat or velocity for sampling, do not perform the BioRecon until sufficient habitat has been wetted and the stream has maintained a minimum of 0.05 m/sec velocity for at least 28 days (unless study objectives dictate otherwise, see SCI Primer).

##### If the water level is >0.5 meters above recent levels, preventing access to substrates that are inhabited by invertebrates, delay sampling until those substrates are accessible or wait 28 days for the invertebrates to colonize the newly inundated substrates.  If you are not confident that the reachable substrates have been inundated for greater than 28 days, do not sample. If water levels have risen, but are < 0.5 meter above the previous level, sample only the ‘deep” habitats where organisms are expected.  If the normal stream channel is not accessible (water in floodplain), postpone sampling until the normal stream channel and habitats may be accessed.

##### Complete the Physical/Chemical Characterization Field Sheet (FD 9000-3), Stream/River Habitat Sketch Sheet (FD 9000-4), and Stream/River Habitat Assessment Field Sheet (FD 9000-5; see FT 3000-FT 3100).  The percent coverage of substrate type refers to how much of each habitat type is actually present and in the water (able to be sampled) at the sampling site.

##### Determine the “best” or “most productive” habitats, which, when sampled, will provide an accurate estimate of the site’s maximum biological diversity.  Generally, the most productive habitat types are as follows:  leaf packs, roots, snags, aquatic vegetation, and rocky outcrops.  All of these are considered “productive” habitats.  Sand and mud/muck are considered “minor” habitats and are not sampled for the BioRecon.  For sampling purposes, select habitats receiving good water velocity (>0.2 m/sec) over those in more sluggish areas.  If there is less than 0.05 m/sec velocity flowing over the substrates, do not sample the site (unless study objectives dictate otherwise, see SCI Primer).  If fewer than four productive habitats are available, select a particularly productive habitat (based on the number of taxa obtained in earlier sweeps) sampled in the earlier sweeps to complete four sweeps. See the SCI Primer for additional description of the methodology for determining productive habitats.

##### Perform four separate 0.5-meter sweeps with the D-frame dip net in the “most productive” habitats as determined by the above procedures.  Sort and remove all organisms between each sweep.  If conditions pose a hazard (e.g., severe weather), the material from all four sweeps may be stored in jugs, placed on ice, and sorted at the laboratory within a 24-hour period.  Three passes over the same 0.5 meter area are required to capture all organisms.  This sampling effort in a discrete 0.5 meter spot is considered to be one sweep.  Catch organisms by using your hands or a brush to dislodge them from the substrate and by creating a flow of water into the net.  Make sure the disturbed material and organism mixture is completely collected by the net.  Where a continuous 0.5 m sweep is not available, take two 0.25 m sweeps or three 0.17 m sweeps of the same habitat type to obtain a full 0.5 m sweep.  Due to the potential spatial heterogeneity in the distribution of the organisms, selecting two separate 0.25 m sweeps (or three 0.17 m sweeps) of a given productive habitat is the preferred technique to comprise the 0.5 m “sweep.”  If you are sampling from a boat, you can get out of the boat and wade in shallow shore areas to obtain the sweeps.  You can also approach a habitat with the boat from downstream, agitate and sweep the reachable portion of the habitat (typically by leaning from the bow of the boat), to capture organisms.  See SCI 1200 section 2.4 and the SCI Primer for further discussion on proper sweeping techniques.

##### There are two picking options for this step.  You must use the field sorting method unless field conditions become hazardous, in which case the laboratory sorting method is allowed.  For both options, refer to Table BRN1100-1 to determine the number of individual specimens in each target taxonomic group to retain for laboratory identification.  Do not retain more than the specified number of individuals.  For both options, perform a quality control (QC) check as outlined in 3.5.3.

##### Field Picking BioRecon Method:  This is the required sorting method for the BioRecon procedure, unless field conditions become hazardous during sampling.  After each habitat sweep, return to a comfortable spot on the bank with the dip net containing the sampled material.  Place small aliquots of the detritus plus organism matrix in a picking pan diluted with a small amount of site water.  Make sure the density of detritus is low enough so organisms are easily seen and captured.  Scan the entire pan for organisms.  When an organism is found, remove it with forceps or a pipette, examine it with the hand lens, determine its identity to the lowest possible taxonomic level in the field (usually family or genus), record on the BioRecon Field Sheet (FD 9000-1).  In a jar filled with alcohol, save up to, but do not exceed, the target number of specimens as indicated in Table BRN 1100-1 for laboratory verification.   For the taxonomic groups listed on Table BRN 1100-1, be sure to include the range of the species encountered for those groups, as they will be identified to lower taxonomic levels in the laboratory (unless otherwise indicated in BRN 2100).  Repeat these procedures until all the material in the net has been examined.  Repeat sorting method until material from four sweeps has been processed.  Organisms from all habitat types, up to the target number of specimens, may be combined in the jar for laboratory verification. Record all data on FD 9000-1.  Examining habitats separately helps the investigator learn which taxa are found in which habitat type and aids in future BioRecon sampling.  Examining habitats separately also helps determine which habitat(s) to target for the fourth sweep if only three productive habitats are available.  If field conditions are tolerable (e.g., no hard rain or lightning that would interfere with sorting efficiency) and time spent in the field is not a limiting factor, this option helps the investigator gain a better understanding of aquatic ecology.  Store samples in alcohol.

##### Laboratory Picking BioRecon Method:  If conditions are hazardous at the site (e.g., severe weather), this option may be used, but it is not allowed for routine use.  To employ this option, sorting must be done within 24 hours on iced samples (do not preserve with alcohol or formalin).  Transfer the material in the net to a wide-mouth jug, performing a cursory examination of the diversity of organisms collected in that given habitat.  Repeat until material from four sweeps (see section 3.4) has been collected and placed into a wide-mouth jug.  Store on ice and sort within 24 hours.  Do not store with alcohol or formalin.  After returning to the laboratory, sort the material with the naked eye, and save up to, but do not exceed, the target number of specimens as indicated in Table BRN 1000-1 for identification.

##### Perform a QC check on all samples sorted in the field or laboratory.

##### Have a second taxonomist randomly select a minimum of 10% of the sorted aliquots and perform a QC check by examining the aliquot and retrieving and counting any new taxa found.  The sorting and QC check can be performed simultaneously by the sampling team.  The aliquot can be discarded after the team reaches a consensus that no new taxa are present.

##### If fewer than 100 individuals in the four D-frame dip net sweeps are collected, this suggests that conditions are inhospitable to invertebrates.  This could be due to recent desiccation, habitat limitation, toxicity, and/or other factors.  If you encounter fewer than 100 individuals, perform a follow-up Stream Condition Index sampling during conditions appropriate for the study objective (see SCI 1100) to further evaluate the stream.

##### See BRN 2100 for laboratory verification of organism identification, BioRecon score determination, and BioRecon score evaluation.

### Required Documentation for Biological Reconnaissance (BioRecon) Sampling

Record the following information or use the BioRecon Field Sheet (Form FD 9000-1).

###### STORET station number

###### Location, including latitude and longitude

###### Watershed or basin name

###### Family or genus of all organisms from all material in all four dipnet sweeps

###### Total taxa tallies

###### Taxa richness, Ephemeroptera taxa, Trichoptera taxa, Long-lived taxa, Clinger taxa, and Sensitive taxa

###### Abundance code for each taxon

###### Name(s) of analysts collecting and sorting samples

###### Habitat types (substrates) sampled

###### Name(s) of analyst(s) performing quality control

###### Signatures

###### Collection date and time

Table BRN 1100-1:

MAXIMUM NUMBER OF SPECIMENS RETAINED FOR BIORECON IDENTIFICATION

## Training for BioRecon Sampling

##### Training for BioRecon Sampling:  Personnel anticipating performing BioRecon sampling according to BRN 1100 shall complete the training specified in SCI 1100 Training Checklist included in Appendix SCI 1000-1.

##### Qualifications for Bioecon Sampling:  Personnel submitting data to DEP from macroinvertebrate sampling according to BRN 1100, Biological Reconnaissance  (BioRecon) Method for the purpose of determining biological indices as calculated per BRN 2100, BioRecon Determination shall successfully complete an audit evaluation administered by DEP according to BRN 1300, Proficiency Criteria for BioRecon Sampling.

## Proficiency Criteria for BioRecon Sampling

##### Scope and Applicability

This auditing protocol is applicable to the BioRecon sampling procedures described in BRN 1100 and the SCI Primer.

##### Personnel must complete the training topics in BRN 1200 and be familiar with FT 3000 prior to requesting an audit.

##### Personnel wishing to submit data to DEP associated with the procedures in BRN 1100, Biological Reconnaissance (BioRecon) Method for the purpose of determining biological indices as calculated per BRN 2100, BioRecon Determination shall be audited by DEP according to the auditing protocol described in section 2 below and produce a satisfactory evaluation and score according to the audit and scoring criteria listed below in sections 3 & 4 prior to collecting samples.

##### After the initial demonstration of proficiency, personnel performing the procedures in BRN 1100, Biological Reconnaissance (BioRecon) Method for the purpose of determining a BioRecon score as calculated per BRN 2100, BioRecon Determination, shall undergo a refresher audit every five years (see section 5, below).

##### First-Time Auditing Protocol for BioRecon Sampling

##### First-Time audit candidates must pass a test of stream and river macroinvertebrate sampling concepts before undergoing the field audit.

##### General Field Auditing Protocols

##### Audits are conducted in an appropriate field setting selected by DEP.

##### Audit candidates are required to provide proper equipment in good working order necessary to conduct sampling.

##### Audit candidates will be asked a series of questions designed to evaluate their conceptual knowledge of appropriate sampling methods.

##### Audit candidates are expected to demonstrate satisfactory skill in performing the procedures detailed in the BioRecon and SCI sampling SOPs.

##### First-Time Auditing Evaluation Criteria for BioRecon Sampling

Personnel must demonstrate a satisfactory working knowledge of and demonstrate the ability to perform the following:

##### Identify the best available habitats in a 100-meter stream reach (snags, leaf packs, roots, aquatic plants, limerock).

##### Identification of best available habitat must include the following:

##### Length of inundation considered

##### Siltation and sedimentation effects considered

##### Condition of leaf packs

##### Flow considerations taken into account

##### Discuss and recognize circumstances where SCI or BioRecon sampling should be postponed, (e.g., in the event of recent increase in water level or prolonged dry period).

##### Know correct number of dip net sweeps for BioRecon (4).

##### Properly apportion dip net sweeps to available habitats.

##### Efficiently capture invertebrates during dip net sweeps while properly agitating substrates with at least 3 passes of the dip net along a 0.5 meter sample sweep length (sweep length sampled is 0.5 meters, plus or minus 0.1 m, without consistently high or low bias).

##### Sample only productive portions of habitats while not diluting sample with unproductive detritus.

##### Properly transfer sampled material to pick pan or sample container without sample loss.

##### BioRecon Sorting

##### Dispense proper density of detritus into pick pan for sorting efficiency.

##### Methodically search for organisms in pick pan.

##### Correctly identify organisms to taxonomic level practical in the field.

##### Efficiently capture organisms using forceps and pipets.

##### Accurately record and achieve the target number of organisms for each taxonomic group, as specified in BRN 1100.

##### Process entire dip net contents.

##### Attain >95% picking efficiency (1 point; between 90% and 95% efficiency, 0.5 point; < 90%, no points, non-attainment).

##### First-Time Audit Evaluation Scoring for BioRecon Sampling

To pass the test in section 2.1 above, the audit candidate must attain a score of 90% correct answers.  For mastery of each component in section 3 above, 1 point is awarded. Only 0.5 point is awarded if the applicable component is evaluated as partially correct. To pass, only 0.5 point can be missed.

##### Refresher Auditing Protocol for BioRecon Sampling

##### Refresher audit candidates must pass a test of stream and river macroinvertebrate sampling concepts with a minimum score of 90% correct answers. The test shall be completed by each individual wishing to conduct BioRecon sampling and submit BioRecon results to the Department, without assistance from others.

##### Field refresher audit consistency demonstration procedures for BioRecon are as described in Section 3 above.

##### Refresher audit evaluation shall be as described in Section 4 above.

# Biological Reconnaissance Laboratory Procedures and Index Determination

See also the following sections:

###### FA 1000 Administrative Procedures

###### FC 1000 Cleaning/Decontamination Procedures

###### FD 1000 Documentation Procedures

###### FM 1000 Field Planning and Mobilization

###### FQ 1000 Field Quality Control Requirements

###### FS 1000 General Sampling Procedures

###### FS 7000 Biological Community Sampling

###### FT 1000 General Field Testing and Measurement

###### FT 3000 Aquatic Habitat Characterization

###### SCI 2200 Quality Control for Biological Community Analysis

## BioRecon Determination

##### Definition:  Biological Reconnaissance (BioRecon) is a rapid community based biological assessment of stream health using benthic macroinvertebrates sampled via four sweeps of a D-frame dipnet, with organisms  identified to the lowest practical taxonomic level in SCI 2230 (with the exception of Oligochaeta, Chironomidae, and Acarina which are identified to higher taxonomic levels).

##### Sampling

##### Perform physical/chemical characterization and habitat mapping according to FT 3001.

##### Perform a Habitat Assessment according to FT 3100.

##### Conduct Biological Reconnaissance (BioRecon) sampling according to BRN 1100.

##### Sort the organisms in the field according to BRN 1100.

##### Laboratory Analyses

##### Laboratory Verification of Field Identifications (Modified from American Public Health Association, Standard Methods for the Examination of Water and Wastewater, 10500 C)

##### Equipment and Supplies

###### Dissecting microscope

###### Identification references

###### Forceps

###### Petri dishes (100 x 15 mm or 60 x 15 mm) or other appropriate containers

###### Laboratory counter (optional)

###### Macroinvertebrate Bench Sheet (may vary from lab to lab)

###### Pen

##### Methods

##### First, empty the contents of the vial into an appropriate container (100-mm diameter petri dish).

##### You may choose to separate the organisms into like groups.  To do this, place the groups into smaller (60-mm diameter) individual petri dishes or group the animals within the larger dish.

##### Use the dissecting microscope to identify the organisms to the lowest practical taxonomic level (as defined in SCI 2230), with the following exception.  For the BioRecon procedure, do not identify Oligochaeta, Chironomidae, and Acarina below class, family, and order, respectively.   Use the most appropriate identification reference for each group.  It will take some time until sufficient experience with identification procedures and references is gained.  In addition to the identification manuals, maintain a reference collection that can be used to compare specimens and facilitate identification.  After using a dichotomous key to arrive at the name of an unknown organism, check the organism’s geographic range, habitat preferences, and morphological diagnosis to confirm that the identification is correct.  Do not identify an organism by simply flipping through some pictures and assigning a name based on a superficial resemblance.

##### Enumerate organisms concurrently with identification and then place them back into the ethanol filled, labeled vial with the forceps.  Remove the animals as you count them in order to avoid counting any of them twice.  If a large number of one or a few taxa are present, use a laboratory counter to keep a running total to facilitate the enumeration process.  You may temporarily label counters to avoid mistakes.  If you do not use a counter, tally the number of each taxon on your bench sheet.

##### If an organism is encountered in your laboratory for the first time, remove it and place it in an individual, labeled  vial for inclusion in the reference collection.  Make a note of this on the bench sheet, so that it can be located in the future, if necessary.

##### Record the individual taxon names and their numbers on the bench sheet.

##### Follow Taxonomic Quality Assurance procedures in SCI 2220.

##### Data Reduction

##### For DEP staff only, enter all data into SBIO (the Florida Statewide Biological Database).

##### Follow the counting and collapsing procedures listed below.  Keep a record of the original taxa list and the resulting collapsed list.

##### When combining biological data (taxa and counts), the number of taxa may become artificially inflated by the incorporation of the same taxon under different names and by counting high-level identifications (family or genus).  An erroneously high taxa count creates additional anomalies among metrics involving these counts, such as the number of Ephemeroptera and Trichoptera taxa.

##### Collapse all taxa to the lowest practical taxonomic level.

##### Collapse taxa further, which follows the “Upward Collapsing” of Taxa (i.e., Preparation for Index Calculations) procedure listed in the Statewide Biological Database Manual, Florida Department of Environmental Protection.

As an example, see the following species list:

Conchapelopia sp. (2)

Helopelopia sp. (4)

Dicrotendipes simpsoni (21)

Dicrotendipes modestus (1)

Dicrotendipes sp. (20)

Hyalella azteca (20)

Planorbella sp. (1)

7 taxa and 69 individuals

Upward collapsing, however, will combine Conchapelopia and Helopelopia as synonyms, and will apportion the individuals from Dicrotendipes sp. to the two identified species of that genus, resulting in:

Conchapelopia sp. (6)

Dicrotendipes simpsoni (40)

Dicrotendipes modestus (2)

Hyalella azteca (20)

Planorbella sp. (1)

5 taxa and 69 individuals

Note that if “Dicrotendipes sp.” had represented a distinct species level entity with no available or known name, it would have properly been entered as “Dicrotendipes sp. 14 Epler” or a similar name, and would then not have been combined.  Collapsing will also take place at other taxonomic levels:  family level identifications will combine with generic level identifications, species will combine with subspecies, etc.

##### Index Calculation

##### Perform the following calculations based on collapsed data as outlined in Section 4.2.

##### Calculate and record the number of long-lived taxa score according to the list below (based on Fore, 2007 with minor modifications).  Use the taxonomic order, family, and genus (or species) name of each taxon to calculate long-lived taxa richness.  Taxonomic synonyms present in the current Florida data base are also shown as additional information.  Long-lived taxa require more than one year to complete their life cycles.

| Order | Family | Genus | Species (Long-lived) | Synonyms |
| --- | --- | --- | --- | --- |
| Decapoda |  |  | All families, genera and species |  |
| Basommatophora | Lymnaeidae | Fossaria | All species |  |
| Mesogastropoda | Ampulariidae | Pomacea | All species |  |
| Veneroida | Corbiculiidae | Corbicula | All species |  |
| Unionoida | Unionidae |  | All genera and species |  |
| Odonata | Aeshnidae | Basiaeschna | Basiaeschna janata |  |
|  |  | Boyeria | All species |  |
|  | Cordulegastridae |  | All species |  |
|  | Gomphidae | Gomphus | All species | Gomphurus |
|  |  | Haegenius | All species |  |
|  |  | Progomphus | All species |  |
| Odonata | Libellulidae | Macromia | All species |  |
|  |  | Somatochlora | All species |  |
|  |  | Tetragoneuria | All species | Epitheca |
|  | Petaluridae | Tachopteryx | Tachopteryx thoreyi |  |
| Plecoptera | Pteronarcidae | Pteronarcys | All species |  |
|  | Leuctridae |  | All genera and species |  |
|  | Peltoperlidae | Tallaperla | Tallaperla cornelia |  |
|  | Perlidae | Acroneuria | All species |  |
|  |  | Agnetina | Agnetina annulipes |  |
|  |  | Eccoptura | Eccoptura xanthenes | Acroneuria xanthenes |
|  |  | Neoperla | All species |  |
|  |  | Paragnetina | All species | Banksiana |
|  |  |  |  | Banksiella |
|  | Perlodidae | Clioperla | Clioperla clio | Isoperla clio |
|  |  | Perlinella | Perlinella drymo | Atoperla ephyre |
|  |  |  | Perlinella ephyre/zwicki |  |
|  |  | Isoperla | Isoperla orata |  |
| Megaloptera | Corydalidae |  | All genera and species |  |
| Trichoptera | Brachycentridae | Brachycentrus | Brachycentrus americanus | Oligoplectrum amercanum |
|  |  | Micrasema | All species |  |
|  | Calamoceratidae | Heteroplectron | Heteroplectron americanum |  |
|  | Hydropsychidae | Diplectrona | Diplectrona modesta |  |
|  |  | Macrostemum | Macrostemum carolina | Macronema carolina |
|  |  |  |  | Macronemum carolina |
|  | Lepidostomidae | Lepidostoma | All species | Alepomyia |
|  |  |  |  | Alepomyiodes |
|  |  |  |  | Arcadopsyche |
|  |  |  |  | Atomyia |
|  |  |  |  | Jenortha |
|  |  |  |  | Mormomyia |
|  |  |  |  | Neuropsyche |
|  |  |  |  | Nosopus |
|  |  |  |  | Notiopsyche |
|  |  |  |  | Olemira |
|  |  |  |  | Oligopsyche |
|  |  |  |  | Phanopsyche |
|  |  |  |  | Pristosilo |
|  | Leptoceridae | Ceraclea | All species | Athripsodes |
|  | Limnephilidae | Ironoquia | Ironoquia punctatissisma |  |
|  |  | Pycnopsyche | All species |  |
|  | Molannidae | Molanna | All species |  |
| Trichoptera | Phryganeidae | Banksiola | Banksiola concatenata | Neuronia concatenata |
|  |  | Ptilostomis | Ptilostomis postica | Neuronia postica |
|  |  | Agrypnia | Agrypnia vestita |  |
|  | Psychomyiidae | Lype | Lype diversa |  |
|  | Rhyacophilidae | Rhyacophila | All species |  |
|  | Uenoidae | Neophylax | All species |  |

##### Calculate and record the number of sensitive taxa score according to the list below (based on Fore, 2007 with minor modifications).  These taxa are identified as sensitive to human disturbance.  Synonyms in the current Florida database are also included as additional information.

| Order | Family | Genus | Taxon (sensitive) | Synonyms |
| --- | --- | --- | --- | --- |
| Acariformes | Lebertiidae | Lebertia | All species |  |
| Amphipoda | Crangonyctidae | Crangonyx | Crangonyx |  |
| Coleoptera | Elmidae | Ancyronyx | Ancyronyx variegatus |  |
|  |  | Gonielmis | All species |  |
|  |  | Macronychus | All species |  |
| Diptera | Chironomidae | Microtendipes | All species |  |
|  |  | Parametriocnemus | All species |  |
|  |  | Polypedilum | Polypedilum aviceps |  |
|  |  | Rheocricotopus | All species |  |
|  |  | Stempellinella | All species |  |
|  |  | Tanytarsus | Tanytarsus sp. d epler |  |
|  |  | Tanytarsus | Tanytarsus sp. m epler |  |
|  |  | Tribelos | Tribelos jucundum |  |
|  | Empididae | Hemerodromia | Hemerodromia |  |
|  | Simuliidae |  | All genera and species |  |
| Ephemeroptera | Baetidae | Acerpenna | Acerpenna pygmaea | Baetis pygmaeus |
|  | Ephemerellidae |  | All genera and species |  |
|  | Heptageniidae |  | All genera and species |  |
|  | Leptophlebiidae |  | All genera and species |  |
|  | Leptohyphidae |  | All genera and species |  |
| Isopoda | Asellidae | Caecidotea | Caecidotea | Asellus |
| Odonata | Libellulidae | Macromia | All species |  |
| Plecoptera |  |  | All families, genera and species |  |
| Trichoptera | Hydropsychidae | Hydropsyche | All species |  |
|  | Leptoceridae | Triaenodes | All species |  |
|  | Philopotamidae | Chimarra | Chimarra |  |
|  | Psychomyiidae | Lype | Lype diversa |  |

##### Tally the number of total taxa (taxa richness), Ephemeroptera taxa, and Trichoptera taxa.

##### Refer to Merritt and Cummins, An Introduction to the Aquatic Insects of North America to determine the number of clinger taxa.  For clinger taxa, include only those taxa whose sole habit is listed as “clinger.”

##### Determine region according to Figure BRN 2100-1.  Regional metric expectations are listed in Table BRN 2100-1.  Evaluate and record the biological metrics according to the scoring system in Tables BRN 2100-2 and 2100-3.

##### References

##### Merritt, R.W., and Cummins, K.W., An Introduction to the Aquatic Insects of North America, Third Edition, 1996.

##### Fore, L., R.B. Frydenborg, D. Miller, T. Frick, D. Whiting, J. Espy, and L. Wolfe.   Development and Testing of Biomonitoring Tools for Macroinvertebrates in Florida Streams.  Florida Department of Environmental Protection, 2007.

##### American Public Health Association, American Water Works Association, Water Pollution Control Federation, SM10500C, Standard Methods for the Examination of Water and Wastewater, 20th Edition, 1999.

### Required Documentation for Laboratory Procedures For BioRecon Determination

###### Sample receipt or log-in record

###### Site, sample identification number, STORET station number, sample type, replicate number and date collected

###### Total number of taxa

###### Name of any organism encountered in the laboratory for the first time

###### Individual taxon names

###### Numbers of each taxon counted

###### Total number of individuals counted

###### Data entry into the Florida Statewide Biological Database (DEP staff only)

###### Method and notes for counting and collapsing taxonomic data

###### Collapsed taxa list

###### Index calculations per BRN 2100, section 5:

###### Long-lived taxa score

  - Sensitive taxa score

  - Taxa richness (total taxa)

  - Number of Ephemeroptera taxa

  - Number of Trichoptera taxa

  - Number of clinger taxa

###### BioRecon index metrics and evaluation as scored per Tables BRN 2100-2 and BRN 2100-3

Appendix  BRN 2100

Tables, Figures and Forms

Table  BRN 2100-1

Table  BRN 2100-2

Table  BRN 2100-3

Figure BRN 2100-1

Table  BRN 2100-1

BioRecon metric name and range of metric values used to assign a score of 0 to 1 by region. Metric values higher or lower than the listed range were assigned a score of 0 or 1 as appropriate.  Bold values indicate scoring ranges that differ from the same SCI metric.

| BioRecon metric | Northeast | Panhandle | Peninsula |
| --- | --- | --- | --- |
| Total number of taxa | 14–37 | 16–49 | 11–36 |
| Number of Ephemeroptera taxa | 0–3.5 | 0–12 | 0–5 |
| Number of Trichoptera taxa | 0–6.5 | 0–7 | 0–7 |
| Number of Long-lived taxa | 0–6 | 0–10 | 0–7 |
| Number of Clinger taxa | 0–7 | 0–15.5 | 0–8 |
| Number of Sensitive taxa | 0–11 | 0–19 | 0–9 |

Table BRN 2100-2

BioRecon metric scoring formula for converting metric values to a metric score ranging from 0 to 1. In each equation, ‘X’ stands for the raw metric value. For calculated values >1, score equals 1; for calculated values <0, score equals 0.

| BioRecon metric | Northeast | Panhandle | Peninsula |
| --- | --- | --- | --- |
| Total number of taxa | (X–14)/23 | (X–16)/33 | (X–11)/25 |
| Number of Ephemeroptera taxa | X /3.5 | X /12 | X /5 |
| Number of Trichoptera taxa | X /6.5 | X /7 | X /7 |
| Number of Long–lived taxa | X /6 | X /10 | X /7 |
| Number of Clinger taxa | X /7 | X /15.5 | X /8 |
| Number of Sensitive taxa | X /11 | X /19 | X /9 |

1.	Check that none of the metric scores are >1 or <0. If so, change the scores to 0 or 1 as appropriate.

2.	Sum the metric scores; check that the sum is between 0 and 6.

3.	Divide the sum of scores by 0.6. This is a correction factor to change the BioRecon range from 0–6 to 0–10.

Table BRN 2100-3

Categorical descriptions and range of index values for the BioRecon Index.  Square brackets indicate a value is included in the range; round brackets indicate a value is not included.

| BioRecon | Index range |
| --- | --- |
| 1 sample |  |
| Category 1 | [7–10] |
| Category 2 | [4–7) |
| Category 3 | [0–4) |

Figure BRN 2100-1.

Map of BioRecon and Stream Condition Index Regions (black lines) and Florida counties (gray lines). Note that the BioRecon has not been calibrated for South Florida Region.